3GPP TSG-RAN WG5 Meeting #91-e                                                                                                                                  R5-212857

# Electronic Meeting, 17th – 28th May 2021

Agenda Item:	5.3.2.17

Source: 	CAICT

Title: 	Discussion on removal of Transmit Off Power for FR2 CA test cases from 38.521-2 and 38.521-3

Document for:	Discussion and Endorsement

Introduction

Transmit Off Power test for FR1 is included in Tx ON/OFF time mask test and no separate Transmit Off Power test will be perfprmed in FR1.

Transmit Off Power test for FR2 is not included in Tx ON/OFF time mask test and separate Transmit Off Power test will be perfprmed in FR2.

Transmit Off Power for FR2 are defined for both single carrier and CA.The requirements from 38.521-2 are as below:

- 6.3.2.3	Minimum conformance requirements

The transmit OFF power is defined as the TRP in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The transmit OFF power shall not exceed the values specified in Table 6.3.2.3-1 for each operating band supported. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 6.3.2.3-1: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257, n258, n259,n261 | -35 | -35 | -35 | -35 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.3.2.

- 6.3A.2.0	Minimum conformance requirements

For intra-band contiguous carrier aggregation, the transmit OFF power is defined as the TRP in the channel bandwidth per component carrier when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the transmitter is not considered OFF.

The transmit OFF power shall not exceed the values specified in Table 6.3A.2.0-1 for each operating band supported.

Table 6.3A.2.0-1: Transmit OFF power for CA

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257, n258, n260, n261 | -35 | -35 | -35 | -35 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

Transmit Off Power requirements for single carrier and CA are same. The test metric for Transmit Off Power for FR2 is TRP which is time consuming.

The off power is same when transmitter is off, no single carrier or CA is transimiting when performing Transmit Off Power test.

There is no need to duplicate the Transmit OFF power test for both single carrier and different kinds of CA.

Conclusions

Proposal 1: Remove the Transmit Off Power for CA test cases from the WP and TS 38.521-2 spec.

Proposal 2: Remove the Transmit Off Power for EN-DC including FR2 (>2 CCs) test cases from the WP and TS 38.521-3 spec.