3GPP TSG-RAN5 Meeting #91-e	R5-213316

# Electronic Meeting, 17th May – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5319 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Reference Sensitivity for CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test configuration in Tables 7.3A.3.5-2, 7.3A.5.4.1-1, 7.3A.5.4.1-2, 7.3A.9.4.1-1, 7.3A.9.4.1-2, 7.3A.10.4.1-2 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test configuration in Tables 7.3A.3.5-2, 7.3A.5.4.1-2, 7.3A.9.4.1-1, 7.3A.9.4.1-2, 7.3A.10.4.1-2 has been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Affected CA configurations cannot be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.3.5, 7.3A.5.4.1, 7.3A.9.4.1, 7.3A.10.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.3A.3.5	Test requirement

For inter-band carrier aggregation with uplink assigned to one E-UTRA band the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Tables 7.3A.3.5-1 and Table 7.3A.3.5-2 (originated from single carrier Tables 7.3.5-1 and 7.3.5-2). The reference sensitivity is defined to be met with both downlink component carriers active and either of the uplink component carriers active.

< Unchanged table omitted >

For the UE which supports inter-band carrier aggregation configurations the ΔRIB,c in Tables 7.3.3-1A, 7.3.3-1B and 7.3.3-1C shall be applied for applicable bands.

In case the UE supports more than one of the inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 7.3.3-1A, truncated to one decimal place that would apply for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL (i.e. bands listed in Table 7.3A.0-0a), then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 7.3.3-1A that would apply for that operating band among the supported CA configurations

The reference receive sensitivity (REFSENS) requirement for inter-band specified in Table 7.3A.3.5-1 shall be met for an uplink CA configurations than or equal to that specified in Table 7.3A.3.5-2.

NOTE:	Table 7.3A.3.5-2 does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors.

Table 7.3A.3.5-2: Inter-band CA uplink configuration for reference sensitivity, LCRB @ RBstart format

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |
| CA_19A-46A | 19 | - | - | 25@0 | 25@25 | 25@50 | - | FDD |
|  | 46 | - | - | - | - | - | - | TDD |
| CA_20A-28A | 20 |  |  |  | 20@0 | 20@11 | 20@16 |  |
|  | 28 |  |  | 25@0 | 25@25 | 25@50 | 25@75 |  |
| CA_20A-31A | 20 | - | - | 25@0 | 20@0 | 20@11 | 20@16 | FDD |
|  | 31 | - | 5@9 | 5@10 | - | - | - |  |
| CA_20A-32A | 20 | - | - | 25@0 | 20@0 |  |  | FDD |
|  | 32 | - | - | N/A | N/A | N/A | N/A |  |
| CA_20A-40A | 20 | - | - | 25@0 | 20@0 | 20@11 | 20@16 | FDD |
|  | 40 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| CA_20A-42A | 20 | - | - | 25@0 | 20@0 | 20@11 | 20@16 | FDD |
|  | 2010,11 |  |  |  |  |  | 8@46 16@42 25@37 | FDD |
|  | 42 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| CA_21A-46A | 21 | - | - | 25@0 | 25@25 | 25@50 | - | FDD |
|  | 46 | - | - | - | - | - | - | TDD |
| CA_20A-67A | 20 | - | - | 25@0 | 20@0 | 20@11 | 20@16 | FDD |
|  | 67 | - | - | N/A | N/A | N/A | N/A |  |
| CA_21A-28A | 21 | - | - | 25@0 | 25@25 | 25@50 | - | FDD |
|  | 28 | - | - | 25@0 | 25@25 | - | - |  |
| CA_21A-42A | 21 | - | - | 25@0 | 25@25 | 25@50 | - | FDD |
|  | 42 | - | - | 25@0 | 50@0 | 75@0 | 100@0 | TDD |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |
| Note 1: Void Note 2: Configuration for the low band for which the 3rd harmonic is within transmission bandwidth of the high band, as specified in Table 7.3A.0-0a Note 5. Note 3: Void Note 4: Void Note 5: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. Note 6: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. Note 7: Void Note 8: Void Note 9: Configuration for the low band for which the 2nd harmonic is within transmission bandwidth of the high band, as specified in Table 7.3A.0-0a Note 9 or Note 11. Note 10: This configuration is used for testing the 3rd harmonic or 4th harmonic interfered high band only. It is for low band with multiple choices of RB number corresponding to different high band bandwidth as indicated in Table 7.3A.0-0aa: Uplink configuration for the low band (exceptions). Only one bandwidth option was used for simplicity. Note 11: Configuration for the low band for which the 4th harmonic is within transmission bandwidth of the high band, as specified in Table 7.3A.0-0a Note 12 and 13. |  |  |  |  |  |  |  |  |

### < Unchanged sections omitted >

### 7.3A.5	Reference sensitivity level for 3DL CA

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	TR 36.905 test point analysis for CA_28A-40C is missing, meaning it is not yet determined if testing of fallback configuration CA_28A_40A can be skipped.

-	Band 70 CA configurations are missing test point analysis.

#### 7.3A.5.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.5.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 3DL with CA configurations in Table 7.1-2a.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

#### 7.3A.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.5.4	Test description

##### 7.3A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.5.4.1-1 through table 7.3A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB ” columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  |  |  | SCC1 |  |  |  |  |  |  | SCC2 |  |  |  |  |  |  |
|  | Band |  | Range |  | NRB |  | Band | Range |  | NRB |  |  |  | Band |  | Range |  |  | NRB |  |
|  | UL MOD |  | DL MOD |  | ULalloc (Note 2,3,4) | DLalloc | UL MOD | DL MOD |  | ULalloc |  | DLalloc |  | UL MOD |  | DL MOD |  |  | ULalloc | DLalloc |
| Default Test Settings for a CA_XD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  | Low/CC1 |  | REFSENS | All RBs | X | Low/CC2 |  | N/A |  | All RBs |  | X |  | Low/CC3 |  |  | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |
| 2 | X |  | Low/CC1 |  | REFSENS | All RBs | X | Low/CC2 |  | N/A |  | All RBs |  | X |  | Low/CC3 |  |  | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |
| 3 | X |  | High/CC1 |  | REFSENS | All RBs | X | High/CC2 |  | N/A |  | All RBs |  | X |  | High/CC3 |  |  | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |
| 4 | X |  | High/CC1 |  | REFSENS | All RBs | X | High/CC2 |  | N/A |  | All RBs |  | X |  | High/CC3 |  |  | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_41C-42A with UL CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 41 |  | Low/CC1 |  | 100@0 | All RBs | 41 | Low/CC2 |  | 50@0 | All RBs | 42 | Mid |  |  | N/A |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 50 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 2 |  | 41 |  | High/CC1 |  | 100@0 | All RBs | 41 | High/CC2 |  | 50@0 | All RBs | 42 | Mid |  |  | N/A |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 50 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 3 |  | 41 |  | Low/CC1 |  | 100@0 | All RBs | 41 | Low/CC2 |  | 100@0 | All RBs | 42 | Mid |  |  | N/A |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 100 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 4 |  | 41 |  | High/CC1 |  | 100@0 | All RBs | 41 | High/CC2 |  | 100@0 | All RBs | 42 | Mid |  |  | N/A |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 100 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 5 |  | 41 |  | Mid/CC1 |  | 50@0 | All RBs | 42 | Mid |  | 50@0 | All RBs | 41 | Mid/CC2 |  |  | N/A |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 50 | QPSK | QPSK |  |  | 50 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 6 |  | 41 |  | Mid/CC1 |  | 100@0 | All RBs | 42 | Mid |  | 100@0 | All RBs | 41 | Mid/CC2 |  |  | N/A |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 100 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| Test Settings for CA_41C-42A with 3UL CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 41 |  | Low/CC1 |  | 100@0 | All RBs | 41 | Low/CC2 |  | 50@0 | All RBs | 42 | Mid |  |  | 100@0 |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 50 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 2 |  | 41 |  | High/CC1 |  | 100@0 | All RBs | 41 | High/CC2 |  | 50@0 | All RBs | 42 | Mid |  |  | 100@0 |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 50 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 3 |  | 41 |  | Low/CC1 |  | 100@0 | All RBs | 41 | Low/CC2 |  | 100@0 | All RBs | 42 | Mid |  |  | 100@0 |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 100 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 4 |  | 41 |  | High/CC1 |  | 100@0 | All RBs | 41 | High/CC2 |  | 100@0 | All RBs | 42 | Mid |  |  | 100@0 |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 100 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 5 |  | 41 |  | Mid/CC1 |  | 50@0 | All RBs | 42 | Mid |  | 50@0 | All RBs | 41 | Mid/CC2 |  |  | 100@0 |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 50 | QPSK | QPSK |  |  | 50 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| 6 |  | 41 |  | Mid/CC1 |  | 100@0 | All RBs | 42 | Mid |  | 100@0 | All RBs | 41 | Mid/CC2 |  |  | 100@0 |  |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | QPSK | QPSK |  |  | 100 | QPSK | QPSK |  |  |  |  |  | 100 |  |
| Test Settings for CA_46C-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 66 |  | Low |  | 100@0 | All RBs | 46 | Mid/CC1 |  | N/A |  | All RBs | 46 | Mid/CC2 |  |  | N/A |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | N/A | QPSK |  |  |  | 100 | N/A | QPSK |  |  |  |  | 100 |  |
| 2 |  | 66 |  | High |  | 100@0 | All RBs | 46 | Mid/CC1 |  | N/A |  | All RBs | 46 | Mid/CC2 |  |  | N/A |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | N/A | QPSK |  |  |  | 100 | N/A | QPSK |  |  |  |  | 100 |  |
| Test Settings for CA_46A-66C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 66 |  | Low/CC1 |  | 100@0 | All RBs | 66 | Mid/CC2 |  | N/A |  | All RBs | 46 | Mid |  |  | N/A |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | N/A | QPSK |  |  |  | 100 | N/A | QPSK |  |  |  |  | 100 |  |
| 2 |  | 66 |  | High/CC1 |  | 100@0 | All RBs | 66 | High/CC2 |  | N/A |  | All RBs | 46 | Mid |  |  | N/A |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 100 | N/A | QPSK |  |  |  | 100 | N/A | QPSK |  |  |  |  | 100 |  |
| Test Settings for CA_70C-71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 70 |  | Mid/CC1 |  | 75@0 | All RBs | 70 | Mid/CC2 |  | N/A |  | All RBs | 71 | Mid |  |  | N/A |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 75 | N/A | QPSK |  |  |  | 50 | N/A | QPSK |  |  |  |  | 50 |  |
| 2 |  | 71 |  | Low |  | 16@12 | All RBs | 70 | Mid/CC1 |  | N/A |  | All RBs | 70 | Mid/CC2 |  |  | N/A |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 50 | N/A | QPSK |  |  |  | 75 | N/A | QPSK |  |  |  |  | 50 |  |
| 3 |  | 71 |  | High |  | 50@0 | All RBs | 70 | Mid/CC1 |  | N/A |  | All RBs | 70 | Mid/CC2 |  |  | N/A |  | All RBs |  |
|  |  | QPSK |  | QPSK |  |  | 50 | N/A | QPSK |  |  |  | 75 | N/A | QPSK |  |  |  |  | 50 |  |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band contiguous & Intra-band contiguous + Inter-band: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band: Use CA Configuration – specific test points if present in the table, Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 4: Inter-band & Intra-band contiguous + Inter-band: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41D, X=41 Note 6: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_1A-3A-19A,X=1,Y=3,Z=19 Note 7: Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3, for CA_1A_42C, X=1,Y=42. Note 8: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC Note 10: Band 12: fUL = 706.7 MHz (NUL = 23087), fDL = 736.7 MHz (NDL = 5087) Band 4: fDL = 2120.1 MHz (NDL = 2051) Note 11: Band 12: fUL = 710.9 MHz (NUL = 23129), fDL = 740.9 MHz (NDL = 5129) Band 4: fDL = 2132.7 MHz (NDL = 2177) Note 12: N/A Note 13: Test points that fulfil criteria of Note 4 in Table 7.3A.5.5-3 Note 14: Only Band 1 and Band 42 need to be tested and Band 3 does not need to be tested Note 15: Only Band 1 and Band 3 need to be tested and Band 42 does not need to be tested Note 16: Band 3: fUL = 1720MHz (NUL = 19300), fDL = 1815MHz (NDL = 1300) Band 42: fUL/DL = 3440MHz (NUL/DL = 41990) Note 17: Band 3: fUL = 1775MHz (NUL = 19850), fDL = 1870MHz (NDL = 1850) Band 42: fUL/DL = 3520MHz (NDL = 42790) Note 18: N/A Note 19: Only Band 1 and Band 19 need to be tested and Band 28 does not need to be tested Note 20: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 21: Band 42: fUL/DL for SCC1 = 3430.2MHz (NUL/DL = 41892), fUL/DL for SCC2 = 3450MHz (NUL/DL = 42090) Note 22: Band 42: fUL/DL for SCC1 = 3500.2MHz (NUL/DL = 42592), fUL/DL for SCC2 = 3520MHz (NUL/DL = 42790). Note 23: Band 28: fUL = 719.3MHz (NUL = 27373), fDL = 774.3MHz (NDL = 9373). Note 24: Band 42: fUL/DL for SCC1 = 3515.8MHz (NUL = 42748), fUL/DL for SCC2 = 3527.5MHz (NDL = 42865). Note 25: Band 8: fUL = 897.5MHz (NUL = 21625), fDL = 942.5MHz (NDL = 3625), Band 42: fUL/DL = 3590MHz (NUL/DL = 43490). Note 26: Band 28: fUL = 722.5 MHz (NUL = 27405), fDL = 777.5 MHz (NDL = 9405), Band 1: fDL = 2167.5 MHz (NDL = 575). Note 27: Band 28: fUL = 743 MHz (NUL = 27610), fDL = 798 MHz (NDL = 9610). Band 11: fDL = 1480.9 MHz (NDL = 4800). Note 28: Band 12: fUL = 704MHz (NUL = 23060), fDL = 734MHz (NDL = 5060) Band 66: fUL = 1712.5MHz (NUL = 131997), fDL = 2112.5MHz (NDL = 66461) Note 29: Band 12: fUL = 707.5MHz (NUL = 23095), fDL = 737.5MHz (NDL = 5095) Band 66: fUL = 1720MHz (NUL = 132072), fDL = 2120MHz (NDL = 66536) Note 30: Band 12: fUL = 711MHz (NUL = 23130), fDL = 741MHz (NDL = 5130) Band 66: fUL = 1717.5MHz (NUL = 132047), fDL = 2117.5MHz (NDL = 66511). Note 31: Band 3: fUL = 1757.4MHz (NUL = 19674), fDL = 1852.4MHz (NDL = 1674), Band 11: fDL = 1480.9MHz (NDL = 4800). Note 32: Band 3: fDL = 1852.4MHz (NDL = 1674), Band 11: fUL = 1432.9MHz (NUL = 22800), fDL = 1480.9MHz (NDL = 4800). Note 33: Band 2: fUL = 1868.3MHz (NUL = 18783), fDL = 1948.3MHz (NDL = 783). Band 4: fUL = 1735MHz (NUL = 20200), fDL = 2135MHz (NDL = 2200). Note 34: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 35: Band 3: fUL = 1737MHz (NUL = 19470), fDL = 1832MHz (NDL = 1470). Band 7: fUL = 2543MHz (NUL = 23430), fDL = 2663MHz (NDL = 3180). Band 20: fUL = 847MHz (NUL = 24300), fDL = 806MHz (NDL = 6300). Note 36: Band 3: fUL = 1775MHz (NUL = 19850), fDL = 1870MHz (NDL = 1850). Band 7: fUL = 2510MHz (NUL = 23100), fDL = 2630MHz (NDL = 2850). Band 20: fUL = 855MHz (NUL = 24380), fDL = 814MHz (NDL = 6380). Note 37: Band 7: fUL = 2512MHz (NUL = 23120), fDL = 2632MHz (NDL = 2512). Band 20: fUL = 851MHz (NUL = 24340), fDL = 851MHz (NDL = 6340). Note 38: Test frequency for each CA configuration shall follow Table 7.3A.0-0f. If test configurations of each ID in a CA configuration are same, test frequency shall follow the order of Table 7.3A.0-0f. Note 39: Test frequency for each CA configuration shall follow Table 7.3A.0-0g. If test configurations of each ID in a CA configuration are same, test frequency shall follow the order of Table 7.3A.0-0g. Note 40: Band 71:fUL = 679.3 MHz (NUL = 133285), fDL = 633.3 MHz (NDL = 68749) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.5.4.1-2: Test Configuration Table Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB" and “SCCs NRB ” columns |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  | PCC |  |  |  |  | Wgap [MHz] | SCC1 |  |  |  | Wgap [MHz] | SCC2 |  |  |
|  |  | Band | Range |  | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |
|  |  | UL MOD | DL MOD |  | UL alloc (Note 2,3,4) | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_12A-66A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | CC1 | 100@0 |  | All RBs | 65 | 66 | CC2 | N/A | All RBs | - | 12 | Mid | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | 100 |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 50 |
| 2 | 66 |  | CC1 | 100@0 |  | All RBs | 50 | 66 | CC2 | N/A | All RBs | - | 12 | Mid | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| 3 | 12 |  | Low | 20@5 |  | All RBs | - | 66 | CC2 | N/A | All RBs | 65 | 66 | CC1 | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | 25 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 25 |
| 4 | 12 |  | Mid | 20@5 |  | All RBs | - | 66 | CC2 | N/A | All RBs | 60 | 66 | CC1 | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | 25 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| 5 | 12 |  | High | 20@30 |  | All RBs | - | 66 | CC2 | N/A | All RBs | 75 | 66 | CC1 | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 75 |
| 6 | 12 |  | Mid | 20@30 |  | All RBs | - | 66 | CC2 | N/A | All RBs | 50 | 66 | CC1 | N/A | All RBs |
|  | QPSK |  | QPSK |  |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for CA_29A-66A-66A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 66 | CC1 |  | 100@0 | All RBs | 50 | 66 | CC2 | N/A | All RBs | - | 29 | Mid | N/A | All RBs |
|  |  | QPSK | QPSK |  |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| 2 |  | 66 | CC1 |  | 100@0 | All RBs | 65 | 66 | CC2 | N/A | All RBs | - | 29 | Mid | N/A | All RBs |
|  |  | QPSK | QPSK |  |  | 100 |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 50 |
| Group Test Settings for CA_66A-66A-XA or CA_XA-66A-66A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 66 | CC1 |  | 100@0 | All RBs | 65 | 66 | CC2 | N/A | All RBs | - | X | Mid | N/A | All RBs |
|  |  | QPSK | QPSK |  |  | 100 |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | Highest NRB |
| 2 |  | 66 | CC1 |  | 100@0 | All RBs | 50 | 66 | CC2 | N/A | All RBs | - | X | Mid | N/A | All RBs |
|  |  | QPSK | QPSK |  |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | Highest NRB |
| 3 |  | X | Mid |  | REFSENS | All RBs | - | 66 | CC1 | N/A | All RBs | 50 | 66 | CC2 | N/A | All RBs |
|  |  | QPSK | QPSK |  |  | Highest NRB |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-5A,X=2,Y=5 or Y=5, X =2 Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A,X=41. Note 7: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band non-contiguous + Inter-band: Configuration for the low band for which the 3rd harmonic is within transmission bandwidth of the high band, as specified in Table 7.3A.0-0a Note 5. Note 9: Intra-band non-contiguous + Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: Not applicable if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 11: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 12: Void Note 13: Void Note 14: Void Note 15: Void Note 16: Void Note 17 Void Note 18: Applicable test configuratons are specified in Table 7.3A.5.4.1-2a. Note 19: Applicable test configurations are specified in Table 7.3A.5.4.1-2b. Note 20: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 21: Band 71:fUL = 679.3 MHz (NUL = 133285), fDL = 633.3 MHz (NDL = 68749) Note 22: Applicable test configurations are specified in Table 7.3A.5.4.1-2c. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.3A.9	Reference sensitivity level for 4DL CA

Editor’s note: Following aspects missing or not yet determined:

-	Reference sensitivity QPSK PREFSENS has defined and made for some CA bands, some of are incomplete.

#### 7.3A.9.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.9.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 4DL with CA configurations in Table 7.1-2b.

#### 7.3A.9.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.9.4	Test description

##### 7.3A.9.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.9.4.1-1 through table 7.3A.9.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.9.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  | PCC |  |  |  |  |  |  |  | SCC1 |  |  |  |  |  |  |  | SCC2 |  |  |  |  |  |  |  | SCC3 |  |  |  |  |  |  |
|  |  | Band |  | Range |  | NRB |  |  |  | Band |  | Range |  | NRB |  |  |  | Band |  | Range |  | NRB |  |  |  | Band |  | Range |  | NRB |  |  |
|  |  | UL MOD |  | DL MOD |  | ULalloc (Note 2,3,4) |  | DLalloc |  | UL MOD |  | DL MOD |  | ULalloc |  | DLalloc |  | UL MOD |  | DL MOD |  | ULalloc |  | DLalloc |  | UL MOD |  | DL MOD |  | ULalloc |  | DLalloc |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings CA_3A-18A-42C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 |  | Low |  | 50@25 |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 50 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 |  | 3 |  | High |  | 50@25 |  | ALL RBs |  | 18 |  | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 |  | 3 |  | Low |  | 50@25 |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 25 |  |
| 4 |  | 3 |  | High |  | 50@25 |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 5 |  | 3 |  | Low |  | 50@50 |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 50 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 6 |  | 3 |  | Low |  | 50@50 |  | ALL RBs |  | 18 |  | Mid |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 7 |  | 18 |  | Low |  | 25@25 |  | ALL RBs |  | 3 |  | Low |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 50 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 8 |  | 18 |  | Mid |  | 25@25 |  | ALL RBs |  | 3 |  | Low |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 9 |  | 18 |  | Mid |  | 25@50 |  | ALL RBs |  | 3 |  | Low |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 10 |  | 42 |  | Low |  | 100@0 |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  | 3 |  | Mid |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 50 |  |
| 11 |  | 42 |  | High |  | 100@0 |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  | 3 |  | Mid |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 25 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 50 |  |
| 12 |  | 42 |  | Low |  | 100@0 |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  | 3 |  | Low |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 25 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 50 |  |
| 13 |  | 42 |  | High |  | 100@0 |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  | 3 |  | Mid |  | N/A |  | ALL RBs |  | 18 |  | Mid |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 14 |  | 42 |  | Low |  | 100@0 |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  | 3 |  | Mid |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band contiguous & Intra-band contiguous + Inter-band: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band: Use CA Configuration – specific test points if present in the table, Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41E, X=41. Note 6: Inter-band: X,Y,Z,R correspond to the different bands in the CA Configuration. E.g. for CA_1A-3A-7A-8A,X=1,Y=3,Z=7,R=8. Note 7: Intra-band contiguous + Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_3C-7A-28A, X=3,Y=7 and Z=28. Note 8: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 10: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 11: Test points that fulfil criteria of Note 4 in Table 7.3A.5.5-3. Note 12: Only Band 1 and Band 42 need to be tested and Band 3 does not need to be tested. Note 13: Band 3: fUL = 1720MHz (NUL = 19300), fDL = 1815MHz (NDL = 1300) Band 42: fUL/DL = 3440MHz (NUL/DL = 41990) Note 14: Only Band 1 and Band 3 need to be tested and Band 42 does not need to be tested. Note 15: Band 3: fUL = 1775MHz (NUL = 19850), fDL = 1870MHz (NDL = 1850) Band 42: fUL/DL = 3520MHz (NDL = 42790) Note 16: Band 42: fUL/DL for SCC1 = 3430.2MHz (NUL/DL = 41892), fUL/DL for SCC2 = 3450MHz (NUL/DL = 42090) Note 17: Band 42: fUL/DL for SCC1 = 3500.2MHz (NUL/DL = 42592), fUL/DL for SCC2 = 3520MHz (NUL/DL = 42790) Note 18: Band 12: fUL = 706.7 MHz (NUL = 23087), fDL = 736.7 MHz (NDL = 5087) Band 4: fDL = 2120.1 MHz (NDL = 2051) Note 19: Band 12: fUL = 710.9 MHz (NUL = 23129), fDL = 740.9 MHz (NDL = 5129) Band 4: fDL = 2132.7 MHz (NDL = 2177) Note 20: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 21: Test point that fulfils criteria of Note 16 in Table 7.3A.0-0a. Band 71: fUL = 679.3 MHz (NUL = 133285), fDL = 633.3 MHz (NDL = 68749) Note 22: Band 28: fDL = 790.0 MHz (NUL = 9530) Note 23: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. CA_XA-YA-YD represents the set of CA_XA-YD-YA, YD-YA-XA, YA-XA-YD and YA-YD-XA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.9.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  |  |  | Wgap [MHz] |  | SCC1 |  |  |  |  |  |  |  | Wgap [MHz] |  | SCC2 |  |  |  | Wgap [MHz] |  | SCC3 |  |  |  |
|  | Band | Range |  | NRB |  |  |  |  | Band |  | Range |  | NRB |  |  |  |  |  | Band | Range | NRB |  |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD |  | ULalloc (Note 2,3,4) | DLalloc |  |  |  | UL MOD |  | DL MOD |  | ULalloc |  | DLalloc |  |  |  | UL MOD | UL MOD | ULalloc | DLalloc |  |  | UL MOD | DL MOD | ULalloc | DLalloc |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_2A-66A-66A-71A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | High | 50@50 | ALL RBs | - |  | 71 |  | NOTE 18 |  | N/A |  | ALL RBs |  | - |  | 66 |  | CC2 | N/A | ALL RBs |  | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 100 |  |  | N/A |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| 2 | 2 |  | Low | 100@0 | ALL RBs | - |  | 71 |  | High |  | N/A |  | ALL RBs |  | - |  | 66 |  | CC2 | N/A | ALL RBs |  | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 100 |  |  | N/A |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| 3 | 66 |  | CC1 | 100@0 | ALL RBs | Max |  | 66 |  | CC2 |  | N/A |  | ALL RBs |  | - |  | 2 |  | Mid | N/A | ALL RBs |  | - | 71 | Mid | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 100 |  |  | N/A |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| 4 | 66 |  | CC1 | 100@0 | ALL RBs | Max |  | 66 |  | CC2 |  | N/A |  | ALL RBs |  | - |  | 2 |  | Mid | N/A | ALL RBs |  | - | 71 | Mid | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 100 |  |  | N/A |  | QPSK |  |  |  | 25 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| 5 | 66 |  | CC1 | 100@0 | ALL RBs | Max |  | 66 |  | CC2 |  | N/A |  | ALL RBs |  | - |  | 2 |  | Mid | N/A | ALL RBs |  | - | 71 | Mid | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 100 |  |  | N/A |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 50 |
| 6 | 66 |  | CC1 | 100@0 | ALL RBs | Max |  | 66 |  | CC2 |  | N/A |  | ALL RBs |  | - |  | 2 |  | Mid | N/A | ALL RBs |  | - | 71 | Mid | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 100 |  |  | N/A |  | QPSK |  |  |  | 25 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 50 |
| 7 | 71 |  | Mid | 50@0 | ALL RBs | - |  | 2 |  | Mid |  | N/A |  | ALL RBs |  | - |  | 66 |  | CC2 | N/A | ALL RBs |  | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 50 |  |  | N/A |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| 8 | 71 |  | Low | 25@0 | ALL RBs | - |  | 2 |  | High |  | N/A |  | ALL RBs |  | - |  | 66 |  | CC2 | N/A | ALL RBs |  | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 25 |  |  | N/A |  | QPSK |  |  |  | 25 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| 9 | 71 |  | Low | 25@12 | ALL RBs | - |  | 2 |  | High |  | N/A |  | ALL RBs |  | - |  | 66 |  | CC2 | N/A | ALL RBs |  | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 50 |  |  | N/A |  | QPSK |  |  |  | 50 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| 10 | 71 |  | Low | 20@25 | ALL RBs | - |  | 2 |  | High |  | N/A |  | ALL RBs |  | - |  | 66 |  | CC2 | N/A | ALL RBs |  | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 75 |  |  | N/A |  | QPSK |  |  |  | 75 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| 11 | 71 |  | Low | 20@40 | ALL RBs | - |  | 2 |  | High |  | N/A |  | ALL RBs |  | - |  | 66 |  | CC2 | N/A | ALL RBs |  | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 100 |  |  | N/A |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| 12 | 71 |  | Mid | 100@0 | ALL RBs | - |  | 2 |  | Mid |  | N/A |  | ALL RBs |  | - |  | 66 |  | CC2 | N/A | ALL RBs |  | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK |  | QPSK |  | 100 |  |  | N/A |  | QPSK |  |  |  | 100 |  |  |  | N/A |  | QPSK |  | 100 |  |  | N/A | QPSK |  | 100 |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X, Y, Z correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-4A-12A,X=2,Y=4,Z=12. Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A-41A,X=41. Note 7: Intra-band non-contiguous + Intra-band non-contiguous: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-4A-4A,X=2,Y=4. Note 8: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: Applicable if only Bandwidth Combination Set 0 is supported by the UE for the CA Configuration. (This note is not applicable for any existing test point and only reserved for future use) Note 11: Not applicable if only Bandwidth Combination Set 0 is supported by the UE for the CA Configuration. Note 12: Applicable if only Bandwidth Combination Set 1 is supported by the UE for the CA Configuration. (This note is not applicable for any existing test point and only reserved for future use) Note 13: Not applicable if only Bandwidth Combination Set 1 is supported by the UE for the CA Configuration. Note 14: Not applicable if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 15: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 16: Band 2: fUL = 1868.3MHz (NUL = 18783), fDL = 1948.3MHz (NDL = 783). Band 4: fUL = 1735MHz (NUL = 20200), fDL = 2135MHz (NDL = 2200). Note 17: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 18: Test point that fulfils criteria of Note 16 in Table 7.3A.0-0a. Band 71: fUL = 679.3 MHz (NUL = 133285), fDL = 633.3 MHz (NDL = 68749) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.3A.10	Reference sensitivity level for 5DL CA

Editor’s note: Following aspects missing or not yet determined:

-	Test configuration table is under investigating

-	Test requirement for some CA configuration is TBD

-	Test applicability with related information in Table 7.1-2a needs to add to TS36.521-2

#### 7.3A.10.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.10.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 5DL with CA configurations in Table 7.1-2c.

#### 7.3A.10.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.10.4	Test description

##### 7.3A.10.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.10.4.1-1 through table 7.3A.10.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.3A.10.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | Wgap [MHz] | SCC1 |  |  |  | Wgap [MHz] | SCC2 |  |  |  | Wgap [MHz] | SCC3 |  |  |  | Wgap [MHz] | SCC4 |  |  |  |
|  | Band | Range | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | UL alloc (Note 2,3,4) | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |  | UL MOD | UL MOD | UL alloc | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |
| Default Test Settings for a CA_XA-XA-YA-ZA-RA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REFSENS | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Y | Mid | N/A | ALL RBs | - | Z | Mid | N/A | ALL RBs | - | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |
| 2 | Y | Mid | REFSENS | ALL RBs | - | X | CC2 | N/A | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Z | Mid | N/A | ALL RBs | - | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |
| 3 | Z | Mid | REFSENS | ALL RBs | - | X | CC2 | N/A | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Y | Mid | N/A | ALL RBs | - | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |
| 4 | R | Mid | REFSENS | ALL RBs | - | X | CC2 | N/A | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Y | Mid | N/A | ALL RBs | - | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |
| Default Test Settings for CA_2A-5B-66A-66A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 2 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 3 | 2 | Mid | REFSENS | ALL RBs | - | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC1 | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 4 | 5 | Mid/CC1 | REFSENS | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs | - | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Default Test Settings for CA_5B-30A-66A-66A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 5 | Mid | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 30 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 2 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 5 | Mid | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 30 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 3 | 30 | Mid | REFSENS | ALL RBs | - | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC1 | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 4 | 5 | Mid/CC1 | REFSENS | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs | - | 30 | Mid | N/A | ALL RBs | - | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Default Test Settings for a CA_29A-66A-66A-70C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 29 | Mid | N/A | ALL RBs | - | 70 | Mid/CC1 | N/A | ALL RBs | - | 70 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 75 |  | N/A | QPSK |  | 50 |
| 2 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 29 | Mid | N/A | ALL RBs | - | 70 | Mid/CC1 | N/A | ALL RBs | - | 70 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 75 |  | N/A | QPSK |  | 50 |
| 3 | 70 | Mid/CC1 | REFSENS | ALL RBs | - | 70 | Mid/CC2 | N/A | ALL RBs | - | 29 | Mid | N/A | ALL RBs | - | 66 | CC1 | N/A | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 75 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |

< End of changes >