3GPP TSG-RAN5 Meeting #92-e	R5-214182

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0146 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update applicability for OTDOA (LTE) test cases for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2021-07-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For NR Test Configuration B (NG-RAN NR) there is no LTE environment and testing OTDOA (LTE) is therefore not applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Testing of OTDOA (LTE) (sub-tests 5 and 7) is deleted for NR Test Configuration B (NG-RAN NR). 2. Minor clarifications made for ECID (LTE) and OTDOA (LTE) and other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sub-tests for OTDOA (LTE) which are not applicable for NR Test Configuration B (NG-RAN NR) will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2, 8.2.3, 8.3.1, 8.4.1.2, 9.3.1.2.3.1, 9.3.1.2.3.2, 9.3.3.1B.1, 9.3.3.1B.3.1, 9.3.3.1B.3.3, 9.3.4.2.3.1, 9.3.4.2.3.2, 9.3.4.4.3.1, 9.3.4.4.3.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 8.2	Default signal conditions

### 8.2.1	Simulated GNSS environment

Same as defined in clause 5.2.1.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

### 8.2.2	Simulated OTDOA (LTE) environment

For OTDOA (LTE) signalling test cases a multi cell environment is used with LTE Cell 1 and LTE Cell 2 (where required) as defined in 3GPP TS 36.508 [8].

All LTE cells transmit PRS according to the PRS configuration provided in the OTDOA (LTE) assistance data defined in subclause 8.4.1.2. The positioning subframes are low-interference subframes, i.e. contain no PDSCH transmissions.

Normal propagation condition is used for all cells.

LTE Cell 1 is the OTDOA reference cell, LTE Cell 2 (where required) is an OTDOA neighbour cell.

Where two LTE cells are required, the two LTE Cells 1 and 2 shall be synchronized, and the timing offset (the RSTD) between the cells, referenced to the UE’s antenna input, shall be set equal to the expectedRSTD value provided in the OTDOA (LTE) assistance data, as defined in subclause 8.4.1.2.

For Test Configuration B (Table 8.1A-1) there is no LTE environment and testing OTDOA (LTE) is therefore not applicable.

For Test Configuration D (Table 8.1A-1), the LTE Cell 1 and LTE Cell 2 the E-UTRA frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

### 8.2.3	Simulated ECID (LTE) environment

Same as defined in clause 5.2.3.

For Test Configuration B (Table 8.1A-1) up to and including LPP Rel-15, UE Rx-Tx measurement is not possible and therefore there is little value in testing ECID (LTE) for positioning purposes and ECID (LTE) shall not be tested.

For Test Configuration D (Table 8.1A-1), the LTE Cell 1 and LTE Cell 2 the E-UTRA frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

### 8.2.4	Simulated MBS environment

Same as defined in clause 5.2.4.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

### 8.2.5	Simulated WLAN environment

Same as defined in clause 5.2.5.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

### 8.2.6	Simulated Bluetooth environment

Same as defined in clause 5.2.6.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

### 8.2.7	Simulated Sensor environment

Same as defined in clause 5.2.7.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

### 8.2.8	Simulated general NR environment

For NR FR2, the connection between the SS and the DUT shall be OTA. The SS shall ensure that the NR cell is suitable throughout the test. The OTA link shall be sufficient to provide stable LPP message transmissions between the SS and the DUT.

For NR FR1 and the other technologies used for the test cases in Chapter 9 (e.g. LTE, GNSS, WLAN…) the connection shall be conducted, following the settings described in this section.

### 8.2.9	Simulated Multi-RTT environment

For Multi-RTT signalling test cases NR Cell 1 is used, as defined in 3GPP TS 38.508-1 [30].

NR cell transmits DL-PRS according to the DL-PRS configuration provided in the Multi-RTT assistance data defined in subclause 8.4.1.6. The UE transmits UL-SRS according to the UL-SRS configuration provided in the RRC message as defined in subclause 8.3.2.

Normal propagation condition is used for all cells.

The simulated general NR environment as specified in 8.2.8 also applies.

Multi-RTT tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 frequency to be used for testing and other default conditions is as specified for signalling test cases in 3GPP TS 38.508-1 [30].

### 8.2.10	Simulated DL-AoD environment

For DL-AoD signalling test cases NR Cell 1 and NR Cell 2 (if required) are used, as defined in 3GPP TS 38.508-1 [30].

All NR cells transmit DL-PRS according to the DL-PRS configuration provided in the DL-AoD assistance data defined in subclause 8.4.1.7.

Normal propagation condition is used for all cells.

The simulated general NR environment as specified in 8.2.8 also applies.

DL-AoD tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 and NR Cell 2 frequencies to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

### 8.2.11	Simulated DL-TDOA environment

For DL-TDOA signalling test cases a multi cell environment with NR Cell 1, NR Cell 2, NR Cell 3 (if required)  and NR Cell 4 (if required) are used, as defined in 3GPP TS 38.508-1 [30].

All NR cells transmit DL-PRS according to the DL-PRS configuration provided in the DL-TDOA assistance data defined in subclause 8.4.1.8.

Normal propagation condition is used for all cells.

NR Cell 1 is the DL-TDOA reference cell. NR Cell 2, NR Cell 3 and NR Cell 4 are DL-TDOA neighbour cells.

The simulated general NR environment as specified in 8.2.8 also applies.

DL-TDOA tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1, NR Cell 2, NR Cell 3 and NR Cell 4 frequencies to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

### 8.2.12	Simulated NR E-CID environment

For NR E-CID signalling test cases NR Cell 1 is used, as defined in 3GPP TS 38.508-1 [30].

The simulated general NR environment as specified in 8.2.8 also applies.

NR E-CID tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 frequency to be used for testing and other default conditions is as specified for signalling test cases in 3GPP TS 38.508-1 [30].

## 8.3	Default RRC and NAS message and information elements contents

The default values of common RRC and NAS messages and information elements are used as defined in 3GPP
TS 38.508-1 [30] with the following exceptions:

Table 8.3-1: Void

Table 8.3-2: Void

### -	REGISTRATION ACCEPT

Table 8.3-3: REGISTRATION ACCEPT

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS network feature support |  | Set according to Table 8.3-4 |  |  |

Table 8.3-4: 5GS network feature support

| Derivation Path: 24.501 clause 9.11.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in 38.508-1 Table 4.7.1-7 with the following exceptions: |  |  |  |  |
| Emergency service support indicator for 3GPP access (EMC) (octet 3, bit 3 and bit 4) |  | 01 | Emergency services supported in NR connected to 5GCN only |  |

### 8.3.1	RRC message contents for measurement gaps

Table 8.3.1-1: RRCReconfiguration for meas gaps

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
|  |  | Not present |  |  |
| measConfig |  | MeasConfig | Table 8.3.1-2 |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.3.1-2: MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | Not present |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| s-MeasureConfig | Not present |  |  |
| quantityConfig | Not present |  |  |
| measGapConfig | MeasGapConfig | Table 8.3.1-3 |  |
| measGapSharingConfig | Not present |  |  |
| } |  |  |  |

Table 8.3.1-3: MeasGapConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 | Not present |  |  |
| gapFR1 | Not present |  |  |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 3 |  |  |
| mgl | ms3 |  |  |
| mgrp | ms160 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.3.2	RRC message contents for UL-SRS Configuration

Table 8.3.2-1: RRCReconfiguration

| Derivation Path: TS 38.508-1, table 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.3.2-2: CellGroupConfig

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| srs-Config CHOICE { |  |  |  |
| setup | srs-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.2-3: SRS-Config

| Derivation Path: TS 38.331, clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-PosResourceSetToReleaseList-r16 | Not present |  |  |
| srs-PosResourceSetToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16 { | 1 entry |  |  |
| SRS-PosResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| srs-PosResourceSetId-r16 | 0 |  |  |
| srs-PosResourceIdList-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16 { | 1 entry |  |  |
| SRS-PosResourceId-r16[1] | 0 | entry 1 |  |
| } |  |  |  |
| resourceType-r16 CHOICE { |  |  |  |
| periodic-r16 SEQUENCE { |  |  |  |
| } |  |  |  |
| alpha-r16 | Not present |  |  |
| p0-r16 | -100 |  |  |
| pathlossReferenceRS-Pos-r16 CHOICE { |  |  |  |
| ssb-IndexServing-r16 | 1 | Set according to Table 4.4.2-2 in TS 38.508-1. |  |
| } |  |  |  |
| } |  |  |  |
| srs-PosResourceToReleaseList-r16 | Not present |  |  |
| srs-PosResourceToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16 { | 1 entry |  |  |
| SRS-PosResource-r16[1] | SRS-PosResource-r16 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.2-4: SRS-PosResource-r16

| Derivation Path: TS 38.331, clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-PosResource-r16::=SEQUENCE { |  |  |  |
| srs-PosResourceId-r16 | 0 |  |  |
| transmissionComb-r16 CHOICE { |  |  |  |
| n2-r16 SEQUENCE { |  |  |  |
| combOffset-n2-r16 | 0 |  |  |
| cyclicShift-n2-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping-r16 SEQUENCE { |  |  |  |
| startPosition-r16 | 0 |  |  |
| nrofSymbols-r16 | n1 |  |  |
| } |  |  |  |
| freqDomainShift-r16 | 0 |  |  |
| freqHopping-r16 SEQUENCE { |  |  |  |
| c-SRS-r16 | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping-r16 | groupHopping |  |  |
| resourceType-r16 CHOICE { |  |  |  |
| periodic-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-p-r16 CHOICE { | sl1 |  |  |
| sl1 | Null |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId-r16 | 0 |  |  |
| spatialRelationInfoPos-r16 CHOICE { |  |  |  |
| servingRS-r16 SEQUENCE { |  |  |  |
| servingCellId | 0 |  |  |
| referenceSignal-r16 CHOICE { |  |  |  |
| ssb-IndexServing-r16 | 1 | Set according to Table 4.4.2-2 in TS 38.508-1. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## 8.4	Default LPP message and information elements contents

The default values of LPP messages and information elements used, unless indicated otherwise in specific clauses of this specification are as defined in clause 5.4 with the following exceptions:

#### -	LPP PROVIDE ASSISTANCE DATA

Table 8.4-1: ProvideAssistanceData

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { | Dependent on test case. |  |  |
| initiator |  |  |  |
| transactionNumber |  |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData SEQUENCE { |  |  | Sub-tests 7 and 15 only; and as defined in Table 5.4.1.1-1. |
| gnss-CommonAssistData SEQUENCE { |  |  |  |
| gnss-ReferenceTime | As defined in 37.571-5 [12] |  |  |
| gnss-ReferenceLocation | As defined in 37.571-5 [12] |  |  |
| gnss-IonosphericModel | As defined in 37.571-5 [12] |  |  |
| gnss-EarthOrientationParameters | Not present |  |  |
| } |  |  |  |
| gnss-GenericAssistData(SIZE(1..4))OF{ | SIZE is dependent on the number of GNSSs supported by the UE. If one GNSS supported by the UE, SIZE = 1 If two GNSSs supported by the UE, SIZE = 2 If three GNSSs supported by the UE, SIZE = 3 If four GNSSs supported by the UE, SIZE = 4 |  |  |
| gnss-ID | For each GNSS supported by the UE. |  |  |
| sbas-ID | Not present |  |  |
| gnss-TimeModels | As defined in 37.571-5 [12] |  |  |
| gnss-DifferentialCorrections | Not present |  |  |
| gnss-NavigationModel | As defined in 37.571-5 [12] |  |  |
| gnss-RealTimeIntegrity | Not present |  |  |
| gnss-DataBitAssistance | Not present |  |  |
| gnss-AcquisitionAssistance | As defined in 37.571-5 [12] |  |  |
| gnss-Almanac | As defined in 37.571-5 [12] |  |  |
| gnss-UTC-Model | As defined in 37.571-5 [12] |  |  |
| gnss-AuxiliaryInformation | As defined in 37.571-5 [12] |  |  |
| } |  |  |  |
| gnss-Error | Not present |  |  |
| } |  |  |  |
| otdoa-ProvideAssistanceData SEQUENCE { |  |  | Sub-tests 5 and 7 only |
| otdoa-ReferenceCellInfo | As defined in Table 8.4.1.2-1 |  |  |
| otdoa-NeighbourCellInfo | As defined in Table 8.4.1.2-2 |  |  |
| otdoa-Error | Not present |  |  |
| } |  |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 18 only as defined in clause 5.4.1.5 |
| sensor-AssistanceDataList-r14 | As defined in Table 5.4.1.5-2 |  |  |
| sensor-Error-r14 | Not present |  |  |
| } |  |  |  |
| tbs-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 16 only as defined in clause5.4.1.3 |
| tbs-AssistanceDataList-r14 SEQUENCE { |  |  |  |
| mbs-AssistanceDataList-r14 SEQUENCE (SIZE(1..n)) OF SEQUENCE{ |  |  |  |
| mbs-AlmanacAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| mbs-AcquisitonAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| tbs-Error-r14 | Not present |  |  |
| } |  |  |  |
| wlan-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 17 only as defined in clause 5.4.1.4 |
| wlan-DataSet-r14 | As defined in Table 5.4.1.4-2 |  |  |
| wlan-Error-r14 | Not present |  |  |
| } |  |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 SEQUENCE { |  | Rel-16 onwards | Sub-test 19 only |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 8.4.1.6-1 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | As defined in Table 8.4.1.6-2 |  |  |
| nr-Multi-RTT-Error-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 SEQUENCE { |  | Rel-16 onwards | Sub-test 20 only |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 8.4.1.6-1 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | As defined in Table 8.4.1.6-2 |  |  |
| nr-PositionCalculationAssistance-r16 | Present or not present dependent on pc_UEB_DL_AoD. As defined in Table 8.4.1.7-1 |  |  |
| nr-DL-AoD-Error-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 SEQUENCE { |  | Rel-16 onwards | Sub-test 21 only |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 8.4.1.6-1 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | As defined in Table 8.4.1.6-2 |  |  |
| nr-PositionCalculationAssistance-r16 | Present or not present dependent on pc_UEB_DL_TDOA. As defined in Table 8.4.1.7-1 |  |  |
| nr-DL-TDOA-Error-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	LPP REQUEST CAPABILITIES

Table 8.4-2: RequestCapabilities

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| Initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestCapabilities SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestCapabilities-r9 SEQUENCE { |  |  |  |
| commonIEsRequestCapabilities SEQUENCE { | Present | Rel-14 onwards |  |
| lpp-message-segmentation-req-r14 | 00 | Server is not able to send segmented LPP messages. Server is not able to receive segmented LPP messages. Rel-14 onwards |  |
| } |  |  |  |
| a-gnss-RequestCapabilities SEQUENCE { |  |  |  |
| gnss-SupportListReq | TRUE |  |  |
| assistanceDataSupportListReq | TRUE |  |  |
| locationVelocityTypesReq | TRUE |  |  |
| } |  |  |  |
| otdoa-RequestCapabilities SEQUENCE { | Present |  |  |
| } |  |  |  |
| ecid-RequestCapabilities SEQUENCE { | Present |  |  |
| } |  |  |  |
| epdu-RequestCapabilities | Not present |  |  |
| sensor-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| tbs-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| wlan-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| bt-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| nr-ECID-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| nr-Multi-RTT-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| nr-DL-AoD-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| nr-DL-TDOA-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| nr-UL-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	LPP REQUEST LOCATION INFORMATION

Table 8.4-3: RequestLocationInformation

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | Dependent on test case |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | 32 |  |  |
| responseTimeEarlyFix-r12 | Not present | Rel-12 onwards |  |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | As defined in Table 5.4-4 |  | Sub-tests 7 and 15 |
| otdoa-RequestLocationInformation | As defined in Table 5.4-5 |  | Sub-test 5 and 7 |
| ecid-RequestLocationInformation | As defined in Table 5.4-6 |  | Sub-test 6 |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | As defined in Table 5.4-10 | Rel-13 onwards | Sub-test 14, 18 |
| tbs-RequestLocationInformation-r13 | As defined in Table 5.4-7 | Rel-13 onwards | Sub-tests 12, 16 |
| wlan-RequestLocationInformation-r13 | As defined in Table 5.4-8 | Rel-13 onwards | Sub-test 11, 17 |
| bt-RequestLocationInformation-r13 | As defined in Table 5.4-9 | Rel-13 onwards | Sub-test 13 |
| nr-ECID-RequestLocationInformation-r16 | As defined in Table 8.4-4 | Rel-16 onwards | Sub-test 22 |
| nr-Multi-RTT-RequestLocationInformation-r16 | As defined in Table 8.4-5 | Rel-16 onwards | Sub-test 19 |
| nr-DL-AoD-RequestLocationInformation-r16 | As defined in Table 8.4-6 | Rel-16 onwards | Sub-test 20 |
| nr-DL-TDOA-RequestLocationInformation-r16 | As defined in Table 8.4-7 | Rel-16 onwards | Sub-test 21 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	NR E-CID REQUEST LOCATION INFORMATION

Table 8.4-4: NR-ECID-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.9.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-ECID-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| requestedMeasurements-r16 | All measurements supported by the UE |  |  |
| } |  |  |  |

#### -	NR Multi-RTT REQUEST LOCATION INFORMATION

Table 8.4-5: NR-Multi-RTT-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.12.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | Not present |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-Multi-RTT-ReportConfig-r16 | Not present |  |  |
| additionalPaths-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	NR DL-AoD REQUEST LOCATION INFORMATION

Table 8.4-6: NR-DL-AoD-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.11.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-AoD-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSRP-MeasurementsPerTRP-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	NR DL-TDOA REQUEST LOCATION INFORMATION

Table 8.4-7: NR-DL-TDOA-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.10.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-RstdMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 0 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-TDOA-ReportConfig-r16 | Not present |  |  |
| additionalPaths-r16 | Not present |  |  |
| } |  |  |  |

### 8.4.1	Default assistance data information elements

#### 8.4.1.1	GNSS Assistance Data Elements

The GNSS assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message are as defined in clause 5.4.1.1.

#### 8.4.1.2	OTDOA (LTE) Assistance Data Elements

This clause defines the OTDOA (LTE) assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages.

#### -	OTDOA (LTE) REFERENCE CELL INFO

Table 8.4.1.2-1: OTDOA-ReferenceCellInfo

| Derivation Path: 37.355 clause 6.5.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OTDOA-ReferenceCellInfo ::= SEQUENCE { |  | LTE Cell 1 |  |
| physCellId | 0 |  |  |
| cellGlobalId SEQUENCE { |  |  |  |
| mcc | As defined for Cell 1 in 36.508 [8] |  |  |
| mnc | As defined for Cell 1 in 36.508 [8] |  |  |
| cellidentity | As defined for E-UTRAN Cell Identifier for Cell 1 in 36.508 [8] |  |  |
| } |  |  |  |
| earfcnRef | As defined for Cell 1 in 36.508 [8] |  |  |
| antennaPortConfig | As defined for Cell 1 in 36.508 [8] |  |  |
| cpLength | Normal |  |  |
| prsInfo SEQUENCE { |  |  |  |
| prs-Bandwidth | PRS are transmitted over the used system bandwidth (see clause 5.2.2) |  |  |
| prs-ConfigurationIndex | FDD: 2 TDD: 4 |  |  |
| numDL-Frames | sf-1 |  |  |
| prs-MutingInfo-r9 | Not present | PRS muting is not used. |  |
| prsID-r14 | Not present | PRS-ID not used | Rel-14 onwards |
| add-numDL-Frames-r14 | Not present | Not required | Rel-14 onwards |
| prsOccGroupLen-r14 | Not present | No PRS occasion group configured | Rel-14 onwards |
| prsHoppingInfo-r14 | Not present | PRS frequency hopping not used | Rel-14 onwards |
| } |  |  |  |
| earfcnRef-v9a0 | As defined for Cell 1 in 36.508 [8] |  |  |
| tpId-r14 | Not present | Transmission Points not used | Rel-14 onwards |
| cpLengthCRS-r14 | Normal |  | Rel-14 onwards |
| sameMBSFNconfigRef-r14 | FALSE | Not the same as the serving cell | Rel-14 onwards |
| dlBandwidth-r14 | Not present | PRS frequency hopping not used | Rel-14 onwards |
| addPRSconfigRef-r14 | Not present | No additional PRS configuration(s) | Rel-14 onwards |
| nr-LTE-SFN-Offset-r15 | Not present |  | Rel-15 onwards |
| tdd-config-v1520 | Not present |  | Rel-15 onwards |
| nr-LTE-fineTiming-Offset-r15 | Not present |  | Rel-15 onwards |
| } |  |  |  |

#### -	OTDOA (LTE) NEIGHBOUR CELL INFO LIST

Table 8.4.1.2-2: OTDOA-NeighbourCellInfoList

| Derivation Path: 37.355 clause 6.5.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE { |  |  |  |
| SEQUENCE (SIZE(2)) OF SEQUENCE { |  | Cell 2 |  |
| physCellId | 2 |  |  |
| cellGlobalId SEQUENCE { |  |  |  |
| mcc | As defined for Cell 2 in 36.508 [8] |  |  |
| mnc | As defined for Cell 2 in 36.508 [8] |  |  |
| cellidentity | As defined for E-UTRAN Cell Identifier for Cell 2 in 36.508 [8] |  |  |
| } |  |  |  |
| earfcn | Not present | Same as for the reference cell |  |
| cpLength | Not present | Same as for the reference cell |  |
| prsInfo | Not present | Same as for the reference cell |  |
| antennaPortConfig | Not present | Same as for the reference cell |  |
| slotNumberOffset | Not present | Slot timing is the same as for reference cell |  |
| prs-SubframeOffset | Not present |  |  |
| expectedRSTD | 8192 | Value 0 |  |
| expectedRSTD-Uncertainty | 10 | About 1 s |  |
| earfcn-v9a0 | Not present | Same as for the reference cell |  |
| tpId-r14 | Not present | Transmission Points not used | Rel-14 onwards |
| prs-only-tp-r14 | Not present | Not required | Rel-14 onwards |
| cpLengthCRS-r14 | Not present | Not required | Rel-14 onwards |
| sameMBSFNconfigNeighbour-r14 | TRUE | Same as for the reference cell | Rel-14 onwards |
| dlBandwidth-r14 | Not present | Same as for the reference cell and PRS frequency hopping not used | Rel-14 onwards |
| addPRSconfigNeighbour-r14 | Not present | No additional PRS configuration(s) | Rel-14 onwards |
| tdd-config-v1520 | Not present |  | Rel-15 onwards |
| } |  |  |  |
| SEQUENCE { |  | Cell 4 |  |
| physCellId | 4 |  |  |
| cellGlobalId SEQUENCE { |  |  |  |
| mcc | As defined for Cell 4 in 36.508 [8] |  |  |
| mnc | As defined for Cell 4 in 36.508 [8] |  |  |
| cellidentity | As defined for E-UTRAN Cell Identifier for Cell 4 in 36.508 [8] |  |  |
| } |  |  |  |
| earfcn | Not present | Same as for the reference cell |  |
| cpLength | Not present | Same as for the reference cell |  |
| prsInfo | Not present | Same as for the reference cell |  |
| antennaPortConfig | Not present | Same as for the reference cell |  |
| slotNumberOffset | Not present | Slot timing is the same as for reference cell |  |
| prs-SubframeOffset | Not present |  |  |
| expectedRSTD | 8192 | Value 0 |  |
| expectedRSTD-Uncertainty | 10 | About 1 s |  |
| earfcn-v9a0 | Not present | Same as for the reference cell |  |
| tpId-r14 | Not present | Transmission Points not used | Rel-14 onwards |
| prs-only-tp-r14 | Not present | Not required | Rel-14 onwards |
| cpLengthCRS-r14 | Not present | Not required | Rel-14 onwards |
| sameMBSFNconfigNeighbour-r14 | TRUE | Same as for the reference cell | Rel-14 onwards |
| dlBandwidth-r14 | Not present | Same as for the reference cell and PRS frequency hopping not used | Rel-14 onwards |
| addPRSconfigNeighbour-r14 | Not present | No additional PRS configuration(s) | Rel-14 onwards |
| tdd-config-v1520 | Not present |  | Rel-15 onwards |
| } |  |  |  |
| } |  |  |  |

#### NEXT CHANGED SECTION

#### 9.3.1.2	LPP Abort

Editor's note: Test configuration D is incomplete:

-	The corresponding attach procedure for NG-RAN E-UTRA has not yet been defined.

-	The message contents need to be revised for Test Configuration D.

9.3.1.2.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {
  when {	UE receives a LPP Abort message carrying the transaction ID of an on-going procedure }

then { UE aborts the on-going procedure }

}

9.3.1.2.2	Conformance requirements

As defined in clause 7.3.5.1.2.

9.3.1.2.3	Test description

9.3.1.2.3.1	Pre-test conditions

System Simulator:

-	For Test Configuration B (Table 9.3.1.2.3.2-1):

-	Sub-tests 11, 12, 13, 15, 16, 17, 19, 20: NR Cell 1.

-	Sub-test 21: NR Cell 1 and NR Cell 2.

-	For Test Configuration D (Table 9.3.1.2.3.2-1):

-	Sub-tests 11, 12, 13, 15, 16, 17: LTE Cell 1.

-	Sub-test 5: LTE Cell 1, as specified in 8.2.2.

UE:

-	The UE shall begin the tests with no assistance data stored.

Preamble:

-	For Test Configuration B (Table 9.3.1.2.3.2-1): The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	Sub-test 19: After the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.1.2.3.1-1 for the configuration of measurement gaps for Multi-RTT and then the SS shall execute the steps in Table 9.3.1.2.3.1-2 for the configuration of UL-SRS for Multi-RTT.

-	Sub-test 20: After the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.1.2.3.1-1 for the configuration of measurement gaps for DL-AoD.

-	Sub-test 21: After the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.1.2.3.1-1 for the configuration of measurement gaps for DL-TDOA.

Table 9.3.1.2.3.1-1: Configuration of measurement gaps

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an RRCReconfiguration message as in Table 8.3.1-1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |

Table 9.3.1.2.3.1-2: Configuration of UL-SRS for Multi-RTT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an RRCReconfiguration message as in Table 8.3.2-1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |

-	For Test Configuration D (Table 9.3.1.2.3.2-1): FFS.

Related PICS/PIXIT Statements:

-

9.3.1.2.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 9.3.1.2.3.2-0 below:

Table 9.3.1.2.3.2-0: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 5 | UE supporting OTDOA (LTE) (Rel-15 onwards) (Test Configuration D only) |
| 11 | UE supporting WLAN (Rel-13 only) |
| 12 | UE supporting MBS (Rel-13 only) |
| 13 | UE supporting Bluetooth |
| 15 | UE supporting GNSS(1) |
| 16 | UE supporting MBS (Rel-14 onwards) |
| 17 | UE supporting WLAN (Rel-14 onwards) |
| 19 | UE supporting Multi-RTT (Rel-16 onwards) |
| 20 | UE supporting DL-AoD (Rel-16 onwards) |
| 21 | UE supporting DL-TDOA (Rel-16 onwards) |
| NOTE 1: The GNSS combination of GPS, GLONASS, Galileo, BDS supported by the UE |  |

Note that this test case does not include a sub-test for the case where ECID (LTE), NR E-CID or Sensor is supported by the UE as the behaviour required cannot be guaranteed in these cases.

Table 9.3.1.2.3.2-1: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| A | EN-DC | Functionality is tested by test case 7.3.5.1 |
| B | NG-RAN NR |  |
| C | NE-DC | Functionality is tested by test configuration B |
| D | NG-RAN E-UTRA |  |
| E | NGEN-DC | Functionality is tested by test configuration D |

Main behaviour is as defined in Table 7.3.5.1.3.2-1.

### Next changed section

### 9.3.3	LPP Error Handling

#### 9.3.3.1	Void

#### 9.3.3.1A	Void

#### 9.3.3.1B	LPP Requested Method not Supported – UE-Assisted

Editor's note: Test configuration D is incomplete:

- The corresponding attach procedure for NG-RAN E-UTRA has not yet been defined.

- The message contents need to be revised for Test Configuration D.

9.3.3.1B.1	Test Purpose (TP)

(1)

with { 	a UE supporting at least one of UE-assisted GNSS, UE-assisted OTDOA (LTE) (LPP Rel-15 onwards) (Test Configuration D only), UE-assisted ECID (LTE) (Test Configuration D only), UE-assisted WLAN, UE-assisted Bluetooth, UE-assisted Sensor, UE-assisted MBS, UE-assisted DL-TDOA, UE-assisted DL-AoD, UE-assisted Multi-RTT or UE-assisted NR E-CID but not all of them }

and with { a NAS signalling connection existing }

ensure that {
  when {	UE receives a LPP message requesting at least one location method not supported }

then { the UE provides location information for the supported methods}

}

9.3.3.1B.2	Conformance requirements

As defined in clause 7.3.3.1B.2.

9.3.3.1B.3	Test description

9.3.3.1B.3.1	Pre-test conditions

System Simulator:

For Test Configuration B (Table 9.3.3.1B.3.2-1):

-	If GNSS is supported by the UE: NR Cell 1 and satellite signals, as specified in 8.2.1.

-	If WLAN is supported by the UE: NR Cell 1 and WLAN signals, as specified in 8.2.5.

-	If Bluetooth is supported by the UE: NR Cell 1 and Bluetooth signals, as specified in 8.2.6.

-	If Sensor is supported by the UE: NR Cell 1.

-	If MBS is supported by the UE: NR Cell 1 and MBS signals, as specified in 8.2.4.

-	If DL-TDOA is supported by the UE: NR Cell 1 and NR Cell 2, as specified in 8.2.11.

-	If DL-AoD is supported by the UE: NR Cell 1, as specified in 8.2.10.

-	If Multi-RTT is supported by the UE: NR Cell 1, as specified in 8.2.9.

-	If NR E-CID is supported by the UE: NR Cell 1, as specified in 8.2.12.

For Test Configuration D (Table 9.3.3.1B.3.2-1):

-	If GNSS is supported by the UE: LTE Cell 1 and satellite signals, as specified in 8.2.1.

-	If OTDOA (LTE) is supported by the UE: LTE Cell 1 and LTE Cell 2, as specified in 8.2.2

-	If ECID (LTE) is supported by the UE: LTE Cell 1 and LTE Cell 2, as specified in 8.2.3.

-	If WLAN is supported by the UE: LTE Cell 1 and WLAN signals, as specified in 8.2.5.

-	If Bluetooth is supported by the UE: LTE Cell 1 and Bluetooth signals, as specified in 8.2.6.

-	If Sensor is supported by the UE: LTE Cell 1.

-	If MBS is supported by the UE: LTE Cell 1 and MBS signals, as specified in 8.2.4.

UE:

-

Preamble:

-	For Test Configuration B (Table 9.3.3.1B.3.2-1): The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	If DL-TDOA is supported by the UE then after the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.3.1B.3.1-1 for the configuration of measurement gaps for DL-TDOA.

-	If DL-AoD is supported by the UE then after the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.3.1B.3.1-1 for the configuration of measurement gaps for DL-AoD.

-	If Multi-RTT is supported by the UE then after the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.3.1B.3.1-1 for the configuration of measurement gaps for Multi-RTT and then the SS shall execute the steps in Table 9.3.3.1B.3.1-2 for the configuration of UL-SRS for Multi-RTT.

Table 9.3.3.1B.3.1-1: Configuration of measurement gaps

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an RRCReconfiguration message as in Table 8.3.1-1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |

Table 9.3.3.1B.3.1-2: Configuration of UL-SRS for Multi-RTT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an RRCReconfiguration message as in Table 8.3.2-1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |

-	For Test Configuration D (Table 9.3.3.1B.3.2-1): FFS.

Related PICS/PIXIT Statements:

-

9.3.3.1B.3.2	Test procedure sequence

Table 9.3.3.1B.3.2-1: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| A | EN-DC | Functionality is tested by test case 7.3.3.1B |
| B | NG-RAN NR |  |
| C | NE-DC | Functionality is tested by test configuration B |
| D | NG-RAN E-UTRA |  |
| E | NGEN-DC | Functionality is tested by test configuration D |

Main behaviour as defined in clause 7.3.3.1B.3.2.

9.3.3.1B.3.3	Specific message contents

As defined in clause 7.3.3.1B.3.3, with the following exceptions:

Table 9.3.3.1B.3.3-1 replaces Table 7.3.3.1B.3.3-1, Table 9.3.3.1B.3.3-2 replaces Table 7.3.3.1B.3.3-2, Table 9.3.3.1B.3.3-2a replaces Table 7.3.3.1B.3.3-3, Table 9.3.3.1B.3.3-3 replaces Table 7.3.3.1B.3.3-5, Table 9.3.3.1B.3.3-4 replaces Table 7.3.3.1B.3.3-6, Table 9.3.3.1B.3.3.5 replaces Table 7.3.3.1B.3.3-4, Table 9.3.3.1B.3.3.6 replaces Table 7.3.3.1B.3.3-6a and Table 9.3.3.1B.3.3-7 replaces Table 7.3.3.1B.3.3-7.

Table 9.3.3.1B.3.3-1: DLInformationTransfer (steps 0, 0b, 1, 2 and 3a, Table 7.3.3.1B.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.3.1B.3.3-2 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.3.1B.3.3-2: DL NAS TRANSPORT (steps 0, 0b, 1, 2 and 3a, Table 7.3.3.1B.3.2-1)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 0: Set according to Table 8.4-2 | LPP Request Capabilities. |  |
|  |  | Step 1: Set according to Table 9.3.3.1B.3.3-2a | LPP Provide Assistance Data |  |
|  |  | Step 2: Set according to Table 9.3.3.1B.3.3-5 | LPP Request Location Information |  |
|  |  | Steps 0b and 3a: Set according to Table 7.3.3.1B.3.3-8 | LPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.3.3.1B.3.3-2a: LPP Provide Assistance data (step 1, Table 7.3.3.1B.3.2-1)

| Derivation Path: Table 8.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 8.4-1 with the following exceptions: |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| Initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) |  |  |
| } |  |  |  |  |
| a-gnss-ProvideAssistanceData |  | Present for all supported GNSSs if UE supports UE-assisted A-GNSS. As defined in clause 8.4 |  |  |
| otdoa-ProvideAssistanceData |  | Present if UE supports UE-assisted OTDOA (LTE). As defined in clause 8.4 |  |  |
| sensor-ProvideAssistanceData-r14 |  | Present if UE supports UE-assisted Sensor. As defined in clause 8.4 | Rel-14 onwards |  |
| tbs-ProvideAssistanceData-r14 |  | Present if UE supports UE-assisted MBS. As defined in clause 8.4 | Rel-14 onwards |  |
| wlan-ProvideAssistanceData-r14 |  | Present if UE supports UE-assisted WLAN. As defined in clause 8.4 | Rel-14 onwards |  |
| nr-Multi-RTT-ProvideAssistanceData |  | Present if UE supports UE-assisted Multi-RTT. As defined in clause 8.4 | Rel-16 onwards |  |
| nr-DL-AoD-ProvideAssistanceData-r16 |  | Present if UE supports UE-assisted DL-AoD. As defined in clause 8.4 | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 |  | Present if UE supports UE-assisted DL-TDOA. As defined in clause 8.4 | Rel-16 onwards |  |

Table 9.3.3.1B.3.3-3: ULInformationTransfer (steps 0a and 3, Table 7.3.3.1B.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.3.1B.3.3-4 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.3.1B.3.3-4: UL NAS TRANSPORT (steps 0a and 3, Table 7.3.3.1B.3.2-1)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 0a: Set according to Table 9.3.3.1B.3.3-6 | LPP Provide Capabilities |  |
|  |  | Step 3: Set according to Table 9.3.3.1B.3.3-7 | LPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1 Table 7.3.3.1B.3.2-1) |  |

Table 9.3.3.1B.3.3-5: LPP Request Location Information (step 2, Table 7.3.3.1B.3.2-1)

| Derivation Path: Table 5.4-3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| As defined in Table 5.4-3 with the following exceptions: |  |  |  |  |  |  |  |
| locationInformationType |  | locationMeasurementsRequired |  |  |  |  |  |
| a-gnss-RequestLocationInformation |  | Present. As defined in Table 5.4-4 |  |  |  |  |  |
| gnss-Methods |  | GNSS-ID-Bitmap: bits 0, 3, 4, 5 = 1 |  |  |  |  |  |
| otdoa-RequestLocationInformation |  | Present. As defined in Table 5.4-5 |  |  |  |  |  |
| ecid-RequestLocationInformation |  | Present. As defined in Table 5.4-6 |  |  |  |  |  |
| requestedMeasurements |  | Test Configuration D: bits 0, 1, 2 = 1 |  |  |  |  |  |
| tbs-RequestLocationInformation-r13 |  | Present. As defined in Table 5.4-7 |  | Rel-13 onwards |  |  |  |
| sensor-RequestLocationInformation-r13 |  | Present. As defined in Table 5.4-10 |  | Rel-13 onwards |  |  |  |
| wlan-RequestLocationInformation-r13 |  | Present. As defined in Table 5.4-8 |  | Rel-13 onwards |  |  |  |
| bt-RequestLocationInformation-r13 |  | Present. As defined in Table 5.4-9 |  | Rel-13 onwards |  |  |  |
| nr-ECID-RequestLocationInformation-r16 |  | Present. As defined in Table 8.4-4 |  | Rel-16 onwards |  |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 |  | Present. As defined in Table 8.4-5 |  | Rel-16 onwards |  |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 |  | Present. As defined in Table 8.4-6 |  | Rel-16 onwards |  |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 |  | Present. As defined in Table 8.4-7 |  | Rel-16 onwards |  |  |  |

Table 9.3.3.1B.3.3-6: LPP Provide Capabilities (step 0a, Table 7.3.3.1B.3.2-1)

| Derivation Path: Table 7.3.3.1B.3.3-6a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.3.1B.3.3-6a with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| nr-ECID-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.3.1B.3.3-7: LPP Provide Location Information (step 3, Table 7.3.3.1B.3.2-1)

| Derivation Path: Table 7.3.3.1B.3.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.3.1B.3.3-7 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 SEQUENCE { |  | Present if UE supports UE-assisted Multi-RTT. | Rel-16 onwards |  |
| nr-Multi-RTT-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-Multi-RTT-Error-r16 |  | May be present |  |  |
| } |  |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { |  | Present if UE supports UE-assisted DL-AoD. | Rel-16 onwards |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-DL-AoD-Error-r16 |  | May be present |  |  |
| } |  |  |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  | Present if UE supports UE-assisted DL-TDOA. | Rel-16 onwards |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-DL-TDOA-Error-r16 |  | May be present |  |  |
| } |  |  |  |  |
| nr-ECID-ProvideLocationInformation-r16 SEQUENCE { |  | Present if UE supports UE-assisted NR E-CID. | Rel-16 onwards |  |
| nr-ECID-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-ECID-Error-r16 |  | May be present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

### 9.3.4	LPP Positioning Procedures

#### 9.3.4.1	E-SMLC Initiated Assistance Data Delivery followed by Location Information Transfer: UE-Based

Editor's note: Test configuration D is incomplete:

- The corresponding attach procedure for NG-RAN E-UTRA has not yet been defined.

- The message contents need to be revised for Test Configuration D.

9.3.4.1.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { UE receives assistance data and a location request for UE-based  }

then { UE sends a PROVIDE LOCATION INFORMATION  message containing a location estimate }

}

9.3.4.1.2	Conformance requirements

As defined in clause 7.3.4.1.2.

9.3.4.1.3	Test description

9.3.4.1.3.1	Pre-test conditions

System Simulator:

For Test Configuration B (Table 9.3.4.1.3.2-1):NR Cell 1.

-	Satellite signals (sub-test case 15): as specified in 8.2.1.

-	MBS signals (Sub-test 16): as specified in 8.2.4.

-	WLAN signals (Sub-test 17): as specified in 8.2.5.

-	Sub-test 20: NR Cell 1 and NR Cell 2, as specified in 8.2.10.

-	Sub-test 21: NR Cell 1, NR Cell 2, NR Cell 3 and NR Cell 4, as specified in 8.2.11.

For Test Configuration D (Table 9.3.4.1.3.2-1): LTE Cell 1.

-	Satellite signals (sub-test case 15): as specified in 8.2.1.

-	MBS signals (Sub-test 16): as specified in 8.2.4.

-	WLAN signals (Sub-test 17): as specified in 8.2.5.

UE:

The UE shall begin the test with no assistance data stored.

Preamble:

-	For Test Configuration B (Table 9.3.4.1.3.2-1): The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	For Test Configuration D (Table 9.3.4.1.3.2-1): FFS.

Related PICS/PIXIT Statements:

-

9.3.4.1.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 9.3.4.1.3.2-0 below:

Table 9.3.4.1.3.2-0: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 15 | UE supporting GNSS(1) |
| 16 | UE supporting MBS (Rel-14 onwards) |
| 17 | UE supporting WLAN (Rel-14 onwards) |
| 18 | UE supporting Sensor (Rel-14 onwards) |
| 20 | UE supporting DL-AoD (Rel-16 onwards) |
| 21 | UE supporting DL-TDOA (Rel-16 onwards) |
| NOTE 1: The GNSS combination of GPS, GLONASS, Galileo, BDS supported by the UE |  |

Table 9.3.4.1.3.2-1: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| A | EN-DC | Functionality is tested by test case 7.3.4.1 |
| B | NG-RAN NR |  |
| C | NE-DC | Functionality is tested by test configuration B |
| D | NG-RAN E-UTRA |  |
| E | NGEN-DC | Functionality is tested by test configuration D |

Main behaviour as defined in Table 7.3.4.1.3.2-1.

9.3.4.1.3.3	Specific message contents

As defined in clause 7.3.4.1.3.3, with the following exceptions:

Table 9.3.4.1.3.3-1 replaces Table 7.3.4.1.3.3-2, Table 9.3.4.1.3.3-2 replaces Table 7.3.4.1.3.3-3, Table 9.3.4.1.3.3-2a replaces Table 7.3.4.1.3.3-4, Table 9.3.4.1.3.3-3 replaces Table 7.3.4.1.3.3-6, Table 9.3.4.1.3.3-4 replaces Table 7.3.4.1.3.3-7, Table 9.3.4.1.3.3-2b replaces Table 7.3.4.1.3.3-5, Table 9.3.4.1.3.3-5 replaces Table 7.3.4.1.3.3-7a and Table 9.3.4.1.3.3-6 replaces Table 7.3.4.1.3.3-8.

Table 9.3.4.1.3.3-1: DLInformationTransfer (steps 1a, 1c, 2, 3 and 4a, Table 7.3.4.1.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.4.1.3.3-2 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.1.3.3-2: DL NAS TRANSPORT (steps 1a, 1c, 2, 3 and 4a, Table 7.3.4.1.3.2-1)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1a: Set according to Table 8.4-2 | LPP Request Capabilities. |  |
|  |  | Step 2: Set according to Table 9.3.4.1.3.3-2a | LPP Provide Assistance Data |  |
|  |  | Step 3: Set according to Table 9.3.4.1.3.3-2b | LPP Request Location Information |  |
|  |  | Steps 1c and 4a: Set according to Table 7.3.4.1.3.3-9 | LPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.3.4.1.3.3-2a: LPP Provide Assistance data (step 2, Table 7.3.4.1.3.2-1)

| Derivation Path: Table 8.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 8.4-1 with the following exceptions: |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) |  |  |
| } |  |  |  |  |

Table 9.3.4.1.3.3-2b: LPP Request Location Information (step 3, Table 7.3.4.1.3.2-1)

| Derivation Path: Table 8.4-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 8.4-3 with the following exceptions: |  |  |  |  |
| locationInformationType |  | locationEstimateRequired |  |  |

Table 9.3.4.1.3.3-3: ULInformationTransfer (steps 1b and 4, Table 7.3.4.1.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.4.1.3.3-4 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.1.3.3-4: UL NAS TRANSPORT (steps 1b and 4, Table 7.3.4.1.3.2-1)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1b: Set according to Table 9.3.4.1.3.3-5 | LPP Provide Capabilities |  |
|  |  | Step 4: Set according to Table 9.3.4.1.3.3-6 | LPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1 Table 7.3.4.1.3.2-1) |  |

Table 9.3.4.1.3.3-5: LPP Provide Capabilities. (step 1b, Table 7.3.4.1.3.2-1)

| Derivation Path: Table 7.3.4.1.3.3-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.1.3.3-7a with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.1.3.3-6: LPP Provide Location Information (step 4, Table 7.3.4.1.3.2-1)

| Derivation Path: Table 7.3.4.1.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.1.3.3-8 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { |  | Present for sub-test 20 | Rel-16 onwards |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-DL-AoD-Error-r16 |  | Not present |  |  |
| } |  |  |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  | Present for sub-test 21 | Rel-16 onwards |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-DL-TDOA-Error-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 9.3.4.2	E-SMLC Initiated Assistance Data Delivery followed by Location Information Transfer: UE-Assisted

Editor's note: Test configuration D is incomplete:

- The corresponding attach procedure for NG-RAN E-UTRA has not yet been defined.

- The message contents need to be revised for Test Configuration D.

9.3.4.2.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {
  when { UE receives assistance data and a location request for UE-assisted  }

then { UE sends a PROVIDE LOCATION INFORMATION  message containing location measurements }

}

9.3.4.2.2	Conformance requirements

As defined in clause 7.3.4.2.2.

9.3.4.2.3	Test description

9.3.4.2.3.1	Pre-test conditions

System Simulator:

For Test Configuration B (Table 9.3.4.2.3.2-1):

-	Sub-tests 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 22: NR Cell 1.

-	Sub-tests 5 and 7: not applicable.

-	Sub-test 6: not applicable.

-	Satellite signals (Sub-test 15): as specified in 8.2.1.

-	WLAN signals (Sub-tests 11, 17): as specified in 8.2.5.

-	MBS signals (Sub-tests 12, 16): as specified in 8.2.4

-	Bluetooth signals (Sub-test 13): as specified in 8.2.6.

-	Sub-test 21: NR Cell 1 and NR Cell 2, as specified in 8.2.11.

For Test Configuration D (Table 9.3.4.2.3.2-1):

-	Sub-tests 11, 12, 13, 14, 15, 16, 17, 18: LTE Cell 1.

-	Sub-tests 5 and 7: LTE Cell 1 and LTE Cell 2, as specified in 8.2.2.

-	Sub-test 6: LTE Cell 1 and LTE Cell 2, as specified in 8.2.3.

-	Satellite signals (Sub-test 15): as specified in 8.2.1.

-	WLAN signals (Sub-tests 11, 17): as specified in 8.2.5.

-	MBS signals (Sub-tests 12, 16): as specified in 8.2.4

-	Bluetooth signals (Sub-test 13): as specified in 8.2.6.

UE:

The UE shall begin the test with no assistance data stored.

Preamble:

-	For Test Configuration B (Table 9.3.4.2.3.2-1): The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	Sub-test 19: After the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.4.2.3.1-1 for the configuration of measurement gaps for Multi-RTT and then the SS shall execute the steps in Table 9.3.4.2.3.1-2 for the configuration of UL-SRS for Multi-RTT.

-	Sub-test 20: After the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.4.2.3.1-1 for the configuration of measurement gaps for DL-AoD.

-	Sub-test 21 After the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.4.2.3.1-1 for the configuration of measurement gaps for DL-TDOA.

Table 9.3.4.2.3.1-1: Configuration of measurement gaps

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an RRCReconfiguration message as in Table 8.3.1-1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |

Table 9.3.4.2.3.1-2: Configuration of UL-SRS for Multi-RTT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an RRCReconfiguration message as in Table 8.3.2-1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |

-	For Test Configuration D (Table 9.3.4.2.3.2-1): FFS.

Related PICS/PIXIT Statements:

-

9.3.4.2.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 9.3.4.2.3.2-0 below:

Table 9.3.4.2.3.2-0: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 5 | UE supporting OTDOA (LTE) (Rel-15 onwards) (Test Configuration D only) |
| 6 | UE supporting ECID (LTE) (Test Configuration D only) |
| 7 | UE supporting GNSS(1) and OTDOA (LTE) (Rel-15 onwards) (Test Configuration D only) |
| 11 | UE supporting WLAN (Rel-13 only) |
| 12 | UE supporting MBS (Rel-13 only) |
| 13 | UE supporting Bluetooth |
| 14 | UE supporting Sensor (Rel-13 only) |
| 15 | UE supporting GNSS(1) |
| 16 | UE supporting MBS (Rel-14 onwards) |
| 17 | UE supporting WLAN (Rel-14 onwards) |
| 18 | UE supporting Sensor (Rel-14 onwards) |
| 19 | UE supporting Multi-RTT (Rel-16 onwards) |
| 20 | UE supporting DL-AoD (Rel-16 onwards) |
| 21 | UE supporting DL-TDOA (Rel-16 onwards) |
| 22 | UE supporting NR E-CID (Rel-16 onwards) |
| NOTE 1: The GNSS combination of GPS, GLONASS, Galileo, BDS supported by the UE |  |

Table 9.3.4.2.3.2-1: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| A | EN-DC | Functionality is tested by test case 7.3.4.2 |
| B | NG-RAN NR |  |
| C | NE-DC | Functionality is tested by test configuration B |
| D | NG-RAN E-UTRA |  |
| E | NGEN-DC | Functionality is tested by test configuration D |

Main behaviour as defined in clause 7.3.4.2.3.2.

For sub-test 22 NR E-CID, the SS does not send LPP message of type Provide Assistance Data at step 2.

9.3.4.2.3.3	Specific message contents

As defined in clause 7.3.4.2.3.3, with the following exceptions:

Table 9.3.4.2.3.3-1 replaces Table 7.3.4.2.3.3-3, Table 9.3.4.2.3.3-2 replaces Table 7.3.4.2.3.3-4, Table 9.3.4.2.3.3-2a replaces Table 7.3.4.2.3.3-5, Table 9.3.4.2.3.3-3 replaces Table 7.3.4.2.3.3-7, Table 9.3.4.2.3.3-4 replaces Table 7.3.4.2.3.3-8, Table 9.3.4.2.3.3-5 replaces Table 7.3.4.2.3.3-6, Table 9.3.4.2.3.3-6 replaces Table 7.3.4.2.3.3-8a and Table 9.3.4.2.3.3-7 replaces Table 7.3.4.2.3.3-9.

Table 9.3.4.2.3.3-1: DLInformationTransfer (steps 1b, 1d, 2, 3, 4a2, 4b2 and 4b4, Table 7.3.4.2.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.4.2.3.3-2 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.2.3.3-2: DL NAS TRANSPORT (steps 1b, 1d, 2, 3, 4a2, 4b2 and 4b4, Table 7.3.4.2.3.2-1)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1b: Set according to Table 8.4-2 | LPP Request Capabilities. |  |
|  |  | Step 2: Set according to Table 9.3.4.2.3.3-2a | LPP Provide Assistance Data |  |
|  |  | Step 3: Set according to Table9.3.4.2.3.3-5 | LPP Request Location Information |  |
|  |  | Steps 1d, 4a2, 4b2 and 4b4: Set according to Table 7.3.4.2.3.3-10 | LPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.3.4.2.3.3-2a: LPP Provide Assistance data (step 2, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 8.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 8.4-1 with the following exceptions: |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) |  |  |
| } |  |  |  |  |

Table 9.3.4.2.3.3-3: ULInformationTransfer (steps 1c, 4 a1, 4b1 and 4b3, Table 7.3.4.2.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.4.2.3.3-4 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.2.3.3-4: UL NAS TRANSPORT (steps 1c, 4 a1, 4b1 and 4b3, Table 7.3.4.2.3.2-1)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1c: Set according to Table 9.3.4.2.3.3-6 | LPP Provide Capabilities |  |
|  |  | Steps 4 a1, 4b1 and 4b3: Set according to Table 9.3.4.2.3.3-7 | LPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1 Table 7.3.4.2.3.2-1) |  |

Table 9.3.4.2.3.3-5: LPP Request Location Information (step 3, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 8.4-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 8.4-3 with the following exceptions: |  |  |  |  |
| ecid-RequestLocationInformation |  |  | Present |  |
| requestedMeasurements |  | Test Configuration D: bits 0, 1, 2 = 1 |  |  |

Table 9.3.4.2.3.3-6: LPP Provide Capabilities. (step 1c, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 7.3.4.2.3.3-8a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.2.3.3-8a with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.2.3.3-7: LPP Provide Location Information (steps 4 a1, 4b1 and 4b3, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 7.3.4.2.3.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.2.3.3-9 with the following exceptions: |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| nr-ECID-ProvideLocationInformation-r16 SEQUENCE { | Present for sub-test 22 | Rel-16 onwards |  |
| nr-ECID-SignalMeasurementInformation-r16 | Present. Any value acceptable |  |  |
| nr-ECID-Error-r16 | May be present |  |  |
| } |  |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 SEQUENCE { | Present for sub-test 19 | Rel-16 onwards |  |
| nr-Multi-RTT-SignalMeasurementInformation-r16 | Present. Any value acceptable |  |  |
| nr-Multi-RTT-Error-r16 | May be present |  |  |
| } |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { | Present for sub-test 20 | Rel-16 onwards |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 | Present. Any value acceptable |  |  |
| nr-DL-AoD-Error-r16 | May be present |  |  |
| } |  |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { | Present for sub-test 21 | Rel-16 onwards |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 | Present. Any value acceptable |  |  |
| nr-DL-TDOA-Error-r16 | May be present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 9.3.4.3	E-SMLC Initiated Position Measurement without assistance data: UE-Based

Editor's note: Test configuration D is incomplete:

- The corresponding attach procedure for NG-RAN E-UTRA has not yet been defined.

- The message contents need to be revised for Test Configuration D.

9.3.4.3.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { UE has no assistance data stored and receives a location request for UE-based and the UE

requires assistance data in order to fulfil the location request }

then {    UE sends a REQUEST ASSISTANCE DATA message followed by a PROVIDE LOCATION INFORMATION

message containing a location estimate }

}

9.3.4.3.2	Conformance requirements

As defined in clause 7.3.4.3.2.

9.3.4.3.3	Test description

9.3.4.3.3.1	Pre-test conditions

System Simulator:

For Test Configuration B (Table 9.3.4.3.3.2-1): NR Cell 1.

-	Satellite signals (sub-test 15): as specified in 8.2.1.

-	MBS signals (sub-test 16): as specified in 8.2.4.

-	WLAN signals (sub-test 17): as specified in 8.2.5.

-	Sub-test 20: NR Cell 1 and NR Cell 2, as specified in 8.2.10.

-	Sub-test 21: NR Cell 1, NR Cell 2, NR Cell 3 and NR Cell 4, as specified in 8.2.11.

For Test Configuration D (Table 9.3.4.3.3.2-1): LTE Cell 1.

-	Satellite signals (sub-test 15): as specified in 8.2.1.

-	MBS signals (sub-test 16): as specified in 8.2.4.

-	WLAN signals (sub-test 17): as specified in 8.2.5.

UE:

The UE shall begin the test with no assistance data stored.

Preamble:

-	For Test Configuration B (Table 9.3.4.3.3.2-1): The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	For Test Configuration D (Table 9.3.4.3.3.2-1): FFS.

Related PICS/PIXIT Statements:

Method of triggering an LPP Request Assistance Data message.

9.3.4.3.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 9.3.4.3.3.2-0 below:

Table 9.3.4.3.3.2-0: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 15 | UE supporting GNSS(1) |
| 16 | UE supporting MBS (Rel-14 onwards) |
| 17 | UE supporting WLAN (Rel-14 onwards) |
| 18 | UE supporting Sensor (Rel-14 onwards) |
| 20 | UE supporting DL-AoD (Rel-16 onwards) |
| 21 | UE supporting DL-TDOA (Rel-16 onwards) |
| NOTE 1: The GNSS combination of GPS, GLONASS, Galileo, BDS supported by the UE |  |

Table 9.3.4.3.3.2-1: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| A | EN-DC | Functionality is tested by test case 7.3.4.3 |
| B | NG-RAN NR |  |
| C | NE-DC | Functionality is tested by test configuration B |
| D | NG-RAN E-UTRA |  |
| E | NGEN-DC | Functionality is tested by test configuration D |

Main behaviour as defined in Table 7.3.4.3.3.2-1.

9.3.4.3.3.3	Specific message contents

As defined in clause 7.3.4.3.3.3, with the following exceptions:

Table 9.3.4.3.3.3-1 replaces Table 7.3.4.3.3.3-2, Table 9.3.4.3.3.3-2 replaces Table 7.3.4.3.3.3-3, Table 9.3.4.3.3.3-2a replaces Table 7.3.4.3.3.3-9, Table 9.3.4.3.3.3-3 replaces Table 7.3.4.3.3.3-6, Table 9.3.4.3.3.3-4 replaces Table 7.3.4.3.3.3-7, Table 9.3.4.3.3.3-5 replaces Table 7.3.4.3.3.3-7a, Table 9.3.4.3.3.3-6 replaces Table 7.3.4.3.3.3-8 and Table 9.3.4.3.3.3-7 replaces Table 7.3.4.3.3.3-10.

Table 9.3.4.3.3.3-1: DLInformationTransfer (steps 1a, 1c, 2, 4 and 5a, Table 7.3.4.3.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.4.3.3.3-2 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.3.3.3-2: DL NAS TRANSPORT (steps 1a, 1c, 2, 4 and 5a, Table 7.3.4.3.3.2-1)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1a: Set according to Table 8.4-2 | LPP Request Capabilities. |  |
|  |  | Step 2: Set according to Table 9.3.4.3.3.3-2b | LPP Request Location Information |  |
|  |  | Step 4: Set according to Table 9.3.4.3.3.3-2a | LPP Provide Assistance Data |  |
|  |  | Steps 1c and 5a: Set according to Table 7.3.4.3.3.3-11 | LPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.3.4.3.3.3-2a: LPP Provide Assistance Data (step 4, Table 7.3.4.3.3.2-1)

| Derivation Path: Table 8.4-1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |  |  |  |
| initiator |  | targetDevice |  |  |  |  |  |
| transactionNumber |  | (0..255) |  | Contains the same value as the corresponding field in the LPP Request Assistance Data message in step 3 Table 7.3.4.3.3.2-1 |  |  |  |
| } |  |  |  |  |  |  |  |
| endTransaction |  | TRUE |  |  |  |  |  |
| sequenceNumber |  | Not present |  |  |  |  |  |
| acknowledgement SEQUENCE { |  | Present if acknowledgement field is included by the UE at step 3, Table 7.3.4.3.3.2-1. |  |  |  |  |  |
| ackRequested |  | FALSE |  |  |  |  |  |
| ackIndicator |  | (0..255) |  | Contains the same value as the sequenceNumber in step 3, Table 7.3.4.3.3.2-1 |  |  |  |
| } |  |  |  |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |  |  |  |  |
| a-gnss-ProvideAssistanceData |  | The SS provides the assistance data requested by the UE at step 3, Table 7.3.4.3.3.2-1 which are available according to TS 37.571-5 [12]. |  |  |  |  |  |
| sensor-ProvideAssistanceData-r14 |  | The SS provides the assistance data requested by the UE at step 3, Table 7.3.4.3.3.2-1 which are available according to subclause 8.4.1.5. |  | Rel-14 onwards |  |  |  |
| tbs-ProvideAssistanceData-r14 |  | The SS provides the assistance data requested by the UE at step 3, Table 7.3.4.3.3.2-1 which are available according to subclause 8.4.1.3. |  | Rel-14 onwards |  |  |  |
| wlan-ProvideAssistanceData-r14 |  | The SS provides the assistance data requested by the UE at step 3, Table 7.3.4.3.3.2-1 which are available according to subclause 8.4.1.4. |  | Rel-14 onwards |  |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 |  | The SS provides the assistance data requested by the UE at step 3, Table 7.3.4.3.3.2-1 which are available according to subclause 8.4.1.7. |  | Rel-16 onwards |  |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 |  | The SS provides the assistance data requested by the UE at step 3, Table 7.3.4.3.3.2-1 which are available according to subclause 8.4.1.8. |  | Rel-16 onwards |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 9.3.4.3.3.3-2b: LPP Request Location Information (step 2, Table 7.3.4.3.3.2-1)

| Derivation Path: Table 5.4-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 5.4-3 with the following exceptions: |  |  |  |  |
| locationInformationType |  | locationEstimateRequired |  |  |
| a-gnss-RequestLocationInformation |  | Set according to Table 7.3.4.3.3.3-5 |  | Sub-test 15 |
| sensor-RequestLocationInformation-r14 |  | Set according to Table 7.3.4.3.3.3-5B | Rel-14 onwards | Sub-test 18 |
| tbs-RequestLocationInformation-r13 |  | Set according to Table 7.3.4.3.3.3-5A | Rel-13 onwards | Sub-test 16 |
| wlan-RequestLocationInformation-r14 |  | Set according to Table 7.3.4.3.3.3-5C | Rel-14 onwards | Sub-test 17 |
| nr-DL-AoD-RequestLocationInformation-r16 |  | As defined in Table 9.3.4.3.3.3-2c | Rel-16 onwards | Sub-test 20 |
| nr-DL-TDOA-RequestLocationInformation-r16 |  | As defined in Table 9.3.4.3.3.3-2d | Rel-16 onwards | Sub-test 21 |

Table 9.3.4.3.3.3-2c: NR DL-AoD Request Location Information (step 2, Table 7.3.4.3.3.2-1)

| Derivation Path: Table 8.4-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.4-6 with the following exceptions: |  |  |  |
| nr-AssistanceAvailability-r16 | TRUE |  |  |

Table 9.3.4.3.3.3-2d: NR DL-TDOA Request Location Information (step 2, Table 7.3.4.3.3.2-1)

| Derivation Path: Table 8.4-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.4-7 with the following exceptions: |  |  |  |
| nr-AssistanceAvailability-r16 | TRUE |  |  |

Table 9.3.4.3.3.3-3: ULInformationTransfer (steps 1b, 3 and 5, Table 7.3.4.3.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.3.4.3.3.3-4 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.3.3.3-4: UL NAS TRANSPORT (steps 1b, 3 and 5, Table 7.3.4.3.3.2-1)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1b: Set according to Table 9.3.4.3.3.3-5 | LPP Provide Capabilities |  |
|  |  | Step 3: Set according to Table 9.3.4.3.3.3-6 | LPP Request Assistance Data |  |
|  |  | Step 5: Set according to Table 9.3.4.3.3.3-7 | LPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1 Table 7.3.4.3.3.2-1) |  |

Table 9.3.4.3.3.3-5: LPP Provide Capabilities. (step 1b, Table 7.3.4.3.3.2-1)

| Derivation Path: Table 7.3.4.3.3.3-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.3.3.3-7a with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in the LPP Request Capabilities message in step 1a, Table 7.3.4.3.3.2-1 |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| nr-DL-AoD-RequestCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-RequestCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.3.3.3-6: LPP Request Assistance Data (step 3, Table 7.3.4.3.3.2-1)

| Derivation Path: Table 7.3.4.3.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.3.3.3-8 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| requestAssistanceData-r9 SEQUENCE { |  |  |  |  |
| nr-DL-AoD-RequestAssistanceData-r16 |  | Present for sub-test 20 | Rel-16 onwards |  |
| nr-DL-TDOA-RequestAssistanceData-r16 |  | Present for sub-test 21 | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.3.4.3.3.3-7: LPP Provide Location Information (step 5, Table 7.3.4.3.3.2-1)

| Derivation Path: Table 7.3.4.3.3.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.3.3.3-10 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 |  | Present for sub-test 20 | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 |  | Present for sub-test 21 | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 9.3.4.4	E-SMLC Initiated Position Measurement without assistance data: UE-Assisted

Editor's note: Test configuration D is incomplete:

- The corresponding attach procedure for NG-RAN E-UTRA has not yet been defined.

- The message contents need to be revised for Test Configuration D.

9.3.4.4.1	Test Purpose (TP)

with { a NAS signalling connection existing }

ensure that {
  when { 	UE has no assistance data stored and receives a location request for UE-assisted and the
 			UE requires assistance data in order to fulfil the location request }

then { 	UE sends a REQUEST ASSISTANCE DATA message followed by a PROVIDE LOCATION INFORMATION
 			message containing location measurements }

}

9.3.4.4.2	Conformance requirements

As defined in clause 7.3.4.4.2.

9.3.4.4.3	Test description

9.3.4.4.3.1	Pre-test conditions

System Simulator:

For Test Configuration B (Table 9.3.4.4.3.2-1):

-	Sub-tests 15, 16, 17, 18, 19, 20: NR Cell 1.

-	Sub-tests 5 and 7: not applicable.

-	Satellite signals (sub-test 15): as specified in 8.2.1.

-	MBS signals (sub-test 16): as specified in 8.2.4.

-	WLAN signals (sub-test 17): as specified in 8.2.5.

-	Sub-test 21: NR Cell 1 and NR Cell 2, as specified in 8.2.11.

For Test Configuration D (Table 9.3.4.4.3.2-1):

-	Sub-tests 15, 16, 17, 18: LTE Cell 1.

-	Sub-tests 5 and 7: LTE Cell 1 and LTE Cell 2, as specified in 8.2.2.

-	Satellite signals (sub-test 15): as specified in 8.2.1.

-	MBS signals (sub-test 16): as specified in 8.2.4.

-	WLAN signals (sub-test 17): as specified in 8.2.5.

UE:

The UE shall begin the test with no assistance data stored.

Preamble:

-	For Test Configuration B (Table 9.3.4.4.3.2-1): The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	Sub-test 19: After the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.4.4.3.1-1 for the configuration of measurement gaps for Muti-RTT and then the SS shall execute the steps in Table 9.3.4.4.3.1-2 for the configuration of UL-SRS for Muti-RTT.

-	Sub-test 20: After the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.4.4.3.1-1 for the configuration of measurement gaps for DL-AoD.

-	Sub-test 21: After the UE is in state 3N-A, the SS shall execute the steps in Table 9.3.4.4.3.1-1 for the configuration of measurement gaps for DL-TDOA.

Table 9.3.4.4.3.1-1: Configuration of measurement gaps

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an RRCReconfiguration message as in Table 8.3.1-1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |

Table 9.3.4.4.3.1-2: Configuration of UL-SRS for Multi-RTT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an RRCReconfiguration message as in Table 8.3.2-1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |

-	For Test Configuration D (Table 9.3.4.4.3.2-1): FFS.

Related PICS/PIXIT Statements:

Method of triggering an LPP Request Assistance Data message.

9.3.4.4.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 9.3.4.4.3.2-0 below:

Table 9.3.4.4.3.2-0: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 5 | UE supporting OTDOA (LTE) (Rel-15 onwards) (Test Configuration D only) |
| 7 | UE supporting GNSS(1) and OTDOA (LTE) (Rel-15 onwards) (Test Configuration D only) |
| 15 | UE supporting GNSS(1) |
| 16 | UE supporting MBS (Rel-14 onwards) |
| 17 | UE supporting WLAN (Rel-14 onwards) |
| 18 | UE supporting Sensor (Rel-14 onwards) |
| 19 | UE supporting Multi-RTT (Rel-16 onwards) |
| 20 | UE supporting DL-AoD (Rel-16 onwards) |
| 21 | UE supporting DL-TDOA (Rel-16 onwards) |
| NOTE 1: The GNSS combination of GPS, GLONASS, Galileo, BDS supported by the UE |  |

Table 9.3.4.4.3.2-1: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| A | EN-DC | Functionality is tested by test case 7.3.4.4 |
| B | NG-RAN NR |  |
| C | NE-DC | Functionality is tested by test configuration B |
| D | NG-RAN E-UTRA |  |
| E | NGEN-DC | Functionality is tested by test configuration D |

Main behaviour as defined in Table 7.3.4.4.3.2-1.