3GPP TSG-RAN5 Meeting #92-e	R5-214277

# Electronic Meeting, 16 - 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0463 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of spurious emission TP analysis for Rel-15 EN-DC configuration DC_66A_n5A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test point analysis for EN-DC configuration DC_66A_n5A needs update for: - Test cases 6.5B.3.3.1 (General spurious emissions): - Current TP analysis has for E-UTRA band 66 used DL frequencies (2110 to 2200 MHz) instead of UL frequencies (1710-1780 MHz). The TP analysis need to be updated using the correct UL frequencies. - According to the minimum requirements defined in 38.101-3 clause 6.5B.3.3.1, The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band. Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur the general spurious emission for inter-band EN-DC can be verified at the specific frequency where 2nd and 3rd order intermodulation products may occur. Therefore, the test requirements need to be updated for the purpose of reducing test time. - Test case 6.5B.3.3.2 (Spurious emission band UE co-existence): - Current TP analysis has for E-UTRA band 66 used DL frequencies (2110 to 2200 MHz) instead of UL frequencies (1710-1780 MHz). The TP analysis need to be updated using the correct UL frequencies. - Minimum requirement need to be updated. - No test requirement is shown in the TP analysis reflecting the list of protected bands where IM products may occur. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test cases 6.5B.3.3.1 (General spurious emissions): - TP analysis updated using E-UTRA band 66 UL frequency range. - Test requirement updated limiting to 2nd and 3rd order intermodulation products. - Test case 6.5B.3.3.2 (Spurious emission band UE co-existence): - TP analysis updated using E-UTRA band 66 UL frequency range. - Minimum requirement updated and TP analysis updated accordingly. - Adding section 7 with test requirement showing the impacted bands (the table that will be introduced in TS 38.521-3 for the test case). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Incorrect test requirments for test cases 6.5B.3.3.1 remains. - Test points and test requirement for test case 6.5B.3.3.2 not correctly reflecting the impacted protected bands for DC_66A_n5A. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.521-3 CR 1064 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Note! Attached zip-file need to be added replacing the current zip-file for DC_66A_n5A. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 4.3.3.2	Spurious emissions test cases for EN-DC

In this case, it is sufficient to verify the minimum requirements in frequency ranges affected by 2nd and 3rd order intermodulation products. The frequency ranges and UL RB allocations used in the test are calculated here.

The analyses are performed per EN-DC configuration and are stored as zip-files as defined in annex A.

Table 4.3.3.2-1: Frequency range analysis availability per EN-DC configuration

| EN-DC config | Justification | Comments |
| --- | --- | --- |
| DC_1A_n3A | 38.521-3_TpAnalysisSpur(DC_1A-n3A).zip | Added at RAN5#89-e |
| DC_1A_n78A | 38.521-3_TpAnalysisSpur(DC_1A_n78A).zip | Added at RAN5#88-e |
| DC_2A_n5A | 38.521-3_TpAnalysisSpur(DC_2A_n5A).zip | Added at RAN5#89-e |
| DC_2A_n41A | 38.521-3_TpAnalysisSpur(DC_2A_n41A).zip | Added at RAN5#90-e |
| DC_2A_n66A | 38.521-3_TpAnalysisSpur(DC_2A_n66A).zip | Added at RAN5#88-e |
| DC_2A_n71A | 38.521-3_TpAnalysisSpur(DC_2A_n71A).zip | Added at RAN5#91-e |
| DC_2A_n78A | 38.521-3_TpAnalysisSpur(DC_2A_n78A).zip | Added at RAN5#88-e |
| DC_3A_n1A | 38.521-3_TpAnalysisSpur(DC_3A_n1A).zip | Added at RAN5#88-e |
| DC_3A_n7A | 38.521-3_TpAnalysisSpur(DC_3A_n7A)_v1.zip | Added at RAN5#91-e |
| DC_3A_n41A | 38.521-3_TpAnalysisSpur(DC_3A-n41A)_v1.zip | Added at RAN5#86-e |
| DC_3A_n78A | 38.521-3_TpAnalysisSpur(DC_3A_n78A)_v1.zip | Added at RAN5#91-e |
| DC_3A_n79A | 38.521-3_TpAnalysisSpur(DC_3A-n79A).zip | Added at RAN5#83 |
| DC_5A_n2A | 38.521-3_TpAnalysisSpur(DC_5A_n2A).zip | Added at RAN5#90-e |
| DC_5A_n66A | 38.521-3_TpAnalysisSpur(DC_5A_n66A)_v1.zip | Added at RAN5#88-e |
| DC_5A_n78A | 38.521-3_TpAnalysisSpur(DC_5A_n78A)_v1.zip | Added at RAN5#88-e |
| DC_7A_n1A | 38.521-3_TpAnalysisSpur(DC_7A_n1A).zip | Added at RAN5#88-e |
| DC_7A_n3A | 38.521-3_TpAnalysisSpur(DC_7A_n3A).zip | Added at RAN5#90-e |
| DC_7A_n66A | 38.521-3_TpAnalysisSpur(DC_7A_n66A).zip | Added at RAN5#88-e |
| DC_7A_n78A | 38.521-3_TpAnalysisSpur(DC_7A_n78A).zip | Added at RAN5#88-e |
| DC_8A_n1A | 38.521-3_TpAnalysisSpur(DC_8A_n1A).zip | Added at RAN5#88-e |
| DC_8A_n3A | 38.521-3_TpAnalysisSpur(DC_8A_n3A).zip | Added at RAN5#90-e |
| DC_8A_n41A | 38.521-3_TpAnalysisSpur(DC_8A-n41A)_v1.zip | Added at RAN5#86-e |
| DC_8A_n77A | 38.521-3_TpAnalysisSpur(DC_8A_n77A)_v1.zip | Added at RAN5#91-e |
| DC_8A_n78A | 38.521-3_TpAnalysisSpur(DC_8A_n78A).zip | Added at RAN5#89-e |
| DC_11A_n77A | 38.521-3_TpAnalysisSpur(DC_11A_n77A).zip | Added at RAN5#90-e |
| DC_11A_n78A | 38.521-3_TpAnalysisSpur(DC_11A_n78A).zip | Added at RAN5#90-e |
| DC_11A_n79A | 38.521-3_TpAnalysisSpur(DC_11A_n79A).zip | Added at RAN5#90-e |
| DC_12A_n66A | 38.521-3_TpAnalysisSpur(DC_12A_n66A)_v1.zip | Added at RAN5#91-e |
| DC_12A_n78A | 38.521-3_TpAnalysisSpur(DC_12A_n78A).zip | Added at RAN5#88-e |
| DC_13A_n2A | 38.521-3_TpAnalysisSpur(DC_13A_n2A).zip | Added at RAN5#90-e |
| DC_13A_n66A | 38.521-3_TpAnalysisSpur(DC_13A_n66A).zip | Added at RAN5#89-e |
| DC_14A_n2A | 38.521-3_TpAnalysisSpur(DC_14A_n2A).zip | Added at RAN5#91-e |
| DC_14A_n66A | 38.521-3_TpAnalysisSpur(DC_14A_n66A).zip | Added at RAN5#91-e |
| DC_20A_n1A | 38.521-3_TpAnalysisSpur(DC_20A-n1A).zip | Added at RAN5#90-e |
| DC_20A_n3A | 38.521-3_TpAnalysisSpur(DC_20A-n3A).zip | Added at RAN5#89-e |
| DC_25A_n41A | 38.521-3_TpAnalysisSpur(DC_25A_n41A).zip | Added at RAN5#90-e |
| DC_26A_n41A | 38.521-3_TpAnalysisSpur(DC_26A_n41A)_v1.zip | Added at RAN5#91-e |
| DC_26A_n77A | 38.521-3_TpAnalysisSpur(DC_26A_n77A).zip | Added at RAN5#90-e |
| DC_26A_n78A | 38.521-3_TpAnalysisSpur(DC_26A_n78A).zip | Added at RAN5#90-e |
| DC_26A_n79A | 38.521-3_TpAnalysisSpur(DC_26A_n79A)_v1.zip | Added at RAN5#91-e |
| DC_28A_n3A | 38.521-3_TpAnalysisSpur(DC_28A_n3A).zip | Added at RAN5#88-e |
| DC_30A_n5A | 38.521-3_TpAnalysisSpur(DC_30A_n5A)_v1.zip | Added at RAN5#91-e |
| DC_39A_n41A | 38.521-3_TpAnalysisSpur(DC_39A-n41A).zip | Added at RAN5#83 |
| DC_39A_n79A | 38.521-3_TpAnalysisSpur(DC_39A-n79A).zip | Added at RAN5#83 |
| DC_40A_n1A | 38.521-3_TpAnalysisSpur(DC_40A_n1A).zip | Added at RAN5#88-e |
| DC_40A_n41A | 38.521-3_TpAnalysisSpur(DC_40A-n41A).zip | Added at RAN5#83 |
| DC_40A_n78A | 38.521-3_TpAnalysisSpur(DC_40A_n78A).zip | Added at RAN5#88-e |
| DC_41A_n77A | 38.521-3_TpAnalysisSpur(DC_41A_n77A).zip | Added at RAN5#90-e |
| DC_41A_n78A | 38.521-3_TpAnalysisSpur(DC_41A_n78A).zip | Added at RAN5#90-e |
| DC_41A_n79A | 38.521-3_TpAnalysisSpur(DC_41A-n79A).zip | Added at RAN5#83 |
| DC_48A_n5A | 38.521-3_TpAnalysisSpur(DC_48A_n5A).zip | Added at RAN5#90-e |
| DC_48A_n66A | 38.521-3_TpAnalysisSpur(DC_48A_n66A).zip | Added at RAN5#90-e |
| DC_66A_n2A | 38.521-3_TpAnalysisSpur(DC_66A_n2A).zip | Added at RAN5#88-e |
| DC_66A_n5A | 38.521-3_TpAnalysisSpur(DC_66A-n5A)_v1.zip | Added at RAN5#92-e |
| DC_66A_n41A | 38.521-3_TpAnalysisSpur(DC_66A-n41A).zip | Added at RAN5#90-e |
| DC_66A_n71A | 38.521-3_TpAnalysisSpur(DC_66A-n71A).zip | Added at RAN5#91-e |
| DC_66A_n78A | 38.521-3_TpAnalysisSpur(DC_66A_n78A)_v1.zip | Added at RAN5#88-e |

<End of modified section>

## Replace current version of the zip-file for DC_5A_n66A attached to TR 38.905 with the zip file “38.521-3_TpAnalysisSpur(DC_66A_n5A)_v1.zip” attached to this CR.