3GPP TSG-RAN WG5 Meeting #92-e	R5-214331

# Electronic Meeting, 16th - 27th August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-2 | CR | 0964 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability for eFD-MIMO demod test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eFDMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The work plan for eFD-MIMO demod test cases with aperiodic CSI-RS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding applicability for new test cases: - 8.3.1.1.10 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eFD-MIMO - 8.3.2.1.11 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eFD-MIMO Adding required ICS table item |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work item cannot be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, A.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.1	RF conformance test cases

NOTE:	To determine applicability of a test case, FGI support in combined or fdd-Add-UE-EUTRA-Capabilities or tdd-Add-UE-EUTRA-Capabilities is taken into account.

Table 4.1-1: Applicability of RF conformance test cases, ref. TS 36.521-1 [1]

| Clause | Title | Release |  | Applicability |  | Tested Bands / CA-Configurations Selection |  | Branch |  | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Condition | Comments |  |  |  |  |  |
| {unchanged rows skipped} |  |  |  |  |  |  |  |  |  |  |
|  | Performance Requirement |  |  |  |  |  |  |  |  |  |
| 8.2.1.1.1 | FDD PDSCH Single Antenna Port Performance | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each ""Test Number"" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.1.1_A.1 or 8.2.1.1.1_A.2 is executed. Note 7 |
| 8.2.1.1.1_1 | FDD PDSCH Single Antenna Port Performance (Release 9 and forward) | Rel-9 |  | C31 | UE supporting E-UTRA FDD (UE categories 1, 2) |  | Each ""Test Number"" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.1.1.1_A.1 or 8.2.1.1.1_A.2 is executed. |
| 8.2.1.1.1_A.1 | FDD PDSCH Single Antenna Port Performance for CA (2 DL CA) | Rel-10 |  | C102 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 2) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.1.1_A.2 or 8.2.1.1.1_A.4 or 8.2.1.1.1_A.5 or 8.13.1.2.2 or 8.13.1.2.3 or 8.13.1.2.4 or 8.13.1.2.5 is executed. Note 7 |
|  |  | Rel-11 |  | C103 | UE supporting E-UTRA FDD and Downlink Intra-band non-contiguous CA (UE Category >= 2) |  |  |  |  |  |
| 8.2.1.1.1_A.2 | FDD PDSCH Single Antenna Port Performance for CA (3DL CA) | Rel-10 |  | C124 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.1.1_A.4 or 8.2.1.1.1_A.5 or 8.13.1.2.3 or 8.13.1.2.4 or 8.13.1.2.5 is executed. |
|  |  | Rel-11 |  | C125 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  |  | Note 7 |
| 8.2.1.1.1_A.4 | FDD PDSCH Single Antenna Port Performance for CA (4DL CA) | Rel-11 |  | C214 | UE supporting E-UTRA FDD and 4DL CA configurations in Table 4.1-4 (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.1.1_A.5 or 8.13.1.2.4 or 8.13.1.2.5 is executed. Note 7 |
| 8.2.1.1.1_A.5 | FDD PDSCH Single Antenna Port Performance for CA (5DL CA) | Rel-11 |  | C215 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.13.1.2.5 is executed. Note 7 |
|  |  | Rel-12 |  | C216 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  |  |  |
| 8.2.1.1.1_A.6 | FDD PDSCH Single Antenna Port Performance for CA (6DL CA) | Rel-14 |  | C349 | UE supporting E-UTRA FDD and 6DL with CA configurations in Table 4.1-6 (UE Category 8, >= 11) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.1.1.2 | FDD PDSCH Single Antenna Port Performance with 1 PRB in presence of MBSFN | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.2.1 | FDD PDSCH Transmit Diversity 2x2 | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.2.1_1 | FDD PDSCH Transmit Diversity 2x2 (Release 9 and forward) | Rel-9 |  | C15 | UE supporting E-UTRA FDD (UE category 1) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.2.2 | FDD PDSCH Transmit Diversity 4x2 | Rel-8 |  | C09 | UE supporting E-UTRA FDD and operating bands supporting 1,4 MHz Bandwidth |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.2.2_1 | FDD PDSCH Transmit Diversity 4x2 (Release 9 and forward) | Rel-9 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.2.3_C.1 | FDD PDSCH Transmit diversity 2x2 for eICIC (non-MBFSN ABS) | Rel-10 |  | C29 | UEs supporting E-UTRA FDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.2.3_E.1 | FDD PDSCH Transmit diversity 2x2 for feICIC (non-MBFSN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.2.4 | FDD PDSCH Transmit Diversity 2x2 with TM3 Interference Model -- Enhanced Performance Requirement Type A | Rel-11 |  | C44 | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.2.5 | FDD PDSCH Transmit Diversity 2x2 with TM2 Interference Model -- Enhanced Performance Requirement Type B | Rel-12 |  | C150 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.2.6 | FDD PDSCH Transmit Diversity 2x2 with TM9 Interference Model -- Enhanced Performance Requirement Type B | Rel-12 |  | C150 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.3.1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 | Rel-8 |  | C13b | UE supporting E-UTRA FDD (UE categories >=2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.1.3.1_A.1 or 8.2.1.3.1_A.2 is executed. |
| 8.2.1.3.1_1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 (Release 11 and forward) | Rel-11 |  | C13b | UE supporting E-UTRA FDD (UE categories >=2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.1.3.1_A.1 or 8.2.1.3.1_A.2 is executed. |
| 8.2.1.3.1_A.1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (2 DL CA) | Rel-10 |  | C101 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >=2) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | If 8.2.1.3.1_A.2 is executed for a CA capability, test execution is not necessary for that CA capability. |
|  |  | Rel-11 |  | C103 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 2) |  |  |  |  | Note 7 |
| 8.2.1.3.1_A.2 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) | Rel-10 |  | C124 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | If 8.2.1.3.1_A.3 is executed for a CA capability, test execution is not necessary for that CA capability. Note 7 |
|  |  | Rel-11 |  | C125 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  |  |  |
| 8.2.1.3.1_A.3 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) | Rel-11 |  | C214 | UE supporting E-UTRA FDD and 4DL CA configurations in Table 4.1-4 (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | If 8.2.1.3.1_A.4 is executed for a CA capability, test execution is not necessary for that CA capability. Note 7 |
| 8.2.1.3.1_A.4 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) | Rel-11 |  | C215 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | If 8.2.1.3.1_A.5 is executed for a CA capability, test execution is not necessary for that CA capability. Note 7 |
|  |  | Rel-12 |  | C216 | UE supporting E-UTRA FDD and 5DL CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  |  |  |
| 8.2.1.3.1_A.5 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (6DL CA) | Rel-14 |  | C349 | UE supporting E-UTRA FDD and 6DL CA configurations in Table 4.1-6 (UE Category 8, >= 11) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.1.3.1A_A.1 | FDD Soft buffer management test for CA (2 DL CA) | Rel-10 |  | C104 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE category 3 and 4) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-11 |  | C106 | UE supporting E-UTRA FDD and Downlink Intra-band non-contiguous CA (UE categories 3 and 4) |  |  |  |  |  |
| 8.2.1.3.1B | FDD PDSCH Open Loop Spatial Multiplexing 2x2 -- Enhanced Performance Requirement Type C | Rel-12 |  | C142 | UE supporting E-UTRA FDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  | Each ""Test Number"" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.3.1C | FDD PDSCH Open Loop Spatial Multiplexing 2x2 with TM1 Interference -- Enhanced Performance Requirement Type C | Rel-12 |  | C142 | UE supporting E-UTRA FDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  | Each ""Test Number"" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.3.2 | FDD PDSCH Open Loop Spatial Multiplexing 4x2 | Rel-8 |  | C13 b | UE supporting E-UTRA FDD (UE categories >=2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.3.3_C.1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for eICIC (non-MBSFN ABS) | Rel-10 |  | C29 | UEs supporting E-UTRA FDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.3.3_C.2 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for eICIC (MBSFN ABS) | Rel-10 |  | C29 | UEs supporting E-UTRA FDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.3.3_E.1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.4.1 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 | Rel-8 only |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.4.1_1 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 (Release 9 and forward) | Rel-9 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.4.1_E.1 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.4.1_H | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for 256QAM in DL | Rel-12 |  | C01h | UE supporting E-UTRA FDD and 256QAM in DL |  |  |  |  |  |
| 8.2.1.4.2 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 4x2 | Rel-8 only |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.1.4.2_A.1 or 8.2.1.4.2_A.2 is executed. |
| 8.2.1.4.2_1 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 4x2 (Release 9 and forward) | Rel-9 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.1.4.2_A.1 or 8.2.1.4.2_A.2 is executed. |
| 8.2.1.4.2_A.1 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (2 DL CA) | Rel-10 |  | C102 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 2) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.4.2_A.2 or 8.2.1.4.2_A.3 or 8.2.1.4.2_A.4 or 8.13.1.1.1.2 or 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |
|  |  | Rel-11 |  | C103 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 2) |  |  |  |  | Note 7 |
| 8.2.1.4.2_A.2 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) | Rel-10 |  | C124 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.4.2_A.3 or 8.2.1.4.2_A.4 or 8.13.1.1.1.2 or 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. Note 7 |
|  |  | Rel-11 |  | C125 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  |  |  |
| 8.2.1.4.2_A.3 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (4DL CA) | Rel-11 |  | C212 | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.4.2_A.4 or 8.13.1.1.1.2 or 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. Note 7 |
| 8.2.1.4.2_A.4 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (5DL CA) | Rel-11 |  | C212a | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.13.1.1.1.2 or 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. Note 7 |
|  |  | Rel-12 |  | C212b | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  |  |  |
| 8.2.1.4.2_A.5 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (6DL CA) | Rel-14 |  | C349 | UE supporting E-UTRA FDD and 6DL with CA configurations in Table 4.1-6 (UE Category 8, >= ≥ 11) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.1.4.2A | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 - Enhanced Performance Requirement Type C | Rel-12 |  | C142 | UE supporting E-UTRA FDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 8.2.1.4.2A |
| 8.2.1.4.3 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference model - Enhanced Performance Requirement Type A | Rel-11 |  | C44 | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.4.3A | FDD PDCSH Closed Loop Multi-Layer Spatial Multiplexing 4X2 for Dual Connectivity | Rel-12 |  | C169 | UE supporting E-UTRA FDD and Dual Connectivity (UE Category >= 3) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.2.1.4.4 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model - Enhanced Performance Requirement Type B | Rel-12 |  | C150 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.1.7_A.1 | FDD Carrier aggregation with power imbalance (intra-band contiguous DL CA) | Rel-10 |  | C22 | UE supporting E-UTRA FDD and intra-band contiguous DL CA |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.2.1.9 | FDD PDSCH in HST-SFN scenario | Rel-14 |  | C299 | UEs supporting E-UTRA FDD and high speed enhancement for measurement |  |  |  |  |  |
| 8.2.1.10 | FDD PDSCH minimum channel spacing for intra-band contiguous CA | Rel-15 |  | C125a | UE supporting E-UTRA FDD and intra-band contiguous DL CA (UE Category >= 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.2.2.1 | Void |  |  |  |  |  |  |  |  |  |
| 8.2.2.1.1 | TDD PDSCH Single Antenna Port Performance | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.2.1.1_A.1 or 8.2.2.1.1_A.2 is executed. |
| 8.2.2.1.1_1 | TDD PDSCH Single Antenna Port Performance (Release 9 and forward) | Rel-9 |  | C54 | UE supporting E-UTRA TDD (UE categories 1, 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.2.1.1_A.1 or 8.2.2.1.1_A.2 is executed. |
| 8.2.2.1.1_A.1 | TDD PDSCH Single Antenna Port Performance for CA (2DL CA) | Rel-10 |  | C110 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or interband DL CA (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.2.2.1.1_A.2 is executed. |
|  |  | Rel-11 |  | C109 | UE supporting E-UTRA TDD and Intra-band non-contiguous DL CA(UE Category >= 5) |  |  |  |  | Note 7 |
| 8.2.2.1.1_A.2 | TDD PDSCH Single Antenna Port Performance for CA (3DL CA) | Rel-10 |  | C128 | UE supporting E-UTRA TDD and 3DL with i CA configurations in Table 4.1-3 (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-11 |  | C129 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  |  |  |
| 8.2.2.1.1_A.3 | TDD PDSCH Single Antenna Port Performance for CA (4DL CA) | Rel-11 |  | C194 | CA configurations in Table 4.1-4 (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.2.1.1_A.4 | TDD PDSCH Single Antenna Port Performance for CA (5DL CA) | Rel-11 |  | C194a | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-12 |  | C194b | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  |  |  |
| 8.2.2.1.2 | TDD PDSCH Single Antenna Port Performance with 1PRB in the presence of MBSFN | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.2 | Void |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.1 | TDD PDSCH Transmit Diversity 2x2 | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.2.1_1 | TDD PDSCH Transmit Diversity 2x2 (Release 9 and forward) | Rel-9 |  | C16 | UE supporting E-UTRA TDD (UE category 1) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.2.2 | TDD PDSCH Transmit Diversity 4x2 | Rel-8 |  | C10 | UE supporting E-UTRA TDD and operating bands supporting 1,4 MHz Bandwidth |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.2.2_1 | TDD PDSCH Transmit Diversity 4x2 (Release 9 and forward) | Rel-9 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.2.3_C.1 | TDD PDSCH Transmit diversity 2x2 for eICIC (non-MBFSN ABS) | Rel-10 |  | C30 | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.2.3_E.1 | TDD PDSCH Transmit diversity 2x2 for feICIC (non-MBFSN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.2.4 | TDD PDSCH Transmit Diversity 2x2 with TM3 Interference Model - Enhanced Performance Requirement Type A | Rel-11 |  | C45 | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.2.5 | TDD PDSCH Transmit Diversity 2x2 for eIMTA (when EIMTA-MainConfigServCell-r12 is configured) | Rel-12 |  | C274 | UE supporting E-UTRA TDD and eIMTA TDD UL-DL reconfiguration |  |  |  |  |  |
| 8.2.2.2.6 | TDD PDSCH Transmit Diversity 2x2 with TM2 Interference Model - Enhanced Performance Requirement Type B | Rel-12 |  | C151 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.2.7 | TDD PDSCH Transmit Diversity 2x2 with TM9 Interference Model - Enhanced Performance Requirement Type B | Rel-12 |  | C151 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.3 | Void |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.2.3.1_A.1 or .2 is executed. |
| 8.2.2.3.1_1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 (Release 11 and forward) | Rel-11 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.2.3.1_A.1 or .2 is executed. |
| 8.2.2.3.1_A.1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (2DL CA) | Rel-10 |  | C110 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or interband DL CA (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | If 8.2.2.3.1_A.2 is executed for a CA capability, test execution is not necessary for that CA capability |
|  |  | Rel-11 |  | C109 | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE Category >= 5) |  |  |  |  | Note 7 |
| 8.2.2.3.1_A.2 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) | Rel-10 |  | C128 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-11 |  | C129 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  |  |  |
| 8.2.2.3.1_A.3 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA(4DL CA) | Rel-11 |  | C194 | UE supporting 4DL CA configurations in Table 4.1-4.(UE category >=8) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.2.3.1_A.4 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) | Rel-11 |  | C194a | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-12 |  | C194b | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  |  |  |
| 8.2.2.3.1A_A.1 | TDD Soft buffer management for CA (2 DL CA) | Rel-10 |  | C105 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE category 3 and 4) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | TBD |
|  |  | Rel-11 |  | C72 | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE category 3 and 4) |  |  |  |  | Note 7 |
| 8.2.2.3.1B | TDD PDSCH Open Loop Spatial Multiplexing 2x2 - Enhanced Performance Requirement Type C | Rel-12 |  | C143 | UE supporting E-UTRA TDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.3.1C | TDD PDSCH Open Loop Spatial Multiplexing 2x2 with TM1 Interference - Enhanced Performance Requirement Type C | Rel-12 |  | C143 | UE supporting E-UTRA TDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.3.2 | TDD PDSCH Open Loop Spatial Multiplexing 4x2 | Rel-8 |  | C02 | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.3.3_C.1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for eICIC (non-MBSFN ABS) | Rel-10 |  | C30 | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.3.3_C.2 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for eICIC (MBSFN ABS) | Rel-10 |  | C30 | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.3.3_E.1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | TBD |  |  |  |
| 8.2.2.4 | Void |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.1 | TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 | Rel-8 only |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.4.1_1 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 (Release 9 and forward) | Rel-9 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.4.1_E.1 | TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.4.1_H | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for 256QAM in DL | Rel-12 |  | C02h | UE supporting E-UTRA TDD and 256QAM in DL |  |  |  |  |  |
| 8.2.2.4.2 | TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 4x2 | Rel-8 only |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.2.4.2_A.1 or 8.2.2.4.2_A.2 is executed. |
| 8.2.2.4.2_1 | TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 4x2 (Release 9 and forward) | Rel-9 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | Test execution not necessary if 8.2.2.4.2_A.1 or 8.2.2.4.2_A.2 is executed. |
| 8.2.2.4.2_A.1 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (2DL CA) | Rel-10 |  | C110 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.2.2.4.2_A.2 or 8.2.2.4.2_A.3 or 8.13.2.1.1.2 or 8.13.2.1.1.3 or 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |
|  |  | Rel-11 |  | C109 | UE supporting E-UTRA TDD and Intra-band non-contiguous DL CA(UE Category >= 5) |  |  |  |  | Note 7 |
| 8.2.2.4.2_A.2 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) | Rel-10 |  | C128 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.2.2.4.2_A.3 or 8.13.2.1.1.2 or 8.13.2.1.1.3 or 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. Note 7 |
|  |  | Rel-11 |  | C129 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  |  |  |
| 8.2.2.4.2_A.3 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (4DL CA) | Rel-11 |  | C194 | UE supporting E-UTRA TDD and 4DL CA configurations in Table 4.1-4 (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.13.2.1.1.2 or 8.13.2.1.1.3 or 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. Note 7 |
| 8.2.2.4.2_A.4 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (5DL CA) | Rel-11 |  | C194a | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-12 |  | C194b | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  |  |  |
| 8.2.2.4.2A | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 - Enhanced Performance Requirement Type C | Rel-12 |  | C143 | UE supporting E-UTRA TDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.4.3 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model - Enhanced Performance Requirement Type A | Rel-11 |  | C45 | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.4.4 | TDD PDSCH Closed Loop Multi-Layer Spatial Multiplexing 4x2 for Dual Connectivity | Rel-12 |  | C170 | UE supporting E-UTRA TDD and Dual Connectivity (UE Category >= 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.2.2.4.5 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model - Enhanced Performance Requirement Type B | Rel-12 |  | C151 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.2.7_A.1 | TDD Carrier aggregation with power imbalance (intra-band contiguous DL CA) | Rel-10 |  | C24 | UE supporting E-UTRA TDD and intra-band contiguous DL CA |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.2.8.1 | Intra-band contiguous carrier aggregation with minimum channel spacing (2DL CA) | Rel-12 |  | C24 | UE supporting E-UTRA TDD and intra-band contiguous DL CA |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.2.8.2 | Intra-band contiguous carrier aggregation with minimum channel spacing (3DL CA) | Rel-12 |  | C24 | UE supporting E-UTRA TDD and intra-band contiguous DL CA |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.2.9 | TDD PDSCH in HST-SFN scenario | Rel-14 |  | C300 | UEs supporting E-UTRA TDD and high speed enhancement for measurement |  |  |  |  |  |
| 8.2.3.1.1.1 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD Pcell (2DL CA) | Rel-12 |  | C154 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.13.3.2.3 or 8.13.3.2.4 or 8.13.3.2.5 or 8.13.3.2.6 is executed. Note 7 |
| 8.2.3.1.1.2 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (3DL CA) | Rel-12 |  | C133 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.13.3.2.3 or 8.13.3.2.4 or 8.13.3.2.5 or 8.13.3.2.6 is executed. Note 7 |
| 8.2.3.1.1.3 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (4DL CA) | Rel-12 |  | C133a | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  | TBD |  |  | Test execution not necessary if 8.13.3.2.3 or 8.13.3.2.4 or 8.13.3.2.5 or 8.13.3.2.6 is executed. |
| 8.2.3.1.1.4 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (5DL CA) | Rel-12 |  | C133b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.3 or 8.13.3.2.4 or 8.13.3.2.5 or 8.13.3.2.6 is executed. |
| 8.2.3.1.1.5 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (6DL CA) | Rel-14 |  | C350 | UE supporting E-UTRA FDD and TDD and 6DL CA with FDD as PCell (UE Category8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.1.1.6 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (7DL CA) | Rel-14 |  | C358 | UE supporting E-UTRA FDD and TDD and 7DL CA with FDD as PCell (UE Category8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.1.2.1 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell(2DL CA) | Rel-12 |  | C155 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.13.3.2.7 or 8.13.3.2.8 or 8.13.3.2.9 or 8.13.3.2.10 is executed. Note 7 |
| 8.2.3.1.2.2 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (3DL CA) | Rel-12 |  | C135 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.13.3.2.7 or 8.13.3.2.8 or 8.13.3.2.9 or 8.13.3.2.10 is executed. Note 7 |
| 8.2.3.1.2.3 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (4DL CA) | Rel-12 |  | C135a | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  | TBD |  |  | Test execution not necessary if 8.13.3.2.7 or 8.13.3.2.8 or 8.13.3.2.9 or 8.13.3.2.10 is executed. |
| 8.2.3.1.2.4 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (5DL CA) | Rel-12 |  | C135b | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) |  | TBD |  |  | Test execution not necessary if 8.13.3.2.7 or 8.13.3.2.8 or 8.13.3.2.9 or 8.13.3.2.10 is executed. |
| 8.2.3.1.2.5 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (6DL CA) | Rel-14 |  | C351 | UE supporting E-UTRA FDD and TDD and 6DL CA with TDD as PCell (UE Category 8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.1.2.6 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (7DL CA) | Rel-14 |  | C359 | UE supporting E-UTRA FDD and TDD and 7DL CA with TDD as PCell (UE Category8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.2.1.1 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (2DL CA) | Rel-12 |  | C154 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.2.1.2 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (3DL CA) | Rel-12 |  | C133 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.2.1.3 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (4DL CA) | Rel-12 |  | C133a | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.2.3.2.1.4 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (5DL CA) | Rel-12 |  | C133b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.2.3.2.1.5 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (6DL CA) | Rel-14 |  | C350 | UE supporting E-UTRA FDD and TDD and 6DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.2.1.6 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (7DL CA) | Rel-14 |  | C358 | UE supporting E-UTRA FDD and TDD and 7DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.2.1A | TDD FDD CA PDSCH Soft buffer management test for FDD PCell (2DL CA) | Rel-12 |  | C136 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE categories 3 and 4) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.2.2.1 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (2DL CA) | Rel-12 |  | C155 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.2.2.2 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (3DL CA) | Rel-12 |  | C135 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  |  |
| 8.2.3.2.2.3 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell(4DL CA) | Rel-12 |  | C135a | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.2.3.2.2.4 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell(5DL CA) | Rel-12 |  | C135b | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.2.3.2.2.5 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell(6DL CA) | Rel-14 |  | C351 | UE supporting E-UTRA FDD and TDD and 6DL CA with TDD as PCell (UE Category 8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.2.2.6 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell(7DL CA) | Rel-14 |  | C359 | UE supporting E-UTRA FDD and TDD and 7DL CA with TDD as PCell (UE Category 8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.2.2A | TDD FDD CA PDSCH Soft buffer management test for TDD PCell (2DL CA) | Rel-12 |  | C137 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD PCell (UE categories 3 and 4) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.1.1 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (2DL CA) | Rel-12 |  | C154 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.1.2 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (3DL CA) | Rel-12 |  | C133 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  | TBD |  |  |  |
| 8.2.3.3.1.3 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (4DL CA) | Rel-12 |  | C133a | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.1.4 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (5DL CA) | Rel-12 |  | C133b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.1.5 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (6DL CA) | Rel-14 |  | C350 | UE supporting E-UTRA FDD and TDD and 6DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.1.6 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (7DL CA) | Rel-14 |  | C358 | UE supporting E-UTRA FDD and TDD and 7DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.2.1 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (2DL CA) | Rel-12 |  | C155 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >=5) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.2.2 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (3DL CA) | Rel-12 |  | C135 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.2.3 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (4DL CA) | Rel-12 |  | C135a | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.2.4 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (5DL CA) | Rel-12 |  | C135b | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.2.5 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (6DL CA) | Rel-14 |  | C351 | UE supporting E-UTRA FDD and TDD and 6DL CA with TDD as PCell (UE Category 8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.3.3.2.6 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (7DL CA) | Rel-14 |  | C359 | UE supporting E-UTRA FDD and TDD and 7DL CA with TDD as PCell (UE Category 8, and Category 11 and onwards) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Note 7 |
| 8.2.4.1.1 | LAA PDSCH CA Closed Loop Spatial Multiplexing Performance-4 Tx Antenna port with FDD as Pcell | Rel-13 |  | C209 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  | Each “Test Number” to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.2.4.1.2 | LAA PDSCH CA Closed Loop Spatial Multiplexing Performance-4 Tx Antenna port with TDD as Pcell | Rel-13 |  | C210 | UE supporting E-UTRA TDD and downlink LAA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.1 | Void |  |  |  |  |  |  |  |  |  |
| 8.3.1.1.1_D | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO | Rel-10 to Rel-14 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.1.1.1_H | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for 256QAM in DL | Rel-12 to Rel-14 |  | C25h | UE supporting E-UTRA FDD and eDL-MIMO and 256QAM in DL and Feature Group Indicator 103 |  |  |  |  |  |
|  |  | Rel-15 |  | C25hm | UE supporting E-UTRA FDD and eDL-MIMO and 256QAM in DL and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and eDL-MIMO and 256QAM in DL and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.1.1.2_D | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with a simultaneous transmission for eDL-MIMO | Rel-10 to Rel-14 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.1.1.3 | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A | Rel-11 to Rel-14 |  | C40 | UE supporting E-UTRA FDD and Feature Group Indictor 103 and supporting the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C40m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and supporting the enhanced performance requirements type A for LTE and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and supporting the enhanced performance requirements type A for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.1.1.4 | FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type B | Rel-12 to Rel-14 |  | C262 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C262m | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.1.1.6 | FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM3 interference model - Enhanced Performance Requirement Type B | Rel-12 |  | C150 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.1.1.7 | FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM10 serving cell configuration and TM9 interference model - Enhanced Performance Requirement Type B | Rel-12 |  | C175 | UE supporting E-UTRA FDD, enhanced performance requirements type B and PDSCH Transmission mode 10 for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.1.1.9 | FDD PDSCH Single-layer Spatial Multiplexing for FD-MIMO | Rel-13 to Rel-14 |  | C323 | UE supporting E-UTRA FDD and Feature Group Indicator 103 and dmrs-Enhancements-r13 |  |  |  |  |  |
|  |  | Rel-15 |  | C323m | UE supporting E-UTRA FDD and Feature Group Indictor 103 and dmrs-Enhancements-r13 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and dmrs-Enhancements-r13 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.1.1.10 | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eFD-MIMO | Rel-14 |  | CXX1 | UE supporting E-UTRA FDD and Feature Group Indicator 103 and aperiodic ZP-CSI-RS reporting |  |  |  |  |  |
| 8.3.1.2.1_D | FDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO | Rel-10 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.1.2.1_D_1 | FDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO (Release 11 and forward) | Rel-11 to Rel-14 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.1.2.2 | FDD PDSCH Dual-layer Spatial Multiplexing - Enhanced Performance Requirement Type C | Rel-12 to Rel-14 |  | C144 | UE supporting E-UTRA FDD and Feature Group Indicator 103 and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C144m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and Enhanced Performance Requirement TypeC for LTE and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and Enhanced Performance Requirement TypeC for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.1.3.1_F | FDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and single NZP CSI-RS resource for CoMP | Rel-11 |  | C50 | UE supporting E-UTRA FDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.1.3.2_F | FDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and multiple NZP CSI-RS resources for CoMP | Rel-11 |  | C52 | UE supporting E-UTRA FDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.1.3.3_F | FDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Different Cell ID, Colliding CRS and single NZP CSI-RS resource for CoMP | Rel-11 |  | C117 | UE supporting E-UTRA FDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.1.1 | TDD PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 8 and forward) | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.1.1_1 | TDD PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 9 and forward) | Rel-9 |  | C16 | UE supporting E-UTRA TDD (UE category 1) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.1.2 | TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 without a simultaneous transmission | Rel-9 only |  | C34 | UE supporting E-UTRA TDD and supporting enhanced dual layer TDD. |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-10 |  | C02 | UE supporting E-UTRA TDD. |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.1.2_D | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO | Rel-10 to Rel-14 |  | C26 | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C26m | UE supporting E-UTRA TDD and Feature Group Indicator 104 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.2.1.2_H | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for 256QAM in DL | Rel-12 to Rel-14 |  | C26h | UE supporting E-UTRA TDD and 256QAM in DL and Feature Group Indicator 104 |  |  |  |  |  |
|  |  | Rel-15 |  | C26hm | UE supporting E-UTRA TDD and 256QAM in DL and Feature Group Indicator 104 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and 256QAM in DL and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.2.1.3 | TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission | Rel-9 only |  | C34 | UE supporting E-UTRA TDD and supporting enhanced dual layer TDD. |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-10 |  | C02 | UE supporting E-UTRA TDD. |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.1.3_D | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with a simultaneous transmission for eDL-MIMO | Rel-10 to Rel-14 |  | C26 | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C26m | UE supporting E-UTRA TDD and Feature Group Indicator 104 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.2.1.4 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A | Rel-11 to Rel-14 |  | C41 | UE supporting E-UTRA TDD and Feature Group Indictor 103 and supporting the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C41m | UE supporting E-UTRA TDD and Feature Group Indicator 103 and supporting the enhanced performance requirements type A for LTE and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and supporting the enhanced performance requirements type A for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.2.1.5 | TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type B | Rel-12 to Rel-14 |  | C263 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C263m | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.2.1.7 | TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM3 interference model - Enhanced Performance Requirement Type B | Rel-12 |  | C151 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.1.8 | TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM10 serving cell configuration and TM9 interference model - Enhanced Performance Requirement Type B | Rel-12 |  | C176 | UE supporting E-UTRA TDD, enhanced performance requirements type B and PDSCH Transmission mode 10 for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.1.10 | TDD PDSCH Single-layer Spatial Multiplexing for FD-MIMO | Rel-13 to Rel-14 |  | C324 | UE supporting E-UTRA TDD and Feature Group Indicator 103 and dmrs-Enhancements-r13 |  |  |  |  |  |
|  |  | Rel-15 |  | C324m | UE supporting E-UTRA TDD and Feature Group Indictor 103 and dmrs-Enhancements-r13 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and dmrs-Enhancements-r13 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.2.1.11 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eFD-MIMO | Rel-14 |  | CXX2 | UE supporting E-UTRA TDD and Feature Group Indicator 104 and aperiodic ZP-CSI-RS reporting |  |  |  |  |  |
| 8.3.2.2.1 | TDD PDSCH Dual-layer Spatial Multiplexing | Rel-9 only |  | C34 | UE supporting E-UTRA TDD and supporting enhanced dual layer TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-10 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.2.1_D | TDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO | Rel-10 |  | C25a | UE supporting E-UTRA TDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.2.1_D_1 | TDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO (Release 11 and forward) | Rel-11 to Rel-14 |  | C25a | UE supporting E-UTRA TDD and Feature Group Indicator 103 |  | TBD |  |  |  |
|  |  | Rel-15 |  | C25am | UE supporting E-UTRA TDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.3.2.2.2 | TDD PDSCH Dual-layer Spatial Multiplexing - Enhanced Performance Requirement Type C | Rel-12 |  | C143 | UE supporting E-UTRA TDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.4.1_F | TDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and single NZP CSI-RS resource for CoMP | Rel-11 |  | C51 | UE supporting E-UTRA TDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.4.2_F | TDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and multiple NZP CSI-RS resources for CoMP | Rel-11 |  | C53 | UE supporting E-UTRA TDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.2.4.3_F | TDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Different Cell ID, Colliding CRS and single NZP CSI-RS resource for CoMP | Rel-11 |  | C118 | UE supporting E-UTRA TDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.3.1.1 | LAA Dual-Layer Spatial Multiplexing with DM-RS with FDD as PCell | Rel-13 |  | C264 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and TM9 on LAA cells |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.3.3.1.2 | LAA Dual-Layer Spatial Multiplexing with DM-RS with TDD as Pcell | Rel-13 |  | C265 | UE supporting E-UTRA TDD and downlink LAA and TM9 on LAA cells |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.1 | FDD PCFICH/PDCCH Single-antenna Port Performance | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.2 | Void |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.1 | FDD PCFICH/PDCCH Transmit Diversity 2x2 | Rel-8 only |  | C09 | UE supporting E-UTRA FDD and operating bands supporting 1,4 MHz Bandwidth |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.2.1_1 | FDD PCFICH/PDCCH Transmit Diversity 2x2 (Release 9 and forward) | Rel-9 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.2.2 | FDD PCFICH/PDCCH Transmit Diversity 4x2 | Rel-8 only |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.2.3_E.1 | FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.2.3_E.2 | FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.2.2_1 | FDD PCFICH/PDCCH Transmit Diversity 4x2 (Release 9 and forward) | Rel-9 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.2.3_C.1 | FDD PCFICH/PDCCH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) | Rel-10 |  | C29 | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.2.3_C.2 | FDD PCFICH/PDCCH Transmit Diversity 2x2 for eICIC (MBSFN ABS) | Rel-10 |  | C29 | UEs supporting E-UTRA FDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.1.2.5 | FDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port under Asynchronous Network | Rel-13 |  | C285 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.4.1.2.6 | FDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | Rel-13 |  | C285 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.4.1.2.7 | FDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | Rel-13 |  | C286 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |
| 8.4.1.2.8 | FDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | Rel-13 |  | C286 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |
| 8.4.2.1 | TDD PCFICH/PDCCH Single-antenna Port Performance | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.2.2 | Void |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.1 | TDD PCFICH/PDCCH Transmit Diversity 2x2 | Rel-8 only |  | C10 | UE supporting E-UTRA TDD and operating bands supporting 1,4 MHz Bandwidth |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.2.2.1_1 | TDD PCFICH/PDCCH Transmit Diversity 2x2 (Release 9 and forward) | Rel-9 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.2.2.2 | TDD PCFICH/PDCCH Transmit Diversity 4x2 | Rel-8 only |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.2.2.2_1 | TDD PCFICH/PDCCH Transmit Diversity 4x2 (Release 9 and forward) | Rel-9 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.2.2.3_C.1 | TDD PCFICH/PDCCH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) | Rel-10 |  | C30 | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.2.2.3_C.2 | TDD PCFICH/PDCCH Transmit Diversity 2x2 for eICIC (MBSFN ABS) | Rel-10 |  | C30 | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.2.2.3_E.1 | TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.2.2.3_E.2 | TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.2.2.5 | TDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | Rel-13 |  | C287 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.4.2.2.6 | DD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | Rel-13 |  | C287 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.4.2.2.7 | TDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer | Rel-13 |  | C288 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |
| 8.4.2.2.8 | TDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer | Rel-13 |  | C288 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |
| 8.4.3.1.1 | LAA PCFICH/PDCCH Transmit Diversity 2x2 with FDD as Pcell | Rel-13 |  | C209 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.4.3.1.2 | LAA PCFICH/PDCCH Transmit Diversity 2x2 with TDD as Pcell | Rel-13 |  | C217 | UE supporting E-UTRA TDD and downlink LAA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.1.1 | FDD PHICH Single-antenna Port Performance | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.1.2 | Void |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.1 | FDD PHICH Transmit Diversity 2x2 | Rel-8 only |  | C09 | UE supporting E-UTRA FDD and operating bands supporting 1,4 MHz Bandwidth |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.1.2.1_1 | FDD PHICH Transmit Diversity 2x2 (Release 9 and forward) | Rel-9 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.1.2.2 | FDD PHICH Transmit Diversity 4x2 | Rel-8 only |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.1.2.2_1 | FDD PHICH Transmit Diversity 4x2 (Release 9 and forward) | Rel-9 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.1.2.3_C.1 | FDD PHICH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) | Rel-10 |  | C29 | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.1.2.3_E.1 | FDD PHICH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.1.2.5 | FDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH- 2 Tx Antenna Ports under Asynchronous Network | Rel-13 |  | C285 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.5.1.2.6 | FDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer | Rel-13 |  | C285 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.5.1.2.7 | FDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer | Rel-13 |  | C286 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |
| 8.5.1.2.8 | FDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer | Rel-13 |  | C286 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |
| 8.5.2.1 | TDD PHICH Single-antenna Port Performance | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.2.2 | Void |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.1 | TDD PHICH Transmit Diversity 2x2 | Rel-8 only |  | C10 | UE supporting E-UTRA TDD and operating bands supporting 1,4 MHz Bandwidth |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.2.2.1_1 | TDD PHICH Transmit Diversity 2x2 (Release 9 and forward) | Rel-9 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.2.2.2 | TDD PHICH Transmit Diversity 4x2 | Rel-8 only |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.2.2.2_1 | TDD PHICH Transmit Diversity 4x2 (Release 9 and forward) | Rel-9 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.2.2.3_C.1 | TDD PHICH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) | Rel-10 |  | C30 | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.2.2.3_E.1 | TDD PHICH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.5.2.2.5 | TDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH- 2 Tx Antenna Ports with Colliding CRS Dominant Interfere | Rel-13 |  | C287 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.5.2.2.6 | TDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer | Rel-13 |  | C287 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.5.2.2.7 | TDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH- 2 Tx Antenna Ports with Colliding CRS Dominant Interferer | Rel-13 |  | C288 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |
| 8.5.2.2.8 | TDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH- 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer | Rel-13 |  | C288 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |
| 8.7.1.1 | FDD sustained data rate performance (Rel-9 and forward) | Rel-9 |  | C76 | UE supporting E-UTRA FDD and not supporting 256QAM in DL (UE categories from1 to 4) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.1.1_A.1 or 8.7.3.1 is executed. |
| 8.7.1.1_1 | FDD sustained data rate performance (Rel-10 and forward) | Rel-10 |  | C42 | UE supporting E-UTRA FDD and not supporting 256QAM in DL (UE categories 6, 7) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.1.1_A.1or 8.7.3.1 is executed. |
| 8.7.1.1_2 | FDD sustained data rate performance for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.7.1.1_A.1 | FDD Sustained data rate performance for CA (2 DL CA ) | Rel-10 |  | C107 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.2 is executed. |
|  |  | Rel-11 |  | C93 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  |  |  | Note 7 |
| 8.7.1.1_A.2 | FDD Sustained data rate performance for CA (3DL CA) | Rel-10 |  | C126 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.4 is executed |
|  |  | Rel-10 |  | C126a | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and supporting at most 40MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 6 and 7) |  | Refer to 36.521-1 8.1.2.3 |  |  | Note 7 |
|  |  | Rel-11 |  | C127 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | TBD |  |  |  |
|  |  | Rel-11 |  | C127a | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and supporting at most 40MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 6 and 7) |  | TBD |  |  |  |
| 8.7.1.1_A.4 | FDD Sustained data rate performance for CA (4DL CA) | Rel-11 |  | C189 | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and not supporting 256QAM in DL (UE category 11 and 12) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.5 is executed. Note 7 |
|  |  | Rel-11 |  | C189a | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and supporting at most 60MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 9 and 10) |  |  |  |  |  |
| 8.7.1.1_A.5 | FDD Sustained data rate performance for CA (5DL CA) | Rel-11 |  | C266 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 and not supporting 256QAM in DL (UE DL category 11,12) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.6 is executed. Note 7 |
|  |  | Rel-12 |  | C267 | UE supporting E-UTRA FDD and 5DL CA configurations in Table 4.1-5 and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  |  |  |  |
| 8.7.1.1_A.6 | FDD Sustained data rate performance for CA (6DL CA) | Rel-11 |  | C330 | UE supporting E-UTRA FDD and 6DL with CA configurations in Table 4.1-6 and not supporting 256QAM in DL (UE DL category 11,12) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.7 is executed. Note 7 |
|  |  | Rel-12 |  | C331 | UE supporting E-UTRA FDD and 6DL CA configurations in Table 4.1-6 and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  |  |  |  |
| 8.7.1.1_A.7 | FDD Sustained data rate performance for CA (7DL CA) | Rel-15 |  | C343 | UE supporting E-UTRA FDD and 7DL CA and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  | 2Rx, 4Rx |  | Note 7 |
| -8.7.1.1_H.1 | FDD sustained data rate performance (Single Carrier) for 256QAM in DL | Rel-12 |  | C42h | UE supporting E-UTRA FDD and 256QAM and UE DL category 13 |  |  |  |  | Test execution not necessary if 8.7.1.1_H.2 is executed |
| 8.7.1.1_H.2 | FDD Sustained data rate performance for CA (2DL CA) for 256QAM in DL | Rel-12 |  | C107h | UE supporting E-UTRA FDD and 2DL CA and 256QAM in DL (UE DL category 11, 12 and 13) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_H.3 is executed Note 7 |
| 8.7.1.1_H.3 | FDD Sustained data rate performance for CA (3DL CA) for 256QAM in DL | Rel-12 |  | C126h | UE supporting E-UTRA FDD and 3DL CA ,and supporting 256QAM in DL (UE DL category 11, 12 and 15) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_H.4 is executed Note 7 |
|  |  | Rel-12 |  | C126ha | UE supporting E-UTRA FDD and 3DL CA ,and supporting 256QAM in DL and supporting at most 40MHz aggregated bandwidth (UE DL category 13) |  |  |  |  | Test execution not necessary if 8.7.1.1_H.4 is executed |
| 8.7.1.1_H.4 | FDD Sustained data rate performance for CA (4DL CA) for 256QAM in DL | Rel-13 |  | C189h | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and supporting 256QAM in DL |  |  |  |  | Test execution not necessary if 8.7.1.1_H.5 is executed. |
| 8.7.1.1_H.5 | FDD Sustained data rate performance for CA (5DL CA) for 256QAM in DL | Rel-13 |  | C266h | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 and supporting 256QAM in DL |  |  |  |  |  |
| 8.7.1.1_H.6 | FDD Sustained data rate performance for CA (6DL CA) for 256QAM in DL | Rel-13 |  | C331h | UE supporting E-UTRA FDD and 6DL with CA configurations in Table 4.1-6 and supporting 256QAM in DL |  |  |  |  |  |
| 8.7.1.1_H.7 | FDD Sustained data rate performance for CA (7DL CA) for 256QAM in DL | Rel-15 |  | C344 | UE supporting E-UTRA FDD and 7DL and supporting 256QAM in DL |  |  |  |  | Test execution not necessary if 8.7.1.1_H.8 is executed |
| 8.7.2.1 | TDD sustained data rate performance (Rel-9 and forward) | Rel-9 |  | C111 | UE supporting E-UTRA TDD and not supporting 256QAM in DL (UE categories from 1 to 4) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.2.1_A.1 or 8.7.4.1 is executed. |
| 8.7.2.1_1 | TDD sustained data rate performance (Rel-10 and forward) | Rel-10 |  | C73 | UE supporting E-UTRA TDD and not supporting 256QAM in DL (UE category 6 and 7) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.2.1_A.1or 8.7.4.1 is executed. |
| 8.7.2.1_2 | TDD sustained data rate performance for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.7.2.1_A.1 | TDD sustained data rate performance for CA (2DL CA) | Rel-10 |  | C74 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA and not supporting 256QAM in DL (UE category 6, 7, 9 and 10) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.2 is executed. |
|  |  | Rel-11 |  | C75 | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA and not supporting 256QAM in DL (UE category 6, 7, 9 and 10) |  |  |  |  | Note 7 |
| 8.7.2.1_A.2 | TDD Sustained data rate performance for CA (3DL CA) | Rel-10 |  | C130 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.3 is executed. |
|  |  | Rel-11 |  | C131 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  |  |  | Note 7 |
| 8.7.2.1_A.3 | TDD Sustained data rate performance for CA (4DL CA) | Rel-11 |  | C213 | UE supporting E-UTRA TDD and 4DL CA configurations in Table 4.1-3 (UE DL category 11, 12 and 15) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.4 is executed. Note 7 |
| 8.7.2.1_A.6 | TDD Sustained data rate performance for CA (6DL CA) | Rel-15 |  | C331a | UE supporting E-UTRA TDD and 6DL CA configurations in Table 4.1-3 (UE DL category 11, 12 and 15) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.7 is executed. Note 7 |
| 8.7.2.1_A.7 | TDD Sustained data rate performance for CA (7DL CA) | Rel-15 |  | C345 | UE supporting E-UTRA TDD and 7DL CA (UE DL category 11, 12 and 15) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.8 is executed. Note 7 |
| 8.7.2.1_H.1 | TDD sustained data rate performance (Single Carrier) for 256QAM in DL | Rel-12 |  | C73h | UE supporting E-UTRA TDD and 256QAM in DL and UE DL category 13 |  |  |  |  | Test execution not necessary if 8.7.2.1_H.2 is executed. |
| 8.7.2.1_H.2 | TDD sustained data rate performance for CA (2DL CA) for 256QAM in DL | Rel-12 |  | C74h | UE supporting E-UTRA TDD and 2DL CA, and supporting 256QAM in DL (UE DL category 11, 12 and 13) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.3 is executed. Note 7 |
| 8.7.2.1_H.3 | TDD Sustained data rate performance for CA (3DL CA) for 256QAM in DL | Rel-12 |  | C130h | UE supporting E-UTRA TDD and 3DL CA and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.4 is executed. Note 7 |
| 8.7.2.1_H.4 | TDD Sustained data rate performance for CA (4DL CA) for 256QAM in DL | Rel-13 |  | C213h | UE supporting E-UTRA TDD and 4DL CA and supporting 256QAM in DL (UE DL Category 11, 12,15 and 16) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.5 is executed. Note 7 |
| 8.7.2.1_H.5 | TDD Sustained data rate performance for CA (5DL CA) for 256QAM in DL | Rel-14 |  | C332 | UE supporting E-UTRA TDD and 5DL CA and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Note 7 |
| 8.7.2.1_H.6 | TDD Sustained data rate performance for CA (6DL CA) for 256QAM in DL | Rel-15 |  | C331ha | UE supporting E-UTRA TDD and 6DL CA and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.7 is executed. Note 7 |
| 8.7.2.1_H.7 | TDD Sustained data rate performance for CA (7DL CA) for 256QAM in DL | Rel-15 |  | C346 | UE supporting E-UTRA TDD and 7DL CA and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.8 is executed. Note 7 |
| 8.7.3.1 | FDD sustained data rate performance for EPDCCH scheduling | Rel-11 |  | C55 | UE supporting E-UTRA FDD and EPDCCH |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.7.4.1 | TDD sustained data rate performance for EPDCCH scheduling | Rel-11 |  | C56 | UE supporting E-UTRA TDD and EPDCCH |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.7.5.1.1 | TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) | Rel-12 |  | C138 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.2 is executed. Note 7 |
| 8.7.5.1.2 | TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) | Rel-12 |  | C139 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.3 is executed. Note 7 |
| 8.7.5.1.3 | TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) | Rel-12 |  | C139a | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 11 and 12) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.4 is executed. Note 7 |
| 8.7.5.1.4 | TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) | Rel-12 |  | C139b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell and not supporting 256QAM in DL (UE DL category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.1.5 | TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) | Rel-12 |  | C139c | UE supporting E-UTRA FDD and TDD and 6DL CA with FDD as PCell and not supporting 256QAM in DL (UE DL category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.1_H.1 | TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) for 256QAM in DL | Rel-12 |  | C138h | UE supporting E-UTRA FDD and TDD and 2DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL (UE DL category 11, 12 and 13) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1_H.2 is executed. Note 7 |
| 8.7.5.1_H.2 | TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) for 256QAM in DL | Rel-12 |  | C139h | UE supporting E-UTRA FDD and TDD and 3DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1_H.3 is executed. Note 7 |
| 8.7.5.1_H.3 | TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) for 256QAM in DL | Rel-12 |  | C139ha | UE supporting E-UTRA FDD and TDD and 4DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  | Test execution not necessary if 8.7.5.1_H.4 is executed. |
| 8.7.5.1_H.4 | TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) for 256QAM in DL | Rel-12 |  | C139hb | UE supporting E-UTRA FDD and TDD and 5DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  |  |
| 8.7.5.1_H.5 | TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) for 256QAM in DL | Rel-12 |  | C139hc | UE supporting E-UTRA FDD and TDD and 6DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  |  |
| 8.7.5.2.1 | TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) | Rel-12 |  | C140 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.2 is executed. Note 7 |
| 8.7.5.2.2 | TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) | Rel-12 |  | C141 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.3 is executed. Note 7 |
| 8.7.5.2.3 | TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) | Rel-12 |  | C141a | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 11 and 12) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.4 is executed. Note 7 |
| 8.7.5.2.4 | TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) | Rel-12 |  | C141b | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.2.5 | TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) | Rel-12 |  | C141c | UE supporting E-UTRA FDD and TDD and 6DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.2_H.1 | TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) for 256QAM in DL | Rel-12 |  | C140h | UE supporting E-UTRA FDD and TDD and 2DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 13) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2_H.2 is executed. Note 7 |
| 8.7.5.2_H.2 | TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) for 256QAM in DL | Rel-12 |  | C141h | UE supporting E-UTRA FDD and TDD and 3DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2_H.3 is executed. Note 7 |
| 8.7.5.2_H.4 | TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) for 256QAM in DL | Rel-12 |  | C141hb | UE supporting E-UTRA FDD and TDD and 5DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.2_H.5 | TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) for 256QAM in DL | Rel-12 |  | C142h | UE supporting E-UTRA FDD and TDD and 6DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Note 7 |
| 8.7.6.1 | FDD sustained data rate performance for Dual Connectivity 64QAM | Rel-12 |  | C171 | UE supporting E-UTRA FDD and Dual Connectivity and not supporting 256QAM in DL (UE Category 3, 4, 6, 7, 9, and 10) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.7.6.2 | FDD sustained data rate performance for Dual Connectivity 256QAM | Rel-12 |  | C173 | UE supporting E-UTRA FDD and Dual Connectivity and supporting 256QAM in DL |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.7.7.1 | TDD sustained data rate performance for Dual Connectivity 64QAM | Rel-12 |  | C172 | UE supporting E-UTRA TDD and Dual Connectivity and not supporting 256QAM in DL (UE Category 6, 7, 9, and 10) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.7.7.2 | TDD sustained data rate performance for Dual Connectivity 256QAM | Rel-12 |  | C174 | UE supporting E-UTRA TDD and Dual Connectivity and supporting 256QAM in DL |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.7.9.2 | FDD sustained data rate performance for 4 layer MIMO (single carrier) | Rel-10 |  | C226 | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 6 and 7 and UE DL category 13) |  | One "Test Number" to be performed. The selected band shall lead to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.9.3 | FDD sustained data rate performance for 4 layer MIMO (2DL CA) | Rel-10 |  | C227 | UE supporting 2DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band contiguous DL CA or inter-band DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  | One "Test Number" to be performed. The selected CA configuration shall lead to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-11 |  | C228 | UE supporting 2DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band non-contiguous DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  |  |  |
| 8.7.9.4 | FDD sustained data rate performance for 4 layer MIMO (3DL CA) | Rel-10 |  | C229 | UE supporting 3DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-11 |  | C230 | UE supporting 3DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 and 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  |  |  |
| 8.7.9.5 | FDD sustained data rate performance for 4 layer MIMO (4DL CA) | Rel-11 |  | C236 | UE supporting 4DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 4DL with CA configurations in Table 4.1-4 (UE DL category 15, 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.9.6 | FDD sustained data rate performance for 4 layer MIMO (5DL CA) | Rel-11 |  | C237 | UE supporting 5DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 15, 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-12 |  | C238 | UE supporting 5DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 15, 16, 18 and 19) |  |  |  |  |  |
| 8.7.10.2 | TDD sustained data rate performance for 4 layer MIMO (single carrier) | Rel-10 |  | C239 | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 6 and 7 and UE DL category 13) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.10.3 | TDD sustained data rate performance for 4 layer MIMO (2DL CA) | Rel-10 |  | C240 | UE supporting 2DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band contiguous DL CA or inter-band DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-11 |  | C241 | UE supporting 2DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band non-contiguous DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  |  |  |
| 8.7.10.4 | TDD sustained data rate performance for 4 layer MIMO (3DL CA) | Rel-10 |  | C242 | UE supporting 3DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-11 |  | C243 | UE supporting 3DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  |  |  |
| 8.7.10.5 | TDD sustained data rate performance for 4 layer MIMO (4DL CA) | Rel-11 |  | C244 | UE supporting 4DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 4DL with CA configurations in Table 4.1-4 (UE DL category 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.10.6 | TDD sustained data rate performance for 4 layer MIMO (5DL CA) | Rel-11 |  | C245 | UE supporting 5DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-12 |  | C246 | UE supporting 5DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 18 and 19) |  |  |  |  |  |
| 8.7.11.2 | TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) | Rel-12 |  | C247 | UE supporting 2DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.11.3 | TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) | Rel-12 |  | C248 | UE supporting 3DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.11.4 | TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) | Rel-12 |  | C249 | UE supporting 4DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE DL category 15, 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.11.5 | TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) | Rel-12 |  | C250 | UE supporting 5DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE DL category 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.12.1.2.1 | LAA sustained data rate performance with FDD PCell with 2DL CA (2Rx) | Rel-13 |  | C209 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |
| 8.7.12.1.2.2 | LAA sustained data rate performance with FDD PCell for 4 Layer MIMO (2DL CA) | Rel-13 |  | C328 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell abd 4-layer spatial multiplexing |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |
| 8.7.12.2.2.1 | LAA sustained data rate performance with TDD PCell with 2DL CA (2Rx) | Rel-13 |  | C210 | UE supporting E-UTRA TDD and downlink LAA |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |
| 8.7.12.2.2.2 | LAA sustained data rate performance with TDD PCell for 4 Layer MIMO (2DL CA) | Rel-13 |  | C329 | UE supporting E-UTRA TDD and downlink LAA with FDD as Pcell abd 4-layer spatial multiplexing |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |
| 8.8.1.1 | FDD distributed EPDCCH performance | Rel-11 |  | C55 | UE supporting E-UTRA FDD and EPDCCH |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.8.1.2 | TDD distributed EPDCCH performance | Rel-11 |  | C56 | UE supporting E-UTRA TDD and EPDCCH |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.8.2.1 | FDD localized EPDCCH performance with TM9 | Rel-11 to Rel-14 |  | C91 | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C91m | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and EPDCCH and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.8.2.2 | TDD localized EPDCCH performance with TM9 | Rel-11 to Rel-14 |  | C92 | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C92m | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and EPDCCH and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.8.3.1 | FDD localized EPDCCH transmission with TM10 Type B quasi co-location type | Rel-11 |  | C57 | UE supporting E-UTRA FDD and EPDCCH and Multiple CSI processes on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.8.3.2 | TDD localized EPDCCH transmission with TM10 Type B quasi co-location type | Rel-11 |  | C58 | UE supporting E-UTRA TDD and EPDCCH and Multiple CSI processes on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.8.4.1 | FDD Enhanced Downlink Control Channel Performance Requirements Type A for EDPCCH - Localized Transmission with CRS Interference Model | Rel-13 |  | C289 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.8.4.2 | TDD Enhanced Downlink Control Channel Performance Requirements Type A for EDPCCH - Localized Transmission with CRS Interference Model | Rel-13 |  | C290 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.8.5.1 | TDD Enhanced Downlink Control Channel Performance Requirements Type A for EDPCCH - Distributed Transmission with TM9 Interference Model | Rel-13 |  | C290 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.8.6.1 | FDD Enhanced Downlink Control Channel Performance Type A for EDPCCH- Distributed Transmission with TM3 Interference Model | Rel-13 |  | C289 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.9.1.1.1 | Transmit diversity performance for UE category 0 (Cell-Specific Reference Symbols) | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.1_1 | FDD PDSCH Transmit Diversity 4x1 for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.2 | FDD closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.2_1 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 4x1 for UE Category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.3 | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | Rel-12 |  | C157 | UE supporting E-UTRA FDD (UE category 0) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.3_1 | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis | Rel-13 |  | C157a | UE supporting E-UTRA FDD (UE category 1bis) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.1 | TDD PDSCH Transmit Diversity for UE category 0 | Rel-12 |  | C156 | UE supporting E-UTRA TDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.1_1 | TDD PDSCH Transmit Diversity for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.2 | TDD closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.2_1 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.3 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | Rel-12 |  | C158 | UE supporting E-UTRA TDD (UE category 0) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.3_1 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis | Rel-13 |  | C158a | UE supporting E-UTRA TDD (UE category 1bis) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.2.1.1 | FDD PHICH Transmit Diversity for UE category 0 | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.2.1.1_1 | FDD PHICH Transmit Diversity for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.2.2.1 | TDD PHICH Transmit Diversity for UE category 0 | Rel-12 |  | C156 | UE supporting E-UTRA TDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.2.2.1_1 | TDD PHICH Transmit Diversity for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.10.1.1.1 | FDD PDSCH Transmit Diversity 2x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.2 | FDD PDSCH Open Loop Spatial Multiplexing 2x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.3 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A | Rel-11 |  | C113d | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.4 | FDD PDSCH Closed Loop Spatial Multiplexing 4x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.5 | FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A | Rel-11 to Rel-14 |  | C113e | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C113em | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 and (UE Category < 8 and 8 < UE Category < 11 and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.1.1.6 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) | Rel-10 to Rel-14 |  | C113c | UE supporting E-UTRA FDD and Feature Group Indicator 103 and 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C113cm | UE supporting E-UTRA FDD and Feature Group Indicator 103 and 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and 4Rx antenna ports (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.1.1.7 | FDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports | Rel-10 |  | C220 | UE supporting E-UTRA FDD with 4Rx antenna ports and 3-layer spatial multiplexing |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.8 | FDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas | Rel-10 |  | C220 | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.9 | FDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) | Rel-10 to Rel-14 |  | C113c | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 and 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C113cm | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 and 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and eDL-MIMO and 4Rx antenna ports (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.1.2.1 | TDD PDSCH Transmit Diversity 2x4 | Rel-10 |  | C198 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.2 | TDD PDSCH Open Loop Spatial Multiplexing 2x4 | Rel-10 |  | C198 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.3 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A | Rel-11 |  | C198 a | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.4 | TDD PDSCH Closed Loop Spatial Multiplexing 4x4 | Rel-10 |  | C198 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.5 | TDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A | Rel-11 |  | C198c | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.6 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) | Rel-10 |  | C198b | UE supporting E-UTRA TDD with 4Rx antenna ports and UE Category 2 or higher and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.7 | TDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports | Rel-10 |  | C235 | UE supporting E-UTRA TDD with 4Rx antenna ports and 3-layer spatial multiplexing |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.8 | TDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas | Rel-10 |  | C235 | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.9 | TDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) | Rel-10 to Rel-14 |  | C183 | UE supporting E-UTRA TDD and UE Category ≥ 5 and Feature Group Indicator 103 and 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C183m | UE supporting E-UTRA TDD and Feature Group Indicator 103 with 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13)), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.2.1.1 | FDD PCFICH/PDCCH Single-antenna Port Performance 1x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.1.2 | FDD PCFICH/PDCCH Transmit Diversity Performance 2x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.1.3 | FDD PCFICH/PDCCH Transmit Diversity Performance 4x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.2.1 | TDD PCFICH/PDCCH Single-antenna Port Performance 1x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.2.2 | TDD PCFICH/PDCCH Transmit Diversity Performance 2x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.2.3 | TDD PCFICH/PDCCH Transmit Diversity Performance 4x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.1.1 | FDD PHICH Single-antenna Port Performance 1x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.1.2 | FDD PHICH Transmit Diversity Performance 2x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.1.3 | FDD PHICH Transmit Diversity Performance 4x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.2.1 | TDD PHICH Single-antenna Port Performance 1x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.2.2 | TDD PHICH Transmit Diversity Performance 2x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.2.3 | TDD PHICH Transmit Diversity Performance 4x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.4.1.1 | FDD distributed EPDCCH performance 2x4 | Rel-10 |  | C164 | UE supporting E-UTRA FDD and EPDCCH with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.4.1.2 | TDD distributed EPDCCH performance 2x4 | Rel-10 |  | C165 | UE supporting E-UTRA TDD and EPDCCH with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.4.2.1 | FDD localized EPDCCH performance with TM9 2x4 | Rel-10 to Rel-14 |  | C166 | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C166m | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and EPDCCH with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.4.2.2 | TDD localized EPDCCH performance with TM9 2x4 | Rel-10 to Rel-14 |  | C167 | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C167m | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and EPDCCH with 4Rx antenna ports (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.11.1.1.1 | FDD and half-duplex FDD Closed-loop spatial multiplexing performance for UE supporting CE | Rel-13 |  | C145e | UE supporting E-UTRA FDD and UE category M1 and TM6 in CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C145f | UE supporting E-UTRA FDD and UE category M2 and TM6 in CEModeA |  |  |  |  |  |
| 8.11.1.1.2 | FDD and half-duplex FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE supporting CE | Rel-13 |  | C145g | UE supporting E-UTRA FDD and UE category M1 and TM9 in CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C145h | UE supporting E-UTRA FDD and UE category M2 and TM9 in CEModeA |  |  |  |  |  |
| 8.11.1.1.3.1 | FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M1 | Rel-13 |  | C145a | UE supporting E-UTRA FDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.1.1.3.1_1 | FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M1 (CEmodeB) | Rel-13 |  | C156c | UE supporting E-UTRA FDD and (UE category M1 and CEModeB) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.1.2.1 | TDD Closed-loop spatial multiplexing performance for CE UE in CE Mode A (Cell-Specific Reference Symbols) | Rel-13 |  | C156q | UE supporting E-UTRA TDD and UE category M1 and TM6 in CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C156h | UE supporting E-UTRA TDD and UE category M2 and TM6 in CEModeA |  |  |  |  |  |
| 8.11.1.2.2 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for CE UE in CE Mode A | Rel-13 |  | C156e | UE supporting E-UTRA TDD and UE category M1 and TM9 in CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C156i | UE supporting E-UTRA TDD and UE category M2 and TM9 in CEModeA |  |  |  |  |  |
| 8.11.1.2.3.1 | TDD PDSCH Transmit Diversity for UE category M1 | Rel-13 |  | C156b | UE supporting E-UTRA TDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.1.2.3.1_1 | TDD PDSCH Transmit Diversity for UE category M1 (CEModeB) | Rel-13 |  | C156d | UE supporting E-UTRA TDD and UE category M1 and CEModeB |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.1.2.3.2 | TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 | Rel-14 |  | C316a | UE supporting E-UTRA TDD and UE category M2 and CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.1.2.3.2_1, | TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 (CEModeB) | Rel-14 |  | C316b | UE supporting E-UTRA TDD and UE category M2 and CEModeB |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.2.1.1 | FDD demodulation of MPDCCH in CE Mode A | Rel-13 |  | C145b | UE supporting E-UTRA FDD and (UE category M1 or CEModeA) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C314 | UE supporting E-UTRA FDD and (UE category M2 or CEModeA) |  |  |  |  |  |
| 8.11.2.1.2 | FDD and half-duplex FDD demodulation of MPDCCH in CE Mode B | Rel-13 |  | C156c | UE supporting E-UTRA FDD and (UE category M1 and CEModeB) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C315 | UE supporting E-UTRA FDD and (UE category M1 and CEModeB) |  |  |  |  |  |
| 8.11.2.2.1 | TDD demodulation of MPDCCH in CE Mode A | Rel-13 |  | C156b | UE supporting E-UTRA TDD and (UE category M1 or UE category M2 or CEModeA) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C316 | UE supporting E-UTRA TDD and (UE category M2 or CEModeA) |  |  |  |  |  |
| 8.11.2.2.2 | TDD demodulation of MPDCCH in CE Mode B | Rel-13 |  | C156d | UE supporting E-UTRA TDD and (UE category M1 or UE category M2 and CEModeB) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C317 | UE supporting E-UTRA TDD and (UE category M1 and CEModeB) |  |  |  |  |  |
| 8.12.1.1.1 | Demodulation of NPDSCH (Cell-Specific Reference Symbols) in In-band mode for Category NB1 and NB2 | Rel-13 |  | C347 | UE supporting NB-IoT FDD |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |
| 8.12.1.1.2 | Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone and Guard-band mode for category NB1 and NB2 | Rel-13 |  | C347 | UE supporting NB-IoT FDD |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |
| 8.12.1.1.3 | Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 | Rel-14 |  | C298 | UE supporting E-UTRA FDD and category NB2 |  | Each “Test Number” to be performed once, in a chosen band |  |  |  |
| 8.12.2.1.1 | Demodulation of NPDCCH single-antenna performance for category NB1 and NB2 | Rel-13 |  | C347 | UE supporting NB-IoT FDD |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |
| 8.12.2.1.2 | Demodulation of NPDCCH in In-band mode Transmit Diversity performance for Category NB1 and NB2 | Rel-13 |  | C347 | UE supporting NB-IoT FDD |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |
| 8.13.1.1.3 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM (2DL CA) | Rel-10 |  | C278 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA and 4 Rx antenna ports and 256QAM in DL (UE Category >= 5) |  |  |  |  |  |
|  |  | Rel-11 |  | C279 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA and 4 Rx antenna ports and 256QAM in DL (UE Category >= 5) |  |  |  |  |  |
| 8.13.1.1.4 | FDD PDSCH Closed Loop Four-Layer Spatial Multiplexing for CA (2DL CA) | Rel-11 |  | C280 | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category >= 5) |  |  |  |  |  |
| 8.13.1.1.1.2 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (2DL CA) | Rel-10 |  | C253 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |
|  |  | Rel-11 |  | C254 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.1.1.3 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (3DL CA) | Rel-10 |  | C255 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |
|  |  | Rel-11 |  | C256 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.1.1.4 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (4DL CA) | Rel-11 |  | C257 | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.1.1.5 is executed. |
| 8.13.1.1.1.5 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (5DL CA) | Rel-11 |  | C258 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |
|  |  | Rel-12 |  | C259 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.2.2 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (2DL CA) | Rel-10 |  | C253 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.2.3 or 8.13.1.2.4 or 8.13.1.2.5 is executed. |
|  |  | Rel-11 |  | C254 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.2.3 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (3DL CA) | Rel-10 |  | C255 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.2.4 or 8.13.1.2.5 is executed. |
|  |  | Rel-11 |  | C256 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.2.4 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (4DL CA) | Rel-11 |  | C257 | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.2.5 is executed. |
| 8.13.1.2.5 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (5DL CA) | Rel-11 |  | C258 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |
|  |  | Rel-12 |  | C259 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.3.1 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA) | Rel-11 |  | C281 | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A (UE Category >= 5) |  |  |  |  |  |
| 8.13.1.4.1 | FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA) | Rel-11to Rel-14 |  | C282 | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 (UE Category >= 5) |  |  |  |  |  |
|  |  | Rel-15 |  | C282m | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.13.2.1.1.2 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (2DL CA) | Rel-10 |  | C291 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.2.1.1.3 or 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |
|  |  | Rel-11 |  | C292 | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.1.1.3 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (3DL CA) | Rel-10 |  | C293 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |
|  |  | Rel-11 |  | C294 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.1.1.4 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (4DL CA) | Rel-11 |  | C295 | UE supporting E-UTRA TDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.1.1.5 is executed. |
| 8.13.2.1.1.5 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (5DL CA) | Rel-11 |  | C296 | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |
|  |  | Rel-12 |  | C297 | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.2.2 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (2DL CA) | Rel-10 |  | C291 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.2.3 or 8.13.2.2.4 or 8.13.2.2.5 is executed. |
|  |  | Rel-11 |  | C292 | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.2.3 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (3DL CA) | Rel-10 |  | C293 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.2.4 or 8.13.2.2.5 is executed. |
|  |  | Rel-11 |  | C294 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.2.4 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (4DL CA) | Rel-11 |  | C295 | UE supporting E-UTRA TDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.2.5 is executed. |
| 8.13.2.2.5 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (5DL CA) | Rel-11 |  | C296 | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |
|  |  | Rel-12 |  | C297 | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.3.1 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA) | Rel-11 |  | C283 | UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A (UE Category >= 5) |  |  |  |  |  |
| 8.13.2.4.1 | TDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA) | Rel-11 to Rel-14 |  | C284 | UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 (UE Category >= 5) |  |  |  |  |  |
|  |  | Rel-15 |  | C284m | UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.13.3.1.1.2 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (2DL CA) | Rel-12 to Rel-14 |  | C306 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.1.3 or 8.13.3.1.1.4 or 8.13.3.1.1.5 is executed. |
|  |  | Rel-15 |  | C234m | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and Feature Group Indicator 103 and the enhanced performance requirements type A for LTE and TDD as PCell and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A for LTE and TDD as PCell and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.13.3.1.1.3 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (3DL CA) | Rel-12 |  | C307 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.1.4 or 8.13.3.1.1.5 is executed. |
| 8.13.3.1.1.4 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (4DL CA) | Rel-12 |  | C308 | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.1.5 is executed. |
| 8.13.3.1.1.5 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (5DL CA) | Rel-12 |  | C309 | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category >= 8) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.13.3.1.2.2 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (2DL CA) | Rel-12 |  | C310 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >=5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.2.3 or 8.13.3.1.2.4 or 8.13.3.1.2.5 is executed. |
| 8.13.3.1.2.3 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (3DL CA) | Rel-12 |  | C311 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.2.4 or 8.13.3.1.2.5 is executed. |
| 8.13.3.1.2.4 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (4DL CA) | Rel-12 |  | C312 | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.2.5 is executed. |
| 8.13.3.1.2.5 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (5DL CA) | Rel-12 |  | C322 | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.13.3.2.3 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for FDD PCell (User-Specific Reference Symbols) (2DL CA) | Rel-12 |  | C154 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.4 or 8.13.3.2.5 or 8.13.3.2.6 is executed. |
| 8.13.3.2.4 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for FDD PCell (User-Specific Reference Symbols) (3DL CA) | Rel-12 |  | C133 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.5 or 8.13.3.2.6 is executed. |
| 8.13.3.2.5 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for FDD PCell (User-Specific Reference Symbols) (4DL CA) | Rel-12 |  | C133a | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.6 is executed. |
| 8.13.3.2.6 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for FDD PCell (User-Specific Reference Symbols) (5DL CA) | Rel-12 |  | C133b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.13.3.2.7 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for TDD PCell (User-Specific Reference Symbols) (2DL CA) |  |  | C155 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >=5) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.8 or 8.13.3.2.9 or 8.13.3.2.10 is executed. |
| 8.13.3.2.8 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for TDD PCell (User-Specific Reference Symbols) (3DL CA) |  |  | C135 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.9 or 8.13.3.2.10 is executed. |
| 8.13.3.2.9 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for TDD PCell (User-Specific Reference Symbols) (4DL CA) |  |  | C135a | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.10 is executed. |
| 8.13.3.2.10 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for TDD PCell (User-Specific Reference Symbols) (5DL CA) |  |  | C135b | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.13.3.3.1 | TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA) | Rel-12 |  | C231 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A for LTE and FDD as PCell (UE Category >=5) |  |  |  |  |  |
| 8.13.3.3.2 | TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA) | Rel-12 |  | C232 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A for LTE and TDD as PCell (UE Category >=5) |  |  |  |  |  |
| 8.13.3.4.1 | TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A for FDD PCell (2DL CA) | Rel-12 |  | C233 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and Feature Group Indictor 103 and the enhanced performance requirements type A for LTE and FDD as PCell (UE Category >=5) |  |  |  |  |  |
| 8.13.3.4.2 | TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A for TDD PCell (2DL CA) | Rel-12 |  | C234 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and Feature Group Indictor 103 and the enhanced performance requirements type A for LTE and TDD as PCell (UE Category >=5) |  |  |  |  |  |
| 8.13.3.6.1 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for FDD PCell (2DL CA) | Rel-12 |  | C251 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and 256QAM in DL and FDD as Pcell(UE Category >=5) |  |  |  |  |  |
| 8.13.3.6.2 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for TDD PCell (2DL CA) | Rel-12 |  | C252 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and 256QAM in DL and TDD as Pcell(UE Category >=5) |  |  |  |  |  |
| 8.14.1.1.1 | FDD slot/subslot-PDSCH Open Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols) | Rel-15 |  | C354 | UE supporting E-UTRA FDD and slot/subslot TTI (UE categories >=2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.1.1.2 | FDD slot/subslot-PDSCH Closed Loop Spatial Multiplexing Performance (User-Specific Reference Signals) | Rel-15 |  | C355 | UE supporting E-UTRA FDD and Feature Group Indicator 103 and slot/subslot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.1.2.1 | TDD slot-PDSCH Open Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols) | Rel-15 |  | C356 | UE supporting E-UTRA TDD and slot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.1.2.2 | TDD slot-PDSCH Closed Loop Spatial Multiplexing Performance (User-Specific Reference Signals) | Rel-15 |  | C355a | UE supporting E-UTRA TDD and Feature Group Indicator 103 and slot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.2.1 | FDD SPDCCH Transmit diversity performance | Rel-15 |  | C355 | UE supporting E-UTRA FDD and Feature Group Indicator 103 and slot/subslot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.2.2 | TDD SPDCCH Transmit diversity performance | Rel-15 |  | C355a | UE supporting E-UTRA TDD and Feature Group Indicator 103 and slot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| Reporting of Channel State Information |  |  |  |  |  |  |  |  |  |  |
| 9.2.1.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.3_C.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for eICIC (non-MBSFN ABS) | Rel-10 |  | C29 | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.4_C.1 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for eICIC (non-MBSFN ABS) | Rel-10 |  | C30 | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.5_E.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.6_E.1 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.7 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for 256QAM in DL | Rel-12 |  | C01h | UE supporting E-UTRA FDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) |  |  | 2Rx, 4Rx |  | Note 7 |
| 9.2.1.8 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for 256QAM in DL | Rel-12 |  | C02h | UE supporting E-UTRA TDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) |  |  | 2Rx, 4Rx |  | Note 7 |
| 9.2.2.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 | Rel-8 |  | C13 b | UE supporting E-UTRA FDD (UE categories >=2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.2.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 | Rel-8 to Rel-14 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 Note 7 |
| 9.2.3.1A | FDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) | Rel-13 |  | C337 | UE supporting E-UTRA FDD and Feature Group Indicator 103 and channeleMeasRestriction-r13 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.3.1_D | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25a | UE supporting E-UTRA FDD and Feature Group Indicators 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.2.3.2A | TDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) | Rel-13 |  | C338 | UE supporting E-UTRA TDD and Feature Group Indicator 103 and channeleMeasRestriction-r13 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.3.2_D | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C26a | UE supporting E-UTRA TDD and Feature Group Indicators 104 and 110 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C26am | UE supporting E-UTRA TDD and Feature Group Indicators 104 and 110 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and Feature Group Indicator 110 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.2.4.1A | FDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured | Rel-13 |  | C339 | UE supporting E-UTRA FDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 and interferenceMeasRestriction-13 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.4.1_F | FDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP | Rel-11 |  | C117 | UE supporting E-UTRA FDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.4.2A | TDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured | Rel-13 |  | C340 | UE supporting E-UTRA TDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 and interferenceMeasRestriction-13 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.4.2_F | TDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP | Rel-11 |  | C118 | UE supporting E-UTRA TDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.5 | TDD CQI Reporting under AWGN conditions – PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured) | Rel-12 |  | C275 | UE supporting E-UTRA TDD and eIMTA TDD UL-DL reconfiguration and Rel-12 CSI subframe sets (UE Category >= 2) |  |  |  |  |  |
| 9.2.6.1 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-0) | Rel-13 |  | C209 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.6.2 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-0) | Rel-13 |  | C217 | UE supporting E-UTRA TDD and downlink LAA with TDD as Pcell |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.7.1 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-1) | Rel-13 |  | C218 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and TM9 on LAA cells |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.7.2 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-1) | Rel-13 |  | C219 | UE supporting E-UTRA TDD and downlink LAA and TM9 on LAA cells |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.1.1.1 | FDD CQI Reporting under fading conditions - PUSCH 3-0 | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.1.1.2 | TDD CQI Reporting under fading conditions - PUSCH 3-0 | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.1.1.5 | TDD CQI Reporting under fading conditions – PUCCH 3-0 (when csi-SubframeSet –r12 is configured) | Rel-12 |  | C276 | UE supporting E-UTRA TDD and Rel-12 CSI subframe sets (UE category >= 1) |  |  |  |  |  |
| 9.3.1.2.1_D | FDD CQI Reporting under fading conditions - PUSCH 3-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 Note 7 |
| 9.3.1.2.2_D | TDD CQI Reporting under fading conditions - PUSCH 3-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25a | UE supporting E-UTRA TDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C25am | UE supporting E-UTRA TDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 Note 7 |
| 9.3.1.2.3 | FDD CQI Reporting under fading conditions - PUSCH 3-1 for 256QAM in DL | Rel-12 to Rel-14 |  | C260 | UE supporting E-UTRA FDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) and Feature Group Indicator 103 |  |  | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C260m | UE supporting E-UTRA FDD and 256QAM in DL and Feature Group Indicator 103 and UE DL Category = 13, or UE supporting E-UTRA FDD and 256QAM in DL and (UE category 11-12 and UE DL category 11, 12, 14 or higher) |  |  |  |  | Note 6 Note 7 |
| 9.3.1.2.4 | TDD CQI Reporting under fading conditions - PUSCH 3-1 for 256QAM in DL | Rel-12 to Rel-14 |  | C261 | UE supporting E-UTRA TDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) and Feature Group Indicator 103 |  |  | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C261m | UE supporting E-UTRA TDD and 256QAM in DL and Feature Group Indicator 103 and UE DL Category = 13, or UE supporting E-UTRA TDD and 256QAM in DL and (UE category 11-12 and UE DL category 11, 12, 14 or higher) |  |  |  |  | Note 6 Note 7 |
| 9.3.1.2.6 | TDD CQI Reporting under fading conditions – PUCCH 3-1 (when csi-SubframeSet –r12 is configured with one CSI process) | Rel-12 |  | C277 | UE supporting E-UTRA TDD and Rel-12 CSI subframe sets and TM10 ( UE Category >= 1). |  |  | 2Rx |  |  |
| 9.3.1.3.1_E.1 | FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C79 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.1.3.2_E.1 | TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C80 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.2.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 | Rel-8 |  | C13 b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.2.1.1_1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 (Release 9 and forward) | Rel-9 |  | C15 | UE supporting E-UTRA FDD (UE category 1) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.2.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-0 | Rel-8 |  | C14 | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.2.1.2_1 | TDD CQI Reporting under fading conditions - PUCCH 1-0 (Release 9 and forward) | Rel-9 |  | C16 | UE supporting E-UTRA TDD (UE category 1) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.2.2.1_D | FDD CQI Reporting under fading conditions - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25x | UE supporting E-UTRA FDD and Feature Group Indicator 103 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25xm | UTRA FDD and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.3.2.2.2_D | TDD CQI Reporting under fading conditions - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C28y | UE supporting E-UTRA TDD and Feature Group Indicators 104 and 110 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C28ym | UE supporting E-UTRA TDD and Feature Group Indicators 104 and 110 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and Feature Group Indicator 110 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.3.3.1.1 | FDD CQI Reporting under fading conditions and frequency-selective interference - PUSCH 3-0 | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.3.1.2 | TDD CQI Reporting under fading conditions and frequency-selective interference - PUSCH 3-0 | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.4.1.1 | FDD CQI Reporting under fading conditions - PUSCH 2-0 | Rel-9 |  | C32 | UE supporting E-UTRA FDD and Feature Group Indicator 1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.4.1.2 | TDD CQI Reporting under fading conditions - PUSCH 2-0 | Rel-9 |  | C37 | UE supporting E-UTRA TDD and Feature Group Indicator 1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.4.2.1 | FDD CQI Reporting under fading conditions - PUCCH 2-0 | Rel-9 |  | C36 | UE supporting E-UTRA FDD and Feature Group Indicator 2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.4.2.2 | TDD CQI Reporting under fading conditions - PUCCH 2-0 | Rel-9 |  | C38 | UE supporting E-UTRA TDD and Feature Group Indicator 2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.5.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A | Rel-11 |  | C44 | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.5.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A | Rel-11 |  | C45 | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.5.2.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A | Rel-11 |  | C44z | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.5.2.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A | Rel-11 |  | C45i | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.6.1_F.1 | FDD CQI Reporting under fading conditions with Single CSI process for CoMP | Rel-11 |  | C50a | UE supporting E-UTRA FDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.1_F.2 | FDD CQI Reporting under fading conditions with Three CSI processes for CoMP | Rel-11 |  | C96 | UE supporting E-UTRA FDD and Maximum CSI processes of Three on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.1_F.3 | FDD CQI Reporting under fading conditions with Four CSI processes for CoMP | Rel-11 |  | C97 | UE supporting E-UTRA FDD and Maximum CSI processes of Four on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.2_F.1 | TDD CQI Reporting under fading conditions with Single CSI process for CoMP | Rel-11 |  | C51a | UE supporting E-UTRA TDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.2_F.2 | TDD CQI Reporting under fading conditions with Three CSI processes for CoMP | Rel-11 |  | C98 | UE supporting E-UTRA TDD and Maximum CSI processes of Three on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.2_F.3 | TDD CQI Reporting under fading conditions with Four CSI processes for CoMP | Rel-11 |  | C99 | UE supporting E-UTRA TDD and Maximum CSI processes of Four on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.7.1 | FDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement | Rel-12 to Rel-14 |  | C25b | UE supporting E-UTRA FDD and Feature Group Indicator 103 and PUSCH feedback mode 3-2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | 9.3.7.1 Note 7 |
|  |  | Rel-15 |  | C25bm | UE supporting E-UTRA FDD and Feature Group Indicator 103 and PUSCH feedback mode 3-2 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and PUSCH feedback mode 3-2 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.3.7.2 | TDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement | Rel-12 to Rel-14 |  | C25c | UE supporting E-UTRA TDD and Feature Group Indicator 103 and PUSCH feedback mode 3-2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25cm | UE supporting E-UTRA TDD and Feature Group Indicator 103 and PUSCH feedback mode 3-2 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and PUSCH feedback mode 3-2 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.3.8.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 (Cell-Specific Reference Symbols) TM4 - Enhanced Receiver Type B | Rel-12 |  | C152 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 (Cell-Specific Reference Symbols) TM4 - Enhanced Receiver Type B | Rel-12 |  | C153 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.2.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B | Rel-12 |  | C152 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.2.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B | Rel-12 |  | C153 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.3.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B | Rel-12 |  | C152 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.3.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B | Rel-12 |  | C153 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.4.1.1.1 | FDD PMI Reporting - PUSCH 3-1 (Single PMI) | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.1.2 | TDD PMI Reporting - PUSCH 3-1 (Single PMI) | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.2.1 | FDD PMI Reporting - PUCCH 2-1 (Single PMI) | Rel-9 |  | C36 | UE supporting E-UTRA FDD and Feature Group Indicator 2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.2.2 | TDD PMI Reporting - PUCCH 2-1 (Single PMI) | Rel-9 |  | C38 | UE supporting E-UTRA TDD and Feature Group Indicator 2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.3.1_D | FDD PMI Reporting - PUSCH 3-1 (Single PMI) for eDL-MIMO | Rel-10 to Rel-14 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.1.3.2_D | TDD PMI Reporting - PUSCH 3-1 (Single PMI) for eDL-MIMO | Rel-10 to Rel-14 |  | C26 | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C26m | UE supporting E-UTRA TDD and Feature Group Indicator 104 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.1.3.3 | FDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO | Rel-13 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.3.4 | TDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO | Rel-13 |  | C14b | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.4.1 | FDD PMI Reporting with 4Tx enhanced codebook - PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement | Rel-12 to Rel-14 |  | C25d | UE supporting E-UTRA FDD and Feature Group Indicator 103 and enhanced 4Tx codebook |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.1.4.2 | TDD PMI Reporting with 4Tx enhanced codebook - PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement | Rel-12 to Rel-14 |  | C25e | UE supporting E-UTRA TDD and Feature Group Indicator 103 and enhanced 4Tx codebook |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25em | UE supporting E-UTRA TDD and Feature Group Indicator 103 and enhanced 4Tx codebook and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and enhanced 4Tx codebook and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.1.4.3 | FDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO | Rel-13 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.4.4 | TDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO | Rel-13 |  | C14b | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.1.1 | FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) | Rel-8 only |  | C11 | UE supporting E-UTRA FDD and operating bands supporting 20 MHz Bandwidth (UE categories 2, 3, 4, 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.1.1_1 | FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) (Release 9 and forward) | Rel-9 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.1.2 | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) | Rel-8 only |  | C12 | UE supporting E-UTRA TDD and operating bands supporting 20 MHz Bandwidth (UE categories 2, 3, 4, 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.1.2_1 | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) (Release 9 and forward) | Rel-9 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.2.1 | FDD PMI Reporting - PUSCH 2-2 (Multiple PMI) | Rel-9 |  | C32 | UE supporting E-UTRA FDD and Feature Group Indicators 1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.2.2 | TDD PMI Reporting - PUSCH 2-2 (Multiple PMI) | Rel-9 |  | C33 | UE supporting E-UTRA TDD and Feature Group Indicators 1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.3.1_D | FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | Rel-10 to Rel-14 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.2.3.2_D | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | Rel-10 to Rel-14 |  | C26 | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C26m | UE supporting E-UTRA TDD and Feature Group Indicator 104 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.2.3.3 | FDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement | Rel-12 to Rel-14 |  | C25d | UE supporting E-UTRA FDD and Feature Group Indicator 103 and enhanced 4Tx codebook |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25dm | UE supporting E-UTRA FDD and Feature Group Indicator 103 and enhanced 4Tx codebook and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and enhanced 4Tx codebook and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.2.3.4 | TDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement | Rel-12 to Rel-14 |  | C25e | UE supporting E-UTRA TDD and eDL-MIMO Enhancement and Feature Group Indicator 103 and enhanced 4Tx codebook |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C25em | UE supporting E-UTRA TDD and Feature Group Indicator 103 and enhanced 4Tx codebook and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and enhanced 4Tx codebook and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.2.3.5 | FDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO | Rel-13 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.3.6 | TDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO | Rel-13 |  | C14b | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.5.1.1 | FDD RI Reporting - PUCCH 1-1 | Rel-8 and Rel-9 only |  | C13a | UE supporting E-UTRA FDD (UE Category 2-5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.1_1 | FDD RI Reporting - PUCCH 1-1 (Release 10) | Rel-10 only |  | C13 | UE supporting E-UTRA FDD (UE Category 2-8) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.1_2 | FDD RI Reporting- PUCCH 1-1 (Release 11) | Rel-11 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.2 | TDD RI Reporting - PUSCH 3-1 | Rel-8 and Rel-9 only |  | C14a | UE supporting E-UTRA TDD (UE Category 2-5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.2_1 | TDD RI Reporting - PUSCH 3-1 (Release 10) | Rel-10 only |  | C14 | UE supporting E-UTRA TDD (UE Category 2-8) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.2_2 | TDD RI Reporting- PUSCH 3-1 (Release 11) | Rel-11 |  | C14b | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.2.1_D | FDD RI Reporting - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25x | UE supporting E-UTRA FDD and Feature Group Indicators 103 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C25xm | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.5.2.2_D | TDD RI Reporting - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25y | UE supporting E-UTRA TDD and Feature Group Indicator 103 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C25ym | UE supporting E-UTRA TDD and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.5.3.1_C.1 | FDD RI Reporting - PUCCH 1-0 for eICIC (non-MBSFN ABS) | Rel-10 |  | C29 | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.3.2_C.1 | TDD RI Reporting - PUCCH 1-0 for eICIC (non-MBSFN ABS) | Rel-10 |  | C30 | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.4.1_E.1 | FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.4.2_E.1 | TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.5.1_F.1 | FDD RI Reporting with Single CSI processes for CoMP | Rel-11 |  | C50 | UE supporting E-UTRA FDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.5.5.1_F.2 | FDD RI Reporting with Multiple CSI processes for CoMP | Rel-11 |  | C52 | UE supporting E-UTRA FDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.5.5.2_F.1 | TDD RI Reporting with Single CSI process for CoMP | Rel-11 |  | C51 | UE supporting E-UTRA TDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.5.5.2_F.2 | TDD RI Reporting with Multiple CSI processes for CoMP | Rel-11 |  | C53 | UE supporting E-UTRA TDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.6.1.1_A.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (2 DL CA) | Rel-10 |  | C108 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 3) |  | Refer to 36.521-1 9.1.1.2 | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.2 is executed. Note 7 |
|  |  | Rel-11 |  | C103 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA(UE Category >= 3) |  |  |  |  |  |
| 9.6.1.1_A.2 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (3 DL CA) | Rel-10 |  | C124 | UE supporting E-UTRA FDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA (UE Category >= 5) |  | Refer to 36.521-1 9.1.1.2 | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.3 is executed. Note 7 |
|  |  | Rel-11 |  | C125 | UE supporting E-UTRA FDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.3 is executed. Note 7 |
| 9.6.1.1_A.3 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (4DL CA) | Rel-11 |  | C192 | UE supporting E-UTRA FDD and 4DL with intra-band contiguous CA, or 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA (UE Category >= 8) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.4 is executed. Note 7 |
|  |  | Rel-11 |  | C193 | UE supporting E-UTRA FDD and 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 8) |  |  |  |  | Test execution not necessary if 9.6.1.1_A.4 is executed. |
| 9.6.1.1_A.4 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (5DL CA) | Rel-11 |  | C192a | UE supporting E-UTRA FDD and 5DL with Intra-band contiguous and Inter-band CA or 5DL with Intra-band non-contiguous and Inter-band CA or 5DL with Intra-band non-contiguous and Intra-band contiguous CA (UE Category 8 and >= 11) |  |  | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-12 |  | C193a | UE supporting E-UTRA FDD and 5DL Inter-band CA (UE Category 8 and>= 11) |  |  |  |  |  |
| 9.6.1.2_A.1 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (2DL CA) | Rel-10 |  | C114 | UE supporting E-UTRA TDD and intra-band contiguous DL CA (UE Category >= 3) |  | Refer to 36.521-1 9.1.1.2 | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.2_A.2 is executed. Note 7 |
| 9.6.1.2_A .2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (3 DL CA) | Rel-10 |  | C128 | UE supporting E-UTRA TDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA (UE Category >= 5) |  | Refer to 36.521-1 9.1.1.2 | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.2_A.3 is executed. Note 7 |
|  |  | Rel-11 |  | C129 | UE supporting E-UTRA TDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 5) |  |  |  |  | Test execution not necessary if 9.6.1.2_A.3 is executed. |
| 9.6.1.2_A.3 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (4 DL CA) | Rel-11 |  | C270 | UE supporting E-UTRA TDD and 4DL with intra-band contiguous CA, or 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA (UE Category >= 8) |  |  |  |  | Test execution not necessary if 9.6.1.2_A.4 is executed. |
|  |  | Rel-11 |  | C271 | UE supporting E-UTRA TDD and 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 8) |  |  |  |  | Test execution not necessary if 9.6.1.2_A.4 is executed. |
| 9.6.1.2_A.4 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (5 DL CA) | Rel-11 |  | C272 | UE supporting E-UTRA TDD and 5DL with intra-band contiguous CA, or 5DL with inter-band CA, or 5DL with intra-band contiguous and inter-band CA (UE Category 8 and >=11) |  |  |  |  |  |
|  |  | Rel-11 |  | C273 | UE supporting E-UTRA TDD and 5DL with intra-band non-contiguous and inter-band CA, or 5DL with intra-band non-contiguous and intra-band contiguous CA (UE Category 8 and >=11) |  |  |  |  |  |
| 9.6.1.3.1 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (2DL CA) | Rel-12 |  | C132 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 3) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.3.2 is executed. Note 7 |
| 9.6.1.3.2 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (3DL CA) | Rel-12 |  | C133 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.3.3 is executed. Note 7 |
| 9.6.1.3.3 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (4DL CA) | Rel-12 |  | C133a | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.3.4 is executed. Note 7 |
| 9.6.1.3.4 | TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (5DL CA) | Rel-12 |  | C133b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 9.6.1.4.1 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for TDD PCell (2DL CA) | Rel-12 |  | C134 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >= 3) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.4.2 is executed. Note 7 |
| 9.6.1.4.2 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for TDD PCell (3DL CA) | Rel-12 |  | C135 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.4.3 is executed. Note 7 |
| 9.6.1.4.3 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for TDD PCell (4DL CA) | Rel-12 |  | C135a | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.4.4 is executed. Note 7 |
| 9.6.1.4.4 | TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (5DL CA) | Rel-12 |  | C133b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 9.7.1.1 | FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category 0 | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.7_1.1.1 | FDD CQI reporting under AWGN conditions – PUCCH 1-0 for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting |  |  |  |
| 9.7_1.1.2 | TDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting |  |  |  |
| 9.7_1.1.3 | FDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting |  |  |  |
| 9.7_1.1.4 | TDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting |  |  |  |
| 9.7.1.2 | TDD CQI reporting definition under AWGN conditions for UE category 0 | Rel-12 |  | C119 | UE supporting E-UTRA TDD (UE category 0) |  |  |  |  |  |
| 9.7.2.1 | FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category 0 | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.7.2.2 | TDD CQI reporting definition under fading conditions for UE category 0 | Rel-12 |  | C156 | UE supporting E-UTRA TDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.1.1 | FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE supporting coverage enhancement | Rel-13 |  | C145a | UE supporting E-UTRA FDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.1.2 | TDD CQI reporting definition under AWGN conditions for UE supporting coverage enhancement | Rel-13 |  | C156a | UE supporting E-UTRA TDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.2.1 | FDD and Half-duplex FDD UE-selected subband CQI for UE supporting coverage enhancement | Rel-13 |  | C145a | UE supporting E-UTRA FDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.2.2 | TDD UE-selected subband CQI for UE supporting coverage enhancement | Rel-13 |  | C156a | UE supporting E-UTRA TDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.3.1 | FDD and half duplex FDD CQI reporting under AWGN conditions for UE supporting coverage enhancement and 64QAM | Rel-15 |  | C145i | UE supporting E-UTRA FDD and CEModeA and 64QAM |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.3.2 | TDD CQI reporting under AWGN conditions for UE supporting coverage enhancement and 64QAM | Rel-15 |  | C156k | UE supporting E-UTRA TDD and CEModeA and 64QAM |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.4.1 | FDD and half duplex FDD CQI reporting under AWGN conditions for UE supporting coverage enhancement alternative table | Rel-15 |  | C145j | UE supporting E-UTRA FDD and CEModeA and alternative table |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.4.2 | TDD CQI reporting under AWGN conditions for UE supporting coverage enhancement alternative table | Rel-15 |  | C156l | UE supporting E-UTRA TDD and CEModeA and alternative table |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.1.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 with Rank 1 1x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.1.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 with Rank 1 1x4 | Rel-10 |  | C177 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.2.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 2 4x4 | Rel-10 to Rel-14 |  | C178 | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C178m | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.1.2.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 2 8x4 | Rel-10 to Rel-14 |  | C179 | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C179m | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.1.3.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 4 4x4 | Rel-10 |  | C180 | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category >= 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.3.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 4 4x4 | Rel-10 |  | C181 | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category >= 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.4.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 3 4x4 | Rel-10 to Rel-14 |  | C182 | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C182m | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.1.4.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 3 4x4 | Rel-10 to Rel-14 |  | C183 | UE supporting E-UTRA TDD andUE Category ≥ 5 and Feature Group Indicator 103 with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C183m | UE supporting E-UTRA TDD and Feature Group Indicator 103 with 4Rx antenna ports and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.2.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 | Rel-11 |  | C197 | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 9.9.2.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 | Rel-11 |  | C198 | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 9.9.2.2.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A2 x4 | Rel-11 to Rel-14 |  | C199 | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C199m | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.2.2.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A2 x4 | Rel-11 to Rel-14 |  | C200 | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C200m | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.3.1.1 | TDD PMI Reporting - PUSCH 3-1 (Single PMI) 8x4 | Rel-10 to Rel-14 |  | C179 | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports (UE Category >= 2) |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C179m | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.4.1.1 | FDD RI Reporting- PUCCH 1-1 4x4 | Rel-10 |  | C203 | UE supporting E-UTRA FDD with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 9.9.4.1.2 | TDD RI Reporting- PUSCH 3-1 4x4 | Rel-10 |  | C204 | UE supporting E-UTRA TDD with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 9.9.4.2.1 | FDD RI Reporting- PUCCH 1-1 for eDL-MIMO 4x4 | Rel-10 to Rel-14 |  | C205 | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C205m | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.4.2.2 | TDD RI Reporting- PUCCH 1-1 for eDL-MIMO 4x4 | Rel-10 to Rel-14 |  | C206 | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C206m | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.10.1.1 | FDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 | Rel-13 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.10.1.2 | TDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 | Rel-13 |  | C341 | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.12.1.1 | FDD CQI reporting under fading conditions for slot/subslot TTI (Cell-Specific Reference Symbol) | Rel-15 |  | C354 | UE supporting E-UTRA FDD and slot/subslot TTI (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.12.1.2 | TDD CQI reporting under fading conditions for slot/subslot TTI (Cell-Specific Reference Symbol) | Rel-15 |  | C354a | UE supporting E-UTRA TDD and slot TTI (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.12.2.1 | FDD CQI reporting under fading conditions for slot/subslot TTI (CSI Reference Symbol) | Rel-15 |  | C357 | UE supporting E-UTRA FDD and Feature Group Indicators 103 and slot/subslot TTI (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.12.2.2 | TDD CQI reporting under fading conditions for slot/subslot TTI (CSI Reference Symbol) | Rel-15 |  | C357a | UE supporting E-UTRA TDD and Feature Group Indicators 103 slot TTI (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| MBMS Performance Testing |  |  |  |  |  |  |  |  |  |  |
| 10.1 | FDD MBMS performance (Fixed Reference Channel) | Rel-9 | C03 |  | UE supporting E-UTRA FDD and MBMS |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 10.1_1 | FDD MBMS performance (Fixed Reference Channel) (Release 13 and forward) | Rel-13 | C03 |  | UE supporting E-UTRA FDD and MBMS |  | Performed once |  |  |  |
| 10.2 | TDD MBMS performance (Fixed Reference Channel) | Rel-9 | C04 |  | UE supporting E-UTRA TDD and MBMS |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 10.2_1 | TDD MBMS performance (Fixed Reference Channel) (Release 13 and forward) | Rel-13 | C04 |  | UE supporting E-UTRA TDD and MBMS |  | Performed once |  |  |  |
| V2X Sidelink Performance Testing |  |  |  |  |  |  |  |  |  |  |
| 14.2 | Demodulation of PSSCH / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 |  | UE supporting V2X Sidelink communication |  |  |  |  |  |
| 14.3 | Demodulation of PSCCH / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 |  | UE supporting V2X Sidelink communication |  |  |  |  |  |
| 14.4 | Power imbalance performance with two links / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 |  | UE supporting V2X Sidelink communication |  |  |  |  |  |
| 14.6 | Demodulation of PSSCH with eNB based synchronization | Rel-14 | C320 |  | UE supporting E-UTRA and V2X Sidelink communication |  | E16 | FDD,TDD |  |  |
| 14.7 | Soft buffer test | Rel-14 | C313 |  | UE supporting V2X Sidelink communication |  | D14 | TDD |  |  |
| 14.8 | PSCCH/PSSCH decoding capability test / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C321 |  | UE supporting V2X Sidelink communication |  |  |  |  |  |
| 14.9 | Sustained downlink data rate with active sidelink | Rel-14 | C320 |  | UE supporting E-UTRA and V2X Sidelink communication |  | E16 | FDD,TDD |  |  |
| Note 1: Due to UE capability signalling for UL 64QAM is introduced from Rel-12, this test case can optionally be executed with a Rel-12 UE. Note 2: For a transition period until RAN5#72, this condition in version 13.0.0 of 36.521-2 shall be used. This is to ensure no test coverage is lost before the UL 64QAM test case becomes available. Note 3: Equivalent aggregated bandwidth is defined as: . Where is number of CCs, and is MIMO layer and bandwidth of CC . The number of MIMO layer for CC in each CA configuration is according to Table A.4.6.1-3, Table A.4.6.2-3, Table A.4.6.3-3, Table A.4.6.3-4 or Table A.4.6.3-5. Note 4: For a transition period until RAN5#83, this condition in version 15.3.1 of 36.521-2 shall be used. This is to ensure no test coverage is lost before the UL 256QAM test cases become available. Note 5: Void. Note 6: The categories for which FGI 103/104 are mandated are (UE category >=11 OR UE DL Category = 11 OR UE DL Category = 12 OR UE DL Category >= 14) according to TS 36.331. The UE DL Categories higher than 14 are not explicitly listed in the corresponding expression in Table 4.1-1a because in this version of core specification UE DL Category > 14 requires supporting of UE Category 11 or 12. Note 7: Skipping 2RX testing if UE is verified with four RX antenna ports in operating bands where it is equipped with four RX antenna port. |  |  |  |  |  |  |  |  |  |  |

Table 4.1-1a: Applicability of RF conformance test cases Conditions

C01 IF NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 THEN R ELSE N/A

C01h IF (A.4.1-1/1 AND A.4.5-1/18) THEN R ELSE N/A

C02 IF NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 THEN R ELSE N/A

C02h IF (A.4.1-1/2 AND A.4.5-1/18) THEN R ELSE N/A

C03 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/1) THEN R ELSE N/A

C04 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/1) THEN R ELSE N/A

C05 Void

C06 Void

C07 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-3b/2 AND A.4.2-1/3) THEN R ELSE N/A

C08 Void

C09 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-3a/1) THEN R ELSE N/A

C10 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-3a/1) THEN R ELSE N/A

C11 IF A.4.1-1/1 AND A.4.3-3a/6 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5) THEN R ELSE N/A

C12 IF A.4.1-1/2 AND A.4.3-3a/6 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5) THEN R ELSE N/A

C13 IF ((A.4.1-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C13a IF ((A.4.1-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5)) THEN R ELSE N/A

C13b IF ((A.4.1-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 )) THEN R ELSE N/A

C14 IF ((A.4.1-1/2) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C14a IF ((A.4.1-1/2) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5)) THEN R ELSE N/A

C14b IF ((A.4.1-1/2) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C15 IF (A.4.1-1/1 AND A.4.3-4/1) THEN R ELSE N/A

C16 IF (A.4.1-1/2 AND A.4.3-4/1) THEN R ELSE N/A

C17 Void

C18 Void

C19 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2) THEN R ELSE N/A

C19a IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/3 AND A.4.6.1-2/3) THEN R ELSE N/A

C19h IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2 AND A.4.5-1/18) THEN R ELSE N/A

C20 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2) AND NOT A.4.6.1-2/2) THEN R ELSE N/A

C20h IF ((A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2) AND (NOT (A.4.6.1-2/1 OR A.4.6.1-2/2)) AND A.4.5-1/18) THEN R ELSE N/A

C21 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1) THEN R ELSE N/A

C21h IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1) AND A.4.5-1/18 THEN R ELSE N/A

C22 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.6.1-1/2) THEN R ELSE N/A

C23 Void

C24 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.6.1-1/2) THEN R ELSE N/A

C25 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.4-3a/103) THEN R ELSE N/A

C25m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C25a IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2) AND A.4.4-3b/103) THEN R ELSE N/A

C25am IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2) AND ((A.4.4-3b/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C25b IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.5-1/81 AND A.4.4-3a/103) THEN R ELSE N/A

C25bm IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.5-1/81 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C25c IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2) AND A.4.5-1/81 AND A.4.4-3b/103) THEN R ELSE N/A

C25cm IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2) AND A.4.5-1/81 AND ((A.4.4-3b/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C25d IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.5-1/80 AND A.4.4-3a/103) THEN R ELSE N/A

C25dm IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.5-1/80 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C25e IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2) AND A.4.5-1/80 AND A.4.4-3b/103) THEN R ELSE N/A

C25em IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2) AND A.4.5-1/80 AND ((A.4.4-3b/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C25h IF (A.4.1-1/1 AND A.4.4-3a/103 AND A.4.5-1/18) THEN R ELSE N/A

C25hm IF (A.4.1-1/1 AND A.4.5-1/18 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C25x IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.4-3a/103) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C25xm IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND ((A.4.4-3a/103 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C25y IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3b/103) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C25ym IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND ((A.4.4-3b/103 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C26 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3b/104)) THEN R ELSE N/A

C26m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND ((A.4.4-3b/104 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C26a IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-3b/104 AND A.4.4-3b/110) THEN R ELSE N/A

C26am IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-3b/110 AND ((A.4.4-3b/104 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C26h IF (((A.4.1-1/1 AND A.4.4-3a/104) OR (A.4.1-1/2 AND A.4.4-3b/104)) AND A.4.5-1/18) THEN R ELSE N/A

C26hm IF ((((A.4.1-1/1 AND A.4.4-3a/104) OR (A.4.1-1/2 AND A.4.4-3b/104)) AND A.4.5-1/18 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/18 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C27 Void

C28 Void

C28y IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3a/104 AND A.4.4-3a/110) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C28ym IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3a/110) AND ((A.4.4-3a/104 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C29 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.4-3a/115) THEN R ELSE N/A

C30 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-3b/115) THEN R ELSE N/A

C31 IF (A.4.1-1/1 AND (A.4.3-4/1 OR A.4.3-4/2)) THEN R ELSE N/A

C32 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.4-1a/1) THEN R ELSE N/A

C33 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-1b/1) THEN R ELSE N/A

C34 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/5) THEN R ELSE N/A

C35 Void

C36 IF NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.4-1a/2 THEN R ELSE N/A

C37 IF NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-1b/1 THEN R ELSE N/A

C38 IF NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-1b/2 THEN R ELSE N/A

C39 IF(NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3-3b/1 OR A.4.3-3b/4)) THEN R ELSE N/A

C39a IF(NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-3b/1) THEN R ELSE N/A

C39b IF(NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-3b/4) THEN R ELSE N/A

C39c IF(NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3-3b/1 OR A.4.3-3b/4) THEN R ELSE N/A

C40 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.4-3a/103 AND A.4.3-7/1) THEN R ELSE N/A

C40m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/1 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C41 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-3b/103 AND A.4.3-7/1) THEN R ELSE N/A

C41m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/1 AND ((A.4.4-3b/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C42 IF ((A.4.1-1/1) AND (NOT A.4.5-1/18) AND (A.4.3-4/6 OR A.4.3-4/7)) THEN R ELSE N/A

C42h IF ((A.4.1-1/1) AND (A.4.3-4/6 OR A.4.3-4/7) AND A.4.5-1/18 AND A.4.3-4a/8) THEN R ELSE N/A

C43 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND NOT A.4.6.2-2/1) THEN R ELSE N/A

C43h IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND NOT A.4.6.2-2/1 AND A.4.5-1/18) THEN R ELSE N/A

C44 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/1) THEN R ELSE N/A

C44z IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/1 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8) AND A.4.4-3a/103) THEN R ELSE N/A

C45 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/1) THEN R ELSE N/A

C45i IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/1 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8) AND A.4.4-3b/103) THEN R ELSE N/A

C46 Void

C47 Void

C48 Void

C49 Void

C50 IF (A.4.1-1/1 AND A.4.5-1/8 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C50a IF (A.4.1-1/1 AND A.4.5-1/8) THEN R ELSE N/A

C51 IF (A.4.1-1/2 AND A.4.5-1/8 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C51a IF (A.4.1-1/2 AND A.4.5-1/8) THEN R ELSE N/A

C52 IF (A.4.1-1/1 AND (A.4.5-1/11 OR A.4.5-1/12) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C53 IF (A.4.1-1/2 AND (A.4.5-1/11 OR A.4.5-1/12) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C54 IF (A.4.1-1/2 AND (A.4.3-4/1 OR A.4.3-4/2)) THEN R ELSE N/A

C55 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/6) THEN R ELSE N/A

C56 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/6) THEN R ELSE N/A

C57 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/6 AND (A.4.5-1/11 OR A.4.5-1/12)) THEN R ELSE N/A

C58 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/6 AND (A.4.5-1/11 OR A.4.5-1/12)) THEN R ELSE N/A

C59 Void

C60 Void

C61 Void

C62 void

C63 void

C64 Void

C65 Void

C66 Void

C67 Void

C68 Void

C69 void

C70 void

C71 Void

C72 IF ((A.4.1-1/2) AND (A.4.6.2-1/1) AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C73 IF ((A.4.1-1/2) AND (NOT A.4.5-1/18) AND (A.4.3-4/6 OR A.4.3-4/7)) THEN R ELSE N/A

C73h IF ((A.4.1-1/2) AND A.4.5-1/18 AND A.4.3-4a/8) THEN R ELSE N/A

C74 IF ((A.4.1-1/2) AND (NOT A.4.5-1/18) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) THEN R ELSE N/A

C74h IF ((A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/8)) THEN R ELSE N/A

C75 IF ((A.4.1-1/2) AND (NOT A.4.5-1/18) AND (A.4.6.2-1/1) AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) THEN R ELSE N/A

C76 IF A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.3-4/1 OR A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4) THEN R ELSE N/A

C77 IF (A.4.1-1/1 AND A.4.5-2/1 AND A.4.4-3a/115 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C78 IF (A.4.1-1/2 AND A.4.5-2/1 AND A.4.5-2/2 AND A.4.4-3b/115 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C79 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.5-2/1 AND A.4.4-3a/115) THEN R ELSE N/A

C80 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.5-2/1 AND A.4.5-2/2 AND A.4.4-3b/115) THEN R ELSE N/A

C81 void

C82 void

C83 IF ((A.4.1-1/2) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7) AND (A.4.6.3-1/1)) THEN R ELSE N/A

C84 void

C85 Void

C86 Void

C87 void

C88 Void

C89 Void

C90 IF ((A.4.1-1/1) AND (A.4.6.2-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C91 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/6 AND A.4.4-3a/103) THEN R ELSE N/A

C91m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/6 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C92 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/6 AND A.4.4-3b/103) THEN R ELSE N/A

C92m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/6 AND ((A.4.4-3b/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C93 IF ((A.4.1-1/1) AND (NOT A.4.5-1/18) AND (A.4.6.2-1/1) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) THEN R ELSE N/A

C94 Void

C95 void

C96 IF (A.4.1-1/1 AND A.4.5-1/11) THEN R ELSE N/A

C97 IF (A.4.1-1/1 AND A.4.5-1/12) THEN R ELSE N/A

C98 IF (A.4.1-1/2 AND A.4.5-1/11) THEN R ELSE N/A

C99 IF (A.4.1-1/2 AND A.4.5-1/12) THEN R ELSE N/A

C100 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/13) THEN R ELSE N/A

C101 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 or A.4.6.3-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C102 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C103 IF ((A.4.1-1/1) AND (A.4.6.2-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C104 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C105 IF ((A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C106 IF ((A.4.1-1/1) AND (A.4.6.2-1/1) AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C107 IF ((A.4.1-1/1) AND (NOT A.4.5-1/18) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) THEN R ELSE N/A

C107h IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/8)) THEN R ELSE N/A

C108 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C109 IF (A.4.1-1/2 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8) AND (A.4.6.2-1/1 OR A.4.6.3-1/1)) THEN R ELSE N/A

C110 IF (A.4.1-1/2 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8) AND (A.4.6.1-1/1 OR A.4.6.1-1/2)) THEN R ELSE N/A

C111 IF A.4.1-1/2 AND (NOT A.4.5-1/18) AND (A.4.3-4/1 OR A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4) THEN R ELSE N/A

C112 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-4a/1) THEN R ELSE N/A

C112a IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-4aa/1) THEN R ELSE N/A

C112b IF (A.4.1-1/8 OR A.4.1-1/8a) THEN R ELSE N/A

C112c IF ((A.4.1-1/1 OR A.4.1-1/2) AND( A.4.3-4a/1a OR A.4.3-4b/1a)) THEN R ELSE N/A

C112d IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-4aa/1 AND A.4.3-4aa/2) THEN R ELSE N/A

C113 IF (A.4.1-1/1 OR A.4.1-1/2) AND NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) THEN R ELSE N/A

C113a IF (A.4.5-1/22) THEN R ELSE N/A

C113b IF (A.4.1-1/1 AND A.4.5-1/37) THEN R ELSE N/A

C113c IF (A.4.1-1/1 AND A.4.5-1/37 AND A.4.4-3a/103) THEN R ELSE N/A

C113cm IF (A.4.1-1/1 AND A.4.5-1/37 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C113d IF (A.4.1-1/1 AND A.4.5-1/37 AND A.4.3-7/1) THEN R ELSE N/A

C113e IF (A.4.1-1/1 AND A.4.5-1/37 AND A.4.3-7/1 AND A.4.4-3a/103) THEN R ELSE N/A

C113em IF (A.4.1-1/1 AND A.4.5-1/37 AND A.4.3-7/1 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C113h IF (A.4.5-1/18) THEN R ELSE N/A

C114 IF (A.4.1-1/2 AND A.4.6.1-1/2) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10) THEN R ELSE N/A

C115 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND A.4.6.2-2/1) THEN R ELSE N/A

C116 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1) THEN R ELSE N/A

C116a IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/3 AND A.4.6.3-2/2) THEN R ELSE N/A

C116b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND NOT (A.4.5-1/17 OR A.4.5-1/58) THEN R ELSE N/A

C117 IF (A.4.1-1/1 AND (A.4.5-1/8 OR A.4.5-1/11 OR A.4.5-1/12) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C118 IF (A.4.1-1/2 AND (A.4.5-1/8 OR A.4.5-1/11 OR A.4.5-1/12) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C119 IF A.4.1-1/2 AND A.4.3-4a/1 THEN R ELSE N/A

C120 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/5) THEN R ELSE N/A

C121 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4)) THEN R ELSE N/A

C122 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.3-1/2 OR A.4.6.2-1/2)) THEN R ELSE N/A

C122h IF ((A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4 OR A.4.6.3-1/2 OR A.4.6.2-1/2)) AND A.4.5-1/18 THEN R ELSE N/A

C123 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2) THEN R ELSE N/A

C124 IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C125 IF (A.4.1-1/1 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C125a IF (A.4.1-1/1 AND (A.4.6-1/2 OR A.4.6-1/3 OR A.4.6-1/4 OR A.4.6-1/5 OR A.4.6-1/6) AND A.4.6.1-1/2 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C126 IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C126a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/6 OR A.4.3-4/7) AND A.4.3-3a/9) THEN R ELSE N/A

C126h IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C126ha IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/18 AND ( A.4.3-4a/8) AND A.4.3-3a/9) THEN R ELSE N/A

C127 IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C127a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/6 OR A.4.3-4/7) AND A.4.3-3a/9) THEN R ELSE N/A

C128 IF (A.4.1-1/2 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C129 IF (A.4.1-1/2 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C130 IF (A.4.1-1/2 AND (NOT A.4.5-1/18) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C130h IF (A.4.1-1/2 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C131 IF (A.4.1-1/2 AND (NOT A.4.5-1/18) AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C132 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C133 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C133a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C133b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/15 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C134 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C135 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C135a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/14 AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C135b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/14 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C136 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C137 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C138 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10) THEN R ELSE N/A

C138h IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C139 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) THEN R ELSE N/A

C139a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C139b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND A.4.3-4a/10) THEN R ELSE N/A

C139c IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND A.4.3-4a/10 AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23)) THEN R ELSE N/A

C139hc IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.3-4a/11) AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23)) THEN R ELSE N/A

C139h IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C139ha IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C139hb IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.3-4a/11)) THEN R ELSE N/A

C140 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10) THEN R ELSE N/A

C140h IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/8)) THEN R ELSE N/A

C141 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) THEN R ELSE N/A

C141a IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12) THEN R ELSE N/A

C141b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND A.4.3-4a/10 THEN R ELSE N/A)

C141c IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND A.4.3-4a/10 AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23)) THEN R ELSE N/A)

C141h IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C141hb IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND A.4.5-1/18 AND A.4.3-4a/11) THEN R ELSE N/A

C142 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/3) THEN R ELSE N/A

C142h IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.5-1/14 AND A.4.5-1/18 AND A.4.3-4a/11 AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23)) THEN R ELSE N/A

C143 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/3) THEN R ELSE N/A

C144 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/3 AND A.4.4-3a/103) THEN R ELSE N/A

C144m IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/3 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C145 IF A.4.1-1/1 AND A.4.3-4a/1 THEN R ELSE N/A

C145a IF A.4.1-1/1 AND A.4.3-4aa/1 THEN R ELSE N/A

C145b IF A.4.1-1/1 AND (A.4.3-4aa/1 OR A.4.5-1/25) THEN R ELSE N/A

C145c Void

C145d IF A.4.1-1/1 AND A.4.3-4a/1a THEN R ELSE N/A

C145e IF A.4.1-1/1 AND A.4.3-4aa/1 AND A.4.5-1/63 THEN R ELSE N/A

C145f IF A.4.1-1/1 AND A.4.3-4aa/2 AND A.4.5-1/63 THEN R ELSE N/A

C145g IF A.4.1-1/1 AND A.4.3-4aa/1 AND A.4.5-1/51 THEN R ELSE N/A

C145h IF A.4.1-1/1 AND A.4.3-4aa/2 AND A.4.5-1/51 THEN R ELSE N/A

C145i IF A.4.1-1/1 AND A.4.5-1/25 AND A.4.5-1/84 THEN R ELSE N/A

C145j IF A.4.1-1/1 AND A.4.5-1/25 AND A.4.5-1/85 THEN R ELSE N/A

C146 IF (NOT(A.4.3-4a/1) AND (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6.3-1/1 AND (A.4.5-1/14 OR A.4.5-1/15)) THEN R ELSE N/A

C147 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/17) THEN R ELSE N/A

C148 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2 AND A.4.5-1/17) THEN R ELSE N/A

C149 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1 OR A.4.5-1/58) AND A.4.1-1/1 AND A.4.5-1/13 AND A.4.5-1/17) THEN R ELSE N/A

C150 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/4) THEN R ELSE N/A

C151 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/4) THEN R ELSE N/A

C152 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/4) THEN R ELSE N/A

C153 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/4) THEN R ELSE N/A

C154 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C155 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C156 IF A.4.1-1/2 AND A.4.3-4a/1 THEN R ELSE N/A

C156a IF A.4.1-1/2 AND A.4.3-4aa/1 THEN R ELSE N/A

C156b IF A.4.1-1/2 AND (A.4.3-4aa/1 OR A.4.5-1/25) THEN R ELSE N/A

C156c IF A.4.1-1/1 AND (A.4.3-4aa/1 AND A.4.5-1/26 THEN R ELSE N/A

C156d IF A.4.1-1/2 AND (A.4.3-4aa/1 AND A.4.5-1/26 THEN R ELSE N/A

C156e IF A.4.1-1/2 AND (A.4.3-4aa/1 OR A.4.5-1/25) AND A.4.5-1/51 THEN R ELSE N/A

C156f IF A.4.1-1/2 AND A.4.3-4a/1a THEN R ELSE N/A

C156q IF A.4.1-1/2 AND (A.4.3-4aa/1 OR A.4.5-1/25) AND A.4.5-1/63 THEN R ELSE N/A

C156h IF A.4.1-1/2 AND A.4.3-4aa/2 AND A.4.5.1/63 THEN R ELSE N/A

C156i IF A.4.1-1/2 AND A.4.3-4aa/2 AND A.4.5.1/51 THEN R ELSE N/A

C156j IF A.4.1-1/1 AND (A.4.3-4aa/1 AND A.4.3-4aa/2) AND A.4.5-1/26 THEN R ELSE N/A

C156k IF A.4.1-1/2 AND A.4.5-1/25 AND A.4.5-1/84 THEN R ELSE N/A

C156l IF A.4.1-1/2 AND A.4.5-1/25 AND A.4.5-1/85 THEN R ELSE N/A

C157 IF A.4.1-1/1 AND A.4.3-4a/1 AND A.4.4-3a/103 THEN R ELSE N/A

C157a IF A.4.1-1/1 AND A.4.3-4a/1a AND A.4.4-3a/103 THEN R ELSE N/A

C158 IF A.4.1-1/2 AND A.4.3-4a/1 AND A.4.4-3b/103 THEN R ELSE N/A

C158a IF A.4.1-1/2 AND A.4.3-4a/1a AND A.4.4-3b/103 THEN R ELSE N/A

C159 IF (NOT(A.4.3-4a/1 OR A.4.5-1/17 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/13) THEN R ELSE N/A

C160 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.5-1/17) THEN R ELSE N/A

C161 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND A.4.6.2-2/1 AND A.4.5-1/17) THEN R ELSE N/A

C162 IF A.4.5-1/23 THEN R ELSE N/A

C163 IF A.4.5-1/24 THEN R ELSE N/A

C164 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.5-1/37) THEN R ELSE N/A

C165 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.5-1/38) THEN R ELSE N/A

C166 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.4-3a/103 AND A.4.5-1/37) THEN R ELSE N/A

C166m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.5-1/37 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C167 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.4-3a/103 AND A.4.5-1/38) THEN R ELSE N/A

C167m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.5-1/38 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C168 IF (A.4.5-1/22 AND NOT A.4.5-1/18) THEN R ELSE N/A

C169 IF A.4.1-1/1 AND A.4.2-1/8 AND NOT (A.4.3-4/1 OR A.4.3-4/2 OR A.4.3-4a/1) THEN R ELSE N/A

C170 IF A.4.1-1/2 AND A.4.2-1/8 AND NOT (A.4.3-4/1 OR A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4a/1) THEN R ELSE N/A

C171 IF A.4.1-1/1 AND A.4.2-1/8 AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10) AND (NOT A.4.5-1/18) THEN R ELSE N/A

C172 IF A.4.1-1/2 AND A.4.2-1/8 AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10) AND (NOT A.4.5-1/18) THEN R ELSE N/A

C173 IF A.4.1-1/1 AND A.4.2-1/8 AND A.4.5-1/18 THEN R ELSE N/A

C174 IF A.4.1-1/2 AND A.4.2-1/8 AND A.4.5-1/18 THEN R ELSE N/A

C175 IF (NOT(A.4.3-4a/1) AND A.4.1-1/1 AND A.4.5-1/12 AND A.4.3-7/4) THEN R ELSE N/A

C176 IF (NOT(A.4.3-4a/1) AND A.4.1-1/2 AND A.4.5-1/12 AND A.4.3-7/4) THEN R ELSE N

C177 IF (A.4.5-1/38) THEN R ELSE N/A

C178 IF (A.4.4-3a/103 AND A.4.5-1/37 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C178m IF (A.4.5-1/37 AND ((A.4.4-3a/103 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C179 IF (A.4.4-3a/104 AND A.4.5-1/38 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C179m IF (A.4.5-1/38 AND ((A.4.4-3a/104 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C180 IF (A.4.5-1/37 AND A.4.5-1/46 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C181 IF (A.4.5-1/38 AND A A.4.5-1/46 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C182 IF (A.4.4-3a/103 AND A.4.5-1/37 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C182m IF (A.4.5-1/37 AND ((A.4.4-3a/103 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C183 IF (A.4.4-3b/103 AND A.4.5-1/38 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C183m IF (A.4.5-1/38 AND ((A.4.4-3b/103 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C184 IF (A.4.5-1/38) THEN R ELSE N/A

C185 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND A.4.6.2-2/1 AND A.4.5-1/17) THEN R ELSE N/A

C186 IF A.4.3-3b/2 AND NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) THEN R ELSE N/A

C187 IF ((A.4.1-1/1 OR A.4.1-1/2) AND ((A.4.1-1/2 AND A.4.6.1-1/4) OR A.4.6.3-1/6 OR A.4.6.3-1/7)) THEN R ELSE N/A

C187h IF (A.4.1-1/1 OR A.4.1-1/2) AND ((A.4.1-1/2 AND A.4.6.1-1/4) OR ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3) OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) AND A.4.5-1/18 THEN R ELSE N/A

C188 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 THEN R ELSE N/A

C189 IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6-1/3) AND (A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C189a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6-1/3) AND (A.4.3-4/9 OR A.4.3-4/10) AND A.4.3-3a/10) THEN R ELSE N/A

C189a2 Void

C189b Void

C189h IF A.4.1-1/1 AND A.4.5-1/18 AND (A.4.3-4a/10 OR A.4.3-4a/11) AND A.4.6-1/3 AND A.4.3-3a/10 AND (A.4.6.1-1/4 OR A.4.6.2-1/4 OR A.4.6.2-1/5 OR A.4.6.3-1/6 OR A.4.6.3-1/7) THEN R ELSE N/A

C190 Void

C191 Void

C192 IF (A.4.1-1/1 AND (A.4.6.1-1/4 OR A.4.6.3-1/6 OR A.4.6.3-1/7) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C192a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17) AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C193 IF (A.4.1-1/1 AND (A.4.6.3-1/10 OR A4.6.3-1/11 OR A.4.6.3-1/12) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C193a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND A.4.6.3-1/14 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C194 IF (A.4.1-1/2 AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.1-1/4 OR A.4.6.2-1/4 OR 4.6.2-1/5 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12)) THEN R ELSE N/A

C194a IF (A.4.1-1/2 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.2-1/6 OR 4.6.2-1/7 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18)) THEN R ELSE N/A

C194b IF (A.4.1-1/2 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.1-1/5 OR A.4.6.3-1/14)) THEN R ELSE N/A

C195 Void

C196 Void

C197 IF (A.4.5-1/37 AND A.4.3-7/1) THEN R ELSE N/A

C198 IF (A.4.5-1/38 AND A.4.3-7/1) THEN R ELSE N/A

C198a IF A.4.1-1/2 AND A.4.5-1/38 AND A.4.3-7/1 THEN R ELSE N/A

C198b IF (A.4.4-3b/103 AND A.4.5-1/38 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C198c IF A.4.1-1/2 AND A.4.5-1/38 AND A.4.3-7/1 AND A.4.4-3b/103 THEN R ELSE N/A

C199 IF A.4.5-1/37 AND A.4.3-7/1 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.4-3a/103 THEN R ELSE N/A

C199m IF A.4.5-1/37 AND A.4.3-7/1 AND ((A.4.4-3a/103 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C200 IF A.4.5-1/38 AND A.4.3-7/1 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.4-3b/103 THEN R ELSE N/A

C200m IF A.4.5-1/38 AND A.4.3-7/1 AND ((A.4.4-3b/103 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C201 Void

C202 IF ((NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-3b/4 AND A.4.2-1/3) THEN R ELSE N/A

C203 IF ((A.4.5-1/37) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 )) THEN R ELSE N/A

C204 IF ((A.4.5-1/38) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 )) THEN R ELSE N/A

C205 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.4-3a/103 AND A.4.5-1/37) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C205m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.5-1/37) AND ((A.4.4-3a/103 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C206 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3b/103 AND A.4.5-1/38) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C206m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.5-1/38) AND ((A.4.4-3b/103 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C207 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6.3-1/1 AND (A.4.5-1/14 OR A.4.5-1/15) AND A.4.5-1/32) THEN R ELSE N/A

C208 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.5-1/32) THEN R ELSE N/A

C209 IF A.4.1-1/1 AND A.4.5-1/15 AND A.4.5-1/32 THEN R ELSE N/A

C210 IF A.4.1-1/2 AND A.4.5-1/14 AND A.4.5-1/32 THEN R ELSE N/A

C211 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) THEN R ELSE N/A

C212 IF (A.4.1-1/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12)) THEN R ELSE N/A

C212a IF (A.4.1-1/1 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8)) THEN R ELSE N/A

C212b IF (A.4.1-1/1 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/14)) THEN R ELSE N/A

C213 IF (A.4.1-1/2 AND (NOT A.4.5-1/18) AND (A.4.6-1/3 OR A.4.6.1-1/4 OR A.4.6.2-1/4 OR A.4.6.2-1/5 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) AND (A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10)) THEN R ELSE N/A

C213h IF (A.4.1-1/2 AND A.4.5-1/18 AND (A.4.6-1/3 OR A.4.6.1-1/4 OR A.4.6.2-1/4 OR A.4.6.2-1/5 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) AND (A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11)) THEN R ELSE N/A

C214 IF (A.4.1-1/1 AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/10 OR A.4.6.2-1/4 OR A.4.6.2-1/5) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C215 IF (A.4.1-1/1 AND (A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/15) AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C216 IF (A.4.1-1/1 AND A.4.6.3-1/14 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C217 IF A.4.1-1/2 AND A.4.5-1/15 AND A.4.5-1/32 THEN R ELSE N/A

C218 IF A.4.1-1/1 AND A.4.5-1/15 AND A.4.5-1/32 AND A.4.5-1/56 THEN R ELSE N/A

C219 IF A.4.1-1/2 AND A.4.5-1/32 AND A.4.5-1/56 THEN R ELSE N/A

C220 IF A.4.1-1/1 AND (A.4.5-1/37 AND A.4.5-1/46) THEN R ELSE N/A

C221 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8) THEN R ELSE N/A

C221h IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR (A.4.1-1/2 AND A.4.6.1-1/5) OR ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4)) AND A.4.5-1/18 THEN R ELSE N/A

C222 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/14) OR (A.4.1-1/2 AND A.4.6.1-1/5) THEN R ELSE N/A

C223 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 THEN R ELSE N/A

C224 IF A.4.2-1/8 THEN R ELSE N/A

C225 IF (A.4.2-1/8 AND A.4.5-1/27) THEN R ELSE N/A

C226 IF (A.4.1-1/1 AND A.4.5-1/37 AND A.4.5-1/46 AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4a/8)) THEN R ELSE N/A

C227 IF (A.4.5-8/1 AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/4 OR A.4.3-4a/5 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10)) THEN R ELSE N/A

C228 IF (A.4.5-8/1 AND A.4.6.2-1/1 AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/4 OR A.4.3-4a/5 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10)) THEN R ELSE N/A

C229 IF (A.4.5-8/4 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C230 IF (A.4.5-8/4 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C231 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C232 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C233 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.4-3a/103 AND A.4.4-3b/103) AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C233m IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (((A.4.4-3a/103 AND A.4.4-3b/103) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C234 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.4-3a/103 AND A.4.4-3b/103) AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C234m IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (((A.4.4-3a/103 AND A.4.4-3b/103) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C235 IF A.4.1-1/2 AND (A.4.5-1/38 AND A.4.5-1/46) THEN R ELSE N/A

C236 IF (A.4.5-8/7 AND (A.4.6.1-1/4 OR A.4.6.2-1/4 OR A.4.6.2-1/5 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) AND (A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C237 IF (A.4.5-8/10 AND (A.4.6.1-1/5 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17) AND (A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C238 IF (A.4.5-8/10 AND A.4.6.3-1/14 AND (A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C239 IF (A.4.1-1/2 AND A.4.5-1/38 AND A.4.5-1/46 AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4a/8)) THEN R ELSE N/A

C240 IF (A.4.5-8/2 AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1 OR A.4.6.3-1/5) AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4a/8)) THEN R ELSE N/A

C241 IF (A.4.5-8/2 AND A.4.6.1-2/1 AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4a/8)) THEN R ELSE N/A

C242 IF (A.4.5-8/5 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C243 IF (A.4.5-8/5 AND (A.4.6. 2-1/2 OR A.4.6.3-1/2) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C244 IF (A.4.5-8/8 AND (A.4.6.1-1/4 OR A.4.6.2-1/4 OR A.4.6.2-1/5 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) AND (A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C245 IF (A.4.5-8/11 AND (A.4.6.1-1/5 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17) AND (A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C246 IF (A.4.5-8/11 AND A.4.6.3-1/14 AND (A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C247 IF (A.4.5-8/3 AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/4 OR A.4.3-4a/5 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10)) THEN R ELSE N/A

C248 IF (A.4.5-8/6 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C249 IF (A.4.5-8/9 AND (A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C250 IF (A.4.5-8/12 AND (A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C251 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C252 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND A.4.5-1/18 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C253 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C254 IF ((A.4.1-1/1) AND (A.4.6.2-1/1) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C255 IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C256 IF (A.4.1-1/1 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C257 IF (A.4.1-1/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) AND A.4.5-1/39) THEN R ELSE N/A

C258 IF (A.4.1-1/1 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18) AND A.4.5-1/39) THEN R ELSE N/A

C259 IF (A.4.1-1/1 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/14) AND A.4.5-1/39) THEN R ELSE N/A

C260 IF (A.4.1-1/1 AND A.4.5-1/18 AND A.4.4-3a/103) THEN R ELSE N/A

C260m IF (A.4.1-1/1 AND A.4.5-1/18 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C261 IF (A.4.1-1/2 AND A.4.5-1/18 AND A.4.4-3b/103) THEN R ELSE N/A

C261m IF (A.4.1-1/2 AND A.4.5-1/18 AND ((A.4.4-3b/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C262 IF (NOT(A.4.3-4a/1) AND A.4.1-1/1 AND A.4.3-7/4 AND A.4.4-3a/103) THEN R ELSE N/A

C262m IF (NOT(A.4.3-4a/1) AND A.4.1-1/1 AND A.4.3-7/4 AND ((A.4.4-3a/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C263 IF (NOT(A.4.3-4a/1) AND A.4.1-1/2 AND A.4.3-7/4 AND A.4.4-3b/103) THEN R ELSE N/A

C263m IF (NOT(A.4.3-4a/1) AND A.4.1-1/2 AND A.4.3-7/4 AND ((A.4.4-3b/103 AND NOT(A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/9))) THEN R ELSE N/A

C264 IF A.4.1-1/1 AND A.4.5-1/15 AND A.4.5-1/32 AND A.4.5-1/56 THEN R ELSE N/A

C265 IF A.4.1-1/2 AND A.4.5-1/14 AND A.4.5-1/32 AND A.4.5-1/56 THEN R ELSE N/A

C266 IF (A.4.1-1/1 AND A.4.2-1/2 AND NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.1-1/5 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18) THEN R ELSE N/A

C267 IF A.4.1-1/1 AND A.4.2-1/2 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10) AND (A.4.6.1-1/5 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18) THEN R ELSE N/A

C267h IF A.4.1-1/1 AND A.4.2-1/2 AND (A.4.3-4a/10 OR A.4.3-4a/11) AND A.4.6-1/4 AND (A.4.6.1-1/5 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18) AND A.4.5-1/18 THEN R ELSE N/A

C268 IF (NOT(A.4.3-4a/1) AND (A.4.1-1/1 AND A.4.1-1/2) AND (A.4.6.3-1/2 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/32) THEN R ELSE N/A

C269 IF (NOT(A.4.3-4a/1) AND A.4.1-1/2 AND (A.4.6.3-1/2 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/32) THEN R ELSE N/A

C270 IF (A.4.1-1/2 AND (A.4.6.1-1/4 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C271 IF (A.4.1-1/2 AND (A.4.6.3-1/10 OR A.4.6.2-1/4 OR A.4.6.2-1/5) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C272 IF (A.4.1-1/2 AND (A.4.6.1-1/5 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16) AND (A.4.3-4/8 AND (A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C273 IF (A.4.1-1/2 AND (A.4.6.3-1/17 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8) AND (A.4.3-4/8 AND (A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C274 IF A.4.1-1/2 AND A.4.5-1/54 THEN R ELSE N/A

C275 IF A.4.1-1/2 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) AND A.4.5-1/54 AND A.4.5-1/55 THEN R ELSE N/A

C276 IF A.4.1-1/2 AND A.4.5-1/55 THEN R ELSE N/A

C277 IF A.4.1-1/2 AND A.4.5-1/8 AND A.4.5-1/55 THEN R ELSE N/A

C278 IF (A.4.1-1/1 AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND A.4.5-1/18 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C279 IF (A.4.1-1/1 AND A.4.6.2-1/1 AND A.4.5-1/18 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C280 IF (A.4.1-1/1 AND A.4.6-1/1 AND A.4.5-1/37 AND A.4.5-1/46 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C281 IF (A.4.1-1/1 AND A.4.6-1/1 AND A.4.5-1/37 AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C282 IF (A.4.1-1/1 AND A.4.6-1/1 AND A.4.5-1/37 AND A.4.3-7/1 AND A.4.4-3a/103 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C282m IF (A.4.1-1/1 AND A.4.6-1/1 AND A.4.5-1/37 AND A.4.3-7/1 AND ((A.4.4-3a/103 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C283 IF (A.4.1-1/2 AND A.4.6-1/1 AND A.4.5-1/38 AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C284 IF (A.4.1-1/2 AND A.4.6-1/1 AND A.4.5-1/38 AND A.4.3-7/1 AND A.4.4-3b/103 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C284m IF (A.4.1-1/2 AND A.4.6-1/1 AND A.4.5-1/38 AND A.4.3-7/1 AND ((A.4.4-3b/103 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C285 IF ((A.4.1-1/1) AND (A.4.3-7/6)) THEN R ELSE N/A

C286 IF ((A.4.1-1/1) AND (A.4.3-7/7)) THEN R ELSE N/A

C287 IF ((A.4.1-1/2) AND (A.4.3-7/6)) THEN R ELSE N/A

C288 IF ((A.4.1-1/2) AND (A.4.3-7/7)) THEN R ELSE N/A

C289 IF ((A.4.1-1/1) AND (A.4.3-7/6) AND (A.4.2-1/6)) THEN R ELSE N/A

C290 IF ((A.4.1-1/2) AND (A.4.3-7/6) AND (A.4.2-1/6)) THEN R ELSE N/A

C291 IF ((A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C292 IF ((A.4.1-1/2) AND (A.4.6.2-1/1) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C293 IF (A.4.1-1/2 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C294 IF (A.4.1-1/2 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C295 IF (A.4.1-1/2 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/12 OR A.4.6.2-1/3 OR A.4.6.3-1/11) AND A.4.5-1/39) THEN R ELSE N/A

C296 IF (A.4.1-1/2 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18 AND A.4.5-1/39) THEN R ELSE N/A

C297 IF (A.4.1-1/2 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/14) AND A.4.5-1/39) THEN R ELSE N/A

C298 IF A.4.1-1/8 AND A.4.3-4c/2 THEN R ELSE N/A

C299 IF (A.4.1-1/1 AND A.4.3-7/5) THEN R ELSE N/A

C300 IF (A.4.1-1/2 AND A.4.3-7/5) THEN R ELSE N/A

C301 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/58) THEN R ELSE N/A

C302 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2 AND A.4.5-1/58) THEN R ELSE N/A

C303 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.5-1/58) THEN R ELSE N/A

C304 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND A.4.6.2-2/1 AND A.4.5-1/58) THEN R ELSE N/A

C305 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.5-1/62) THEN R ELSE N/A

C306 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C307 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C308 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C309 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C310 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C311 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C312 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C313 IF (A.4.1-1/10 AND A.4.3-4d/2 AND A.4.5-7/1) THEN R ELSE N/A

C314 IF A.4.1-1/1 AND (A.4.3-4aa/2 OR A.4.5-1/25) THEN R ELSE N/A

C315 IF A.4.1-1/1 AND (A.4.3-4aa/2 AND A.4.5-1/26 THEN R ELSE N/A

C316 IF A.4.1-1/2 AND (A.4.3-4aa/2 OR A.4.5-1/25) THEN R ELSE N/A

C316a IF A.4.1-1/2 AND A.4.3-4aa/2 AND A.4.5-1/25 THEN R ELSE N/A

C316b IF A.4.1-1/2 AND A.4.3-4aa/2 AND A.4.5-1/26 THEN R ELSE N/A

C317 IF A.4.1-1/2 AND (A.4.3-4aa/2 AND A.4.5-1/26 THEN R ELSE N/A

C318 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/13 AND A.4.5-1/58) THEN R ELSE N/A

C319 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2 AND NOT(A.4.5-1/17)) THEN R ELSE N/A

C320 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/10 AND A.4.5-1/66 AND A.4.5-1/67) THEN R ELSE N/A

C321 IF (A.4.1-1/10 AND A.4.3-4d/2 AND A.4.5-7/1 AND A.4.5-1/70) THEN R ELSE N/A

C322 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C323 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.4-3a/103 AND A.4.3-7/8) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C323m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.3-7/8) AND ((A.4.4-3a/103 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C324 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3b/103 AND A.4.3-7/8) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C324m IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.3-7/8) AND ((A.4.4-3b/103 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) OR (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C325 IF (A.4.1-1/8 OR A.4.1-1/8a) AND A.4.3-3b/5 THEN R ELSE N/A

C326 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1 OR A.4.5-1/58) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2 AND A.4.5-1/17) THEN R ELSE N/A

C327 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1 OR A.4.5-1/58) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.5-1/17) THEN R ELSE N/A

C328 IF A.4.1-1/1 AND A.4.5-1/15 AND A.4.5-1/32 AND A.4.5-8/1 THEN R ELSE N/A

C329 IF A.4.1-1/2 AND A.4.5-1/14 AND A.4.5-1/32 AND A.4.5-8/2 THEN R ELSE N/A

C330 IF (A.4.1-1/1 AND A.4.2-1/2 AND NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23) THEN R ELSE N/A

C331 IF A.4.1-1/1 AND A.4.2-1/2 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10) AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23) THEN R ELSE N/A

C331a IF A.4.1-1/2 AND A.4.2-1/2 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10) AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23) THEN R ELSE N/A

C331h IF A.4.1-1/1 AND A.4.2-1/2 AND A.4.3-4a/11 AND A.4.6-1/4 AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23) AND A.4.5-1/18 THEN R ELSE N/A

C331ha IF A.4.1-1/2 AND A.4.2-1/2 AND A.4.3-4a/11 AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23) AND A.4.5-1/18 THEN R ELSE N/A

C332 IF A.4.1-1/2 AND A.4.2-1/2 AND A.4.3-4a/11 AND A.4.6-1/4 AND (A.4.6.1-1/5 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18) AND A.4.5-1/18 THEN R ELSE N/A

C333 IF (A.4.1-1/10 AND A.4.3-4d/2 AND A.4.5-7b) THEN R ELSE N/A

C334 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6-1/3 AND A.4.6-2/3) THEN R ELSE N/A

C335 IF (A.4.1-1/10 AND A.4.3-4d/2 AND A.4.5-7/1 AND A.4.3-3b/4) THEN R ELSE N/A

C336 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/6 AND A.4.6-2/3) THEN R ELSE N/A

C337 IF ((A.4.1-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 ) AND A.4.4-3a/103 AND A.4.5-1/82) THEN R ELSE N/A

C338 IF ((A.4.1-1/2) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 ) AND A.4.4-3a/103 AND A.4.5-1/83) THEN R ELSE N/A

C339 IF (NOT((A.4.3-4a/1) or (A.4.3-4/1)) AND A.4.1-1/1 AND ( A.4.5-1/8 or A.5.-1/11 or A.4.5-1/12) AND A.4.5-1/83 ) THEN R ELSE N/A

C340 IF (NOT((A.4.3-4a/1) or (A.4.3-4/1)) AND A.4.1-1/2 AND ( A.4.5-1/8 or A.5.-1/11 or A.4.5-1/12) AND A.4.5-1/83 ) THEN R ELSE N/A

C341 IF ((A.4.1-1/2) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 )) THEN R ELSE N/A

C342 IF( (A.4.1-1/1 AND A.4.2-1/2) AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23)) OR ( (A.4.1-1/2 AND A.4.2-1/2) AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23)) OR ( (A.4.1-1/1 AND A.4.1-1/2 AND A.4.2-1/7) AND (A.4.6.3-1/19 OR A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23)) THEN R ELSE N/A

C343 IF A.4.1-1/1 AND A.4.2-1/2 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10) AND A.4.6-1/6 THEN R ELSE N/A

C344 IF A.4.1-1/1 AND A.4.2-1/2 AND A.4.5-1/18 AND A.4.3-4a/11 AND A.4.6-1/6 THEN R ELSE N/A

C345 IF A.4.1-1/2 AND A.4.2-1/2 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10) AND A.4.6-1/6 THEN R ELSE N/A

C346 IF A.4.1-1/2 AND A.4.2-1/2 AND A.4.5-1/18 AND A.4.3-4a/11 AND A.4.6-1/6 THEN R ELSE N/A

C347 IF A.4.1-1/8 AND (A.4.3-4c/1 or A.4.3-4c/2) THEN R ELSE N/A

C348 Void

C349 IF (A.4.1-1/1 AND A.4.6-1/5 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C350 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/5 AND A.4.5-1/15 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C351 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/5 AND A.4.5-1/14 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C352 IF A.4.3-3b/2 AND NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.5-1/x01 AND (A.4.5-1/x02a OR A.4.5-1/x02b OR A.4.5-1/x02c) AND A.4.1-1/1THEN R ELSE N/A

C352a IF A.4.3-3b/2 AND NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.5-1/x01 AND A.4.5-1/x02a AND A.4.1-1/2 THEN R ELSE N/A

C353 IF A.4.1-1/1 AND NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.5-1/x01 AND (A.4.5-1/x02a OR A.4.5-1/x02b OR A.4.5-1/x02c) THEN R ELSE N/A

C353a IF A.4.1-1/2 AND NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.5-1/x01 AND A.4.5-1/x02a THEN R ELSE N/A

C354 IF ((A.4.1-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 ) AND A.4.5-1/x01 AND (A.4.5-1/x02a OR A.4.5-1/x02b OR A.4.5-1/x02c)) THEN R ELSE N/A

C354a IF ((A.4.1-1/2) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 ) AND A.4.5-1/x01 AND A.4.5-1/x02a) THEN R ELSE N/A

C355 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.4-3a/103 AND A.4.5-1/x01 AND (A.4.5-1/x02a OR A.4.5-1/x02b OR A.4.5-1/x02c)) THEN R ELSE N/A

C355a IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2) AND A.4.4-3a/103 AND A.4.5-1/x01 AND A.4.5-1/x02a) THEN R ELSE N/A

C356 IF NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.5-1/x01 AND A.4.5-1/x02a THEN R ELSE N/A

C357 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.4-3a/103) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 ) AND A.4.5-1/x01 AND (A.4.5-1/x02a OR A.4.5-1/x02b OR A.4.5-1/x02c)) THEN R ELSE N/A

C357a IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3a/103) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 ) AND A.4.5-1/x01 AND A.4.5-1/x02a) THEN R ELSE N/A

C358 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/6 AND A.4.5-1/15 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C359 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/6 AND A.4.5-1/14 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

CXX1 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.4-3a/103) AND A.4.3-7/9 THEN R ELSE N/A

CXX2 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3b/104) AND A.4.3-7/9) THEN R ELSE N/A

Note 1: Cxxxh applicability is defined for small cell enhancements for physical layer related test.

## {Unchanged sections skipped}

## A.4	ICS proforma tables

### A.4.1	UE Implementation Types

Table A.4.1-1: UE Radio Technologies

| Item | UE Radio Technologies | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | E-UTRA FDD | 36.101 | Rel-8 |  |
| 2 | E-UTRA TDD | 36.101 | Rel-8 |  |
| 3 | UTRA FDD | 25.101 | R99 |  |
| 4 | UTRA TDD | 25.102 | R99 |  |
| 5 | GSM | 45.005 | R99 |  |
| 6 | cdma2000 HRPD | C.S0024-A | Rel-8 |  |
| 7 | cdma2000 1xRTT | C.S0002-A | Rel-8 |  |
| 8 | NB-IoT | 36.101 | Rel-13 |  |
| 8a | NB-IoT TDD | 36.101 | Rel-15 |  |
| 9 | WLAN | IEEE Std 802.11 |  |  |
| 10 | V2X Sidelink Communication | 36.101 | Rel-14 |  |

### A.4.2	UE Service Capabilities

Table A.4.2-1: UE Radio Technologies

| Item | UE Radio Technologies | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | LTE MBMS | 36.101 | Rel-9 |  |
| 2 | LTE CA | 36.101 | Rel-10 |  |
| 3 | UL-MIMO | 36.306, 4.3.4.6 | Rel-10 |  |
| 4 | Void |  |  |  |
| 5 | Enhanced Dual Layer TDD | 36.306, 4.3.4.5 | Rel-9 |  |
| 6 | EPDCCH | 36.306, 4.3.4.18 | Rel-11 |  |
| 7 | FDD - TDD CA | 36.306, 4.3.4.28 | Rel-12 |  |
| 8 | Support of DC | 36.306, 4.3.5.9 | Rel-12 | The UE supports of synchronous dual connectivity and power control mode 1 |
| 9 | Support of E-UTRAN WLAN Aggregation - LWA | 36.306, 4.3.18, 4.3.25, 4.3.27, 7.10.2 | Rel-13 |  |
| 10 | Support of E-URAN WLAN Aggregation with IPsec Tunnel - LWIP | 36.306, 4.3.18, 4.3.24, 4.3.27, 7.10.2 | Rel-13 |  |

### A.4.3	Baseline Implementation Capabilities

Table A.4.3-1: Supported protocols

| Item | Supported protocols | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | EPS Mobility Management | 24.301, 5 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 2 | EPS Session Management | 24.301, 6 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 3 | GPRS Mobility Management | 23.060 | R99 | For NB-IoT the release is from Rel-13 |
| 4 | Radio Resource Control | 36.331 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 5 | Packet Data Convergence Protocol | 36.323 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 6 | Radio Link Control | 36.322 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 7 | Medium Access Control | 36.321 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 8 | Physical Layer | 36.201 36.302 | Rel-8 | For NB-IoT the release is from Rel-13 |

Table A.4.3-2: Special Conformance Testing Functions

| Item | Special Conformance Testing Functions | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | UE test loop | 36.509 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 2 | Max UE test loop UL RLC SDU size 65535 bits | 36.509 | Rel-8 |  |

Table A.4.3-3: RF Baseline Implementation Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Frequency band: 1920-1980, 2110-2170 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 1 |
| 2 | Frequency band: 1850-1910, 1930-1990 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 2 |
| 3 | Frequency band: 1710-1785, 1805-1880 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 3 |
| 4 | Frequency band: 1710-1755, 2110-2155 MHz | 36.101, 5.5 | Rel-8 | FDD Band 4 |
| 5 | Frequency band: 824-849, 869-894 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 5 |
| 6 | Frequency band: 830-840, 875-885 MHz | 36.101, 5.5 | Rel-8 | FDD Band 6 |
| 7 | Frequency band: 2500-2570, 2620-2690 MHz | 36.101, 5.5 | Rel-8 | FDD Band 7 |
| 8 | Frequency band: 880-915, 925-960 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 8 |
| 9 | Frequency band: 1749.9-1784.9, 1844.9-1879.9 MHz | 36.101, 5.5 | Rel-8 | FDD Band 9 |
| 10 | Frequency band: 1710-1770, 2110-2170 MHz | 36.101, 5.5 | Rel-8 | FDD Band 10 |
| 11 | Frequency band: 1427.9-1447.9, 1475.9-1495.9 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 11 |
| 12 | Frequency band: 699-716, 729-746 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 12 |
| 13 | Frequency band: 777-787, 746-756 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 13 |
| 14 | Frequency band: 788-798, 758-768 MHz | 36.101, 5.5 | Rel-8 | FDD Band 14 |
| 15 | Reserved | 36.101, 5.5 | Rel-8 | FDD Band 15 |
| 16 | Reserved | 36.101, 5.5 | Rel-8 | FDD Band 16 |
| 17 | Frequency band: 704-716, 734-746 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 17 |
| 18 | Frequency band: 815-830, 860-875 MHz | 36.101, 5.5 | Rel-9 | FDD and HD-FDD Band 18 |
| 19 | Frequency band: 830-845, 875-890 MHz | 36.101, 5.5 | Rel-9 | FDD and HD-FDD Band 19 |
| 20 | Frequency band: 832-862, 791-821MHz | 36.101, 5.5 | Rel-9 | FDD and HD-FDD Band 20 |
| 21 | Frequency band: 1447.9-1462.9, 1495.9-1510.9 MHz | 36.101, 5.5 | Rel-9 | FDD and HD-FDD Band 21Band 21 |
| 22 | Frequency band: 3410-3490, 3510-3590 MHz | 36.101, 5.5 | Rel-10 | FDD Band 22 |
| 23 | Frequency band: 2000-2020, 2180-2200 MHz | 36.101, 5.5 | Rel-10 | FDD Band 23 |
| 24 | Frequency band: 1626.5-1660.5, 1525-1559 MHz | 36.101, 5.5 | Rel-10 | FDD Band 24 |
| 25 | Frequency band: 1850-1915, 1930-1995 MHz | 36.101, 5.5 | Rel-10 | FDD and HD-FDD Band 25 |
| 26 | Frequency band: 814-849, 859-894 MHz | 36.101, 5.5 | Rel-11 | FDD and HD-FDD Band 26 |
| 27 | Frequency band: 807-824, 852-869 MHz | 36.101, 5.5 | Rel-11 | FDD Band 27 |
| 28 | Frequency band: 703-748, 758-803 MHz | 36.101, 5.5 | Rel-11 | FDD and HD-FDD Band 28 |
| 29 | Frequency band: N/A, 717-728 MHz | 36.101, 5. 5 | Rel-11 | FDD Band 29 |
| 30 | Frequency band: 2305-2315, 2350-2360 MHz (Note 2) | 36.101, 5.5 | Rel-12 | FDD Band 30 |
| 31 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | Rel-12 | FDD and HD-FDD Band 31 |
| 32 | Frequency band: N/A, 1452-1496 MHz | 36.101, 5.5 | Rel-12 | FDD Band 32 |
| 33 | Frequency band: 1900-1920, 1900-1920 MHz | 36.101, 5.5 | Rel-8 | TDD Band 33 |
| 34 | Frequency band: 2010-2025, 2010-2025 MHz | 36.101, 5.5 | Rel-8 | TDD Band 34 |
| 35 | Frequency band: 1850-1910, 1850-1910 MHz | 36.101, 5.5 | Rel-8 | TDD Band 35 |
| 36 | Frequency band: 1930-1990, 1930-1990 MHz | 36.101, 5.5 | Rel-8 | TDD Band 36 |
| 37 | Frequency band: 1910-1930, 1910-1930 MHz | 36.101, 5.5 | Rel-8 | TDD Band 37 |
| 38 | Frequency band: 2570-2620, 2570-2620 MHz | 36.101, 5.5 | Rel-8 | TDD Band 38 |
| 39 | Frequency band: 1880-1920, 1880-1920 MHz | 36.101, 5.5 | Rel-8 | TDD Band 39 |
| 40 | Frequency band: 2300-2400, 2300-2400 MHz | 36.101, 5.5 | Rel-8 | TDD Band 40 |
| 41 | Frequency band: 2496-2690, 2496-2690 MHz | 36.101, 5.5 | Rel-10 | TDD Band 41 |
| 42 | Frequency band: 3400-3600, 3400-3600 MHz | 36.101, 5.5 | Rel-10 | TDD Band 42 |
| 43 | Frequency band: 3600-3800, 3600-3800 MHz | 36.101, 5.5 | Rel-10 | TDD Band 43 |
| 44 | Frequency band: 703-803, 703-803 MHz | 36.101, 5.5 | Rel-11 | TDD Band 44 |
| 45 | Frequency band: 1447-1467, 1447-1467 MHz | 36.101, 5.5 | Rel-13 | TDD Band 45 |
| 46 | Frequency band: 5150-5925, 5250-5925 MHz | 36.101, 5.5 | Rel-13 | TDD Band 46 |
| 47 | Frequency band: 5855-5925, 5855-5925 MHz | 36.101, 5.5 | Rel-14 | TDD Band 47 |
| ... |  |  |  |  |
| 48 | Frequency band: 3550-3700, 3550-3700 MHz | 36.101, 5.5 | Rel-14 | TDD Band 48 |
| .. |  |  |  |  |
| 53 | Frequency band: 2483.5-2495, 2483.5-2495 MHz | 36.101, 5.5 | Rel-16 | TDD Band 53 |
| ... |  |  |  |  |
| 65 | Frequency band: 1920-2010, 2110-2200 MHz | 36.101, 5.5 | Rel-13 | FDD Band 65 |
| 66 | Frequency band: 1710-1780, 2110-2200 MHz | 36.101, 5.5 | Rel-13 | FDD and HD-FDD Band 66 |
| ... |  |  |  |  |
| 68 | Frequency band: 698-728, 753-783 MHz | 36.101, 5.5 | Rel-15 | FDD Band 68 |
| 69 | Frequency band: N/A, 2570-2620 MHz | 36.101, 5.5 | Rel-14 | FDD Band 69 |
| 70 | Frequency band: 1695-1710, 1995-2020 MHz | 36.101, 5.5 | Rel-14 | FDD and HD-FDD Band 70 |
| 71 | Frequency band: 663-698, 617-652 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 71 |
| 72 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 72 |
| 73 | Frequency band: 450-455, 460-465 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 73 |
| 74 | Frequency band: 1427-1470, 1475-1518 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 74 |
| ... |  |  |  |  |
| 85 | Frequency band: 698-716, 728-746 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 85 |
| ... |  |  |  |  |
| 87 | Frequency band: 410-415, 420-425 MHz | 36.101, 5.5 | Rel-16 | FDD and HD-FDD Band 87 |
| 88 | Frequency band: 412-417, 422-427 MHz | 36.101, 5.5 | Rel-16 | FDD and HD-FDD Band 88 |
| Note 1: The values indicated in column "Release" are to be understood as the specifications release version in which a band was introduced and not as a mandate that a UE conforming to particular release shall support a particular band. For further guidance to release independent bands see TS 36.307 [16] Note 2: The uplink transmission is not allowed at this band for the UE with the externally vehicle-mounted antennas. |  |  |  |  |

Table A.4.3-3a: RF Additional Baseline Implementation Capabilities

| Item | RF Additional Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | Support of 1.4 MHz channel bandwidth | 36.101, 5.6.1 | Operating bands supporting 1.4 MHz Bandwidth: 2, 3, 4, 5, 8, 12, 23, 25, 26, 27, 31, 35, 36, 53, 65, 66, 72, 73, 74, 87, 88 |
| 2 | Support of 3 MHz channel bandwidth | 36.101, 5.6.1 | Operating bands supporting 3 MHz Bandwidth: 2, 3, 4, 5, 8, 12, 23, 25, 26, 27, 28, 31, 35, 36, 44, 53, 65, 66, 72, 73, 74, 87, 88 |
| 3 | Support of 5 MHz channel bandwidth | 36.101, 5.6.1 | All operating bands support 5 MHz Bandwidth except band 46 and Band 47 |
| 4 | Support of 10 MHz channel bandwidth | 36.101, 5.6.1 | All operating bands support 10 MHz Bandwidth except band 31, 72 and 73 |
| 5 | Support of 15 MHz channel bandwidth | 36.101, 5.6.1 | Operating bands supporting 15 MHz Bandwidth: 1, 2, 3, 4, 7, 9, 10, 18, 19, 20, 21, 22, 23, 25, 26, 28, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 48, 65, 66, 70, 74 |
| 6 | Support of 20 MHz channel bandwidth | 36.101, 5.6.1 | Operating bands supporting 20MHz Bandwidth: 1, 2, 3, 4, 7, 9, 10, 20, 22, 23, 25, 28, 33, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 65, 66, 701, 74 |
| 7 | Support of 20 MHz for both PCell and SCell | 36.101, 5.6A.1 |  |
| 8 | Support of 20 MHz for PCell and 10 MHz for SCell | 36.101, 5.6A.1 |  |
| 9 | Support at most 40 MHz aggregated bandwidth | 36.101, 5.6A.1 |  |
| 10 | Support at most 60 MHz aggregated bandwidth | 36.101, 5.6A.1 |  |
| 11 | Support at most 80 MHz aggregated bandwidth | 36.101, 5.6A.1 |  |
| 12 | Support at most 100 MHz aggregated bandwidth | 36.101, 5.6A.1 |  |
| Note 1: 1 For the 20 MHz channel bandwidth, the additional baseline implementation capabilities are restricted to E-UTRA operation when carrier aggregation is configured. |  |  |  |

Table A.4.3-3b: Additional UE Power Class implementation Capabilities

| Item | RF baseline UE Baseline implementation capability | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UE Power Class 1 | 36.101, 6.2.2 | Applicable to Bands 14, 31, 72, 87, 88 |
| 2 | UE Power Class 3 | 36.101, 6.2.2 | All applicable E-UTRA and NB-IoT bands |
| 3 | UE Power Class 5 | 36.101, 6.2.2E 36.306, 4.3.5.1 and 4.3.5.1A | All applicable E-UTRA and NB-IoT bands 20dBm |
| 4 | UE Power Class 2 | 36.101, 6.2.2. 6.2.2E | Applicable to Bands 31, 41, 47, 72 |
| 5 | UE Power Class 6 | 36.101, 6.2.2E 36.306, 4.3.5.1A.2 | All applicable E-UTRA and NB-IoT bands 14dBm |

Table A.4.3-3c: UE Power Class 1 implementation Capabilities per band

| Item | RF Baseline Implementation Capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Frequency band: 788-798, 758-768 MHz | 36.101, 5.5 | Rel-8 | FDD Band 14 |
| 2 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | Rel-12 | FDD and HD-FDD Band 31 |
| 3 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 72 |
| 4 | Frequency band: 410-415, 420-425 MHz | 36.101, 5.5 | Rel-16 | FDD and HD-FDD Band 87 |
| 5 | Frequency band: 412-417, 422-427 MHz | 36.101, 5.5 | Rel-16 | FDD and HD-FDD Band 88 |

Table A.4.3-3d: UE Power Class 2 implementation Capabilities per band

| Item | RF Baseline Implementation Capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Frequency band: 2496-2690, 2496-2690 MHz | 36.101, 5.5 | Rel-10 | TDD Band 41 |
| 2 | Frequency band: 5855-5925, 5855-5925 MHz | 36.101, 5.5 | Rel-14 | TDD Band 47 |
| 3 | Frequency band: 2570-2620, 2570-2620 MHz | 36.101, 5.5 | Rel-15 | TDD Band 38 |
| 4 | Frequency band: 2300-2400, 2300-2400 MHz | 36.101, 5.5 | Rel-15 | TDD Band 40 |
| 5 | Frequency band: 3400-3600, 3400-3600 MHz | 36.101, 5.5 | Rel-15 | TDD Band 42 |
| 6 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | Rel-12 | FDD and HD-FDD Band 31 |
| 7 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 72 |

Table A.4.3-4: UE Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category 1 | 36.306, 4.1 | Rel-8 |  |
| 2 | Category 2 | 36.306, 4.1 | Rel-8 |  |
| 3 | Category 3 | 36.306, 4.1 | Rel-8 |  |
| 4 | Category 4 | 36.306, 4.1 | Rel-8 |  |
| 5 | Category 5 | 36.306, 4.1 | Rel-8 | Support for 64QAM in UL |
| 6 | Category 6 | 36.306, 4.1 | Rel-10 |  |
| 7 | Category 7 | 36.306, 4.1 | Rel-10 |  |
| 8 | Category 8 | 36.306, 4.1 | Rel-10 | Support for 64QAM in UL |
| 9 | Category 9 | 36.306, 4.1 | Rel-11 |  |
| 10 | Category 10 | 36.306, 4.1 | Rel-11 |  |
| 11 | Category 11 | 36.306, 4.1 | Rel-11 |  |
| 12 | Category 12 | 36.306, 4.1 | Rel-11 |  |

Table A.4.3-4a: UE Downlink Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category DL 0 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 0 |
| 1a | Category DL 1bis | 36.306, 4.1A | Rel-13 | Only in combination with Category UL 1bis |
| 2 | Category DL 6 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 5 or Category UL 16 |
| 3 | Category DL 7 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 13 or Category UL 18 |
| 4 | Category DL 9 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 5 or Category UL 16 |
| 5 | Category DL 10 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 13 or Category UL 18 |
| 6 | Category DL 11 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 5 or Category UL 16 |
| 7 | Category DL 12 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 13 or Category UL 15 or Category UL 18 or Category UL 20 |
| 8 | Category DL 13 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 16 or Category UL 18 |
| 9 | Category DL 14 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 8 or Category UL 17 |
| 10 | Category DL 15 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 16 or Category UL 18 |
| 11 | Category DL 16 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 |
| 12 | Category DL 17 | 36.306, 4.1A | Rel-13 | Only in combination with Category UL 14 or Category UL 19 |
| 13 | Category DL 18 | 36.306, 4.1A | Rel-13 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 |
| 14 | Category DL 19 | 36.306, 4.1A | Rel-13 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 or Category UL 21 |
| 15 | Category DL 4 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 5 |
| 16 | Category DL 20 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 or Category UL 21 |
| 17 | Category DL 21 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 |
| 18 | Category DL 22 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |
| 19 | Category DL 23 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |
| 20 | Category DL 24 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |
| 21 | Category DL 25 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |
| 22 | Category DL 26 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |

Table A.4.3-4aa: Additional UE Downlink Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category DL M1 | 36.306, 4.1A | Rel-13 | Only in combination with Category UL M1 |
| 2 | Category DL M2 (NOTE 1) | 36.306, 4.1A | Rel-14 | Only in combination with Category UL M2 |
| NOTE 1: A UE indicating Category M2 shall also indicate Category M1. |  |  |  |  |

Table A.4.3-4b: UE Uplink Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category UL 0 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 0 |
| 1a | Category UL 1bis | 36.306, 4.1A | Rel-13 | Only in combination with Category DL 1bis |
| 2 | Category UL 3 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 13, Category DL 15, Category DL 16, Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 3 | Category UL 5 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 6, Category DL 9, Category DL 11, Category DL 13, Category DL 15 , Category DL 16, Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 4 | Category UL 7 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 13, Category DL 15, Category DL 16, Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 5 | Category UL 8 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 14 |
| 6 | Category UL 13 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 7, Category DL 10, Category DL 12, Category DL 13, Category DL 15, Category DL 16, Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 7 | Category UL 14 | 36.306, 4.1A | Rel-13 | Only in combination with Category DL 17 |
| 8 | Category UL 15 | 36.306, 4.1A | Rel-13 | Only in combination with Category DL 12 or Category DL 16 or Category DL 18 or Category DL 19 or Category DL 20 or Category DL 21 |
| 9 | Category UL 16 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 6, Category DL 9, Category DL 11, Category DL 13, Category DL 15, Category DL 16. Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 10 | Category UL 17 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 14 |
| 11 | Category UL 18 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 7, Category DL 10, Category DL 12, Category DL 13, Category DL 15, Category DL 16. Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 12 | Category UL 19 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 17 |
| 13 | Category UL 20 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 12, Category DL 16, Category DL 18, Category DL 19, Category DL 20, Category DL 21, Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 14 | Category UL 21 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 19 or Category DL 20 |
| 15 | Category UL 22 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 16 | Category UL 23 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 17 | Category UL 24 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 18 | Category UL 25 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 19 | Category UL 26 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |

Table A.4.3-4ba: Additional UE Uplink Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category UL M1 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL M1 |
| 2 | Category UL M2 (NOTE 1) | 36.306, 4.1A | Rel-14 | Only in combination with Category DL M2 |
| NOTE 1: A UE indicating Category M2 shall also indicate Category M1. |  |  |  |  |

Table A.4.3-4c: UE Category NB

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category NB1 | 36.306, 4.1C | Rel-13 |  |
| 2 | Category NB2 (Note 1) | 36.306, 4.1C | Rel-14 |  |
| NOTE 1: A UE indicating Category NB2 shall also indicate Category NB1. |  |  |  |  |

Table A.4.3-4d: UE Category for V2X sidelink communication and sidelink discovery

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | SL-D Category 1 | 36.306, 4.1B | Rel-14 |  |
| NOTE 1: If a UE of this release supports sidelink discovery, the UE shall support SL-D Category 1 |  |  |  |  |

Table A.4.3-4da: UE Category for reception capabilities for sidelink communication

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | SL-C-RX Category 1 | 36.306, 4.1B | Rel-14 |  |
| 2 | SL-C-RX Category 2 | 36.306, 4.1B | Rel-14 |  |
| 3 | SL-C-RX Category 3 | 36.306, 4.1B | Rel-14 |  |
| 4 | SL-C-RX Category 4 | 36.306, 4.1B | Rel-14 |  |
| NOTE 1: If a UE of this release supports sidelink communication, the UE shall support SL-C-RX Category 1 and SL-C-TX Category 1. If a UE of this release supports V2X sidelink communication, the UE shall support SL-C-RX Category 2 to 4 for reception, and SL-C-TX category 2 to 5 for transmission. |  |  |  |  |

Table A.4.3-4db: UE Category for transmission capabilities for sidelink communication

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | SL-C-TX Category 1 | 36.306, 4.1B | Rel-14 |  |
| 2 | SL-C-TX Category 2 | 36.306, 4.1B | Rel-14 |  |
| 3 | SL-C-TX Category 3 | 36.306, 4.1B | Rel-14 |  |
| 4 | SL-C-TX Category 4 | 36.306, 4.1B | Rel-14 |  |
| 5 | SL-C-TX Category 5 | 36.306, 4.1B | Rel-14 |  |
| NOTE 1: If a UE of this release supports sidelink communication, the UE shall support SL-C-RX Category 1 and SL-C-TX Category 1. If a UE of this release supports V2X sidelink communication, the UE shall support SL-C-RX Category 2 to 4 for reception, and SL-C-TX category 2 to 5 for transmission. |  |  |  |  |

Table A.4.3-5: Void

Table A.4.3-6: Void

Table A.4.3-7: Additional capabilities

| Item | Additional capabilities | Ref. | Release | Comments |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Enhanced performance requirements type A for LTE | 36.101, 8 | Rel-11 | Support for Enhanced performance requirements type A |  |
| 2 | Support of Type B Half-duplex FDD operation | 36.211, 6,2,5 36.306, 4.2.6 | Rel-12 | Support of Half-duplex FDD operation type B for category 0, category M1, category M2, category NB1 and category NB2 UE |  |
| 3 | Enhanced performance requirements type C for LTE | 36.101, 8 | Rel-12 | Support for Enhanced performance requirements type C |  |
| 4 | Enhanced performance requirements type B for LTE | 36.101, 8 36.306, 4.3.4.35 | Rel-12 | Support for Enhanced performance requirements type B |  |
| 5 | Enhanced measurement in high speed scenario | 36.306,4.3.33.1 | Rel-14 | Support measurement enhancements in high speed scenario |  |
| 6 | Enhanced downlink control channel performance requirements type A for LTE | 36.101, 8 | Rel-13 | Support for Enhanced downlink control channel performance requirements type A for LTE |  |
| 7 | Enhanced downlink control channel performance requirements type B for LTE | 36.101, 8 | Rel-13 | Support for Enhanced downlink control channel performance requirements type B for LTE |  |
| 8 | DMRS enhancements for TM9 | 36.306, 4.3.28.4 | Rel-13 | Support for DMRS enhancements in TM9 |
| 9 | aperiodic ZP-CSI-RS reporting | 36.306, 4.3.4.51 | Rel-13 | Support for aperiodic ZP-CSI-RS reporting |

Table A.4.3-8: Void