3GPP TSG-RAN5 Meeting #92-e	R5-214405

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1964 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct CSI-MeasConfig for test cases with 1SSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CSI-MeasConfig in Table 7.3.1-6 states that the value of n2 is 2 if CSI-RS for BM is configured in the test case. That means, according to Table 7.3.0-1, if the test case uses CSI-RS.X.2 RMCs. However, some RRM test cases (e.g. 4.6.1.4) explicitly request to use only one resource of the CSI-RS.X.2 RMC: “CSI-RS.1.2 FDD resource #0”. Therefore, n2 should have value of 1 for these test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that n2 = 1 if the test case explicitly requests for 1-SSB only |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration of Table 7.3.1-6 will not be compatible with test cases explicitly requesting 1SSB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### -	CSI-MeasConfig

Table 7.3.1-6: CSI-MeasConfig for RRM

| Derivation Path: Table 4.6.3-38 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { |  | n1+n2+n3+n4+n5 entries |  | n1=1 if CSI-RS for CSI is configured in test case, n1=0 otherwise; n2=0 if CSI-RS for BM is not configured in the test case. If CSI-RS for BM is configured in the test case , n2 = 2 if 2SSB, n2 = 1 otherwise n3=4 if TRS is configured in test case, n3=0 otherwise; n4=4 if second resource set of TRS is configured in test case, n4=0 otherwise; n5=2 if aperiodic CSI-RS for BM is configured in test case, n5=0 otherwise; |  |  |  |
| NZP-CSI-RS-Resource[k, k=1..n1] |  | NZP-CSI-RS-Resource for CSI |  | entry 1 |  | n1>0 |  |
| NZP-CSI-RS-Resource[k, k=n1+1.. n1+n2] |  | NZP-CSI-RS-Resource for BM (k-n1-1) |  | entry ... |  | n2>0 |  |
| NZP-CSI-RS-Resource[k, k= n1+n2+1.. n1+n2+n3] |  | NZP-CSI-RS-Resource for TRS (k-n1-n2) |  | entry ... |  | n3>0 |  |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+1.. n1+n2+n3+n4] |  | NZP-CSI-RS-Resource for TRS (k-n1-n2-n3) with condition SECOND_SET |  | entry ... |  | n4>0 |  |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+n4+1.. n1+n2+n3+n4+n5] |  | NZP-CSI-RS-Resource for BM (k-n1-n2-n3-n4) with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { |  | m1+m2+m3+m4+m5 entries |  | mi=1 if ni>0, mi=0 otherwise |  |  |  |
| NZP-CSI-RS-ResourceSet[k, k=1..m1] |  | NZP-CSI-RS-ResourceSet for CSI |  | entry 1 |  | n1>0 |  |
| NZP-CSI-RS-ResourceSet[k, k=m1+1.. m1+m2] |  | NZP-CSI-RS-ResourceSet for BM |  | entry ... |  | n2>0 |  |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+1.. m1+m2+m3] |  | NZP-CSI-RS-ResourceSet for TRS |  | entry ... |  | n3>0 |  |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+1.. m1+m2+m3+m4] |  | NZP-CSI-RS-ResourceSet for TRS with condition SECOND_SET |  | entry ... |  | n4>0 |  |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+m4+1.. m1+m2+m3+m4+m5] |  | NZP-CSI-RS-ResourceSet for BM with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { |  | 1 entry |  |  |  | n1>0 |  |
| CSI-IM-Resource[1] |  | CSI-IM-Resource-RRM |  | entry 1 |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-IM-ResourceToAddModList |  | Not present |  |  |  | n1=0 |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { |  | 1 entry |  |  |  | n1>0 |  |
| CSI-IM-ResourceSet[1] |  | CSI-IM-ResourceSet-RRM |  | entry 1 |  |  |  |
| } |  |  |  |  |  |  |  |
| csi-IM-ResourceSetToAddModList |  | Not present |  |  |  | n1=0 |  |
| csi-SSB-ResourceSetToAddModList |  | Not present |  |  |  |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { |  | p1+m2+m3+m5 entries |  | p1=m1+1 if n1>0, p1=0 otherwise. |  |  |  |
| CSI-ResourceConfig[k, k=1..m1] |  | CSI-ResourceConfig for CSI |  | entry 1 |  | n1>0 |  |
| CSI-ResourceConfig[k, k=p1] |  | CSI-ResourceConfig for CSI-IM |  | entry ... |  | n1>0 |  |
| CSI-ResourceConfig[k, k=p1+1.. p1+m2] |  | CSI-ResourceConfig for BM |  | entry ... |  | n2>0 |  |
| CSI-ResourceConfig[k, k= p1+m2+1.. p1+m2+m3] |  | CSI-ResourceConfig for TRS |  | entry ... |  | n3>0 and n4=0 |  |
|  |  | CSI-ResourceConfig for TRS with condition SECOND_SET |  |  |  | n4>0 |  |
| CSI-ResourceConfig[k, k= p1+m2+m3+1.. p1+m2+m3+m5] |  | CSI-ResourceConfig for BM with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { |  | m1+m2+m5 entries |  |  |  |  |  |
| CSI-ReportConfig[k, k=1..m1] |  | CSI-ReportConfig for CSI |  | entry 1 |  | n1>0 |  |
| CSI-ReportConfig[k, k=m1+1.. m1+m2] |  | CSI-ResourceConfig for BM |  | entry ... |  | n2>0 |  |
| CSI-ReportConfig[k, k=m1+m2+1.. m1+m2+m5] |  | CSI-ResourceConfig for BM with condition APERIODIC |  | entry ... |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| reportTriggerSize |  | Not present |  |  |  |  |  |
|  |  | 1 |  |  |  | n5>0 |  |
| aperiodicTriggerStateList CHOICE { |  | Not present |  |  |  |  |  |
| setup |  | CSI-AperiodicTriggerStateList |  |  |  | n5>0 |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2SSB | For configuration with 2 SS Blocks |

#### –	NZP-CSI-RS-Resource for TRS

< End of changes >