3GPP TSG-RAN5 Meeting #92-e	R5-214425

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1299 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion Annex C.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Annex C.2.3 (Multi-path fading propagation conditions) is FFS. The Annex is referenced by RRM test cases using fading, e.g. RLM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Completed Annex C.2.3 with the appropriate extracts from TR 38.901 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multipath fading propagation conditions will remain undefined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

## C.2.3	Multi-path fading propagation conditions

The multipath propagation conditions consist of several parts:

-	A Tapped Delay Line (TDL) model, characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency

-	A set of correlation matrices defining the correlation between the UE and eNodeB antennas in case of multi-antenna systems.

## C.2.3.1	TDL models

The TDL models for simplified evaluations, e.g., for non-MIMO evaluations, are defined for the full frequency range from 0.5 GHz to 100 GHz with a maximum bandwidth of 2 GHz.

Three TDL models, namely TDL-A, TDL-B and TDL-C, are constructed to represent three different channel profiles for NLOS while TDL-D and TDL-E are constructed for LOS, the parameters of which can be found respectively in Table C.2.3.1-1, Table C.2.3.1-2 , Table C.2.3.1-3, Table C.2.3.1-4 and Table C.2.3.1-5.

The Doppler spectrum for each tap is characterized by a classical (Jakes) spectrum shape and a maximum Doppler shift fD where . Due to the presence of a LOS path, the first tap in TDL-D and TDL-E follows a Ricean fading distribution. For those taps the Doppler spectrum additionally contains a peak at the Doppler shift fS = 0.7 fD with an amplitude such that the resulting fading distribution has the specified K-factor.

Each TDL model can be scaled in delay so that the model achieves a desired RMS delay spread, according to the procedure described in Clause C.2.3.2.

Table C.2.3.1-1. TDL-A

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0.0000 | -13.4 | Rayleigh |
| 2 | 0.3819 | 0 | Rayleigh |
| 3 | 0.4025 | -2.2 | Rayleigh |
| 4 | 0.5868 | -4 | Rayleigh |
| 5 | 0.4610 | -6 | Rayleigh |
| 6 | 0.5375 | -8.2 | Rayleigh |
| 7 | 0.6708 | -9.9 | Rayleigh |
| 8 | 0.5750 | -10.5 | Rayleigh |
| 9 | 0.7618 | -7.5 | Rayleigh |
| 10 | 1.5375 | -15.9 | Rayleigh |
| 11 | 1.8978 | -6.6 | Rayleigh |
| 12 | 2.2242 | -16.7 | Rayleigh |
| 13 | 2.1718 | -12.4 | Rayleigh |
| 14 | 2.4942 | -15.2 | Rayleigh |
| 15 | 2.5119 | -10.8 | Rayleigh |
| 16 | 3.0582 | -11.3 | Rayleigh |
| 17 | 4.0810 | -12.7 | Rayleigh |
| 18 | 4.4579 | -16.2 | Rayleigh |
| 19 | 4.5695 | -18.3 | Rayleigh |
| 20 | 4.7966 | -18.9 | Rayleigh |
| 21 | 5.0066 | -16.6 | Rayleigh |
| 22 | 5.3043 | -19.9 | Rayleigh |
| 23 | 9.6586 | -29.7 | Rayleigh |

Table C.2.3.1-2. TDL-B

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0.0000 | 0 | Rayleigh |
| 2 | 0.1072 | -2.2 | Rayleigh |
| 3 | 0.2155 | -4 | Rayleigh |
| 4 | 0.2095 | -3.2 | Rayleigh |
| 5 | 0.2870 | -9.8 | Rayleigh |
| 6 | 0.2986 | -1.2 | Rayleigh |
| 7 | 0.3752 | -3.4 | Rayleigh |
| 8 | 0.5055 | -5.2 | Rayleigh |
| 9 | 0.3681 | -7.6 | Rayleigh |
| 10 | 0.3697 | -3 | Rayleigh |
| 11 | 0.5700 | -8.9 | Rayleigh |
| 12 | 0.5283 | -9 | Rayleigh |
| 13 | 1.1021 | -4.8 | Rayleigh |
| 14 | 1.2756 | -5.7 | Rayleigh |
| 15 | 1.5474 | -7.5 | Rayleigh |
| 16 | 1.7842 | -1.9 | Rayleigh |
| 17 | 2.0169 | -7.6 | Rayleigh |
| 18 | 2.8294 | -12.2 | Rayleigh |
| 19 | 3.0219 | -9.8 | Rayleigh |
| 20 | 3.6187 | -11.4 | Rayleigh |
| 21 | 4.1067 | -14.9 | Rayleigh |
| 22 | 4.2790 | -9.2 | Rayleigh |
| 23 | 4.7834 | -11.3 | Rayleigh |

Table C.2.3.1-3. TDL-C

| Tap # | Normalized delays | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -4.4 | Rayleigh |
| 2 | 0.2099 | -1.2 | Rayleigh |
| 3 | 0.2219 | -3.5 | Rayleigh |
| 4 | 0.2329 | -5.2 | Rayleigh |
| 5 | 0.2176 | -2.5 | Rayleigh |
| 6 | 0.6366 | 0 | Rayleigh |
| 7 | 0.6448 | -2.2 | Rayleigh |
| 8 | 0.6560 | -3.9 | Rayleigh |
| 9 | 0.6584 | -7.4 | Rayleigh |
| 10 | 0.7935 | -7.1 | Rayleigh |
| 11 | 0.8213 | -10.7 | Rayleigh |
| 12 | 0.9336 | -11.1 | Rayleigh |
| 13 | 1.2285 | -5.1 | Rayleigh |
| 14 | 1.3083 | -6.8 | Rayleigh |
| 15 | 2.1704 | -8.7 | Rayleigh |
| 16 | 2.7105 | -13.2 | Rayleigh |
| 17 | 4.2589 | -13.9 | Rayleigh |
| 18 | 4.6003 | -13.9 | Rayleigh |
| 19 | 5.4902 | -15.8 | Rayleigh |
| 20 | 5.6077 | -17.1 | Rayleigh |
| 21 | 6.3065 | -16 | Rayleigh |
| 22 | 6.6374 | -15.7 | Rayleigh |
| 23 | 7.0427 | -21.6 | Rayleigh |
| 24 | 8.6523 | -22.8 | Rayleigh |

Table C.2.3.1-4. TDL-D.

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -13.5 | Rayleigh |
| 2 | 0.035 | -18.8 | Rayleigh |
| 3 | 0.612 | -21 | Rayleigh |
| 4 | 1.363 | -22.8 | Rayleigh |
| 5 | 1.405 | -17.9 | Rayleigh |
| 6 | 1.804 | -20.1 | Rayleigh |
| 7 | 2.596 | -21.9 | Rayleigh |
| 8 | 1.775 | -22.9 | Rayleigh |
| 9 | 4.042 | -27.8 | Rayleigh |
| 10 | 7.937 | -23.6 | Rayleigh |
| 11 | 9.424 | -24.8 | Rayleigh |
| 12 | 9.708 | -30.0 | Rayleigh |
| 13 | 12.525 | -27.7 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 13.3 dB and a mean power of 0dB. |  |  |  |

Table C.2.3.1-5. TDL-E.

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.03 | LOS path |
|  | 0 | -22.03 | Rayleigh |
| 2 | 0.5133 | -15.8 | Rayleigh |
| 3 | 0.5440 | -18.1 | Rayleigh |
| 4 | 0.5630 | -19.8 | Rayleigh |
| 5 | 0.5440 | -22.9 | Rayleigh |
| 6 | 0.7112 | -22.4 | Rayleigh |
| 7 | 1.9092 | -18.6 | Rayleigh |
| 8 | 1.9293 | -20.8 | Rayleigh |
| 9 | 1.9589 | -22.6 | Rayleigh |
| 10 | 2.6426 | -22.3 | Rayleigh |
| 11 | 3.7136 | -25.6 | Rayleigh |
| 12 | 5.4524 | -20.2 | Rayleigh |
| 13 | 12.0034 | -29.8 | Rayleigh |
| 14 | 20.6519 | -29.2 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 22 dB and a mean power of 0dB. |  |  |  |

### C.2.3.2	Scaling of delays

The RMS delay spread values of both CDL and TDL models are normalized and they can be scaled in delay so that a desired RMS delay spread can be achieved. The scaled delays can be obtained according to the following equation:

(C.2.3.2-1)

in which

is the normalized delay value of the nth cluster in a CDL or a TDL model

is the new delay value (in [ns]) of the nth cluster

is the wanted delay spread (in [ns])

The example scaling parameters are selected according to Table C.2.3.2-1 where the values have been chosen such that the RMS delay spreads span the range observed in measurements corresponding to the typical 5G evaluation scenarios.

The example parameters given in Table C.2.3.2-1 does not preclude the use of other scaling values if this is found appropriate, for instance if additional scenarios are introduced or if e.g. the effect of beamforming needs to be captured in a TDL. Both of these examples can potentially result in an increased range of experienced RMS delay spreads.

Table C.2.3.2-1. Example scaling parameters for CDL and TDL models.

| Model |  |
| --- | --- |
| Very short delay spread | 10 ns |
| Short delay spread | 30 ns |
| Nominal delay spread | 100 ns |
| Long delay spread | 300 ns |
| Very long delay spread | 1000 ns |

< End of changes >