3GPP TSG-RAN WG5 Meeting #92-e	R5-214457

# Electronic Meeting, 16th - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0570 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new TC 6.3A.4.4.6 Aggregate power tolerance for CA (7UL CA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | 3in |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The work plan of NR Transmitter and Receiver Test Cases – SA Range 2 (38.521-2) identifies the need of Aggregate power tolerance for CA test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new TC 6.3A.4.4.6 of Aggregate power tolerance for CA (7UL CA) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Aggregate power tolerance for CA test case cannot be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.4.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3A.4.4.6	Aggregate power tolerance for CA (7UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerances are FFS.

- UE transmitted power for PC 1, 2 and 4 are FFS.

- Power window is FFS.

- How to ensure equal PSD between component carriers is FFS.

6.3A.4.4.6.1	Test purpose

To verify the ability of the UE transmitter to maintain its power during non-contiguous transmissions within 21ms in response to 0 dB commands with respect to the first UE transmission and all other power control parameters as specified in TS 38.213 kept constant.

6.3A.4.4.6.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 7UL CA and 7DL CA.

6.3A.4.4.6.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.4.4.0.

6.3A.4.4.6.4	Test description

Same as in clause 6.3A.4.4.1.4 with the following exceptions:

-	Instead of Table 6.3A.4.4.1.4.1-1 use Table 6.3A.4.4.6.4.1-1.

-	Instead of Table 6.3A.4.4.1.5-1 and Table 6.3A.4.4.1.5-2 use Table 6.3A.4.4.6.5-1 and Table 6.3A.4.4.6.5-2.

Table 6.3A.4.4.6.4.1-1: Test Configuration Table: PUSCH

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation (Note 1) |
| 1 | PCC | Default | Default | N/A | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC1 |  |  |  | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC2 |  |  |  | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC3 |  |  |  | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC4 |  |  |  | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC5 |  |  |  | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC6 |  |  |  | DFT-s-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

6.3A.4.4.6.5	Test requirement

The requirement for the power measurements made in step 8 of the test procedure shall not exceed the values specified in Table 6.3A.4.4.6.5-1 and Table 6.3A.4.4.6.5-2. The power measurement period shall be 1 sub-frame (1ms).

Table 6.3A.4.4.6.5-1: Power control tolerance (Pint ≥ P ≥ Pmin)

| TPC command | UL channel | Test requirement measured power |
| --- | --- | --- |
| 0 dB | PUSCH | Given 3 power measurements in the pattern, the 2nd, and later measurements shall be within ± 5.5+TTdB of the 1st measurement. |
| Note 1: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3A.4.4.6.5-3. |  |  |

Table 6.3A.4.4.6.5-2: Power control tolerance (Pmax ≥ P > Pint)

| TPC command | UL channel | Test requirement measured power |
| --- | --- | --- |
| 0 dB | PUSCH | Given 3 power measurements in the pattern, the 2nd, and later measurements shall be within ± 3.5+TTdB of the 1st measurement. |
| Note 1: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3A.4.4.6.5-4. |  |  |

Table 6.3A.4.4.6.5-3: Test Tolerance (Pint ≥ P ≥ Pmin)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | FFS | FFS |

Table 6.3A.4.4.6.5-4: Test Tolerance (Pmax ≥ P > Pint)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | FFS | FFS |