3GPP TSG RAN WG5 Meeting #92-e	R5-214555

Electronic Meeting, 16th Aug 2021 – 27th Aug 2021

Agenda Item:	6.3.2.20

Source:	Ericsson

Title:	NG.114 annex C

Document for:	Endorsement

# 1	Introduction

NG.114 [1] includes “Annex C MNO provisioning and Late Customization”. E.g., an open market device according NG.114 [1], C.1

“1. Supports non-roaming and roaming cases;

2. Has a default configuration suitable for many MNOs; and

3. Can be configured to the MNO’s needs.”

# 2	Discussion

NG.114 main and annex C

The main body of NG.114 [1] includes normative requirements and mandatory parts of annex C.

Examples NG.114 [1] media and features

Speech Media: SDP Considerations, Media 3.2.2, EVS 3.2.2.3

“The configuration of the EVS payload type to be included first in the initial SDP offer for EVS is defined by the EVS/Br and EVS/Bw parameters as specified in Annex C.3, which must be configured to one of the five above EVS Configurations.”

SIP Precondition Considerations, Feature Set 2.2.5

“If the precondition mechanism is enabled by the Precondition_disabling_policy node in Annex C.3, the UE must use the precondition mechanism.”

Observation 2-1:

To be able to verify NG.114 [1] details for media and features the required annex C.3 parts needs to be included.

Default configuration of annex C

Configuration parameters according NG.114 [1], C.3

“Table C.3-1 contains the configuration parameters with their default values, that must be supported by the UE and the network.”

Observation 2-2:

Support of the default values are mandatory.

Proposal 2-1:

Introduce an ICS capability for the annex C default configuration.

Additional configurations of annex C

Configuration parameters according NG.114 [1], C.3

“The UE must use the default value for each parameter in Table C.3-1 unless configured differently as described in Annex C.2.”

Observation 2-3:

Parameters in NG.114 [1], Table C.3.1 can be configured for different MNO’s needs. I.e., additional test coverages can be considered for media and features.

Proposal 2-2:

Allow test coverage based on Table C.3-1 additional configurations.

# 3	Proposal

It’s proposed to implement the proposal 2-1 and 2.2.

# 4	References

[1] GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS, Version 2.0".