3GPP TSG-RAN5 Meeting #92-e	R5-214600

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1307 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Test Frequencies in the test configuration table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test Frequencies selection in the test configuration table is confused. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test Frequencies selection was corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Frequencies selection will remain confused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.6C.3	Out-of-band blocking for SUL

7.6C.3.1	Test Purpose

Same test purpose as in clause 7.6.3.1.

7.6C.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

7.6C.3.3	Minimum conformance requirements

For SUL operation, the out-of-band blocking requirement for downlink bands specified in clause 7.6.3 shall be met. For operation band combination listed in Table 7.6C.3.3-1, exceptions to the requirement specified in Table 7.6C.3.3-2 are allowed when the second order intermodulation product of the SUL carrier and the CW interfering signal fully or partially overlaps with the DL carrier.

Table 7.6C.3.3-1: SUL operating band combination with exceptions allowed

NR Band combination for SUL

SUL_n78-n81

SUL_n78-n82

SUL_n78-n83

SUL_n79-n81

SUL_n79-n83

Table 7.6C.3.3-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when, where and are the channel bandwidths configured for SUL and DL (victim) bands in MHz, respectively. |  |  |

For all interferer frequency ranges specified in clause 7.6.3 a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

The normative reference for this requirement is TS 38.101-1 [2] clauses 7.6C.3.

7.6C.3.4	Test description

Same test description as specified in clause 7.6.3.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1

Instead of table 7.6.3.4.1-1  use Table 7.6C.3.4-1.

Table 7.6C.3.4-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Mid range for SUL carrier One frequency chosen arbitrarily from low or high range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Mid, Highest for Non-SUL carrier For SUL band: n80: 30 MHz n81: 20 MHz n82: 20 MHz n83: 20 MHz n84: 20 MHz n86: 40 MHz n95: 15 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and lowest for Non-SUL carrier |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | DL Configuration |  | UL Configuration |  | SUL Configuration |  |
|  | Mod'n | RB allocation | N/A |  | Mod'n | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) |  |  | DFT-s-OFDM QPSK | REFSENS (NOTE 1) |
| NOTE 1: The specific configuration of SUL and DL are defined in Table 7.3C.2.4.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.9 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Tables 4.6.3-14, 4.6.3-15 with condition SUL, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, additionally the following exception shown in Table 7.6C.3.4-2 is considered.

Table 7.6C.3.4-2: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 7.6C.3.4-3: Void

7.6C.3.5	Test Requirement

For SUL operation, the out-of-band blocking requirement for downlink bands specified in clause 7.6.3.5 shall be met. For operation band combination listed in Table 7.6C.3.5-1, exceptions to the requirement specified in Table 7.6C.3.5-2 are allowed when the second order intermodulation product of the SUL carrier and the CW interfering signal fully or partially overlaps with the DL carrier.

Table 7.6C.3.5-1: SUL operating band combination with exceptions allowed

NR Band combination for SUL

SUL_n78-n81

SUL_n78-n82

SUL_n78-n83

SUL_n79-n81

SUL_n79-n83

Table 7.6C.3.5-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when, where and are the channel bandwidths configured for SUL and DL (victim) bands in MHz, respectively. |  |  |

For all interferer frequency ranges, a maximum of

exceptions are allowed for the spurious response frequencies recorded in the final step of test procedure in each assigned frequency channel when measured using a step size of  MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

<< End of changes >>