3GPP TSG-RAN WG5 Meeting #92-e	R5-214656

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0186 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCPTT test case 5.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - There is a condition POC-SETTINGS-EVENT in 36.579-1 already and default message content is extended accordingly - SIP SUBSCRIBE at step 7 is a re-SUBSCRIBE - SIP 200 (OK) from the SS needs condition SUBSCRIBE-RSP - PoC-Settings has mandatory id attribute - SIP 200 (OK) from the UE does not need any specific message content - New generic test procedure for subscription and notification introduced |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reference to POC-SETTINGS-EVENT added to specific message contents where appropriate; information elements removed accordingly - New Table 5.5.3.3-1A for re-SUBCRIBE - Condition INVITE-RSP replaced by SUBSCRIBE-RSP for table 5.5.3.3-3 - id attribute added to Table 5.5.3.3-5 - Content removed from Table 5.5.3.3-6 - Adaptation to new generic test procedure 5.3.29 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case cannot be implemented and verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.579-1 CR 0156, 0165 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.5	Configuration / Determination of MCPTT Service Settings / Current Active MCPTT Settings / De-subscribe

5.5.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {
  when { MCPTT User requests to verify the currently active MCPTT service settings or to discover MCPTT service settings }

then { UE (MCPTT Client) sends a SIP SUBSCRIBE message to find the MCPTT service settings and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

(2)

with { UE (MCPTT Client) having already subscribed to find the MCPTT service settings }

ensure that {
  when { MCPTT User requests to re-subscribe for MCPTT service settings }

then { UE (MCPTT Client) sends a SIP SUBSCRIBE message to re-subscribe for the MCPTT service settings and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

(3)

with { UE (MCPTT Client) having already subscribed to find the MCPTT service settings }

ensure that {
  when { MCPTT User requests to de-subscribe for MCPTT service settings }

then { UE (MCPTT Client) sends a SIP SUBSCRIBE message to de-subscribe for the MCPTT service settings and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

5.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clause 7.2.4. Unless otherwise stated these are Rel-14 requirements.

[TS 24.281, clause 7.2.4]

In order to discover MCPTT service settings of another MCPTT client of the same MCPTT user or to verify the currently active MCPTT service settings of this MCPTT client, the MCPTT client shall generate an initial SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 6665 [26], and IETF RFC 4354 [55].

In the SIP SUBSCRIBE request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT ID of the MCPTT user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

4)	shall set the Event header field to the 'poc-settings' value;

5)	shall include an Accept header field containing the "application/poc-settings+xml" MIME type;

6)	if the MCPTT client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295; and

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

7)	if the MCPTT client wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [26], to zero.

In order to re-subscribe or de-subscribe, the MCPTT client shall generate an in-dialog SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 6665 [26], IETF RFC 4354 [55]. In the SIP SUBSCRIBE request, the MCPTT client:

1)	shall set the Event header field to the 'poc-settings' value;

2)	shall include an Accept header field containing the "application/poc-settings+xml" MIME type;

3)	if the MCPTT client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295; and

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

4)	if the MCPTT client wants to de-subscribe, shall set the Expires header field according to IETF RFC 6665 [26], to zero.

Upon receiving a SIP NOTIFY request according to 3GPP TS 24.229 [4], IETF RFC 6665 [26] and IETF RFC 4354 [55], that contains an application/poc-settings+xml MIME body the MCPTT client shall cache:

1)	the <am-settings> element of the poc-settings+xml MIME body for each MCPTT client identified by the "id" attribute according to IETF RFC 4354 [55] as the current Answer-mode indication of that MPCTT client; and

2)	the <selected-user-profile-index> element of the poc-settings+xml MIME body for each MCPTT client identified by the "id" attribute according to IETF RFC 4354 [55] as the active MCPTT service user profile of that MCPTT client.

5.5.3	Test description

5.5.3.1		Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	E-UTRA related parameters are set to the default parameters for the basic single cell environment, as defined in TS 36.508 [20] clause 4.4.

IUT:

-	UE (MCPTT Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10, is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

5.5.3.2		Test procedure sequence

Table 5.5.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCPTT User request to verify the currently active MCPTT service settings of the UE (MCPTT Client) and to receive later notifications. ( NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) perform Generic Test Procedure for MCPTT Subscription and Notification as described in TS 36.579-1 [2] Table 5.3.29.3-1 to request to verify the currently active MCPTT service settings of the UE (MCPTT Client)? | - | - | 1 | P |
| 3-5 | Void | -- | - | - | - |
| 6 | Make the MCPTT User request to re-subscribe to MCPTT service settings and to receive later notifications. ( NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCPTT Client) perform Generic Test Procedure for MCPTT Subscription and Notification as described in TS 36.579-1 [2] Table 5.3.29.3-1 to re-subscribe to MCPTT service settings and to receive later notifications? | - | - | 2 | P |
| 8-10 | Void. | - | - | - | - |
| 11 | Make the MCPTT User request to de-subscribe to MCPTT service settings. (NOTE 1) | - | - | - | - |
| 12-14 | Check: Does the UE (MCPTT Client) perform Steps 1a1 to 3 of Generic Test Procedure for MCPTT Subscription and Notification as described in TS 36.579-1 [2] Table 5.3.29.3-1 to de-subscribe to MCPTT service settings? | - | - | 3 | P |
| - | EXCEPTION: SS (MCPTT Server) releases the E-UTRA connection. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI command. |  |  |  |  |  |

5.5.3.3		Specific message contents

Table 5.5.3.3-1: SIP SUBSCRIBE (step 2, Table 5.5.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.29.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.14-1, condition POC-SETTINGS-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  | TS 24.379 [9] clause 7.2.4 |  |
| value | "4294967295" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.3-2 |  |  |  |

Table 5.5.3.3-1A: SIP SUBSCRIBE (step 7, Table 5.5.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.14-1, condition POC-SETTINGS-EVENT, re_SUBSCRIBE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  | TS 24.379 [9] clause 7.2.4 |  |
| value | "4294967295" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.3-2 |  |  |  |

Table 5.5.3.3-2: MCPTT-Info in SIP SUBSCRIBE (Table 5.5.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1, condition POC-SETTINGS-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 5.5.3.3-3: SIP 200 (OK) from the SS (steps 2, 7, 13, Table 5.5.3.2-1; step 3, TS 36.579-1 [2] Table 5.3.29.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition SUBSCRIBE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 5.5.3.3-4: SIP NOTIFY from the SS (steps 2, 7, Table 5.5.3.2-1; step 4, TS 36.579-1 [2] Table 5.3.29.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1, condition POC-SETTINGS-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PoC-Settings | RFC 4354 [103] |  |
| MIME-part-body | PoC-Settings as described in Table 5.5.3.3-5 |  |  |  |

Table 5.5.3.3-5: PoC-Settings in SIP NOTIFY (Table 5.5.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.11.2-1, condition AUTOMATIC |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 5.5.3.3-6: Void

Table 5.5.3.3-7: SIP SUBSCRIBE (step 12, Table 5.5.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.29.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.14-1, condition POC-SETTINGS-EVENT, re_SUBSCRIBE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  | TS 24.379 [9] clause 7.2.4 |  |
| value | "0" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.3-2 |  |  |  |