3GPP TSG RAN WG5 Meeting #92-e	R5-214696

Electronic Meeting, 16th Aug 2021 – 27th Aug 2021

Agenda Item:	6.3.2.20

Source:	Ericsson

Title:	NG.114 PS data off

Document for:	Endorsement

# 1	Introduction

The 3GPP PS data off is one main service access procedure defined by SA1. According TS 22.011[2], 10.1-2

“3GPP PS Data Off is a feature which, when configured by the HPLMN and activated by the user, prevents transport via PDN connections in 3GPP access networks of all data packets except IP packets required by 3GPP PS Data Off Exempt Services.

…

The 3GPP system shall provide a mechanism by which an operator can configure which operator services are defined as 3GPP PS Data Off Exempt Services for their own subscribers.”

# 2	Discussion

NG.114 main

The feature is part of NG.114 [1], 5.4.1

“The UE and the network must support 3GPP PS Data Off. When 3GPP PS Data Off is activated by the user, the UE and the network must not send via any PDU session any IP packet of any service other than 3GPP PS Data Off Exempt Services. This applies to both non-IMS based services and SIP-based IMS services.”

Observation 2-1: 3GPP PS data off is mandatory in NG.114.

Further details are found in e.g., TS 24.229 [3]

4.17 3GPP PS data off

“The UE informs the network about its 3GPP PS data off status by including a g.3gpp.ps-data-off media feature tag specified in subclauce 7.9.8 in all REGISTER requests …”

U.3.1.5 3GPP PS data off

“If the 3GPP PS data off status is "active" the UE shall only send initial requests that:

1)	are associated with a 3GPP IMS service which enforces 3GPP PS data off;

…

2)	are associated with an emergency service; or

3)	are associated with 3GPP PS data off exempt services configured in the UE using one or more of the following methods:

-	the non_3GPP_ICSIs_exempt node specified in 3GPP TS 24.167 [8G], if the UE is in the HPLMN or the EHPLMN, or if the UE is in the VPLMN and the non_3GPP_ICSIs_roaming_exempt node specified in 3GPP TS 24.167 [8G] is not configured;”

Proposal 2-1:

Add NG.114 [1] test coverage for the 3GPP PS data off feature.

# 3	Proposal

It’s proposed to implement the proposal 2-1.

# 4	References

[1] GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS, Version 2.0".

[2] 3GPP TS 22.011: "Service accessibility".

[3] 3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".