3GPP TSG-RAN5 Meeting 92-e	R5-214850

# Electric Meeting, 16th Aug – 27th Aug 2021

Title:							Removal of EN-DC Receiver Spurious Emission test with multiple CCs

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

Introduction

The contribution proposes to remove EN-DC Rx Spurious test case with higher number of CCs from TS 38.521-3.

Discussion

As is already discussed in [1] and [2],  RAN5 agreed to test up to 2 CC CA case for FR1 receiver spurious emission in TS 38.521-1, which is the same policy as done in LTE.

This is because that the purpose of the receiver spurious requirement for CA is to define requirement for an operating band without uplink band(i.e. DL-only band), and not for the purpose to define the requirement for “spurious emission in the CA status”. As DL-only band is not solely testable as single carrier operation, then 2 CC test case is specified. Figure 1 and 2 are the snippets from TS 36.101-1 and TS 38.101-1.

Figure 1 The core requirements on “Spurious emissions for CA” in TS 36.101

Figure 2 The core requirements on "Spurious emissions for CA" in TS 38.101-1

In TS 38.101-3, spurious emission for EN-DC is defined as below.

- 2.1 EN-DC including FR2

In TS 38.101-2 v17.1.0, there is no core requirement for receiver spurious emission for CA. This can be because of no DL-only FR2 band is defined yet (as of 17.1.0), hence there is no need to define the receiver spurious requirement for CA mode. TS 38.521-2 Work Plan also does not specify the section for receiver spurious for CA

Observation 1 : No core requirement for receiver spurious for CA is defined in 38.101-2 due to no DL-only band is specified in FR2.

Hence, the consequence in EN-DC including FR2 is that more than 2 CCs EN-DC test cases are not required.

Proposal 1 : Remove “Spurious Emissions for inter-band EN-DC including FR2 (>2 CCs)” and its sub-sections from the work plan(5GS_NR_LTE-UEConTest) and TS 38.521-3.

Note that, in the future there is a possibility that DL-only FR2 band will be specified eventually, in that case the 4CCs (1LTE + 2 inter-band FR2 NR) test case will be required, again.

|  |  | # 38.521-3 | # 7.9B.4_1 | # Spurious Emissions for inter-band EN-DC including FR2 (>2 CCs) |  |  |  |  |  | # 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 368 | # Y | # 38.521-3 | # 7.9B.4_1.1 | # Spurious Emissions for inter-band EN-DC including FR2 (3 CCs) |  | # Anritsu | # Opt 3: RAN5#92 (Aug-21) MU: RAN5#92 (Aug-21) TT: RAN5#92 (Aug-21) Opt 4: FFS Opt 7: FFS |  |  | # 1 | # 0% |
| # 369 | # Y | # 38.521-3 | # 7.9B.4_1.2 | # Spurious Emissions for inter-band EN-DC including FR2 (4 CCs) |  | # Anritsu | # Opt 3: RAN5#92 (Aug-21) MU: RAN5#92 (Aug-21) TT: RAN5#92 (Aug-21) Opt 4: FFS Opt 7: FFS |  |  | # 1 | # 0% |
| # 370 |  | # 38.521-3 | # 7.9B.4_1.3 | # Spurious Emissions for inter-band EN-DC including FR2 (5 CCs) |  | # Anritsu |  |  |  | # 0 |  |
| # 371 |  | # 38.521-3 | # 7.9B.4_1.4 | # Spurious Emissions for inter-band EN-DC including FR2 (6 CCs) |  | # Anritsu |  |  |  | # 0 |  |

- 2.2 EN-DC including FR1

In FR1, DL-only band(SDL band) is defined, which is n29, n75 and n76 as of v17.1.0. Several EN-DC band combinations which include such DL-only NR band is also defined. We can extend the idea in FR1 NR CA test case to EN-DC test cases as below. Note here that receiver spurious emission is LTE anchor agnostic test.

- 2CCs(1LTE + 1NR) test cases are needed to test the NR receiver spurious performance mainly for NSA-only UE.

- 3CCs test case is required to test the 1 LTE + 2 inter-band NR CCs with DL-only NR band mainly for NSA-only UE.

- 4CCs and upper test cases are not required.

Proposal 2 : Remove “Spurious Emissions for EN-DC within FR1 (x CCs)”(x>=4) from the work plan(5GS_NR_LTE-UEConTest) and TS 38.521-3.

Proposal 3 : Make “Spurious Emissions for EN-DC within FR1 (3 CCs)” applicable for  1 LTE + 2 inter-band NR CCs with DL-only NR band.

Note : More specifically, DC_XA_nYA-nZA, DC_(n)XAA_nZA and DC_XA_nXA-nZA, where nZ is DL-only NR band are applicable.

|  |  | # 38.521-3 | # 7.9B.3_1 | # Spurious Emissions for EN-DC within FR1 (>2 CCs) |  | # Anritsu |  |  |  | # 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 362 | # Y | # 38.521-3 | # 7.9B.3_1.1 | # Spurious Emissions for EN-DC within FR1 (3 CCs) |  | # Anritsu | # Opt 3: RAN5#91 (May-21) Opt 4: FFS Opt 7: FFS | # R5-212867 | # Test can be completed at RAN5#87 | # 1 | # 33% |
| # 363 | # Y | # 38.521-3 | # 7.9B.3_1.2 | # Spurious Emissions for EN-DC within FR1 (4 CCs) |  | # Anritsu | # Opt 3: RAN5#92 (Aug-21) Opt 4: FFS Opt 7: FFS |  | # Test can be completed at RAN5#87 | # 1 | # 0% |
| # 364 | # Y | # 38.521-3 | # 7.9B.3_1.3 | # Spurious Emissions for EN-DC within FR1 (5 CCs) |  | # Anritsu | # Opt 3: RAN5#92 (Aug-21) Opt 4: FFS Opt 7: FFS |  | # Test can be completed at RAN5#87 | # 1 | # 0% |
| # 365 |  | # 38.521-3 | # 7.9B.3_1.4 | # Spurious Emissions for EN-DC within FR1 (6 CCs) |  | # Anritsu |  |  |  | # 0 |  |

- 3. Conclusions

Observation 1 : No core requirement for receiver spurious for CA is defined in 38.101-2 due to no DL-only band is specified in FR2.

Proposal 1 : Remove “Spurious Emissions for inter-band EN-DC including FR2 (>2 CCs)” and its sub-sections from the work plan(5GS_NR_LTE-UEConTest) and TS 38.521-3.

Proposal 2 : Remove “Spurious Emissions for EN-DC within FR1 (x CCs)”(x>=4) from the work plan(5GS_NR_LTE-UEConTest) and TS 38.521-3.

Proposal 3 : Make “Spurious Emissions for EN-DC within FR1 (3 CCs)” applicable for  1 LTE + 2 inter-band NR CCs with DL-only NR band.

- 4. References

[1] R5-190615, “Discussion on the structure of Clause 7.9A in TS 38.521-1”, CMCC et. al. , RAN5 NR#4 AdHoc

[2] R5-192384, “Reply LS to RAN5 on the requirements of 7.9A spurious emissions for CA”, TSG WG RAN4, RAN5#82