3GPP TSG-RAN5 Meeting 92-e	R5-214852

# Electric Meeting, 16th  Aug – 27th Aug. 2021

Title:							On the testable SNR for FR2 demodulation tests

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

- Introduction

# This paper provides our view on the upper limit of testable SNR for FR2 demod/perf test cases.

- Discussion

# Table 1 shows the achievable power at center of QZ at 1dB CW compression point for each band (mid frequency).  The FR2 values in Table 1 are shown for information purpose as those values are endorsed in the RAN5#91[1].

Table 1 Estimated achievable DL Power at center of QZ

|  | n258 mid | n257 mid n261 mid | n260 mid | n259 mid | n262mid |
| --- | --- | --- | --- | --- | --- |
| Frequency[GHz] | 25.875 GHz | 28/27.925 GHz | 38.5 GHz | 41.5 GHz | 48GHz |
| Power at 1dB CW compression point at center of QZ [dBm] | -32.8 (Endorsed in RAN5#91[1] ) | -32.8 (Endorsed in RAN5#91[1] ) | -35.5 | -36.2 | -39.4 |

# Proposal 1 : Adopt the values in Table 1 for deriving the working assumption of testable SNR upper limit.

# n262 value is shown as an input for the LS from RAN4 on the achievable SNR for n262[2]. Though the split values are requested from RAN4 as shown in the below table, it is no longer possible to provide the split value as RAN5 decided not to specify the split values but only specify “achievable DL power at 1dB CW compression point” and “crest factor due to waveform and fading”.

Table 1: Proposed demodulation test setup SNR calculation parameters for band n262

| Parameter | Value | Comment |
| --- | --- | --- |
| REFSENS | -82.8 dBm/50 MHz | Using REFSENS agreed for band n262 |
| Multi-band relaxation | 1.0 dB | Defined as ceil(.); change from 2.0 dB |
| FS path loss | -63.2 dB | Change from -62.3 dB (scaling from 43.5 to 48.2 GHz) |
| Cable loss | -8.7 dB | Additional 0.33 dB/m in cable loss at 48.2 GHz |
| Probe antenna gain | [12.0] dB | Needs checking |
| Backoff from P1dB | [13.0] dB | Needs checking |

# To align the policy among RAN4 and RAN5, it is proposed to inform only “Power at center of QZ at 1dB CW compression point [dBm]” and “Backoff from P1dB”(17.71dB for up to 64QAM) to RAN4 instead of split values.

Table 2: Proposed demodulation test setup SNR calculation parameters for band n262

| Parameter | Value | Comment |
| --- | --- | --- |
| REFSENS | -82.8 dBm/50 MHz | Using REFSENS agreed for band n262 |
| Multi-band relaxation | 1.0 dB | Defined as ceil(.); change from 2.0 dB |
| FS path loss | -63.2 dB | Change from -62.3 dB (scaling from 43.5 to 48.2 GHz) |
| Cable loss | -8.7 dB | Additional 0.33 dB/m in cable loss at 48.2 GHz |
| Probe antenna gain | [12.0] dB | Needs checking |
| Power at 1dB CW compression point at center of QZ [dBm] | -39.4 dBm | Achievable power considering Tx Power of SS, cable loss, probe antenna gain and FS path loss |
| Backoff from P1dB | 17.71 dB | Crest factor of OFDM wave form and fading are considered (applicable up to 64QAM) |

# Proposal 2 : Inform RAN4 of “Power at center of QZ at 1dB CW compression point [dBm]” and “Backoff from P1dB”(17.71dB for up to 64QAM) instead of split values.

- Conclusion

# Proposal 1 : Adopt the values in Table 1 for deriving the working assumption of testable SNR upper limit.

# Proposal 2 : Inform RAN4 of “Power at center of QZ at 1dB CW compression point [dBm]” and “Backoff from P1dB”(17.71dB for up to 64QAM) instead of split values.

# 4.	References

# [1] R5-213806, “On the achievable SNR for demod test cases”, ROHDE & SCHWARZ, RAN5#91

# [2] R5-212018, “LS on band-dependent parameters for the FR2 demodulation setup”, TSG WG RAN4, RAN5#91