3GPP TSG-RAN WG5 Meeting #92-e	R5-214960

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0215 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new IMS over 5GS TC 10.6 Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Fulfilling the work plan by introducing new test case 10.6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 10.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0287 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## 10.6	Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS

10.6.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to start an emergency call }

then { UE performs IMS registration and sets up emergency call }

}

(2)

with { Emergency call ongoing }

ensure that {

when { Network sends BYE }

then { UE sends 200 OK for BYE }

}

(3)

with { Emergency call being terminated }

ensure that {

when { UE is made to initiate a non UE detectable emergency call }

then { UE sends correctly composed INVITE request for normal voice call }

}

(4)

with { UE having sent INVITE for normal voice call }

ensure that {

when { UE receives 380 Alternative Service response }

then { UE acknowledges and initiates an emergency call }

}

10.6.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.3.1]:

In the event the UE receives a 380 (Alternative Service) response to an initial INVITE request the response containing a P-Asserted-Identity header field with a value equal to the value of the last entry of the Path header field value received during registration and the response containing a 3GPP IM CN subsystem XML body that includes an <ims-3gpp> element, including a version attribute, with an <alternative-service> child element with the <type> child element set to "emergency" (see table 7.6.2), the UE shall select a domain in accordance with the conventions and rules specified in 3GPP TS 22.101 [1A] and 3GPP TS 23.167 [4B], and:

-	if the CS domain is selected, the UE behavior is defined in subclause 7.1.2 of 3GPP TS 23.167 [4B] and, where appropriate, in the access technology specific annex; and

-	if the IM CN subsystem is selected, the UE shall apply the procedures in subclause 5.1.6 with the exception of selecting a domain for the emergency call attempt.

NOTE 10:	The last entry on the Path header field value received during registration is the value of the SIP URI of the P-CSCF. If there are multiple registration flows associated with the registration, then the UE has received from the P-CSCF during registration multiple sets of Path header field values. The last entry of the Path header field value corresponding to the flow on which the 380 (Alternative Service) response was received is checked.

[TS 24.229 clause 5.1.6.2A]:

The UE shall perform a new initial emergency registration, as specified in subclause 5.1.6.2, if the UE determines that:

-	it has previously performed an emergency registration which has not yet expired; and

-	it has obtained an IP address from the serving IP-CAN, as specified in subclause 9.2.1, different than the IP address used for the emergency registration.

10.6.3	Test description

10.6.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

10.6.3.2	Test procedure sequence

Table 10.6.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to make an emergency call | - | - | - | - |
| 2-11 | Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 12-15 | Steps 1-4 of Annex A.6 happens | - | - | - | - |
| 16 | Step 5 of Annex A.6 happens Check: Does the UE send an ACK? | --> | ACK | 1 | P |
| 17 | Step 1 of Annex A.8 happens | <-- | BYE | - | - |
| 18 | Step 2 of Annex A.8 happens Check: Does the UE send 200 OK for the BYE request and ends the call? | --> | 200 OK | 2 | P |
| 19 | UE is made to attempt an IMS voice call | - | - | - | - |
| 20 | UE sends INVITE with the first SDP offer Check: Does the UE send correctly composed INVITE request for normal voice call? | --> | INVITE | 3 | P |
| 21 | SS responds with 380 Alternative Service | <-- | 380 Alternative Service | - | - |
| 22 | UE acknowledges the receipt of 380 Alternative Service response | --> | ACK | - | - |
| 23 | Step 1 of Annex A.6 happens Check: Does UE send INVITE message for emergency call? | --> | INVITE | 4 | P |
| 24-27 | Steps 2-5 of Annex A.6 happens | - | - | - | - |
| 28 | Step 1 of Annex A.8 happens | <-- | BYE | - | - |
| 29 | Step 2 of Annex A.8 happens Check: Does the UE send 200 OK for the BYE request and ends the call? | --> | 200 OK | 2 | P |

10.6.3.3	Specific message contents

Table 10.6.3.3-1: INVITE for non UE detectable emergency call (step 20, table 10.6.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.2.1, Conditions A1, A3, A4, A28, A29, and A30 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | The following SDP types and values. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] Time description: t= (start-time) (stop-time) Media description: m=audio (transport port) [Note 2] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: EVS codec shall be present in the media attributes, optionally including channel number "/1". |  | TS 24.229 [7] |

Table 10.6.3.3-2: 380 Alternative Service (step 21, table 10.6.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | <?xml version="1.0" encoding="UTF-8"?> <ims-3gpp version="1"> <alternative-service> <type>emergency</type> <reason/> <action>emergency-registration</action> </alternative-service> </ims-3gpp> |  | TS 24.229 [7] |

Table 10.6.3.3-3: ACK (step 22, table 10.6.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.2.7, Conditions A1 and A4

<End of modifed section>