3GPP TSG-RAN5 Meeting #92-e	R5-215061

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1098 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of REFSENS for inter-band EN-DC 2CC adding DC_28A_n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Exceptional requirements are specified for configuration DC_28A_n78A. The test configuration table of DC_28A_n78A is not aligned with latest RAN5 agreement |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating the test configuration table - Test requirements are already completed therefore no changes are needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundant test points remain for the configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.2B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

<Unchanged text skipped >

Table 7.3B.2.3.4.2.1-2_15: Test configurations table for exceptions due to UL harmonic interference for EN-DC 19_n77 (Requirement of Note 4 and 5)

| E-UTRA Band 19 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 835 MHz/ 24050 | 16@17 | 4175.01 MHz/ 678334 | Lowest | REFSENS (NOTE 2) |
| 2 | 10 | 835 MHz/ 24050 | 25@12 | 4175.01 MHz/ 678334 | 40 | REFSENS (NOTE 2) |
| 3 |  | 838.9 MHz/ 24089 | 16@17 | 4194.99 MHz/ 679666 | Lowest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_16: void

Table 7.3B.2.3.4.2.1-2_17: Void

Table 7.3B.2.3.4.2.1-2_18: Test configurations table for exceptions due to UL harmonic interference for EN-DC 26_n41 (Requirement of Note 8 and 9)

| E-UTRA Band 26 |  |  |  | NR Band n41 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 833.7 MHz/ 26887 | 16@17 | 2501.1 MHz/ 500220 | Lowest | REFSENS (NOTE 2) |
| 3 |  | 844 MHz/ 26990 | 16@17 | 2532 MHz/ 506400 | Lowest | REFSENS (NOTE 2) |
|  |  | 844 MHz/ 26990 | 25@12 | 2532 MHz/ 506400 | 50 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 and 9 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |  |

<Unchanged text skipped >

Table 7.3B.2.3.4.2.1-3e: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 41_n78 (Victim band LTE)

| NR Band n78 |  |  |  | E-UTRA Band 41 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | E-UTRA FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 | 20 | 3789.7 MHz/ 652644 | 50@28 | 2526.4MHz/ 39954 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 in Table 7.3B.2.0.3.2-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3f: Void

Table 7.3B.2.3.4.2.1-3g: Void

Table 7.3B.2.3.4.2.1-3h: Void

<Unchanged text skipped >

Table 7.3B.2.3.4.2.1-5: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to "Range" columns. |  |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  | DC Configuration / NRB_agg |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |
|  |  | DC Configuration |  |  |  |  |  | E-UTRA Ch BW |  | NR Ch BW |  | CC MOD E-UTRA/NR | E-UTRA & NR RB allocation |  | CC MOD E-UTRA/NR |  | E-UTRA & NR allocations (LCRB @ RBstart) |  |  |  |
|  |  | E-UTRA |  |  |  | NR |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |
|  |  | Band |  | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  | Mid |  | Y | Mid | 5 |  | Mid |  | QPSK /CP-OFDM QPSK | All RBs |  | QPSK/ DFT-s-OFDM QPSK |  | REFSENS |  |  | REFSENS |
| Test Settings for DC_1A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 2 |  | Note 5 |  | 66 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 2 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 |  | Note 5 |  | 5 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 |  | Note 5 |  | 7 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_3A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 |  | Note 5 |  | 20 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_3A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 |  | Note 5 |  | 77/ 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 5 |  | Note 5 |  | 66 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 5 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 |  | Low |  | 5 | High | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_7A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 |  | Note 5 |  | 66 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 |  | Low |  | 77 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A-n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 8 |  | Low |  | 1 | High | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 8 |  | Low |  | 3 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 8 |  | Low |  | 41 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A_n77A/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 8 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| 2 |  | 8 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_8A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 8 |  | Note 5 |  | 79 | Note 5 | 5 |  | 40 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 216@0 |
| Test Settings for DC_12A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 12 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | All RBs |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_20A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 20 |  | Note 5 |  | 3 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 20 |  | Note 5 |  | 8 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_20A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 2 |  | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 21 | Note 5 |  | 79 |  | Note 5 | 5 | 40 |  | Note 7 |  | All RBs |  |  | Note 7 |  | 25@0 | 216@0 |  |
| Test Settings for DC_26_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 26 |  | Note 5 |  | 41 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_26A_n77/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 26 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 1 |  | 26 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 48 |  | Note 5 |  | 66 | Note 5 | 20 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 100@0 |  |  | 25@0 |
| Test Settings for DC_28A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 28 |  | Note 5 |  | 41 | Note 5 | 10 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 50@0 |
| Test Settings for DC_28A_n50A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 28 |  | Note 5 |  | 50 | Note 5 | 10 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 50@0 |
| Test Settings for DC_28A_n51A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 28 |  | Note 5 |  | 51 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 66 |  | Note 5 |  | 2 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A_n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 66 |  | Note 5 |  | 71 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for CA_7A_20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 |  | Note 5 |  | 20 | Note 5 | 10 |  | 5 |  | QPSK | All RBs |  | QPSK |  | 50@0 |  |  | 25@0 |
| Note 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. Note 2: Use DC Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the DC Configuration. E.g. for DC_1A-n3A, X=1, Y=3. Note 4: REFSENS refers to the E_UTRA bands and NR band NRB 's single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2 of TS 36.521-1 [10] and Table 7.3.2.4.1-3 of TS 38.521-1 [8], respectively Note 5: Test frequency for each DC configuration shall follow Table 7.3B.2.0.3.5.1-1. If test configurations of each ID in a DC configuration are same, test frequency shall follow the order of Table 7.3B.2.0.3.5.1-1. Note 6: Not applicable if the UE only supports Bandwidth Combination Set 1. Note 7: Same as default. Note 8: RBSTART = 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [6] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  | NRB |  |  | Band |  |  |  | Range |  | NRB |  |  |
|  | UL MOD | DL MOD |  | CH BW |  | DLalloc / UL alloc | UL MOD |  |  |  | DL MOD |  | UL/DL Ch BW | DLalloc / UL alloc |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 | Mid |  |  |  |  | n3 |  |  |  | Mid |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |
| 24,10 | 1 | UL 1950 / DL 2140 |  |  |  |  | n3 |  |  |  | UL 1760 / DL 1855 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 36 | 1 | Low |  |  |  |  | n3 |  |  |  | High |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1950 / DL 2140 |  |  |  |  | n78 |  |  |  | 3710 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 | Mid |  |  |  |  | n1 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |
| 26 | 3 | High |  |  |  |  | n1 |  |  |  | Low |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |
| 34 | 3 | UL 1760 / DL 1855 |  |  |  |  | n1 |  |  |  | UL 1950 / DL 2140 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 | High |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |
| 26 | 3 | High |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |
| 34, 10 | 3 | UL 1740 / DL 1835 |  |  |  |  | n41 |  |  |  | 2657.5 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3495 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |
| 23 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3525 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 35 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3685 |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |
| 3a9 | 3 | Low |  |  |  |  | n78 |  |  |  | High |  |  |  |  |
|  | QPSK | QPSK |  | 20 |  | All RBs/0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 | All RBs / REFSENS_ENDC_2 |  |
| 44 | 3 | UL 1740 / DL 1835 |  |  |  |  | n78 |  |  |  | 3575 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 54 | 3 | UL 1765 / DL 1860 |  |  |  |  | n78 |  |  |  | 3435 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  |  |  | n3 |  |  |  | DL 1825 |  |  |  |  |
|  | QPSK | N/A |  | 10 MHz |  | N/A | N/A |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 932.5 |  |  |  |  | n1 |  |  |  | DL 2155 |  |  |  |  |
|  | QPSK | N/A |  | 5 MHz |  | N/A | N/A |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 945 |  |  |  |  | n3 |  |  |  | DL 1850 |  |  |  |  |
|  | QPSK | N/A |  | 5 MHz |  | N/A | N/A |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 24 | 8 | DL 942.5 |  |  |  |  | n3 |  |  |  | DL 1842.5 |  |  |  |  |
|  | QPSK | N/A |  | 5 MHz |  | N/A | N/A |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_8A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | Mid |  |  |  |  | n77 |  |  |  | 3590 |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 2,8 | 8 | Mid |  |  |  |  | n77 |  |  |  | 3520 |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 34 | 8 | UL 897.5 |  |  |  |  | n77 |  |  |  | 3635 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 | Mid |  |  |  |  | n79 |  |  |  | 4457.7 |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| 2,9 | 11 | Mid |  |  |  |  | n79 |  |  |  | 4512.7 |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 | DL 884 |  |  |  |  | n79 |  |  |  | Low |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |
| 2,9 | 19 | DL 884 |  |  |  |  | n79 |  |  |  | 4445.02 |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 840 / DL 799 |  |  |  |  | n3 |  |  |  | UL 1775 / DL 1870 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 24 | 20 | UL 847 / DL 806 |  |  |  |  | n3 |  |  |  | UL 1735 / DL 1830 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 | Mid |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 23 | 20 | Mid |  |  |  |  | n78 |  |  |  | 3388 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |
| 34 | 20 | UL 850 / DL 809 MHz |  |  |  |  | n78 |  |  |  | 3359 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  | n41 |  | 2524.5 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | QPSK |  | 50 MHz |  |  | All RBs / 0 |
| 2,8 | 26 | High |  |  |  |  |  | n41 |  | 2572 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | QPSK |  | 50 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 839 |  |  |  |  |  | n41 |  | 2562 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  | n77/n78 |  | 3366 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 2,8 | 26 | High |  |  |  |  |  | n77/n78 |  | 3446 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 836.5 |  |  |  |  |  | n77/n78 |  | 3391 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 | High |  |  |  |  |  | n79 |  | 4432.5 |  |  |  |  |  |
|  | N/A | QPSK | 15 MHz |  | All RBs / 0 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 | 26 | High |  |  |  |  |  | n79 |  | 4470.5 |  |  |  |  |  |
|  | N/A | QPSK | 15 MHz |  | All RBs / 0 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 28 | Low |  |  |  |  |  | n77/n78 |  | 3540 |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 28 | 28 | Low |  |  |  |  |  | n77/n78 |  | Low |  |  |  |  |  |
|  | QPSK | QPSK | 10 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 34 | 28 | UL 705.5 / DL 760.5 |  |  |  |  |  | n77/n78 |  | 3582.5 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 45 | 28 | Low |  |  |  |  |  | n77 |  | 3815 |  |  |  |  |  |
|  | N/A | QPSK | 10 MHz |  | All RBs / 0 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 | Mid |  |  |  |  |  | n78 |  | 3525 |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 2,9 | 40 | Low |  |  |  |  |  | n78 |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz |  | All RBs / REFSENS_ENDC_2 |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 | Mid |  |  |  |  |  | n77/n78 |  | 3889.5 |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 26 | 41 | High |  |  |  |  |  | n77/n78 |  | Low |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 41 | High |  |  |  |  |  | n77/n78 |  | Low |  |  |  |  |  |
|  | QPSK | QPSK | 20MHz |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84.. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

<End of Changes>