3GPP TSG-RAN5 Meeting #92-e	R5-215206

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1397 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of n24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specifications for Rel-17 n24 band in RAN4 has been completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying band grouping for n24 for RRM testing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band group specific RRM test cases parameters will not be specified for n24 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3A.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Rel-17 WIC will trigger a spec. upgrade |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 3A.4.1	NR operating bands in FR1

NR frequency bands grouping for FR1 is specified in Table 3A.4.1-1.

Table 3A.4.1-1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation |  | Operating bands |
| A | NR_FDD_FR1_A | n1, n18, n24, n70, n744 | NR_TDD_FR1_A | n34, n38, n39, n40, n50, n51, n53 | NR_SDL_FR1_A |  | n75, n76 |
| B | NR_FDD_FR1_B | n65, n66, n743 | NR_TDD_FR1_B | n387 | NR_SDL_FR1_B | - |  |
| C | NR_FDD_FR1_C | n30 | NR_TDD_FR1_C | n48, n771, n78, n79 | NR_SDL_FR1_C | - |  |
| D | NR_FDD_FR1_D | n28 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - |  |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41 | NR_SDL_FR1_E | - |  |
| F | NR_FDD_FR1_F | n266 | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - |  |
| G | NR_FDD_FR1_G | n3, n8, n12, n14, n20, n71 | NR_TDD_FR1_G | - | NR_SDL_FR1_G | n29 |  |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - |  |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. NOTE 6: The minimum Io condition is reduced by 0.5 dB when the carrier frequency of the assigned NR channel bandwidth is within 865-894 MHz. NOTE 7: When this band is only used for V2X SL service, the band is exclusively used for NR V2X in particular regions. |  |  |  |  |  |  |  |

Table 3A.4.1-2: Power offsets for the test configuration between NR frequency band groups for FR1 with respect to NR_FDD_FR1_A

| Group | NR FDD |  | NR TDD |  | NR SDL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Power Offset [dB], ΔBG_offset | Band group notation | Power Offset [dB], ΔBG_offset | Band group notation | Operating bands |
| A | NR_FDD_FR1_A | - | NR_TDD_FR1_A | 0.0 | NR_SDL_FR1_A | 0.0 |
| B | NR_FDD_FR1_B | 0.5 | NR_TDD_FR1_B | 0.5 | NR_SDL_FR1_B | - |
| C | NR_FDD_FR1_C | 1.0 | NR_TDD_FR1_C | 1.0 | NR_SDL_FR1_C | - |
| D | NR_FDD_FR1_D | 1.5 | NR_TDD_FR1_D | 1.5 | NR_SDL_FR1_D | - |
| E | NR_FDD_FR1_E | 2.0 | NR_TDD_FR1_E | 2.0 | NR_SDL_FR1_E | - |
| F | NR_FDD_FR1_F | 2.5 | NR_TDD_FR1_F | 2.5 | NR_SDL_FR1_F | - |
| G | NR_FDD_FR1_G | 3.0 | NR_TDD_FR1_G | 3.0 | NR_SDL_FR1_G | - |
| H | NR_FDD_FR1_H | 3.5 | NR_TDD_FR1_H | 3.5 | NR_SDL_FR1_H | - |
| NOTE 1: In the test parameters table, only the power configuration for NR_FDD_FR1_A or NR_TDD_FR1_A will be given. |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------