3GPP TSG-RAN5 Meeting #92-e	R5-215309

# Electronic Meeting, 16th – 27th Aug. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2013 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding signalling test frequencies for SUL band n97 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test frequencies for NR standalone signalling testing for SUL band n97 need to be added in TS 38.508-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding signalling test frequencies for n97 in Table 6.2.3.1-7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Signalling test frequencies for n97 is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RF test frequencies for n97 is introduced in R5-215308 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2.3.1	Test frequencies for NR standalone signalling testing

The default channel bandwidth for signalling test is specified per NR band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For signalling test cases, the mapping of frequency ranges to NR test frequencies are as follows:

-	for band with only one test frequency (e.g. n51): Low Range (NRf1);

-	for band with up to two test frequencies: Low Range (NRf1) and High Range (NRf2);

-	for band with up to three test frequencies: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3);

-	for band with up to four test frequencies: Low Range (NRf1), Mid Low Range (NRf2), Mid High Range (NRf3) and High Range (NRf4);

The signalling test frequencies NRf5, NRf6, NRf7 are mapped respectively as NRf1, NRf2, NRf3 on the operating band for inter-band.

The test frequencies, subcarrier spacing, default channel bandwidth, SS/PBCH block and CORESET#0 parameters for signalling is specified in Table 6.2.3.1-1 (FDD FR1 BW 5MHz), Table 6.2.3.1-2 (FDD FR1 BW 10MHz), Table 6.2.3.1-3 (TDD FR1 BW 5MHz), Table 6.2.3.1-4 (TDD FR1 BW 10MHz), Table 6.2.3.1-4A (TDD FR1 BW 60MHz), Table 6.2.3.1-5 (TDD FR1 BW 100MHz), Table 6.2.3.1-6 (TDD FR2 BW 100MHz) and Table 6.2.3.1-7 (NR FDD FR1 SUL bands).

<Unchanged Text Skipped>

Table 6.2.3.1-7: Test frequencies for NR FDD FR1 SUL bands

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | AbsoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.80-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n81 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.81-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n82 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.82-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n83 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.83-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n84 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.84-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n86 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.86-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n97 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.97-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |

<End of Changes>