3GPP TSG-RAN5 Meeting #92-e                                                      R5-215523

Electronic Meeting, 17th May – 28th May

Title: 		        5GS_NR_LTE-UEConTest Work plan extension request

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.2.17

Document for:	Discussion and Endorsement

# 1.	Introduction

Current Work plan for 5GS_NR_LTE-UEConTest work item ([1]) doesn’t include test cases for EN-DC including FR2 with more than 6CC for neither UL nor DL CA test cases.

This document analyses which of those test cases should be added to that work plan.

To be noticed that this discussion paper is related to a similar contribution for NR_CADC_NR_LTE_DC_R16-UEConTest to be discussed in RAN5#92-e ([7]).

# 2.	Core requirements overview

2.1 EN-DC including FR2 for Downlink CA

In latest core requirements for EN-DC for Rel-15 ([2]), there are multiple bands including EN-DC band combos with more than 6CC in FR2. Some examples are shown below:

- Intra-band contiguous in FR2: DC_2A-n260M8CC in FR2 DL

- Intra-band non-contiguous in FR2: DC_66A_n260(E-Q)7CC in FR2 DL

Band combinations used as examples above are already included in 5GS_NR_LTE-UEConTest Work plan.

2.2 EN-DC including FR2 for Uplink CA

In latest core requirements for EN-DC for Rel-15 ([2]), there are only band combos whose UL includes one NR FR2 CC in the UL (maximum). Hence testing of such band combos are already considered in 5GS_NR_LTE-UEConTest Work Plan.

# 3. 38.307 overview

In section 8.1.2.2 in 38.307, it seems the number of CCs within FR2 has been evolving along the releases as follows:

- Rel-15 ([4]):

- Rel-16 ([5]):

- Rel-17 ([6]): Same as Rel-16

Observation 1: Increase of DL NR CCs in FR2 in Rel-16 is supposed to be Release independent from Rel-15.

# 4. Proposals

Considering the analysis in sections above, some extensions are required for EN-DC including FR2 in [1].

Possible options for such extensions are described in table below:

| Option ID | Affected Work Plan | DL/UL CA | Number of FR2 CCs to be considered | Rationale |
| --- | --- | --- | --- | --- |
| #1a | 5GS_NR_LTE-UEConTest | DL | 10 CCs | As per 38.307 Rel-16 |
| #1b | 5GS_NR_LTE-UEConTest | DL | 8CCs | As per: - 38.101-3 (maximum of intra-band contiguous and intra-band non-contiguous) - 38.307 Rel-15 |
| #1c | 5GS_NR_LTE-UEConTest | DL | 8CCs for intra-band contiguous 7CCs for intra-band non-contiguous | As per 38.101-3 (maximum number of CCs included in any defined EN-DC band combo including intra-band contiguous and intra-band non-contiguous CA) |
| #2a | 5GS_NR_LTE-UEConTest | UL | 8CCs | As per 38.307 Rel-15 |
| #2b | 5GS_NR_LTE-UEConTest | UL | 1CC | As per current band combos defined in 38.101-3 Rel-15 |

Taking into account the frequency band combinations considered in the Work Plan, the following proposals are made:

Proposal 1: Extend 5GS_NR_LTE-UEConTest Work Plan to include EN-DC including FR2 with up to 8CCs in NR DL CA (option #1b).

Proposal 2: Keep current 5GS_NR_LTE-UEConTest Work Plan as it is for NR UL CA in FR2, i.e. not considering any NR FR2 UL CA (option #2b).

# 5.	Conclusion

This document identifies one gap in 5GS_NR_LTE-UEConTest work plan related to test cases for EN-DC including FR2 with more than 6CC and makes the following observation and proposals:

Observation 1: Increase of DL NR CCs in FR2 in Rel-16 is supposed to be Release independent from Rel-15.

Proposal 1: Extend 5GS_NR_LTE-UEConTest Work Plan to include EN-DC including FR2 with up to 8CCs in NR DL CA (option #1b).

Proposal 2: Keep current 5GS_NR_LTE-UEConTest Work Plan as it is for NR UL CA in FR2, i.e. not considering any NR FR2 UL CA (option #2b).

# 6.	References

[1] R5-212657, “WP UE Conformance Test Aspects - 5G system with NR and LTE”, Ericsson, RAN5#91-e

[2] 38.101-3 v 15.14.0, “User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[3] 38.521-3 v17.1.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[4] 38.307 v15.8.0, “Requirements on User Equipments (UEs) supporting a release-independent frequency band”

[5] 38.307 v16.7.0, “Requirements on User Equipments (UEs) supporting a release-independent frequency band”

[6] 38.307 v17.2.0, “Requirements on User Equipments (UEs) supporting a release-independent frequency band”

[7] R5-215524, “NR_CADC_NR_LTE_DC_R16-UEConTest Work plan extension request”, Keysight Technologies, RAN5#92-e