3GPP TSG-RAN5 Meeting #92-e	R5-215611

# Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0224 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case 10.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the WP test case 10.9 (based on similar test case 19.4.5 in TS 34.229-1) needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case 10.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage will remaing incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.9	Emergency call without emergency registration / UE credentials are not accepted / 5GS

10.9.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and being made to initiate an emergency call }

ensure that {

when { UE attempts IMS emergency registration and network declines by sending 403 Forbidden }

then { UE initiates and completes IMS emergency calls on non-protected ports }

}

10.9.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 33.203 clause 7.1]

The P-CSCF is allowed to receive only REGISTER messages, messages relating to emergency services in accordance with TS 23.167 [31] and TS 24.229 [8], and error messages related to unprotected messages on unprotected ports. All other messages not arriving on a protected port shall be either discarded or rejected by the P-CSCF.

4.	The UE is allowed to receive only the following messages on an unprotected port:

-	responses to unprotected REGISTER messages;

-	messages relating to emergency services in accordance with TS 23.167 [31] and TS 24.229 [8];

-	error messages related to unprotected messages.

All other messages not arriving on a protected port shall be rejected or silently discarded by the UE.

[TS 24.229 clause 5.1.6.1]

If the IM CN subsystem is selected and the UE is currently attached to its home operator's network (e.g. HPLMN) and the UE is currently registered and the IP-CAN defines emergency bearers and the core network has indicated that it supports emergency bearers, the UE shall:

1)	perform an initial emergency registration, as described in subclause 5.1.6.2; and

2)	attempt an emergency call as described in subclause 5.1.6.8.3.

10.9.3	Test description

10.9.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS.

10.9.3.2	Test procedure sequence

Table 10.9.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE to initiate an emergency call. | - | - | - | - |
| 2-4 | Steps 1-3 of annex A.3 (emergency registration) | - | - | - | - |
| 5 | EXCEPTION: From this point onward the SS shall ignore any Registration message sent by the UE. | - | -- | - | - |
| 6 | The SS sends this message to get the UE in a stable state. | <-- | 403 Forbidden | - | - |
| 7 | Step 1 of annex A.6 (emergency call) Check: Does the UE send a correctly composed INVITE request? | --> | INVITE | 1 | P |
| 8 | Step 2 of annex A.6 (emergency call). | <-- | 100 Trying | - | - |
| 9 | Step 3 of annex A.6 (emergency call). | <-- | 180 Ringing | - | - |
| 10 | Step 4 of annex A.6 (emergency call). | <-- | 200 OK | - | - |
| 11 | Step 5 of annex A.6 (emergency call) Check: Does the UE send ACK? | --> | ACK | 1 | P |
| 12 | Step 1 of annex A.8 (MT Release of Voice Call) | <-- | BYE | - | - |
| 13 | Step 1 of annex A.8 (MT Release of Voice Call) | --> | 200 OK | - | - |

10.9.3.3	Specific message contents

Table 10.9.3.3-1: INVITE (step 7, table 10.9.3.2-1)

Use the default message for INVITE as specified in Annex A.6 for this step except that in addition to the specified conditions the condition A6 “INVITE for creating an emergency session in case of no registration” should also be used when deriving the message from Annex A.2.1 of TS 34.229-1 [2].

Table 10.9.3.3-2: 180 Ringing (step 9, table 10.9.3.2-1)

Use the default message for 180 Ringing as specified in Annex A.6 for this step except that in addition to the specified conditions the condition A7 “Response sent by SS for emergency call without emergency registration” should also be used when deriving the message from Annex A.2.6 of TS 34.229-1 [2].

Table 10.9.3.3-3: 200 OK (step 10, table 10.9.3.2-1)

Use the default message for 200 OK as specified in Annex A.6 for this step except that in addition to the specified conditions the condition A7 “Response sent by SS for INVITE for emergency call without emergency registration” should also be used when deriving the message from Annex A.3.1 of TS 34.229-1 [2].

Table 10.9.3.3-4: BYE (step 12, table 10.9.3.2-1)

Use the default message for BYE as specified in Annex A.6 for this step except that in addition to the specified conditions the condition A6 “BYE for emergency call with no registration”should also be used when deriving the message from Annex A.2.8 of TS 34.229-1 [2].