Source:		Rohde & Schwarz, Huawei

Title:	Discussion on test point selection for EVM equalizer spectrum flatness for Pi/2 BPSK in FR1

Document for:	Endorsement

Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the EVM equalizer spectrum flatness test case for Pi/2 BPSK is analyzed.

Discussion

The EVM equalizer spectrum flatness is a measure for the ripple inside the transmission bandwidth of an UE and is defined in section 6.4.2.4.1 and Annex F Transmit Modulation of TS 38.101-1 [1].

The minimum requirement is specified as follows:

6.4.2.4.1	Requirements for pi/2 BPSK modulation

These requirements apply if the IE powerBoostPi2BPSK is set to 1 for power class 3 capable UE operating in TDD bands n40, n77, n78 and n79 with pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission. These requirements also apply if the IE dmrs-UplinkTransformPrecoding-r16 is configured  and UE indicates support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16.Otherwise the requirements for EVM equalizer spectrum flatness defined in clause 6.4.2.4 apply.

The EVM equalizer coefficients across the allocated uplink block shall be modified to fit inside the mask specified in Table 6.4.2.4.1-1 for normal conditions, prior to the calculation of EVM.

Table 6.4.2.4.1-1: Mask for EVM equalizer coefficients for pi/2 BPSK, normal conditions

| Frequency range | Parameter | Maximum ripple [dB] |
| --- | --- | --- |
| F_meas – F_center ≤ X MHz or F_center – F_meas ≤ X MHz (Range 1) | X1 | 6 (p-p) |
| F_meas – F_center > X MHz or F_center – F_meas > X MHz (Range 2) | X2 | 14 (p-p) |
| NOTE 1: F_meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: F_center refers to the center frequency of an allocated block of PRBs NOTE 3: X, in MHz, is equal to 25% of the bandwidth of the PRB allocation NOTE 4: See Figure 6.4.2.4-1 for description of X1, X2 |  |  |

Figure 6.4.2.4.1-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation. F_center denotes the center frequency of the allocated block of PRBs. F_alloc denotes the bandwidth of the PRB allocation.

For pi/2 BPSK modulation the UE shall be allowed to employ spectral shaping and the shaping filter shall be restricted so that the impulse response of the shaping filter itself shall meet

│ãt(t,0)│ ≥ │ãt(t, τ)│    ∀τ ≠ 0

20log10│ãt(t,τ)│< -15 dB    1< τ < M - 1,

where,│ãt(t, τ)│=IDFT{│ãt(t,f)│ejφ (t,f)},   f  is the frequency of the M allocated subcarriers , ã(t,f) and φ(t,f) are the amplitude and phase response.

0dB reference is defined as 20log10│ãt(t,0)│.

Test Environment

In NR FR1, EVM equalizer spectrum flatness for modulations except Pi/2 BPSK is tested under extreme conditions. However, since TS 38.101-1 does not define the minimum requirements under extreme conditions for EVM equalizer spectrum flatness for Pi/2 BPSK, it is proposed to apply normal condition as test environment.

Proposal 1: Define test environment as Normal for EVM equalizer spectrum flatness for Pi/2 BPSK in FR1.

Test Subcarrier Spacing

In order to maximize the tested bandwidth the smallest SCS is selected as it provides the highest spectrum efficiency.

Proposal 2: Select smallest SCS for EVM equalizer spectrum flatness for Pi/2 BPSK in FR1.

Test Frequencies

In LTE, EVM equalizer spectrum flatness is tested for Low range, Mid range, and High range. Due to the presence of filters in the FR1 UEs it is reasonable to follow the LTE approach for NR.

Proposal 3: Test EVM equalizer spectrum flatness for Pi/2 BPSK for Low range, Mid range, and High range frequencies in FR1.

Test Channel Bandwidth

In LTE, EVM equalizer spectrum flatness is tested for Lowest, 5 MHz and Highest channel bandwidths. Due to the presence of filters in the FR1 UEs it is reasonable to follow the LTE approach for NR.

Proposal 4: Test EVM equalizer spectrum flatness for Pi/2 BPSK for Lowest, Mid and Highest channel bandwidth in FR1.

Test Modulations and waveforms

The test purpose is for Pi/2 BPSK which is therefore selected as modulation.

Proposal 5: Test EVM equalizer spectrum flatness for Pi/2 BPSK with DFT-s-OFDM Pi/2 BPSK for UEs indicating support for UE capability powerBoosting-pi2BPSK.

Proposal 5a: Test EVM equalizer spectrum flatness for Pi/2 BPSK with DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS for UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16.

RB allocation

In LTE, EVM equalizer spectrum flatness is tested with full allocation to test all equalizer coefficients. The same approach is followed for NR.

Proposal 6: Test EVM equalizer spectrum flatness for Pi/2 BPSK with Outer Full allocation.

Conclusion

To sum up, the following selection of test points is proposed for EVM equalizer spectrum flatness in FR1:

Proposal 1: Define test environment as Normal for EVM equalizer spectrum flatness for Pi/2 BPSK in FR1.

Proposal 2: Select smallest SCS for EVM equalizer spectrum flatness for Pi/2 BPSK in FR1.

Proposal 3: Test EVM equalizer spectrum flatness for Pi/2 BPSK for Low range, Mid range, and High range frequencies in FR1.

Proposal 4: Test EVM equalizer spectrum flatness for Pi/2 BPSK for Lowest, Mid and Highest channel bandwidth in FR1.

Proposal 5: Test EVM equalizer spectrum flatness for Pi/2 BPSK with DFT-s-OFDM Pi/2 BPSK for UEs indicating support for UE capability powerBoosting-pi2BPSK.

Proposal 5a: Test EVM equalizer spectrum flatness for Pi/2 BPSK with DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS for UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16.

Proposal 6: Test EVM equalizer spectrum flatness for Pi/2 BPSK with Outer Full allocation.

Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR1.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 3 | 1 | 1 | 9 |
| Note: UEs indicating support for UE capability powerBoosting-pi2BPSK or lowPAPR-DMRS-PUSCHwithPrecoding-r16 |  |  |  |  |  |

- References

- TS 38.101-1 V15.2.0 , 3GPP TSG RAN WG4, June 2018