3GPP TSG-RAN WG5 Meeting #92-e	R5-216273

# Electronic Meeting, 16th Aug 2021 – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2454 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-16 SNPN TC 9.1.11.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NPN TC 9.1.11.1 SNPN / Initial registration / Rejected / Temporarily not authorized for this SNPN |  |  |  |  |  |  |  |  |
| Summary of change: |  | NPN TC 9.1.11.1 SNPN / Initial registration / Rejected / Temporarily not authorized for this SNPN is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NPN WP will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.11.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-215670->R5-215670r1 TP1 editorial changes are done. R5-216273: This is the final version of R5-215670r1 |  |  |  |  |  |  |  |  |

### 9.1.11	SNPN / Mobility management aspects

#### 9.1.11.1	SNPN / Initial registration / Rejected /  Temporarily not authorized for this SNPN

9.1.11.1.1	Test Purpose (TP)

(1)

with { UE in Automatic SNPN selection mode and a SNPN cell is available for which an entry exists in the "list of subscriber data" and the UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when {  the SS sends a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value #74 (Temporarily not authorized for this SNPN) }

then { the UE deletes any 5G-GUTI, last visited registered TAI and ngKSI, and stores the SNPN identity in the "temporarily forbidden SNPNs" }

}

(2)

with {  the initial registration request cannot be accepted by the network }

ensure that {

when {  the SS sends a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value #74 (Temporarily not authorized for this SNPN) }

then { The UE stores the SNPN identity in the "temporarily forbidden SNPNs" and does not remove it atleast until 60 minutes or the UE is switched off }

}

(3)

with { the UE is in 5GMM-DEREGISTERED.PLMN-SEARCH state and the SNPN identity of the current cell belongs to the list of "temporarily forbidden SNPNs" }

ensure that {

when { the UE enters a cell belonging to a SNPN cell for which an entry exists in the "list of subscriber data" }

then { the UE attempts registration on the SNPN cell }

}

9.1.11.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501 clauses 5.5.1.2.5, 5.1.3.2.1, 5.1.3.2.2, TS 23.122 clause 4.9.3.0. Unless otherwise stated these are Rel-16 requirements.

[TS 24.501, clause 5.5.1.2.5]

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

If the initial registration request is rejected due to general NAS level mobility management congestion control, the network shall set the 5GMM cause value to #22 "congestion" and assign a back-off timer T3346.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

…

#74	(Temporarily not authorized for this SNPN).

5GMM cause #74 is only applicable when received from a cell belonging to an SNPN. 5GMM cause #74 received from a cell not belonging to an SNPN is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.5.1.2.7.

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete any 5G-GUTI, last visited registered TAI, TAI list and ngKSI. The UE shall reset the registration attempt counter and store the SNPN identity in the "temporarily forbidden SNPNs" list for the specific access type for which the message was received. The UE shall enter state 5GMM-DEREGISTERED.PLMN-SEARCH and perform an SNPN selection according to 3GPP TS 23.122 [5]. If the message has been successfully integrity checked by the NAS, the UE shall set the SNPN-specific attempt counter for 3GPP access and the SNPN-specific attempt counter for non-3GPP access for the current SNPN to the UE implementation-specific maximum value.

If the message has been successfully integrity checked by the NAS and the UE also supports the registration procedure over the other access to the same SNPN, the UE shall in addition handle 5GMM parameters and 5GMM state for this access, as described for this 5GMM cause value.

NOTE 4:	When 5GMM cause #74 is received over 3GPP access, the term "other access" in "the UE also supports the registration procedure over the other access to the same SNPN" is used to express access to SNPN services via a PLMN.

…

[TS 24.501, clause 5.1.3.2.1.3.5]

The substate 5GMM-DEREGISTERED.PLMN-SEARCH is chosen in the UE, if the UE is searching for PLMNs or SNPNs. This substate is left either when a cell has been selected (the new substate is NORMAL-SERVICE or LIMITED-SERVICE) or when it has been concluded that no cell is available at the moment (the new substate is NO-CELL-AVAILABLE).

This substate is not applicable to non-3GPP access.

[TS 24.501, clause 5.1.3.2.2]

In order to describe the detailed UE behaviour, the 5GS update (5U) status pertaining to a specific subscriber is defined.

If the UE is not operating in SNPN access operation mode (see 3GPP TS 23.501 [8]), the 5GS update status is stored in a non-volatile memory in the USIM if the corresponding file is present in the USIM, else in the non-volatile memory in the ME, as described in annex C.

If the UE is operating in SNPN access operation mode, the 5GS update status for each SNPN whose SNPN identity is included in the "list of subscriber data" configured in the ME (see 3GPP TS 23.122 [5]) is stored in the non-volatile memory in the ME as described in annex C.

The 5GS update status value is changed only after the execution of a registration, network-initiated de-registration, 5GS based primary authentication and key agreement, service request, paging procedure or due to change in TAI which does not belong to the current registration area while T3346 is running.

5U1: UPDATED

The last registration attempt was successful.

5U2: NOT UPDATED

The last registration attempt failed procedurally, e.g. no response or reject message was received from the AMF.

5U3: ROAMING NOT ALLOWED

The last registration, service request, or registration for mobility or periodic registration update attempt was correctly performed, but the answer from the AMF was negative (because of roaming or subscription restrictions).

[TS 23.122, clause 4.9.3.0]

The ME is configured with a "list of subscriber data" containing zero or more entries. Each entry of the "list of subscriber data" consists of:

…

The MS shall maintain a list of "temporarily forbidden SNPNs" and a list of "permanently forbidden SNPNs" in the ME. Each entry of those lists consists of an SNPN identity.

The MS shall add an SNPN to the list of "temporarily forbidden SNPNs", if a message with cause value #74 "Temporarily not authorized for this SNPN" (see 3GPP TS 24.501 [64]) is received by the MS in response to an LR request from the SNPN. In addition, if:

-	the message is integrity-protected; or

-	the message is not integrity-protected, and the value of the SNPN-specific attempt counter for that SNPN is equal to the MS implementation specific maximum value as defined in 3GPP TS 24.501 [64];

then the MS shall start an MS implementation specific timer not shorter than 60 minutes.The MS shall remove an SNPN from the list of "temporarily forbidden SNPNs", if:

a)	there is a successful LR after a subsequent manual selection of the SNPN;

b)	the MS implementation specific timer not shorter than 60 minutes expires;

c)	the timer T3247 expires and the value of the SNPN-specific attempt counter for that SNPN is less than the MS implementation specific maximum value as defined in 3GPP TS 24.501 [64];

d)	the MS is switched off; or

e)	an entry of the "list of subscriber data" with the SNPN identity of the SNPN is updated or the USIM is removed if:

-	EAP based primary authentication and key agreement procedure using EAP-AKA'; or

-	5G AKA based primary authentication and key agreement procedure;

was performed in the selected SNPN.

If an SNPN is removed from the list of "temporarily forbidden SNPNs" list, the MS shall stop the MS implementation specific timer not shorter than 60 minutes, if running.

9.1.11.1.3	Test description

9.1.11.1.3.1	Pre-test conditions

System Simulator:

-	2 SNPN cells NGC Cell A and NGC Cell B are configured according to Table 6.3.2.2-1 broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-12 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NGC Cells.

UE:

-	The UE is in Automatic SNPN selection mode.

-	The UE is provisioned with a “list of subscriber data” to allow access to SNPN identified by NGC Cell A and NGC Cell B.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN. And the UE is in state Switched OFF (state 0-A).

9.1.11.1.3.2	Test procedure sequence

Table 9.1.11.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NGC Cell A as the "Serving cell ". - NGC Cell B as “Non-suitable cell”. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3-14 | Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed | - | - | - | - |
| 15 | The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #74 "Temporarily not authorized for this SNPN". | <-- | REGISTRATION REJECT | - | - |
| 16 | The SS starts timer 1 = 60 min. | - | - | - | - |
| 17 | The SS releases the RRC connection. | - | - | - | - |
| 18 | Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A in the next 60 seconds? | --> | REGISTRATION REQUEST | 1 | F |
| 19 | The SS reconfigures: - NGC cell B as the "Serving cell". | - | - | - | - |
| 20 | Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell B? | --> | REGISTRATION REQUEST | 3 | P |
| 21-27 | Steps 5 to 11 from procedure in TS 38.508-1 [4] Table 4.5.2.2-2 are performed. | - | - | - | - |
| 28 | The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #74 " Temporarily not authorized for this SNPN". | <-- | REGISTRATION REJECT | - | - |
| 29 | The SS releases the RRC connection. | - | - | - | - |
| 30 | The SS reconfigures: - NGC cell B as the " Non-suitable cell". | - | - | - | - |
| 31 | Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A? | --> | REGISTRATION REQUEST | - | - |
| 32 | The SS stops timer 1. | - | - | - | - |
| 33 | The UE is switched OFF | - | - | - | - |
| 34 | The UE is Switched ON | - | - | - | - |
| 35 | Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A? | --> | REGISTRATION REQUEST | 2 | P |
| 36-51 | Steps 5-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NGC Cell A. | - | - | - | - |

9.1.11.1.3.3	Specific message contents

Table 9.1.11.1.3.3-1: REGISTRATION REJECT (step 15 and step 28, Table 9.1.11.1.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | ‘01001010’B | #74 "Temporarily not authorized for this SNPN" |  |

Table 9.1.11.1.3.3-2: REGISTRATION REQUEST (step 20 and step 35, Table 9.1.11.1.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ngKSI |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111" |  |
| 5GS mobile identity | The valid SUCI | Only SUCI is available. |  |
| Last visited registered TAI | Not present |  |  |