3GPP TSG-RAN WG5 Meeting #92-e	R5-214327r1

# Electronic Meeting, 16th - 27th August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0556 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to FR2 minimum output power test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current minimum output power test case is using a test requirement that is too relaxed. This is caused by adding relaxation due to testability issue and TT without a combined statistical analysis on the risk of failing a marginally good UE. As shown in discussion paper R5-214326, a marginal UE will pass the test with at least 4.75 dB margin (400 MHz BW). In other words: A bad UE with 4.75 dB worse performance than the core requirement will be guaranteed to pass the current test case. This can improved by 3.35 dB after performing a combined (TT and Relaxation) statistical analysis of the risk of failing a good UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the test requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-compliant UEs will pass the test leading to performance degradation in networks |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related discussion paper R5-214326 R5-214327r1: Moved back the definition of R from Annex F to test case. Corrected title of Table 6.3.1.5-2 to exclude other power classes than PC3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.1	Minimum output power

Editor’s Note: The following aspects of the clause are for future consideration:

-	Testing of extreme conditions for FR2 is FFS.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

6.3.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

6.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.3.1.3	Minimum conformance requirements

The minimum controlled output power of the UE is defined as the EIRP in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value.

The minimum output power is defined as the mean power in at least one subframe (1ms).

6.3.1.3.1	Minimum output power for power class 1

For power class 1 UE, the minimum output power shall not exceed the values specified in Table 6.3.1.3.1-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3.1.3.1-1: Minimum output power for power class 1

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | 4 | 47.58 |
|  | 100 | 4 | 95.16 |
|  | 200 | 4 | 190.20 |
|  | 400 | 4 | 380.28 |

6.3.1.3.2	Minimum output power for power class 2, 3, and 4

The minimum output power shall not exceed the values specified in Table 6.3.1.3.2-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3.1.3.2-1: Minimum output power for power class 2, 3, and 4

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | -13 | 47.58 |
|  | 100 | -13 | 95.16 |
|  | 200 | -13 | 190.20 |
|  | 400 | -13 | 380.28 |
| NOTE 1: n260 is not applied for power class 2. |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.3.1.

6.3.1.4	Test description

6.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1. |  | Highest |  |
| Test Parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | N/A for minimum output power test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.1.4.3.

6.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "down" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE transmits at its minimum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the measurement bandwidth specified in Table 6.3.1.5-1 and Table 6.3.1.5-2 for the specific channel bandwidth under test. EIRP test procedure is defined in Annex K.1.3. The measuring duration is at least one active subrame (1ms). EIRP is calculated considering both polarizations, theta and phi. For TDD, only slots consisting of only UL symbols are under test.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.3.1.5	Test requirement

The maximum EIRP, derived in step 5 shall not exceed the values specified in Table 6.3.1.5-1 and Table 6.3.1.5-2.

Table 6.3.1.5-1: Minimum output power for power class 1

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | 4+TT | 47.58 |
|  | 100 | 4+TT | 95.16 |
|  | 200 | 4+TT | 190.20 |
|  | 400 | 4+TT | 380.28 |

Table 6.3.1.5-2: Minimum output power for power class 3

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Test Tolerance TT (dB) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| n257, n258, n261 | 50 | -13+TT | 4.21 | 47.58 |
|  | 100 | -13+2.4+TT1 | 2.52 | 95.16 |
|  | 200 | -13+5.4+TT1 | 0 | 190.20 |
|  | 400 | -13+8.4+TT1 | 0 | 380.28 |
| n260 | 50 | -13+4.5+TT1 | 1.17 | 47.58 |
|  | 100 | -13+7.5+TT1 | 0 | 95.16 |
|  | 200 | -13+10.5+TT1 | 0 | 190.20 |
|  | 400 | -13+13.5+TT1 | 0 | 380.28 |
| NOTE 1: Core requirement cannot be tested due to testability issue and test requirement includes relaxation to achieve impact from test system noise to measurement result = 1.0 dB (Minimum requirement + relaxation). |  |  |  |  |

Table 6.3.1.5-3: Minimum output power Test tolerance for power class 1, 2, 4

FFS

Table 6.3.1.5-4: Void

{Unchanged sections skipped}

# F.1	Acceptable uncertainty of Test System (normative)

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. Care should be taken to ensure that each conformance test implementation including the OTA chamber aspects meets the specified measurement uncertainty for each test case by requiring the test laboratory to maintain a detailed measurement uncertainty test report showing compliance to all the measurement uncertainty requirements. The detailed measurement uncertainty report would contain the justification for each measurement uncertainty component and its value and distribution. The derivation of these values is based on the minimum conformance requirements plus relaxation, i.e., test tolerance is not to be considered. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

The downlink signal uncertainties apply at the defined quiet zone with the UE properly positioned in the quiet zone. The uplink signal uncertainties apply at the measurement equipment with the UE positioned properly in the quiet zone.

## F.1.1	Measurement of test environments

Editor’s note: Various measurement accuracies for UE test environments, e.g., pressure, relative humidity, DC&AC voltage, vibration, and vibration frequency, are FFS:

The measurement accuracy of the UE test environments defined in TS 38.508-1 [5] subclause 4.1, Test environments shall be

-	Temperature			±4 degrees.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2-2 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Quiet Zone size ≤ 30 cm ±4.42 dB (FR2a) ±4.62 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-2 in TR 38.903) |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC3 Quiet Zone size ≤ 30 cm ±4.60 dB (FR2a) ±5.20 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-4 in TR 38.903) |
| 6.2.2 UE maximum output power for modulation / channel bandwidth | PC3 Quiet Zone size ≤ 30 cm ±4.92 dB (FR2a) ±5.10 dB (FR2b) | MTSU = 1.00 x MU (from Table B.4.4.2 in TR 38.903) |
| 6.2.3 UE maximum output power with additional requirements | TBD |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD | MTSU = 1.00 x MU (from Table B.4.2-2 in TR 38.903) |
| 6.3.1 Minimum output power | PC1 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm FFS (FR2a) FFS dB (FR2b) PC3 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm ±6.15 dB (FR2a & FR2b) |  |
| 6.3.2 Transmit OFF power | PC3: Quiet Zone size ≤ 30 cm ±5.49 dB (FR2a) | MTSU = 1.00 x MU (from Table B.8-2-2 in TR 38.903) |
| 6.3.3.2 General ON/OFF time mask | PC3: ON power: TBD OFF power: Quiet Zone size ≤ 30 cm ±6.15 dB (FR2a & FR2b) |  |
| 6.3.3.4 PRACH time mask | PC3: PRACH power: TBD OFF power: Quiet Zone size ≤ 30 cm ±6.15 dB (FR2a & FR2b) |  |
| 6.3.3.6 SRS time mask | TBD |  |
| 6.3.4.2 Absolute power tolerance | Same as 6.3.1 |  |
| 6.3.4.3 Relative power tolerance | PC3 Quiet Zone size ≤ 30 cm [±1.4 dB] (FR2a) [±1.4 dB] (FR2b) | MTSU = 1.00 x MU (from Table B.9a2.2-2 in TR 38.903) |
| 6.3.4.4 Aggregate power tolerance | PC3 Quiet Zone size ≤ 30 cm [±1.4 dB] (FR2a) [±1.4 dB] (FR2b) | MTSU = 1.00 x MU (from Table B.9a2.2-2 in TR 38.903) |
| 6.3A.1.1 Minimum output power for CA (2UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.2.1 Transmit OFF power for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.2 Transmit OFF power for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.3 Transmit OFF power for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO | PC3: OFF Power Quiet Zone size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |
| 6.3D.3.4 SRS time mask for UL MIMO | PC3: OFF Power Quiet Zone size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4.1 Frequency error | ± 0.01 ppm (NTC & ETC testing) | MTSU = 1.00 x MU (from B.10.1 and B.10.2 in TR 38.903) |
| 6.4.2.1 Error vector magnitude | TBD |  |
| 6.4.2.2 Carrier leakage | TBD |  |
| 6.4.2.3 In-band emissions | TBD |  |
| 6.4.2.4 EVM equalizer spectrum flatness | TBD |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation | TBD |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) | TBD |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) | TBD |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) | TBD |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) | TBD |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) | TBD |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) | TBD |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) | TBD |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) | TBD |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) | TBD |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) | TBD |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) | TBD |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) | TBD |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) | TBD |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) | TBD |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) | TBD |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) | TBD |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) | TBD |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) | TBD |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) | TBD |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) | TBD |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) | TBD |  |
| 6.5.1 Occupied bandwidth | Quiet Zone size ≤ 30cm PC3: FR2a: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.2 [%CBW] (BW 200MHz) ±1.2 [%CBW] (BW 400MHz) FR2b: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.3 [%CBW] (BW 200MHz) ±1.3 [%CBW] (BW 400MHz) FR2c: TBD |  |
| 6.5.2.1 Spectrum Emission Mask | Quiet Zone size ≤ 30 cm ±4.94 dB (FR2a) ±5.32 dB (FR2b) | MTSU = 1.00 x MU (from Table B.16.2-2, B.16.2-3 in TR 38.903) |
| 6.5.2.3 Adjacent Channel Leakage Ratio | Quiet Zone size ≤ 30cm FR2a: ±5.63 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) FR2b: ±6.09 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) | MTSU = 1.00 x MU (from Table B.17-1 in TR 38.903) |
| 6.5.3.1 Transmitter Spurious emissions | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz) ±5.11 dB (12.75GHz < f ≤ 23.45GHz) ±5.41 dB (23.45GHz < f < 40.8GHz) ±7.42 dB (40.8GHz ≤ f ≤ 66GHz) ±7.72 dB (66GHz ≤ f ≤ 80GHz) | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |
| 6.5.3.2 Spurious emission band UE co-existence | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz Protected band n260, n261, n257: ±6.00 dB Protected frequency 57 GHz ≤ f ≤ 66GHz: ±8.01 dB Protected frequency 36 GHz ≤ f ≤ 37GHz: ±6.00 dB | MTSU = 1.00 x MU (from Table B.18-1a in TR 38.903) |
| 6.5.3.3 Additional Spurious emission | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz), NS_202 ±5.70 dB (12.75GHz < f ≤ 23.45GHz), NS_202 ±6.00 dB (23.45GHz < f < 40.8GHz), NS_202, NS_203 ±8.01 dB (40.8GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band), NS_202 | MTSU = 1.00 x MU (from Table B.18-1b in TR 38.903) |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Quiet Zone size ≤ 30cm PC3: FR2a: TBD FR2b: TBD FR2c: TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) | TBD |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) | TBD |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) | TBD |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) | TBD |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD | MTSU = 1.00 x MU (from Table B.17.2-2 and Table B.17.2-4 in TR 38.309) |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) | TBD |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) | TBD |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) | TBD |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) | TBD |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.4 Transmitter Spurious emissions for CA (5UL CA) | TBD |  |
| 6.5A.3.1.5 Transmitter Spurious emissions for CA (6UL CA) | TBD |  |
| 6.5A.3.1.6 Transmitter Spurious emissions for CA (7UL CA) | TBD |  |
| 6.5A.3.1.7 Transmitter Spurious emissions for CA (8UL CA) | TBD |  |
| 6.5A.3.2.1 Spurious emission band UE co-existence for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.2.2 Spurious emission band UE co-existence for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.2.3 Spurious emission band UE co-existence for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.2.4 Spurious emission band UE co-existence for CA (5UL CA) | TBD |  |
| 6.5A.3.2.5 Spurious emission band UE co-existence for CA (6UL CA) | TBD |  |
| 6.5A.3.2.6 Spurious emission band UE co-existence for CA (7UL CA) | TBD |  |
| 6.5A.3.2.7 Spurious emission band UE co-existence for CA (8UL CA) | TBD |  |
| 6.5A.3.3.1 Additional spurious emissions for CA (2UL CA) | TBD |  |
| 6.5A.3.3.2 Additional spurious emissions for CA (3UL CA) | TBD |  |
| 6.5A.3.3.3 Additional spurious emissions for CA (4UL CA) | TBD |  |
| 6.5A.3.3.4 Additional spurious emissions for CA (5UL CA) | TBD |  |
| 6.5A.3.3.5 Additional spurious emissions for CA (6UL CA) | TBD |  |
| 6.5A.3.3.6 Additional spurious emissions for CA (7UL CA) | TBD |  |
| 6.5A.3.3.7 Additional spurious emissions for CA (8UL CA) | TBD |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause |  | Maximum Test System Uncertainty |  | Derivation of MTSU |  |
| --- | --- | --- | --- | --- | --- |
| 7.3.2 Reference sensitivity power level |  | PC3 ±5.19 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b, NTC testing) ±5.45 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.19-2-2 in TR 38.903) |  |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3.4 EIS spherical coverage |  | PC3 ±4.90 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b) |  | MTSU = 1.00 x MU (from Table B.19-2-2 in TR 38.903) |  |
| 7.4 Maximum input level |  | TBD |  |  |  |
| 7.5 Adjacent channel selectivity |  | PC3 ±7.84 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b) |  | MTSU = 1.00 x MU (from Table B.21-1 in TR 38.903) |  |
| 7.6.2 In-band blocking |  | Same as 7.5 |  |  |  |
| 7.9 Spurious emissions |  | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz For Band n257: ±5.50dB (6GHz ≤ f ≤ 12.75GHz) ±5.46dB (12.75GHz < f ≤ 23.45GHz) ±6.11dB (23.45GHz < f < 40.8GHz) ±7.65dB (40.8GHz ≤ f ≤ 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) |  | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |  |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.1.2-1. |  |  |  |  |  |

# F.2	Interpretation of measurement results (normative)

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause F.1 of the present document.

If the Test System using one of the permitted test methods defined in TR38.903 [20] for a test is known to have a measurement uncertainty greater than that specified in clause F.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows:

Any additional uncertainty in the Test System over and above that specified in clause F.1 shall be used to tighten the Test Requirement, making the test harder to pass. For some tests, for example receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with clause F.1does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause F.1 had been used.

# F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

## F.3.1	Measurement of test environments

TBD

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) |  | PC3 Minimum peak EIRP IFF (Quiet Zone size ≤ 30 cm) 2.87 dB (FR2a), NTC 2.87 dB (FR2b), NTC 3.04 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) Max EIRP 0 dB |  | Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) |  |
| 6.2.1.1 UE maximum output power (TRP) |  | PC3 Max TRP IFF (Quiet Zone size ≤ 30 cm) 2.65 dB (FR2a) 2.77 dB (FR2b) |  | Max TRP TT = 0.60 x MTSUIFF |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) |  | PC1 TBD PC2 TBD PC3 IFF (Quiet Zone size ≤ 30 cm) 2.58 dB (FR2a) 2.58 dB (FR2b) PC4 TBD |  | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) |  |
| 6.2.2 UE maximum output power for modulation / channel bandwidth |  | PC3 Minimum peak EIRP IFF (Quiet Zone size ≤ 30 cm) 3.11 dB (FR2a) 3.11 dB (FR2b) |  | Minimum peak EIRP TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) |  |
| 6.2.3 UE maximum output power with additional requirements |  | TBD |  |  |  |
| 6.2.4 Configured transmitted power |  | TBD |  |  |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) |  | Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3.1 Minimum output power |  | PC3 Minimum EIRP IFF (Quiet Zone size ≤ 30 cm) 4.21 dB (FR2a 50 MHz) 2.52 dB (FR2a 100 MHz) 0 dB (FR2a 200 MHz) 0 dB (FR2a 400 MHz) 1.17 dB (FR2b 50 MHz) 0 dB (FR2b 100 MHz) 0 dB (FR2b 200 MHz) 0 dB (FR2b 400 MHz) |  | Minimum EIRP TT = max(R, ΔSNRmr + 0.65 x (MTSUIFF – 1.0) -R R: Relaxation needed to ensure 6 dB SNR in TE (specified in clause 6.3.1.5) ΔSNRmr: Systematic offset due to noise when measuring at minimum requirement level (-13 dBm) FR2a 50 MHz: ΔSNRmr = 0.86 dB FR2a 100 MHz: ΔSNRmr = 1.57 dB FR2a 200 MHz: ΔSNRmr = 2.71 dB FR2a 400 MHz: ΔSNRmr = 4.35 dB FR2b 50 MHz: ΔSNRmr = 2.32 dB FR2b 100 MHz: ΔSNRmr = 3.82 dB FR2b 200 MHz: ΔSNRmr = 5.82 dB FR2b 400 MHz: ΔSNRmr = 8.21 dB |  |
| 6.3.2 Transmit OFF power |  | 0 dB |  |  |  |
| 6.3.3.2 General ON/OFF time mask |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD |  | ON Power TBD |  |
| 6.3.3.4 PRACH time mask |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD |  | ON Power TBD |  |
| 6.3.4.2 Absolute power tolerance |  | Same as 6.3.1 |  |  |  |
| 6.3.4.3 Relative power tolerance |  | PC3 IFF (Quiet Zone size ≤ 30 cm) [0.26 dB] (FR2a) [0.26 dB] (FR2b) |  | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |  |
| 6.3.4.4 Aggregate power tolerance |  | PC3 IFF (Quiet Zone size ≤ 30 cm) [0.26 dB] (FR2a) [0.26 dB] (FR2b) |  | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.2.1 Transmit OFF power for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.2.2 Transmit OFF power for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.2.3 Transmit OFF power for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3D.3.4 SRS time mask for UL MIMO |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4.1 Frequency error |  | 0.005 ppm (NTC & ETC testing) |  | TT = 0.5 x MTSU |  |
| 6.4.2.1 Error vector magnitude |  | 0%, up to 64QAM |  | Minimum requirement + TT |  |
| 6.4.2.2 Carrier leakage |  | TBD |  |  |  |
| 6.4.2.3 In-band emissions |  | TBD |  |  |  |
| 6.4.2.4 EVM equalizer spectrum flatness |  | TBD |  |  |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation |  | TBD |  |  |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) |  | TBD |  |  |  |
| 6.5.1 Occupied bandwidth |  | 0 kHz |  | Minimum requirement + TT |  |
| 6.5.2.1 Spectrum Emission Mask |  | IFF (Quiet Zone size ≤ 30 cm) 3.21 dB (FR2a) 3.46 dB (FR2b) |  | TT = 0.65 x MTSUIFF |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio |  | Absolute requirement 0 dB Relative requirement IFF (Quiet Zone size ≤ 30 cm) FR2a: ±4.66 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) FR2b: ±4.96 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5.3.1 Transmitter Spurious emissions |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.2 Spurious emission band UE co-existence |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.3 Additional spurious emission |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |  |  |  |

## F.3.3	Measurement of receiver