3GPP TSG-RAN WG5 Meeting #92-e	R5-214636r1

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0165 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.2.14 – SIP SUBSCRIBE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Missing condition POC-SETTINGS-EVENT for Event and Accept header |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Condition POC-SETTINGS-EVENT added for Event and Accept header |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete, inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision1: Format corrections |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.14	SIP SUBSCRIBE

This message is sent by the UE.

Table 5.5.2.14-1: SIP SUBSCRIBE

| Derivation Path: TS 24.229 [16] clause A.2.1.4.13, A.2.2.4.13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "SUBSCRIBE" |  |  |  |
| Request-URI | tsc_MCPTT_PublicServiceId_A | The public service identity identifying the originating participating MCPTT function serving the MCPTT user |  |  |
|  | px_MCVideo_PublicServiceId_A | The public service identity identifying the originating participating MCVideo function serving the MCVideo user |  | MCVIDEO |
|  | px_MCData_PublicServiceId_A | The public service identity identifying the originating participating MCData function serving the MCData user |  | MCDATA |
|  | "sip:" & tsc_MCX_CMS_Hostname | SIP URI of the CMS's domain name: public service identity (PSI) for performing subscription proxy function of the CMS | TS 24.484 [14] clause 6.3.13.2.2 | CONFIG |
|  | tsc_MCX_GMSURI | public service identity (PSI) for performing subscription proxy function of the GMS as configured in the <GMS-URI> element of the initial UE configuration | TS 24.481 [11] clause 6.3.13.2.1 | GROUPCONFIG |
|  | same URI as the SS has sent earlier in the Contact header of a message within the same dialog | Contact URI of the recipient of the previous 200 OK |  | re_SUBSCRIBE |
| SIP-Version | "SIP/2.0" |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | "scscf.3gpp.org" |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Route |  |  | RFC 3261 [22] | re_SUBSCRIBE |
| route-param list | URIs of the Record-Route header sent to the UE in the response which has established the dialog, in reverse order |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  | UDP |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by |  |  |  |  |
| host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE | as assigned during registration |  |  |
| via-branch | value starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | Default public user id (px_MCX_SIP_PublicUserId_A_1) |  |  |  |
| port | not present |  |  |  |
| tag | any value |  |  |  |
| From |  |  | RFC 3261 [22] | re_SUBSCRIBE |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog ID (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | same URI as used as Request URI |  |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| To |  |  | RFC 3261 [22] | re_SUBSCRIBE |
| addr-spec | Same URI of the SS as used earlier in the dialogURI | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog ID (from the UE's point of view) |  |  |
| Contact |  |  | RFC 3261 [22] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  |  |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Data (MCData) communication. |  | MCDATA |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" | This URN indicates that the device has the capabilities to support the mission critical video (MCVideo) service. |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  | MCDATA |
| feature-param | "audio" |  |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA |
| Expires |  |  | RFC 3261 [22] RFC 3903 [43] |  |
| value | any value |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security -Server header sent by SS during registration |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  |  |
|  | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  | re_SUBSCRIBE |
| method | "SUBSCRIBE" |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any allowed value |  |  |  |
|  | same value as in SUBSCRIBE creating the dialog |  |  | re_SUBSCRIBE |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] RFC 7913 [51] |  |
| access-net-spec | Access network technology and, if applicable, the cell ID | Access network technology and, if applicable, the cell ID |  |  |
| Event |  |  | RFC 6665 [39] |  |
| event-type | "presence" |  |  |  |
|  | "xcap-diff" |  |  | CONFIG GROUPCONFIG |
|  | "poc-settings" |  |  | MCDATA, POC-SETTINGS-EVENT |
| Accept |  |  | RFC 3261 [22] |  |
| media-range | "application/pidf+xml" |  |  |  |
|  | "application/xcap-diff+xml" |  |  | CONFIG, GROUPCONFIG |
|  | "application/poc-settings+xml" |  |  | POC-SETTINGS-EVENT |
| P-Preferred-Service |  |  | RFC 6050 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 |  |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | SIMPLE-FILTER |  | PRESENCE-EVENT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/simple-filter+xml" |  |  |  |
| MIME-part-body | SIMPLE-FILTER as described in Table 5.5.3.6-1 |  | TS 24.379 [9] clause 9.3.2 |  |
|  | SIMPLE-FILTER as described in Table 5.5.3.6-2 |  | TS 24.281 [86] clause 8.3.2 | MCVIDEO |
|  | SIMPLE-FILTER as described in Table 5.5.3.6-3 |  | TS 24.282 [87] clause 8.4.2 | MCDATA |
| MIME body part |  | Resource-lists |  | CONFIG, GROUPCONFIG |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/resource-lists+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 |  |  |  |
|  | Resource-lists as described in Table 5.5.3.3.1-2 |  |  | MCVIDEO |
|  | Resource-lists as described in Table 5.5.3.3.1-3 |  |  | MCDATA |
| MIME body part |  | MIKEY | RFC 3830 [24] | CONFIG, GROUPCONFIG |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | MIKEY message as described in Table 5.5.9.1-1 | MIKEY message, containing the CSK | TS 33.180 [94] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| re_SUBSCRIBE | SUBSCRIBE within a dialog |
| For further conditions see table 5.5.1-1 |  |