3GPP TSG-RAN5 Meeting #92-e	R5-214686r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1324 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As an outcome of discussin for interruption test case in R4-2108850 in RAN4, following parts are updated in 38.133. - Defined new RMC of SR.1.2 TDD for SCS15kHz TDD which schedules PDSCH even at partical-downlink slots. - Changed SSB burst position on SCell to at even SFN(SSB.5 FR1 or SSB.6 FR1) - SMTC for monitoring SCell SSB was updated to cover the new SSB burst position(SMTC.4) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test parameter table in 6.5.2.1 and RMC in A.1.1.2 are aligned with 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption cannot be evaluated correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1, A.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency : R4-2115251(Revised from R4-2111856) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 : Aligned with revised RAN4 CR R4-2115251 |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

6.5.2	Interruption

6.5.2.0	Minimum conformance requirements

6.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC

Interruptions on PCell due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [13] is 640 ms or longer.

If the PCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on PCell immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 6.5.2.0.1-1

If the PCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by Table 6.5.2.0.1-2. The interruption shall not exceed requirements in Table 6.5.2.0.1-2.

Interruptions on active SCells due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [13] is 640 ms or longer.

If the activated SCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on an activated SCell immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 6.5.2.0.1-1

If the activated SCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on the activated SCell no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by Table 6.5.2.0.1-2. The interruption shall not exceed requirements in Table 6.5.2.0.1-2.

Table 6.5.2.0.1-1: Interruption duration for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length (slot) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |

Table 6.5.2.0.1-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | X (slots) | Interruption length (slots) |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 | 8 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is the longest SMTC duration among all above active serving cells and the deactivated SCell to be measured; NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [7]. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.2.2.2.

6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC

6.5.2.1.1	Test purpose

To verify UE’s ability to complete  NR PCell interruptions during the measurement on the deactivated NR SCC within the missed ACK/NACK rate in standalone NR requirements.

6.5.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 2DL CA.

6.5.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.2.1.

6.5.2.1.4	Test description

6.5.2.1.4.1	Initial conditions

Test 6.5.2.1 can be run in one of the configurations defined in Table 6.5.2.1.4.1-1.

Table 6.5.2.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.5.2.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD – FDD duplex mode |
| 6.5.2.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD – TDD duplex mode |
| 6.5.2.1-3 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD – FDD duplex mode |
| 6.5.2.1-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD – TDD duplex mode |
| 6.5.2.1-5 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD – TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.2.1.4.1-2.

Table 6.5.2.1.4.1-2: Initial conditions for NR SA FR1 interruptions during measurements on deactivated NR SCC

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.5.2.1.4.1-3

2. Message contents are defined in clause 6.5.2.1.4.3.

3. There are two NR carriers and two cells specified in the test. Cell 1 is the PCell on one NR carrier, Cell 2 is the SCell on the other NR carrier. Cell 1 and Cell 2 shall be configured according to Annex C.1.1 and C.1.2.

Table 6.5.2.1.4.1-3: General test parameters for NR SA FR1 interruptions during measurements on deactivated NR SCC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell2 | Deactivated SCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to Cell1 and Cell 2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

6.5.2.1.4.2	Test procedure

The test consists of two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is deactivated SCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE shall be connected to Cell1 and Cell2 and the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector. Cell1 shall be configured as PCell and Cell2 shall be configured as SCell.. During T1, PCell is continuously scheduled in DL.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall transmit an RRCReconfiguration message including measCycleSCell or allowInterruptions for the deactivated NR SCell.

3.	The UE shall transmit RRCReconfigurationComplete message.

4.	Set the parameters according to Table 6.5.2.1.5-1. Propagation conditions are set according to Annex C.2.1. T1 starts.

5.	SS schedules on PCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

6.	If more than 99.5% of uplink transmissions are received by SS then count a success for the event “ACK/NACK”. Otherwise count a fail for the event “ACK/NACK”.

7.	If no longer than X consecutive DTX is observed by the SS, then count a success for the event “DTX”. Otherwise count a fail for the event “DTX”. Where

-	For test configuration 6.5.2.1-1 and 6.5.2.1-3,

-	X = interruption length+k1 if k1 ≤ interruption length

-	X = interruption length if k1 > interruption length

-	For test configuration 6.5.2.1-2, 6.5.2.1-4 and 6.5.2.1-5,

-	X = interruption length.

-	interruption length is given in table 6.5.2.1.5-2 for inter-band case and in 6.5.2.1.5-3 for intra-band case.

8.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED TS 38.508-1 [14] clause 4.5),

or

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat step 2-8 until a test verdict has been achieved.

Each of the events "ACK/NACK" and "DTX" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.

If all events pass, the test passes. If one event fails, the test fails.

6.5.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition Deactivated SCell; |
| Specific message contents exceptions for Test Configuration 6.5.2.1-1, 6.5.2.1-2, 6.5.2.1-3 and 6.5.2.1-4 | Table H.3.1-3 with Condition Deactivated SCell and SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.5.2.1-5 | Table H.3.1-3 with Condition Deactivated SCell and SSB.2 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

6.5.2.1.5	Test requirement

Table 6.5.2.1.5-1 defines the primary level settings including test tolerances for NR SA FR1 interruptions during measurements on deactivated NR SCC.

Table 6.5.2.1.5-1: NR cell specific test parameters for NR SA FR1 interruptions during measurements on deactivated NR SCC

| Parameter |  | Unit | Cell1 |  | Cell2 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |  | FR1 |
| Duplex mode | Config 1 |  | FDD |  | FDD |
|  | Config 2,5 |  | TDD |  | TDD |
|  | Confiq 3 |  | TDD |  | FDD |
|  | Confiq 4 |  | FDD |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.1 |  | Not Applicable |
|  | Confiq 4 |  | Not Applicable |  | TDDConf.1.1 |
|  | Confiq 5 |  | TDDConf.1.2 |  | TDDConf.1.2 |
| BWchannel | Config 1,2,3,4 |  | 10 MHz: NRB,c = 52 |  | 10 MHz: NRB,c = 52 |
|  | Config 5 |  | 40 MHz: NRB,c = 106 |  | 40 MHz: NRB,c = 106 |
| Initial DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.0.1 | DLBWP.0.1 |  |
|  | Config 5 |  | DLBWP.0.1 | DLBWP.0.1 |  |
| Dedicated DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.1.1 | DLBWP.1.1 |  |
|  | Config 5 |  | DLBWP.1.1 | DLBWP.1.1 |  |
| Initial UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.0.1 | ULBWP.0.1 |  |
|  | Config 5 |  | ULBWP.0.1 | ULBWP.0.1 |  |
| Dedicated UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.1.1 | ULBWP.1.1 |  |
|  | Config 5 |  | ULBWP.1.1 | ULBWP.1.1 |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.2 TDD |  | SR.1.2 TDD |
|  | Config 3 |  | SR.1.2 TDD |  | SR.1.1 FDD |
|  | Confiq 4 |  | SR.1.1 FDD |  | SR.1.2 TDD |
|  | Confiq 5 |  | SR.2.1 TDD |  | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |
|  | Config 3 |  | CR.1.1 TDD |  | CR.1.1 FDD |
|  | Confiq 4 |  | CR.1.1 FDD |  | CR.1.1 TDD |
|  | Confiq 5 |  | CR.2.1 TDD |  | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.1.1 TDD |  | CCR.1.1 FDD |
|  | Config 4 |  | CCR.1.1 FDD |  | CCR.1.1 TDD |
|  | Config 5 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 |  | OP.1 |
| SMTC Configuration |  |  | SMTC.1 |  | SMTC.4 |
| SSB Configuration | Config 1,2,3,4 |  | SSB.1 FR1 |  | SSB.5 FR1 |
|  | Config 5 |  | SSB.2 FR1 |  | SSB.6 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 |  | -104 |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 |  | -87 |
| Ês/Iot |  | dB | 17 |  | 17 |
| Ês/Noc |  | dB | 17 |  | 17 |
| NocNote 2 | Config 1,2,3,4 | dBm/SCS | -104 |  | -104 |
|  | Config 5 |  | -101 |  | -101 |
| IoNote3 | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 |  | -58.96 |
|  | Config 5 | dBm/ 38.16MHz | -52.86 |  | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - |  | 3 |
| Propagation Condition |  |  | AWGN |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2 defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |

The UE shall be continuously scheduled on PCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on PCell.

If the NR PCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on NR PCell immediately before and immediately after an SMTC. Each interruption on NR PCell shall not exceed the value defined in Table 6.5.2.1.5-2.

If the NR PCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. the interruption on NR PCell shall not exceed the value defined in Table 6.5.2.1.5-3.

Table 6.5.2.1.5-2: Interruption duration if the PCell is not in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |

Table 6.5.2.1.5-3: Interruption duration if the PCell is in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 2 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |

The rate of correct events observed during repeated tests shall be at least 90%.

<<End of change>>

A.1.1.2	TDD

Table A.1.1.2-1: PDSCH Reference Measurement Channels for SCS = 15 kHz for TDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | SR.1.1 TDD | SR.1.2 TDD |
| Channel bandwidth | MHz | 10 | Defined in test case |
| Number of transmitter antennas |  | 1 | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 | 24 |
| Allocated slots per Radio Frame |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 |
| Radio frame not containing SSB | slots | 4 | 6 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 |
| PDSCH mapping type |  | Type A | Type A |
| Information Bit Payload |  |  |  |
| For slots with RMSI Note 2 | Bits | 1608 | 1608 |
| For slots without RMSI | Bits | 1864 | 1864 |
| For special slots | Bits | N/A | 1128 |
| Number of Code Blocks per slot |  | 1 |  |
| Binary Channel Bits Per slot |  |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 | 6048 |
| For special slots Note 6 | Bits | - | 3744 |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |  |

Table A.1.1.2-2: PDSCH Reference Measurement Channels for SCS = 30 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.2.1 TDD |
| Channel bandwidth | MHz | 40 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  |  |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 10 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

Table A.1.1.2-3: PDSCH Reference Measurement Channels for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD | SR.3.2 TDD | SR.3.3 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 |
| Number of transmitter antennas |  | 1 | 1 | 1 |
| Allocated resource blocks for PDSCH |  | 24Note 1 | 24Note 7 | 48Note 7 |
| Allocated slots per Radio Frame |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 8 | Note 5 |
| Radio frame not containing SSB | slots | 48 | 48 | 48 |
| MCS table |  | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 | 2 |
| PDSCH mapping type |  | Type A | Type A | Type A |
| Information Bit Payload |  |  |  |  |
| For slots with RMSI | Bits | 1608 | 1608 | 3104 |
| For slots without RMSI | Bits | 1864 | 1864 | 3624 |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |
| Binary Channel Bits Per slot |  |  |  |  |
| For slots with RMSI Note 4 | Bits | 5184 | 5184 | 10368 |
| For slots without RMSI Note 6 | Bits | 6048 | 6048 | 12096 |
| Note 1: Allocated in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: Void Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditionalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: Allocated in the same resource blocks as the CORESET. Note 8: When DRX is configured, PDSCH is scheduled only while drx-onDurationTimer is running, unless otherwise specified in the test case. |  |  |  |  |

<<Unchaged sections skipped>>