3GPP TSG-RAN WG5 Meeting #92-e	R5-214745r1

# Electronic Meeting, 16th Aug 2021 – 27th Aug 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2337 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Rel-16 RACS TC 9.1.9.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - At step 13, there is no need for the SS to include URCID. - To simplify Step 15, UE doesn’t need to be in connected state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated Step 13 accordingly, instead of adding specific message contents for Registration Accept. - Modifying Step 15 to release RRC connection (UE in RRC_IDLE state) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-214745R5-214745r1 Editorial changes are done |  |  |  |  |  |  |  |  |

#### 9.1.9.5	RACS / Handling of delete indication for NW assigned UE radio capability ID

9.1.9.5.1	Test Purpose (TP)

(1)

with { UE in 5GMM-REGISTERED state and a UE Radio Capability ID assigned in REGISTRATION ACCEPT message }

ensure that {

when { UE receives CONFIGURATION UPDATE COMMAND with UE radio capability ID deletion indication  and indicating “registration requested” }

then { UE deletes any stored Network-assigned UE radio capability IDs and then releases the existing N1 NAS signalling connection, starts a mobility registration procedure and does not include any Network-assigned UE radio capability IDs in the REGISTRATION REQUEST message }

}

(2)

with { UE in state 5GMM-REGISTERED, and 5GMM-IDLE mode over 3GPP access and assigned a new UE Radio Capability ID, enters a new tracking area that is not in the list of tracking areas that the UE previously registered in the AMF }

ensure that {

when { UE receives REGISTRATRATION ACCEPT message with UE radio capability ID deletion indication }

then { UE deletes the stored Network-assigned UE radio capability ID and initiate a registration procedure for mobility and periodic registration update over the existing N1 NAS signalling connection}

}

9.1.9.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501, clauses 4.16, 5.4.4.1, 5.4.4.2, 5.4.4.3, 5.5.1.2.4. Unless otherwise stated these are Rel-16 requirements.

[TS 24.501, clause 4.16]

UE radio capability signalling optimisation (RACS) is a feature that is optional at both the UE and the network and which aims to optimise the transmission of UE radio capability over the radio interface (see 3GPP TS 23.501 [8]). RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called the UE radio capability ID. A UE radio capability ID can be either manufacturer-assigned or network-assigned. The UE radio capability ID is an alternative to the signalling of the radio capabilities container over the radio interface.

In this release of the specification, RACS is applicable to neither NB-N1 mode nor non-3GPP access.

If the UE supports RACS:

a)	the UE shall indicate support for RACS by setting the RACS bit to "RACS supported" in the 5GMM capability IE of the REGISTRATION REQUEST message;

b)	if the UE performs a registration procedure for initial registration and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID in the UE radio capability ID IE as a non-cleartext IE in the REGISTRATION REQUEST message. If both a network-assigned UE radio capability ID and a manufacturer-assigned UE Radio Capability ID are applicable, the UE shall include the network-assigned UE radio capability ID in the REGISTRATION REQUEST message;

...

d)	upon receiving a network-assigned UE radio capability ID in the REGISTRATION ACCEPT message or the CONFIGURATION UPDATE COMMAND message, the UE shall store the network-assigned UE radio capability ID and the PLMN ID or SNPN identity of the serving network along with a mapping to the current UE radio configuration in its non-volatile memory as specified in annex C. The UE shall be able to store at least the last 16 received network-assigned UE radio capability IDs with the associated PLMN ID or SNPN identity and the mapping to the corresponding UE radio configuration;

…

f)	upon receiving a UE radio capability ID deletion indication IE set to "delete network-assigned UE radio capability IDs" in the REGISTRATION ACCEPT message or the CONFIGURATION UPDATE COMMAND message, the UE shall delete all network-assigned UE radio capability IDs stored at the UE for the serving network, initiate a registration procedure for mobility and periodic registration update and include an applicable manufacturer-assigned UE radio capability ID for the current UE radio configuration, if available at the UE, in the UE radio capability ID IE of the REGISTRATION REQUEST message; and

g)	if the UE performs a registration procedure for mobility and periodic registration update due to entering a tracking area that is not in the list of tracking areas that the UE previously registered in the AMF and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID in the UE radio capability ID IE as a non-cleartext IE in the REGISTRATION REQUEST message. If both a network-assigned UE radio capability ID and a manufacturer-assigned UE Radio Capability ID are applicable, the UE shall include the network-assigned UE radio capability ID in the REGISTRATION REQUEST message.

If the network supports RACS:

a)	the network may assign a network-assigned UE radio capability ID to a UE which supports RACS by including a UE radio capability ID IE in the REGISTRATION ACCEPT message or in the CONFIGURATION UPDATE COMMAND message;

[TS 24.501, clause 5.4.4.1]

The purpose of this procedure is to:

a)	allow the AMF to update the UE configuration for access and mobility management-related parameters decided and provided by the AMF by providing new parameter information within the command; or

b)	request the UE to perform a registration procedure for mobility and periodic registration update towards the network to update access and mobility management-related parameters decided and provided by the AMF (see subclause 5.5.1.3).

…

[TS 24.501, clause 5.4.4.2]

The AMF shall initiate the generic UE configuration update procedure by sending the CONFIGURATION UPDATE COMMAND message to the UE.

The AMF shall in the CONFIGURATION UPDATE COMMAND message either:

a)	include one or more of the following parameters: 5G-GUTI, TAI list, allowed NSSAI that may include the mapped S-NSSAI(s), LADN information, service area list, MICO indication, NITZ information, configured NSSAI that may include the mapped S-NSSAI(s), rejected NSSAI, network slicing subscription change indication, operator-defined access category definitions, SMS indication, service gap time value, "CAG information list", UE radio capability ID, 5GS registration result, UE radio capability ID deletion indication or truncated 5G-S-TMSI configuration;

b)	include the Configuration update indication IE with the Registration requested bit set to "registration requested"; or

c)	include a combination of both a) and b).

If an acknowledgement from the UE is requested, the AMF shall indicate "acknowledgement requested" in the Acknowledgement bit of the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message and shall start timer T3555. Acknowledgement shall be requested for all parameters except when only NITZ is included.

…

If the AMF includes a UE radio capability ID deletion indication IE in the CONFIGURATION UPDATE COMMAND message, the AMF shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE.

…

If the UE is not in NB-N1 mode and the UE supports RACS, the AMF may include either a UE radio capability ID IE or a UE radio capability ID deletion indication IE in the CONFIGURATION UPDATE COMMAND message.

…

[TS 24.501, clause 5.4.4.3]

Upon receiving the CONFIGURATION UPDATE COMMAND message, the UE shall stop timer T3346 if running and use the contents to update appropriate information stored within the UE.

If "acknowledgement requested" is indicated in the Acknowledgement bit of the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message, the UE shall send a CONFIGURATION UPDATE COMPLETE message.

…

If the CONFIGURATION UPDATE COMMAND message indicates "registration requested" in the Registration requested bit of the Configuration update indication IE and:

…

d)	a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested" is included, and:

1)	the UE is not in NB-N1 mode;

2)	a new allowed NSSAI, a new configured NSSAI or a Network slicing subscription change indication is not included; and

3)	the UE has set the RACS bit to "RACS supported" in the 5GMM capability IE of the REGISTRATION REQUEST message,

the UE shall, after the completion of the generic UE configuration update procedure, start a registration procedure for mobility and registration update as specified in subclause 5.5.1.3.

…

If the UE is not in NB-N1 mode, the UE has set the RACS bit to "RACS supported" in the 5GMM capability IE of the REGISTRATION REQUEST message and the CONFIGURATION UPDATE COMMAND message includes:

a)	a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested", the UE shall delete any network-assigned UE radio capability IDs associated with the RPLMN or RSNPN stored at the UE; and

b)	a UE radio capability ID IE, the UE shall store the UE radio capability ID as specified in annex C

[TS 24.501, clause 5.5.1.2.4]

If the UE is not in NB-N1 mode, the UE has set the RACS bit to "RACS supported" in the 5GMM Capability IE of the REGISTRATION REQUEST message and the REGISTRATION ACCEPT message includes:

a)	a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested", the UE shall delete any network-assigned UE radio capability IDs associated with the RPLMN or RSNPN stored at the UE, then the UE shall, after the completion of the ongoing registration procedure, initiate a registration procedure for mobility and periodic registration update as specified in subclause 5.5.1.3.2 over the existing N1 NAS signalling connection; and

b)	a UE radio capability ID IE, the UE shall store the UE radio capability ID as specified in annex C.

9.1.9.5.3	Test description

9.1.9.5.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [4] belongs to Home PLMN and set as serving cell;

-	NGC Cell B configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [4] and set as Non-Suitable “Off” cell.

-	System information combination NR-2 as defined in TS 38.508[4] clause 4.4.3.1.2 is used.

UE:

None

Preamble:

-	The UE is in state 3N-A on NGC cell A according to TS 38.508-1 [4].

9.1.9.5.3.2	Test procedure sequence

Table 9.1.9.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The SS transmits CONFIGURATION UPDATE COMMAND including UE radio capability ID deletion indication. | <-- | CONFIGURATION UPDATE COMMAND | - | - |
| 2 | Check: Does UE transmit CONFIGURATION UPDATE COMPLETE | --> | CONFIGURATION UPDATE COMPLETE | - | - |
| 3 | SS releases the RRC connection. | - | - | - | - |
| 4 | Check: Does UE transmit a REGISTRATION REQUEST message and does not include any Network-assigned UE radio capability ID? | --> | REGISTRATION REQUEST | 1 | P |
| 5 | The SS transmits a REGISTRATION ACCEPT message including a UE radio capability ID. | <-- | REGISTRATION ACCEPT | - | - |
| 6 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| 7 | The SS transmits an RRCRelease message. | - | - | - | - |
| 8 | The SS configures NGC Cell A as the "Non-suitable cell" and NGC Cell B as the "Serving cell". | - | - | - | - |
| 9 | The UE initiates a Registration procedure for mobility registration update by transmitting a REGISTRATION REQUEST message on NGC Cell B and includes the UE radio capability ID assigned at step 5. | --> | REGISTRATION REQUEST | - | - |
| 10 | The SS transmits REGISTRATION ACCEPT message including UE radio capability ID deletion indication. | <-- | REGISTRATION ACCEPT | - | - |
| 11 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| 12 | Check: Does UE transmit a REGISTRATION REQUEST message and does not include any Network-assigned UE radio capability ID? | --> | REGISTRATION REQUEST | 2 | P |
| 13 | The SS transmits a REGISTRATION ACCEPT message. | <-- | REGISTRATION ACCEPT | - | - |
| 14 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| 15 | The SS transmits an RRCRelease message. | - | - | - | - |

9.1.9.5.3.3	Specific message contents

Table 9.1.9.5.3.3-1: REGISTRATION REQUEST (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '0000 0001'B | Initial registration |  |
| 5GMM capability |  |  |  |
| RACS | ‘1’B | RACS supported |  |

Table 9.1.9.5.3.3-2: REGISTRATION ACCEPT (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘100000000001’H | Type Field (TF): 1 Radio Configuration Identifier (RCI): 00000000001 |  |

Table 9.1.9.5.3.3-3: CONFIGURATION UPDATE COMMAND (step 1, Table 9.1.9.5.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Configuration update indication | ‘11010011’B | Registration and Acknowledgement (ACK) requested |  |
| UE radio capability ID deletion indication | 001 | Network-assigned UE radio capability IDs deletion requested |  |

Table 9.1.9.5.3.3-4: REGISTRATION REQUEST (step 4 and 12, Table 9.1.9.5.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '0000 0001'B | mobility registration updating |  |
| 5GMM capability |  |  |  |
| RACS | ‘1’B | RACS supported |  |

Table 9.1.9.5.3.3-5: REGISTRATION ACCEPT (step 5, Table 9.1.9.5.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘100000000002’H | Type Field (TF): 1 Radio Configuration Identifier (RCI): 00000000002 |  |

Table 9.1.9.2.3.3-6: REGISTRATION REQUEST (step 9, Table 9.1.9.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '00000010'B | mobility registration updating |  |
| 5GMM capability |  |  |  |
| RACS | ‘1’B | RACS supported |  |
| UE radio capability ID | ‘100000000002’H | Type Field (TF): ‘1’H Radio Configuration Identifier (RCI): ‘00000000002’H |  |

Table 9.1.9.5.3.3-7: REGISTRATION ACCEPT (step 10, Table 9.1.9.5.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID deletion indication | 001 | Network-assigned UE radio capability IDs deletion requested |  |