3GPP TSG-RAN5 Meeting #92-e	R5-214884r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0210 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new 5GS IMS generic procedure A.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The generic procedure for MTSI Video Conference Creation / 5GS is needed for test case 8.35 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new generic procedure A.25 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The generic procedure is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Updated message contents of NOTIFY in step 12 using new condition A5 in Annex A.5.3 of TS 34.229-1 [2]. Remove MTSI from the title. |  |  |  |  |  |  |  |  |

A.25	Video Conference Creation / 5GS

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-7 |  |  | Steps 1-7 of Annex A.15.1 | The same messages as in steps 1-7 of Annex A.15.1 |
| 8 | <- |  | 200 OK | The SS responds INVITE with 200 OK and gives the final conference URI within the response |
| 9 | -> |  | ACK | The UE acknowledges the receipt of 200 OK for INVITE |
|  |  |  | EXCEPTION: steps 10–13 describe optional behaviour depending on UE configuration. The SS shall wait up to 3s for the SUBSCRIBE of step 10 |  |
| 10 | -> |  | SUBSCRIBE | UE subscribes the conference event |
| 11 | <- |  | 200 OK | SS responds to the subscription |
| 12 | <- |  | NOTIFY | SS sends the initial state of the conference event to the UE |
| 13 | -> |  | 200 OK | UE responds to the NOTIFY |

Specific Message Contents

The specific message contents for steps 1–7 is otherwise identical to what have been specified in Annex A.15.1, but with the exceptions as below:

INVITE (Step 1)

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |
| To |  |
| addr-spec | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |

183 Session in Progress for INVITE (Step 3)

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | sip:temporary@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | isfocus |
| Record-Route |  |
| rec-route | <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |

200 OK for INVITE (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in Annex A.3.1 of TS 34.229-1 [2], applying conditions A1 and A5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Record-Route |  |
| rec-route | Same value as in the 183 response |
| Contact |  |
| addr-spec | sip:final@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | Isfocus |

ACK (Step 9)

Use the default message “ACK” in Annex A.2.7 of TS 34.229-1 [2], applying conditions A1 and A3, with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:final@conf-factory. appended with px_IMS_HomeDomainName |

SUBSCRIBE (Step 10)

Use the default message “SUBSCRIBE for conference event package” in Annex A.5.1 of TS 34.229-1 [2], applying conditions A1 and A7.

200 OK (Step 11)

Use the default message “200 OK for SUBSCRIBE” in Annex A.5.2 of TS 34.229-1 [2], applying condition A1.

NOTIFY (Step 12)

Use the default message “MT NOTIFY for conference event package” in Annex A.5.3 of TS 34.229-1 [2], applying condition A5.

200 OK (Step 13)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in Annex A.3.1 of TS 34.229-1 [2], applying conditions A5 and A22.