3GPP TSG-RAN5 Meeting #92-e	R5-215044r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5041 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NB-IoT test case for NB-IoT / RACH Procedure / Non-anchor carrier / Preamble format 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of new NB-IoT test case for NB-IoT / RACH Procedure / Non-anchor carrier / Preamble format 2 in accordance with work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding NB-IoT test case 22.3.1.12: NB-IoT / RACH Procedure / Non-anchor carrier / Preamble format 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.12 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1352 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Update the “Category” error on the cover page. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

22.3.1.12	NB-IoT / RACH Procedure / Non-anchor carrier / Preamble format 2

22.3.1.12.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and SystemInformationBlockType23-NB with IE nprach-ParametersListFmt2-r15 is broadcast }

ensure that {

when { UE has need to send a CCCH UL MAC PDU and UE is in enhanced coverage level 0}

then { UE transmits a random access preamble repeated numRepetitionsPerPreambleAttempt from Random Access Preambles group using the non-anchor NPRACH resource corresponding to enhanced coverage level 0}

}

22.3.1.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clauses 5.1.1 and TS 36.331, clauses 5.2.2.4.

[TS 36.321, clause 5.1.1]

…

-	if the UE is an NB-IoT UE:

-	the available set of PRACH resources supported in the Serving Cell on the anchor carrier, nprach-ParametersList, and on the non-anchor carriers, in ul-ConfigList.

-	for EDT, the available set of PRACH resources associated with EDT on anchor carrier, nprach-ParametersList-EDT, and on the non-anchor carriers, in ul-ConfigList.

-	for random access resource selection and preamble transmission:

-	a PRACH resource is mapped into an enhanced coverage level.

-	each PRACH resource contains a set of nprach-NumSubcarriers subcarriers which can be partitioned into one or two groups for single/multi-tone Msg3 transmission by nprach-SubcarrierMSG3-RangeStart and nprach-NumCBRA-StartSubcarriers as specified in TS 36.211 [7, 10.1.6.1]. Each group is referred to as a Random Access Preamble group below in the procedure text.

-	a subcarrier is identified by the subcarrier index in the range:
[nprach-SubcarrierOffset, nprach-SubcarrierOffset + nprach-NumSubcarriers -1]

-	each subcarrier of a Random Access Preamble group corresponds to a Random Access Preamble.

-	when the subcarrier index is explicitly sent from the eNB as part of a PDCCH order ra-PreambleIndex shall be set to the signalled subcarrier index.

-	the mapping of the PRACH resources into enhanced coverage levels is determined according to the following:

-	the number of enhanced coverage levels is equal to one plus the number of RSRP thresholds present in rsrp-ThresholdsPrachInfoList.

-	each enhanced coverage level has one anchor carrier PRACH resource present in nprach-ParametersList and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

-	for EDT, each enhanced coverage level has zero or one anchor carrier PRACH resource present in nprach-ParametersList-EDT and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

-	enhanced coverage levels are numbered from 0 and the mapping of PRACH resources to enhanced coverage levels are done in increasing numRepetitionsPerPreambleAttempt order.

-	when multiple carriers provide PRACH resources for the same enhanced coverage level, the UE will randomly select one of them using the following selection probabilities:

-	the selection probability of the anchor carrier PRACH resource for the given enhanced coverage level, nprach-ProbabilityAnchor, is given by the corresponding entry in nprach-ProbabilityAnchorList

-	the selection probability is equal for all non-anchor carrier PRACH resources and the probability of selecting one PRACH resource on a given non-anchor carrier is (1- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources)

[TS 36.331, clauses 5.2.2.4]

…

1>	if the NB-IoT UE supports NPRACH resources using preamble format 2:

2>	if schedulingInfoList indicates that SystemInformationBlockType23-NB is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType23-NB;

…

22.3.1.12.3	Test description

22.3.1.12.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

NB-IoT Operation Mode set to standalone.

-	System information combination 7 as defined in TS 36.508[18] clause 8.1.4.3.1.1.

- 	SystemInformationBlockType2-NB as specified in Table 22.3.1.12.3.3-1 and SystemInformationBlockType22-NB as specified in Table 22.3.1.12.3.3-3.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 3-NB) according to [18] in Ncell 1.

22.3.1.12.3.2	Test procedure sequence

Table 22.3.1.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts SystemInformationBlockType22-NB to indicate the NPRACH Probablity Anchor is set to zero and notifies the UE of change of System Information. | <-- | Paging-NB | - | - |
| 2 | Wait for 90 seconds for UE to read updated SystemInformationBlockType22-NB | - | - | - | - |
| 3 | The SS transmits a Paging message including a matched identity. | <-- | Paging-NB | - | - |
| 4 | Check: Does the UE transmit a preamble on NPRACH on non-anchor carrier in ul-ConfigList-v1530 in SystemInformationBlockType23-NB? | --> | NPRACH Preamble | 1 | P |
| 5 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing a matching RAPID. | <-- | Random Access Response | - | - |
| 6 | UE sends an RRCConnectionRequest-NB message? | --> | MAC PDU (RRCConnectionRequest-NB) | - | - |
| 7 | The SS transmits a valid MAC PDU containing “UE Contention Resolution Identity” MAC control element with matching “Contention Resolution Identity” and CCCH message (RRCConnectionSetup-NB containing configuration of UE-specific search space). | <-- | MAC PDU (UE Contention Resolution Identity, RRCConnectionSetup-NB) | - | - |
| 8 | UE sends an RRCConnectionSetupComplete-NB message? | --> | RRCConnectionSetupComplete-NB | - | - |

22.3.1.12.3.3	Specific message contents

Table 22.3.1.12.3.3-1: SystemInformationBlockType2-NB (preamble)

| Derivation Path: 36.508 Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon-r13 SEQUENCE {} | RadioResourceConfigCommonSIB-NB-DEFAULT | See table 22.3.1.12.3.3-2 |  |
| } |  |  |  |

Table 22.3.1.12.3.3-2: RadioResourceConfigCommonSIB-NB-DEFAULT (preamble)

| Derivation Path: 36.508 Table 8.1.6.3-9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| nprach-Config-v1530 SEQUENCE { |  |  |  |
| tdd-Parameters-r15 | Not present |  |  |
| fmt2-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-ParametersListFmt2-r15 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ParametersFmt2-NB-r15 SEQUENCE { | 1 entry |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms10 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| npdcch-CarrierIndex-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| nprach-ParametersListFmt2EDT-r15 | Not present |  |  |
| } |  |  |  |
| edt-Parameters-r15 | Not present |  |  |
| } |  |  |  |
| nprach-Config-v1550 | Not present |  |  |
| } |  |  |  |  |

Table 22.3.1.12.3.3-3: SystemInformationBlockType22-NB (preamble)

| Derivation Path: 36.508 Table 8.1.4.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| dl-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigCommon-NB-r14 SEQUENCE { | 1 entry |  |  |
| DL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| dl-CarrierConfig-r14 | DL-CarrierConfigCommon-NB-DEFAULT1 |  |  |
| pcch-Config-r14 SEQUENCE { |  |  |  |
| npdcch-NumRepetitionPaging-r14 | Not present |  |  |
| pagingWeight-r14 | w1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 SEQUENCE { | 1 entry |  |  |
| UL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| ul-CarrierFreq-r14 | Uplink EARFCN for non-anchor carrier under test. For Signalling test cases, see 36.508 clauses 8.3.2.3. Use the anchor carrier EARFCN + 2. |  |  |
| nprach-ParametersList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r14 SEQUENCE { | 1 entry |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms640 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n12 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v4 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pagingWeightAnchor-r14 | w1 |  |  |
| nprach-ProbabilityAnchorList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ProbabilityAnchor-NB-r14 SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | oneHalf |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.12.3.3-4: DL-CarrierConfigCommon-NB-DEFAULT1

| Derivation Path: 36.508 clause 8.1.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DL-CarrierConfigCommon-NB -DEFAULT::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-r14 | Downlink EARFCN for non-anchor carrier under test. For Signalling test cases, see 36.508 clauses 8.3.2.3. Use the anchor carrier EARFCN + 2. |  |  |
| } |  |  |  |

Table 22.3.1.12.3.3-5: SystemInformationBlockType22-NB (Step 1)

| Derivation Path: 22.3.1.12.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchorList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ProbabilityAnchor-NB-r14 SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>