3GPP TSG-RAN5 Meeting #92-e	R5-215158r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5055 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NB-IoT test case 22.4.13 for Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The configuration of NB-IoT test case 22.4.13 is not applicable to Rel-15 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the test case configruation for NB-IoT Rel-15 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 22.4.13 will not be applicable to NB-IoT Rel-15 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1352 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Checked r15 IEs against the PICS parameter. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

22.4.13	NB-IoT / UE capability transfer / Success

22.4.13.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry-NB message }

then { UE transmits an UECapabilityInformation-NB message including UE radio access capability parameters }

}

22.4.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.3.3. Unless and otherwise stated these are Rel-13 requirements.

[TS 36.331, clause 5.6.3.3]

The UE shall:

1>	for NB-IoT, set the contents of UECapabilityInformation message as follows:

2>	include the UE Radio Access Capability Parameters within the ue-Capability-Container;

2>	include ue-RadioPagingInfo;

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

22.4.13.3	Test description

22.4.13.3.1	Pre-test conditions

System Simulator:

-	Ncell 1

UE:

None.

Preamble:

-	The UE is in State Connected Mode (State 2-NB) according to [18].

22.4.13.3.2	Test procedure sequence

Table 22.4.13.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry-NB message to request UE radio access capability information. | <-- | UECapabilityEnquiry-NB | - | - |  |
| 2 | Check: Does the UE transmit a UECapabilityInformation-NB message? | --> | UECapabilityInformation-NB | 1 | P |  |

22.4.13.3.3	Specific message contents

Table 22.4.13.3.3-1: UEcapabilityEnquiry-NB (step 1)

Derivation path: 36.508 clause 8.1.6 Table 8.1.6.1-16

Table 22.4.13.3.3-2: UECapabilityInformation-NB (step 2)

| Derivation path: 36.508 clause 8.1.6 Table 8.1.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityInformation-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation-r13 SEQUENCE { |  |  |  |
| ue-Capability-Container-r13 SEQUENCE { |  |  |  |
| accessStratumRelease-r13 | Not checked | Value should be based on Rel of Access stratum supported |  |
| ue-Category-NB-r13 | nb1 |  |  |
| multipleDRB-r13 | Checked |  | pc_NB_MultiDRB |
| pdcp-Parameters-r13 SEQUENCE {} | Not present |  | NOT (pc_User_Plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) |
| pdcp-Parameters-r13 SEQUENCE {} | Not present or Present | Up to UE implementation (i.e this case is not exactly specified in TS 36.331 / TS 36.306 | (pc_User_Plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) AND NOT (pc_ROHC_profile0x0002 OR pc_ROHC_profile0x0003 OR pc_ROHC_profile0x0004 OR pc_ROHC_profile0x0006 OR pc_ROHC_profile0x0102 OR pc_ROHC_profile0x0103 OR OR pc_ROHC_profile0x0104) |
| pdcp-Parameters-r13 SEQUENCE { |  |  | (pc_User_Plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) AND (pc_ROHC_profile0x0002 OR pc_ROHC_profile0x0003 OR pc_ROHC_profile0x0004 OR pc_ROHC_profile0x0006 OR pc_ROHC_profile0x0102 OR pc_ROHC_profile0x0103 OR OR pc_ROHC_profile0x0104) |
| supportedROHC-Profiles-r13 SEQUENCE { |  |  |  |
| profile0x0002 | Checked |  | pc_ROHC_profile0x0002 |
| profile0x0003 | Checked |  | pc_ROHC_profile0x0003 |
| profile0x0004 | Checked |  | pc_ROHC_profile0x0004 |
| profile0x0006 | Checked |  | pc_ROHC_profile0x0006 |
| profile0x0102 | Checked |  | pc_ROHC_profile0x0102 |
| profile0x0103 | Checked |  | pc_ROHC_profile0x0103 |
| profile0x0104 | Checked |  | pc_ROHC_profile0x0104 |
| } |  |  |  |
| maxNumberROHC-ContextSessions | Not checked |  |  |
| } |  |  |  |
| phyLayerParameters-r13 SEQUENCE { |  |  |  |
| multiTone-r13 | Checked |  | pc_NB_MultiTone |
| multiCarrier-r13 | Checked |  | pc_NB_MultiCarrier |
| } |  |  |  |
| rf-Parameters-r13 SEQUENCE { |  |  |  |
| supportedBandList-r13 SEQUENCE (SIZE (1.. maxBands)) OF SEQUENCE { | n1 entries where n2 is the sum of pc_eBandα_Supp for α= 1 to maxFBI2(256) and n3 is the sum of reported bands without pc_eBandα_Supp for α= 1 to maxFBI2(256) | n1= n2 +n3 |  |
| band-r13 [α = 1..n2] | Any value β such that pc_eBandβ_Supp is TRUE and different from all eutra-Band[k] where k = 1 to α - 1 |  |  |
| powerClassNB-20dBm-r13 [α = 1..n] | Not checked |  |  |
| } |  |  |  |
| multiNS-Pmax-r13 | Not checked |  |  |
| } |  |  |  |
| dummy SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| ue-RadioPagingInfo-r13 SEQUENCE { |  |  |  |
| ue-Category-NB-r13 | nb1 |  |  |
| multiCarrierPaging-r14 | Checked |  | pc_NB_MultiCarrier_Paging |
| mixedOperationMode-r15 | Checked |  | pc_NB_mixedOperationMode |
| wakeUpSignal-r15 | Checked |  | pc_wakeUpSignal |
| wakeUpSignalMinGap-eDRX-r15 | Not checked |  |  |
| multiCarrierPagingTDD-r15 | Checked |  | pc_NB_MultiCarrier_Paging_TDD |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  | Rel-13 |
| nonCriticalExtension SEQUENCE { |  |  | > Rel-13 |
| ue-Capability-ContainerExt-r14 OCTET STRING { |  |  |  |
| ue-Category-NB-r14 | nb2 |  | pc_ue_Category_NB2 |
| mac-Parameters-r14 SEQUENCE { |  |  |  |
| dataInactMon-r14 | Not checked |  |  |
| rai-Support-r14 | Checked |  | pc_NB_Rai_Support |
| } |  |  |  |
| phyLayerParameters-v1430 SEQUENCE { |  |  |  |
| multiCarrier-NPRACH-r14 | Checked |  | pc_NB_MultiCarrier_NPRACH |
| twoHARQ-Processes-r14 | Checked |  | pc_NB_TwoHARQ_Processes |
| } |  |  |  |
| rf-Parameters-v1430 SEQUENCE { |  |  |  |
| powerClassNB-14dBm-r14 | Not checked |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| phyLayerParameters-v1440 |  |  |  |
| interferenceRandomisation-r14 SEQUENCE { |  |  |  |
| interferenceRandomisation-r14 | Checked |  | pc_NB_InterferenceRandomisation |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  | Rel-14 |
| nonCriticalExtension SEQUENCE { |  |  | > Rel-14 |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| earlyData-UP-r15 | Not checked |  |  |
| rlc-Parameters-r15 SEQUENCE { |  |  |  |
| rlc-UM-r15 | Checked |  | pc_NB_RLC_UM |
| } |  |  |  |
| mac-Parameters-v1530 SEQUENCE { |  |  |  |
| sr-SPS-BSR-r15 | Not checked |  |  |
| } |  |  |  |
| phyLayerParameters-v1530 SEQUENCE { |  |  |  |
| mixedOperationMode-r15 | Checked |  | pc_NB_mixedOperationMode |
| sr-WithHARQ-ACK-r15 | Checked |  | pc_SR_WithHARQ_ACK |
| sr-WithoutHARQ-ACK-r15 | Checked |  | pc_SR_WithoutHARQ_ACK |
| nprach-Format2-r15 | Checked |  | pc_NB_nprach_Format2 |
| additionalTransmissionSIB1-r15 | Not checked |  |  |
| npusch-3dot75kHz-SCS-TDD-r15 | Not checked |  |  |
| } |  |  |  |
| tdd-UE-Capability-r15 SEQUENCE {} | Not present |  | FDD |
| tdd-UE-Capability-r15 SEQUENCE { |  |  | TDD |
| ue-Category-NB-r15 | nb2 |  | pc_ue_Category_NB2 |
| phyLayerParametersRel13-r15 SEQUENCE { |  |  |  |
| multiTone-r13 | Checked |  | pc_NB_MultiTone |
| multiCarrier-r13 | Checked |  | pc_NB_MultiCarrier |
| } |  |  |  |
| phyLayerParametersRel14-r15 SEQUENCE { |  |  |  |
| multiCarrier-NPRACH-r14 | Checked |  | pc_NB_MultiCarrier_NPRACH |
| twoHARQ-Processes-r14 | Checked |  | pc_NB_TwoHARQ_Processes |
| } |  |  |  |
| phyLayerParameters-v1530 SEQUENCE { |  |  |  |
| mixedOperationMode-r15 | Checked |  | pc_NB_mixedOperationMode |
| sr-WithHARQ-ACK-r15 | Checked |  | pc_SR_WithHARQ_ACK |
| sr-WithoutHARQ-ACK-r15 | Checked |  | pc_SR_WithoutHARQ_ACK |
| nprach-Format2-r15 | Checked |  | pc_NB_nprach_Format2 |
| additionalTransmissionSIB1-r15 | Not checked |  |  |
| npusch-3dot75kHz-SCS-TDD-r15 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>