3GPP TSG-RAN5 Meeting #92-e	R5-215181r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5342 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Additional Spurious Emission test cases for LTE Band 24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | new NS value of NS_56 and associated additional spurious emission requirements were introduced in TS 36.101 due to the regulatory updates |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additions related to spurious emission requirements for LTE band 24 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance and test requirements will not reflect the updates for LTE band 24 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.3, 6.6.3.3.3, 6.6.3.3.4.1, 6.6.3.3.4.3, 6.6.3.3.5, 6.6.3.3_1, 6.6.3.3_2, 6.6.3B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Rel-17 WIC to trigger spec. upgrade |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Checkmark against ME on the coversheet is removed. |  |  |  |  |  |  |  |  |

#### 6.6.3.3	Additional spurious emissions

Editor’s notes: This test case is incomplete. The following items are missing or incomplete:

- Test configuration table for NS_38 and NS_39 is not defined.

- Test configuration table for NS_56 is FFS.

<<Unchanged text skipped>>

##### 6.6.3.3.3.30	Minimum requirement (network signalled value “NS_44”)

When "NS_44" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.3.30-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.3.30-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 |  |  |
| 2620-2645 | -15.5 | 5MHz | 1 |
| 2645-2690 | -40 | 1MHz | 1 |
| Note 1: The E-UTRA carrier with channel bandwidth is confined within 2570 MHz and 2615 MHz |  |  |  |

##### 6.6.3.3.3.31	Minimum requirement (network signalled value “NS_56”)

When "NS_56" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.3.31-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.3.31-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

The normative reference for this requirement is TS 36.101[2] subclause 6.6.3.3.33.

##### 6.6.3.3.4	Test description

6.6.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3.3.4.1-1 through Table 6.6.3.3.4.1-24. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<Unchanged text skipped>>

Table 6.6.3.3.4.1-25: Test Configuration Table (network signalled value "NS_44")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low Range, Mid Range Additional test frequencies: For 5 MHz bandwidth: 2612.5 MHz, NUL= 38175 For 10 MHz bandwidth: 2610 MHz, NUL= 38150 For 15 MHz bandwidth: 2607.5 MHz, NUL= 38125 For 20 MHz bandwidth: 2605 MHz, NUL= 38100 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_44 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Note 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 24 |
| 2 | Note 1 | 5MHz |  |  | QPSK | 25 | 0 |
| 3 | Note 1 | 5MHz |  |  | 16QAM | 1 | 24 |
| 4 | Note 1 | 5MHz |  |  | 16QAM | 25 | 0 |
| 5 | Note 1 | 10MHz |  |  | QPSK | 1 | 49 |
| 6 | Note 1 | 10MHz |  |  | QPSK | 45 | 5 |
| 7 | Low/Mid | 10MHz |  |  | QPSK | 50 | 0 |
| 8 | Note 1 | 10MHz |  |  | 16QAM | 1 | 49 |
| 9 | Note 1 | 10MHz |  |  | 16QAM | 45 | 5 |
| 10 | Low/Mid | 10MHz |  |  | 16QAM | 50 | 0 |
| 11 | Note 1 | 15MHz |  |  | QPSK | 75 | 0 |
| 12 | Note 1 | 15MHz |  |  | QPSK | 45 | 30 |
| 13 | Note 1 | 15MHz |  |  | QPSK | 1 | 74 |
| 14 | Low/Mid | 15MHz |  |  | QPSK | 75 | 0 |
| 15 | Note 1 | 15MHz |  |  | 16QAM | 75 | 0 |
| 16 | Note 1 | 15MHz |  |  | 16QAM | 45 | 30 |
| 17 | Note 1 | 15MHz |  |  | 16QAM | 1 | 74 |
| 18 | Low/Mid | 15MHz |  |  | 16QAM | 75 | 0 |
| 19 | Mid/Note 1 | 20MHz |  |  | QPSK | 100 | 0 |
| 20 | Mid/Note 1 | 20MHz |  |  | QPSK | 45 | 55 |
| 21 | Mid/Note 1 | 20MHz |  |  | QPSK | 1 | 99 |
| 22 | Low | 20MHz |  |  | QPSK | 100 | 0 |
| 23 | Mid/Note 1 | 20MHz |  |  | 16QAM | 100 | 0 |
| 24 | Mid/Note 1 | 20MHz |  |  | 16QAM | 45 | 55 |
| 25 | Mid/Note 1 | 20MHz |  |  | 16QAM | 1 | 99 |
| 26 | Low | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: Only applicable for additional test frequencies. |  |  |  |  |  |  |  |

Table 6.6.3.3.4.1-26: Test Configuration Table (network signalled value "NS_56")

FFS

<<Unchanged text skipped>>

6.6.3.3.4.3	Message contents

<<Unchanged text skipped>>

6.6.3.3.4.3.26	Message contents exceptions (network signalled value "NS_44")

1.	Information element additionalSpectrumEmission is set to NS_44. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.3.3.4.3.26-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 44 (NS_44) |  |  |

6.6.3.3.4.3.27	Message contents exceptions (network signalled value "NS_56")

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.3.3.4.3.27-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 56 (NS_56) |  |  |

##### 6.6.3.3.5	Test requirement

This clause specifies the requirements for the specified E-UTRA band for an additional spectrum emission requirement with protected bands as indicated from table 6.6.3.3.5.1-1 to table 6.6.3.3.5.25-2.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<<Unchanged text skipped>>

6.6.3.3.5.26	Test requirement (network signalled value “NS_44”)

When "NS 44" is indicated in the cell:

-	the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.5.26-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3.3.5.26-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 |  |  |
| 2620-2645 | -15.5 | 5MHz | 1 |
| 2645-2690 | -40 | 1MHz | 1 |
| Note 1: The E-UTRA carrier with channel bandwidth is confined within 2570 MHz and 2615 MHz. |  |  |  |

6.6.3.3.5.27	Test requirement (network signalled value “NS_56”)

When "NS 56" is indicated in the cell:

-	the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.5.27-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3.3.5.27-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.6.3.3_1	Additional spurious emissions for UL 64QAM

Editor’s notes: This test case is incomplete. The following items are missing or incomplete:

- Test configuration table for NS_56 is FFS.

<<Unchanged text skipped>>

Table 6.6.3.3_1.4.1-22: Test Configuration Table (network signalled value "NS_36")

|  | Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 700.5 MHz (NUL = low range) - 705.5 MHz (NUL =132747) 10 MHz CH BW: - 703 MHz (NUL = low range) - 708 MHz (NUL = 132772) 15 MHz CH BW: - 705.5 MHz (NUL = low range) - 710.5 MHz (NUL =132797 ) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | 5MHz, 10MHz, 15MHz |  |  |
| Test Parameters for NS_36 A-MPR |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID |  | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 |  | 700.5 | 5MHz | N/A for A-MPR testing |  | 64QAM | 25 | 0 |
| 2 |  | 705.5 | 5MHz |  |  | 64QAM | 25 | 0 |
| 3 |  | 703 | 10MHz |  |  | 64QAM | 50 | 0 |
| 4 |  | 708 | 10MHz |  |  | 64QAM | 50 | 0 |
| 5 |  | 705.5 | 15MHz |  |  | 64QAM | 75 | 0 |
| 6 |  | 710.5 | 15MHz |  |  | 64QAM | 75 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Void. Note 3: Void. Note 4: The 64QAM configurations shall not apply to UEs for which TC 6.6.3.3_2 is executed with network signalled value NS_36. |  |  |  |  |  |  |  |  |

Table 6.6.3.3_1.4.1-23: Test Configuration Table (network signalled value "NS_56")

FFS

<<Unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.6.3.3_2	Additional spurious emissions for UL 256QAM

Editor’s notes: This test case is incomplete. The following items are missing or incomplete:

#### - Test configuration table for NS_56 is FFS.

<<Unchanged text skipped>>

Table 6.6.3.3_2.4.1-22: Test Configuration Table (network signalled value "NS_36")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 700.5 MHz (NUL = low range) - 705.5 MHz (NUL =132747) 10 MHz CH BW: - 703 MHz (NUL = low range) - 708 MHz (NUL = 132772) 15 MHz CH BW: - 705.5 MHz (NUL = low range) - 710.5 MHz (NUL =132797 ) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 700.5 | 5MHz | N/A for Additional Spurious Emissions testing |  | 256QAM | 25 | 0 |
| 2 | 705.5 | 5MHz |  |  | 256QAM | 25 | 0 |
| 3 | 703 | 10MHz |  |  | 256QAM | 50 | 0 |
| 4 | 708 | 10MHz |  |  | 256QAM | 50 | 0 |
| 5 | 705.5 | 15MHz |  |  | 256QAM | 75 | 0 |
| 6 | 710.5 | 15MHz |  |  | 256QAM | 75 | 0 |

Table 6.6.3.3_2.4.1-23: Test Configuration Table (network signalled value "NS_56")

FFS

<<Unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.6.3B.3	Additional spurious emissions for UL-MIMO

Editor’s notes: This test case is incomplete. The following items are missing or incomplete:

#### - Test configuration table for NS_56 is FFS.

<<Unchanged text skipped>>

Table 6.6.3B.3.4.1-20: Test Configuration Table (network signalled value "NS_36")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 700.5 MHz (NUL = low range) - 705.5 MHz (NUL =132747) 10 MHz CH BW: - 703 MHz (NUL = low range) - 708 MHz (NUL = 132772) 15 MHz CH BW: - 705.5 MHz (NUL = low range) - 710.5 MHz (NUL =132797 ) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz |  |  |
| Test Parameters for NS_36 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 700.5 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | 700.5 | 5MHz |  |  | QPSK | 25 | 0 |
| 3 | 700.5 | 5MHz |  |  | QPSK | 20 | 1 |
| 4 | 700.5 | 5MHz |  |  | QPSK | 24 | 1 |
| 5 | 700.5 | 5MHz |  |  | QPSK | 12 | 0 |
| 6 (Notes 2,3) | 700.5 | 5MHz |  |  | 16QAM | 12 | 0 |
| 7 | 705.5 | 5MHz |  |  | QPSK | 25 | 0 |
| 8 (Notes 2,3) | 705.5 | 5MHz |  |  | 16QAM | 25 | 0 |
| 9 | 703 | 10MHz |  |  | QPSK | 50 | 0 |
| 10 | 703 | 10MHz |  |  | QPSK | 1 | 0 |
| 11 | 703 | 10MHz |  |  | QPSK | 40 | 9 |
| 12 | 703 | 10MHz |  |  | QPSK | 36 | 13 |
| 13 | 703 | 10MHz |  |  | QPSK | 10 | 40 |
| 14 | 703 | 10MHz |  |  | QPSK | 6 | 44 |
| 15 | 708 | 10MHz |  |  | QPSK | 50 | 0 |
| 16 (Notes 2,3) | 708 | 10MHz |  |  | 16QAM | 50 | 0 |
| 17 | 705.5 | 15MHz |  |  | QPSK | 75 | 0 |
| 18 | 705.5 | 15MHz |  |  | QPSK | 50 | 17 |
| 19 | 705.5 | 15MHz |  |  | QPSK | 50 | 24 |
| 20 | 705.5 | 15MHz |  |  | QPSK | 1 | 57 |
| 21 | 705.5 | 15MHz |  |  | QPSK | 10 | 17 |
| 22 | 705.5 | 15MHz |  |  | QPSK | 18 | 24 |
| 23 (Notes 2,3) | 705.5 | 15MHz |  |  | 16QAM | 18 | 24 |
| 24 | 710.5 | 15MHz |  |  | QPSK | 75 | 0 |
| 25 | 710.5 | 15MHz |  |  | 16QAM | 75 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.6.3B.3.4.1-20: Test Configuration Table (network signalled value "NS_56")

FFS

<<Unchanged text skipped>>

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------