3GPP TSG-RAN5 Meeting #92-e	R5-215201r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1327 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of n24 and n99 to spurious emissions and addition spurious emission test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specifications for Rel-17 n24 and n99 bands in RAN4 have been completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | i) Addition of conformance and test requirements for spurious emissions for UE co-existence test cases for Rel-17 n24 and n99 bands ii)Addition of conformance and test requirements for additional spurious emissions test casses for n24 and n99 bands |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious emission UE coexistence and additional spurious emission test cases will not be specified for n24 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2.3, 6.5.3.2.5, 6.5.3.3, 6.5.3.3.3, 6.5.3.3.4.1, 6.5.3.3.4.3, 6.5.3.3.5, 6.5C.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Checkmark against ME on the coversheet is removed; ii) Removed overlap with R5-215313 by removing Table 6.5.3.2.3-3 and Table 6.5.3.2.5-1. |  |  |  |  |  |  |  |  |

#### 6.5.3.2	Spurious emissions for UE co-existence

6.5.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.3.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified NR band for coexistence with protected bands as indicated in Tables 6.5.3.2.3-1 to 6.5.3.2.3-2.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<<Unchanged Text Skipped>>

NOTE 1:	The frequency range applicable with network signalling values of NS_04, NS_17, NS_18, NS_05, NS_43, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_45 and NS_56 are covered in subclause 6.5.3.3 Additional Spurious Emissions.

NOTE 2:	The following is applied to Note 2 in Table 6.5.3.2.3-1. For frequency with 2nd, 3rd or 4th harmonic spurious emissions, the measurements are covered in subclause 6.5.3.1.

NOTE 3:	The restriction on the maximum uplink transmission to 54 RB in Notes 21 and 22 of Tables 6.5.3.2.3-1 to 6.5.3.2.3-2 is intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.5.3.3	Additional spurious emissions

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined

Initial conditions for NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_45 and NS_56 are incomplete.

6.5.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5.3.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.3.3.3	Minimum conformance requirements

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

<<Unchanged Text Skipped>>

6.5.3.3.3.24	Requirement for network signalling value "NS_44"

When "NS_44" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3.24-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3.24-1: Additional requirements for “NS_44”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2620 | - | 2645 | -15.5 | 5 | 1, 2 |
| Frequency range | 2645 | - | 2690 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for carriers confined in 2570-2615 MHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

6.5.3.3.3.25	Requirement for network signalling value "NS_56"

When "NS_56" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-2 from the edge of the channel bandwidth.

Table 6.5.3.3.25-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

6.5.3.3.4	Test description

6.5.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1.  All these configurations shall be tested with applicable test parameters for each channel bandwidth and sub-carrier spacing, are shown in Tables 6.5.3.3.4.1-1 through Table 6.5.3.3.4.1-24 for different NS values. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<Unchanged Text Skipped>>

Table 6.5.3.3.4.1-25: Test Configuration Table (network signalling value "NS_21")

Same test configuration as listed in Table 6.2.3.4.1-27shall be used with the following exceptions:

Test SCS shall be: Lowest.

Table 6.5.3.3.4.1-26: Test Configuration Table (network signalling value "NS_56")

FFS

<<Unchanged Text Skipped>>

6.5.3.3.4.3	Message contents

<<Unchanged Text Skipped>>

6.5.3.3.4.3.24	Message contents exceptions (network signalling value "NS_44")

Message contents are according to TS 38.508 [5] subclause 4.6, with the following exceptions:

1.	Information element additionalSpectrumEmission is set to NS_44. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.5.3.3.4.3.24-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_44"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_44) |  |  |

6.5.3.3.4.3.25	Message contents exceptions (network signalling value "NS_56")

Message contents are according to TS 38.508 [5] subclause 4.6, with the following exceptions:

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.5.3.3.4.3.25-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.5.3.3.5	Test requirement

This clause specifies the requirements for the specified NR band for an additional spectrum emission requirement with protected bands as indicated from Table 6.5.3.3.5.1 to Table 6.5.3.3.5.25 for different NS_values.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<<Unchanged Text Skipped>>

6.5.3.3.5.24	Test requirement (network signalling value “NS_44”)

When "NS 44" is indicated in the cell,

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-25.

The power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.5.24-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.5.24-1: Additional requirements

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2620 | - | 2645 | -15.5 | 5 | 1, 2 |
| Frequency range | 2645 | - | 2690 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for carriers confined in 2570-2615 MHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

6.5.3.3.5.25	Test requirement (network signalling value “NS_56”)

When "NS 56" is indicated in the cell,

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-35.

The power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.5.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.5.25-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.5C.3.3	Additional spurious emissions for SUL

Editor’s notes:

- Tests for network signalling value NS_56 not complete.

6.5C.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5C.3.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.3.3	Minimum conformance requirements

The additional spurious emission requirements specified in 6.5.3.3.3 apply to the UE operating on SUL bands.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.3.3

6.5C.3.3.4	Test description

Same test description as specified in clause 6.5.3.3.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	For NS_05, instead of table 6.5.3.3.4.4-1 use Table 6.5C.3.3.4-1

-	For NS_43, instead of table 6.5.3.3.4.1-5 use Table 6.5C.3.3.4-2

-	For NS_18, instead of table 6.5.3.3.4.1-3 use Table 6.2C.3.3.4-3

-	For NS_48, instead of table 6.5.3.3.4.1-5 use Table 6.2C.3.3.4-4

[-	For NS_48, instead of table 6.5.3.3.4.1-26 use Table 6.2C.3.3.4-5]

<<Unchanged Text Skipped>>

Table 6.5C.3.3.4-5: Test Configuration Table (network signalled value “NS_49”)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15 kHz for all carriers |  |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A. | N/A | DFT-s-OFDM | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 4 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 5 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 6 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 7 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |
| 8 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 9 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 10 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 11 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 12 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 13 | Default | 40 |  |  |  | QPSK | 159@33 (A2) |
| 14 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 15 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 16 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 17 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 18 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 19 | Default | 50 |  |  |  | QPSK | 175@45 (A2) |
| 20 | Default | 50 |  |  |  | QPSK | 220@0 (A1) |
| 21 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 22 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 23 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 24 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 25 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 26 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 27 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 28 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 29 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 30 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 31 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 32 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 33 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 34 | Default | 40 |  |  |  | 256 QAM | 150@33 (A2) |
| 35 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |
| 36 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 37 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 38 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 39 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 40 | Default | 50 |  |  |  | 256 QAM | 175@45 (A2) |
| 41 | Default | 50 |  |  |  | 256 QAM | 216@0 (A1) |
| 42 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| 43 | Default | 25 |  |  | CP-OFDM | QPSK | Outer_Full (A3) |
| 44 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 45 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 46 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 47 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 48 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 49 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |
| 50 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 51 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 52 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 53 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 54 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 55 | Default | 40 |  |  |  | QPSK | 159@33 (A2) |
| 56 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 57 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 58 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 59 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 60 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 61 | Default | 50 |  |  |  | QPSK | 175@45 (A2) |
| 62 | Default | 50 |  |  |  | QPSK | 216@0 (A1) |
| 63 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 64 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 65 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 66 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 67 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 68 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 69 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 70 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 71 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 72 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 73 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 74 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 75 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 76 | Default | 40 |  |  |  | 256 QAM | 150@33 (A2) |
| 77 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |
| 78 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 79 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 80 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 81 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 82 | Default | 50 |  |  |  | 256 QAM | 175@45 (A2) |
| 83 | Default | 50 |  |  |  | 256 QAM | 220@0 (A1) |
| 84 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

[Table 6.5C.3.3.4-6: Test Configuration Table (network signalled value “NS_56”)

FFS]

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND RF, Tables 4.6.3-14, 4.6.3-15 and 4.6.3-19 with conditions SUL, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, together with exceptions as specified in clause 6.5.3.3.4.3 as appropriate for different NS values.

Table 6.5C.3.3.4-2: Void

6.5C.3.3.5	Test requirement

For SUL operation, the Additional Spurious emission requirement specified in clause 6.5.3.3.5 shall be met for specific NS values.

Table 6.5C.3.3.5-1: Void

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------