3GPP TSG-RAN5 Meeting #92-e	R5-215203r3

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1329 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of n24 - blocking test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specifications for Rel-17 n24 band in RAN4 has been completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of n24 to conformance, configuration and test requirements sections of blocking test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Blocking test cases will not include n24 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2.3, 7.6.2.5, 7.6.3.3, 7.6.3.5, 7.6.4.3, 7.6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Checkmark against ME on the coversheet is removed Rev2: Revised CR catergory from B to F Rev3: Removed blocking requirements for inter-band CA combination since n24 related combinations are not being introduced at this time (although specified in 38.101-1) |  |  |  |  |  |  |  |  |

7.6.2.3	Minimum conformance requirements

<<Unchanged Text Skipped>>

Table 7.6.2.3-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and CBW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |  | -BWChannel/2-11 |
| n1, n2, n3, n5, n7, n8, n12, n14, n20, n24, n25, n26, n28, n34, n38, n39, n40, n41, n483, n50, n51, n53, n65, n66, n70, n74, n75, n76 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. |  |  |  |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

7.6.2.5	Test requirement

<<Unchanged Text Skipped>>

Table 7.6.2.5-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and CBW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |  | -BWChannel/2-11 |
| n1, n2, n3, n5, n7, n8, n12, n14, n20, n24, n25, n26, n28, n34, n38, n39, n40, n41, n483, n50, n51, n53, n65, n66, n70, n74 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. |  |  |  |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

7.6.3.3	Minimum Conformance Requirements

<<Unchanged Text Skipped>>

Table 7.6.3.3-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, n5, n7, n8, n12, n14, n20, n24, n25, n26, n30, n28, n34, n38, n39, n40, n41, n485, n50, n51, n538, n65, n66, n70, n71, n74, n75, n76 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 5: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz. NOTE 6: Void. NOTE 7: For UE supporting both bands 25 and 70, the FDL_high of band 70 is applied as FDL_high for band 25, and the FDL_low of band 25 is applied as FDL_low for band 70. NOTE 8: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2580 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

7.6.3.5	Test Requirement

<<Unchanged Text Skipped>>

Table 7.6.3.5-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, n5, n7, n8, n12, n14, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n485, n50, n51, n538, n65, n66, n70, n71, n74 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 5: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz. NOTE 6: Void. NOTE 7: For UE supporting both bands 25 and 70, the FDL_high of band 70 is applied as FDL_high for band 25, and the FDL_low of band 25 is applied as FDL_low for band 70. NOTE 8: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2580 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

7.6.4.3	Minimum Conformance Requirements

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.4.3-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.4.3-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1,n2, n3, n5, n7, n8, n12, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n70, n71, n74, n75, n76 | Pw | dBm | PREFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 |
|  | Fuw (offset SCS= 15 kHz) | MHz | 2.7075 | 5.2125 | 7.7025 | 10.2075 | 13.0275 | 15.6075 | 20.5575 | 25.7025 | NA | NA | NA | NA |
|  | Fuw (offset SCS= 30 kHz) | MHz | NA | NA | NA | NA | NA | NA | NA | NA | 30.855 | 40.935 | 45.915 | 50.865 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2.3-1 and Table 7.3.2.3-2 for two and four antenna ports, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.6.4.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

7.6.4.5	Test Requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 with parameters specified in Table 7.6.4.5-1.

Table 7.6.4.5-1: Narrow-band blocking

| NR band | Parameter | Unit | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1, n2, n3, n5, n7, n8, n12, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n70, n71, n74 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 |
|  | Fuw (offset SCS= 15 kHz) | MHz | 2.7075 | 5.2125 | 7.7025 | 10.2075 | 13.0275 | 15.6075 | 20.5575 | 25.7025 | NA | NA | NA | NA |
|  | Fuw (offset SCS= 30 kHz) | MHz | NA | NA | NA | NA | NA | NA | NA | NA | 30.855 | 40.935 | 45.915 | 50.865 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.6.4.5-2 Void

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------