3GPP TSG-RAN5 Meeting #92-e	R5-215402r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0734 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 34.229-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | px_IMS_CalleeContactUri and px_IMS_CalleeContactUri2 have a NOTE providing guidance to the test operator on how to set those PIXITs (when set different to the default values). Such guidance is missing for px_IMS_CalleeUri and px_IMS_CalleeUri2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A common NOTE 1 has been added to mandate that px_IMS_CalleeUri, px_IMS_CalleeUri2, px_IMS_CalleeContactUri and px_IMS_CalleeContactUri2 values shall be all different from each other. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test operator may wrongly set the value of px_IMS_CalleeUri2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Created a common NOTE 1 for all PIXITs that shall not have the same value. |  |  |  |  |  |  |  |  |

## B.1.1	PIXITs

Table B.1.1: PIXIT

| Parameter name | Description | Type | Default value | Supported value |
| --- | --- | --- | --- | --- |
| px_IMS_AssociatedTelUri | TEL URI for the user | charstring | “+331234567” | format shall be TEL URI |
| px_IMS_CalleeUri | URI of Callee, send by the UE in INVITE (MO call establishment) to address the remote UE (NOTE 1) | charstring | "sip:User-B@3gpp.org" |  |
| px_IMS_CalleeUri2 | URI of Callee, sent by the UE in INVITE (MO call establishment) to address the remote UE (NOTE 1) | charstring | " sip:User-C@3gpp.org " |  |
| px_IMS_CalleeContactUri | URI provided by the remote side (i.e. by SS) to be used by the UE as contact address in further SIP signalling of the dialog (NOTE 1) | charstring | "sip:User-B-Contact@3gpp.org" |  |
| px_IMS_CalleeContactUri2 | URI provided by the remote side (i.e. by SS) to be used by the UE as contact address in further SIP signalling of the dialog (NOTE 1) | charstring | "sip:User-C-Contact@3gpp.org" |  |
| px_IMS_CiphAlgo_Def | Ciphering Algorithm; NOTE: Unless specified otherwise in the test prose „nociph“ shall not be used for verification | CiphAlgo | des_ede3_cbc | enumerated type: des_ede3_cbc, aes_cbc or nociph |
| px_IMS_HomeDomainName | Home Domain Name. Applicable when using an ISIM: same value as EFDOMAIN. (derived from the IMSI otherwise) FBBA: value as preconfigured at the UE | charstring | As defined in TS 34.229-1 [5], Annex E |  |
| px_IMS_HomeDomainName_Refreshed | used in 8.15 | charstring | "refreshed3gpp.org" |  |
| px_IMS_Private_UserId_Refreshed | used in 8.15 | charstring | "privateuser@refreshed3gpp.org" |  |
| px_IMS_PublicUserIdentity1_Refreshed | used in 8.15 | charstring | "sip:PublicId1@refreshed3gpp.org" |  |
| px_IMS_MessageAccountIdentity | used in 15.15 | charstring | "" |  |
| px_IMS_SPI_ValidValueList_PDN1 | List of comma separated integer values that can be used as SPI values for IMS security on PDN1; each value shall be ≥ 256 | charstring | "" |  |
| px_IMS_SPI_ValidValueList_PDN2 | List of comma separated integer values that can be used as SPI values for IMS security on PDN2; each value shall be ≥ 256 | charstring | "" |  |
| px_IMS_IPSecAlgorithm | Integrity Algorithm | IntAlgo | hmac_sha_1_96 | enumerated type; hmac_md5_96, hmac_sha_1_96 |
| px_IMS_Private_UserId | Private User Identity. Applicable when using an ISIM: same value as EFIMPI. (derived from the IMSI otherwise) FBBA: username as used for authorization | charstring | As defined in TS 34.229-1 [5], Annex E |  |
| px_IMS_PublicUserIdentity1 | Public User Identity. It is set to the same value as the first record in EFIMPU. FBBA: IMPU preconfigured in the UE | Charstring | As defined in TS 34.229-1 [5], Annex E |  |
| px_IMS_PublicUserIdentity2 | It is set to the same value as the second record in EFIMPU. | Charstring | As defined in TS 34.229-1 [5], Annex E |  |
| px_IMS_PublicUserIdentity3 | It is set to the same value as the third record in EFIMPU. | Charstring | As defined in TS 34.229-1 [5], Annex E |  |
| px_SMS_SMSC_InternationalNumber | international number of the SMSC: Dialled number string of the TS-Service Centre Address according to clause 4.5.3.9 "EFSMSP (Short message service parameters)" in TS 31.121 [52] (TON is assumed to be "International Number"). | charstring | As defined in Annex E.3.2.14 of TS 34.229-1 [5]: “112233445566778” according to TS 31.121 [52]. |  |
| px_UEwithISIM | true: UE has ISIM false: UE has USIM only | boolean | true |  |
| px_DigestPasswordForXCAP | XCAP password | charstring | "XCAP-Password" |  |
| px_XCAP_RootUri | XCAP Root URI according to section 6.1 of RFC 4825 [70] as configured at the UE | charstring | "/XCAP.3gpp.org" |  |
| px_XCAP_TargetUri | Target (SIP or TEL URI) where to forward to | charstring | "sip:user@domain.com" |  |
| px_XCAP_Username | username to be used for HTTP authentication in case of pc_XCAP_UsernameIsConfiguredInUE==true | charstring |  |  |
| NOTE 1: px_IMS_CalleeUri, px_IMS_CalleeUri2, px_IMS_CalleeContactUri and px_IMS_CalleeContactUri2 values shall be all different from each other. |  |  |  |  |