3GPP TSG-RAN5 Meeting #92-e	R5-214508r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2018 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to NR cell configurations for SIG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In subclause 6.3.1, an Editor’s Note remains, which should be resolved. The need for NR cell configuration types, similar to those specified in TS 36.508 subclause 6.3.3 for E-UTRA cells, has not been identified/requested. However the need for NR SCell configuration types, similar to those specified in TS 36.508 subclause 6.3.4 for E-UTRA SCells, has been requested. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Editor’s Note has been deleted. The title of clause 6.3.1 has been updated to better reflect the scope of this subclause. A new clause 6.3.3 has been introduced for NR SCell configuration types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Issues captured via Editor’s Notes, remain unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Proposal updated based on the outcome of RAN5#92-e TTCN sidebar discussions. |  |  |  |  |  |  |  |  |

## 6.3	Reference system configurations

### 6.3.1	Default System Information configurations

#### 6.3.1.1	Intra-frequency neighbouring cell list in SIB3 for NR cells

Intra-frequency neighbouring cell list for signalling test cases is defined in table 6.3.1.1-1. This table is referred to in the default contents of IE intraFreqNeighCellList in SIB3 defined in table 4.6.2-2.

Table 6.3.1.1-1: Intra-frequency neighbouring cell lists for NR cells

| cell ID | Test Frequency | intra-frequency neighbouring cell list |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | number of entries | physCellId[n] |  |  |
|  |  |  | 1 | 1 | 3 |
| NR Cell 1 | NRf1 | 3 | NR Cell 2 | NR Cell 4 | NR Cell 11 |
| NR Cell 2 | NRf1 | 3 | NR Cell 1 | NR Cell 4 | NR Cell 11 |
| NR Cell 4 | NRf1 | 3 | NR Cell 1 | NR Cell 2 | NR Cell 11 |
| NR Cell 11 | NRf1 | 3 | NR Cell 1 | NR Cell 2 | NR Cell 4 |
| NR Cell 3 | NRf2 | 1 | NR Cell 23 | - | - |
| NR Cell 23 | NRf2 | 1 | NR Cell 3 | - | - |

Editor’s Note: The intra-frequency NR neighbouring cell list for signalling NAS test cases when cells are on same PLMN is FFS.

#### 6.3.1.2	Inter-frequency carrier frequency list in SIB4 for NR cells

Inter-frequency NR carrier frequency list for signalling test cases is defined in table 6.3.1.2-1. This table is referred to in the default contents of IE interFreqCarrierFreqList in SIB4 defined in table 4.6.2-3.

Table 6.3.1.2-1: Inter-frequency carrier frequency lists for NR cells

| cell ID | Test Frequency | interFreqCarrierFreqList |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | number of entries | dl-CarrierFreq[n] |  |  |
|  |  |  |  | 1 | 2 | 3 |
| NR Cell 1 NR Cell 2 NR Cell 4 NR Cell 11 | NRf1 (Note 2) | 3 | NRf2 | NRf3 | NRf5 |
| NR Cell 3 NR Cell 23 | NRf2 (Note 2) | 3 | NRf1 | NRf3 | NRf5 |
| NR Cell 6 | NRf3 (Note 2) | 3 | NRf1 | NRf2 | NRf5 |
| NR Cell 10 | NRf5 (Note 3) | 3 | NRf1 | NRf2 | NRf3 |
| Note 1: Depending on the Band under test, NRf3 may not be applicable. Note 2: In case of Test frequency NRf1, NRf2 and NRf3, dl-CarrierFreq NRf5 as part of inter-frequency list is applicable only in case of multi-band scenarios. Note 3: Test frequency NRf5 is applicable only in case of multi-band scenarios. |  |  |  |  |  |

Editor’s Note: The inter-frequency NR carrier frequency list for signalling NAS test cases when cells are on same PLMN is FFS.

#### 6.3.1.3	E-UTRA carrier frequency list in SIB5 for NR cells

The frequency mapping of E-UTRA cells are defined as per TS 36.508 [2] clause 4.4.2 and TS 36.508 [2] clause 6.3.2 for NGC NAS test cases, E-UTRA frequency mapping is according to clause 6.2.3.3. E-UTRA carrier frequency list for signalling test cases is defined in table 6.3.1.3-1. This table is referred to in the default contents of IE carrierFreqListEUTRA in SIB5 defined in table 4.6.2-4.

Table 6.3.1.3-1: E-UTRA carrier frequency lists for NR cells

| interFreqCarrierFreqList |  |
| --- | --- |
| number of entries | carrierFreq[n] |
|  | 1 |
| 1 | E-UTRA f1 |
| 2 | E-UTRA f2 |
| 3 | E-UTRA f3 |
| 4 | E-UTRA f4 |
| Note 1: E-UTRAf1, E-UTRAf2, E-UTRAf3, E-UTRAf4 are according to clause 6.2.3.3 Note 2: Depending on the Band under test, E-UTRA f2 or E-UTRA f3 or E-UTRA f4 may not be applicable. |  |

Table 6.3.1.3-2: Void

### 6.3.2	Default configurations for NAS test cases

The default configurations specified in this subclause apply only to NAS test cases. They apply to all NAS test cases unless otherwise specified.

#### 6.3.2.1	Simulated network scenarios for NAS test cases

Simulated network scenarios for NAS test cases to be tested are specified in the pre-test conditions of each individual test case.

Any combination is allowed with the following restrictions:

-	NGC Cell B shall not be used if NGC Cell D is used

-	a maximum 3 cells on the same frequency can be used, i.e. only 3 cells out of NGC Cell A, NGC Cell B, NGC Cell C and NGC Cell D may be used simultaneously in each individual test case when cells in the test case are in different PLMNs (refer to Table 6.3.2.2-3).

#### 6.3.2.2	Simulated NAS cells

Simulated NAS cells and default NAS parameters are specified in Table 6.3.2.2-1. Unless otherwise specified in a test case, default radio parameters of the NAS cells are specified as per Table 6.3.2.2-2.

Unless otherwise specified, the default parameters specified in clause 4.4.2 will also apply to all NAS cells.

Table 6.3.2.2-1: Default NAS parameters for simulated NAS cells

| NAS cell ID | Tracking Area |  |  |  |  | TA# list (Note 1) | 5G-GUTI (Note 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  |  | TAC |  | AMF Identifier |  |  | 5G-TMSI |
|  |  | MCC |  | MNC |  |  | AMF Region ID | AMF Set ID | AMF Pointer |  |
| NGC Cell A | TAI-1 | (Note 3) |  |  | 1 | TAI-1 | 254 | 1 | 1 | Arbitrarily selected according to TS 23.003 subclause 2.10.1 [26]. |
| NGC Cell B | TAI-2 | (Note 3) |  |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NGC Cell C | TAI-3 | (Note 3) |  |  | 3 | TAI-3 | 252 | 1 | 1 |  |
| NGC Cell D | TAI-4 | (Note 3) |  |  | 4 | TAI-4 | 252 | 1 | 1 |  |
| NGC Cell E | TAI-12 | 002 |  | 101 | 3 | TAI-12 | 244 | 1 | 1 |  |
| NGC Cell F | TAI-11 | 003 | 101 |  | 2 | TAI-11 | 239 | 1 | 1 |  |
| NGC Cell G | TAI-7 | (Note 4) |  | 02 | 1 | TAI-7 | 238 | 1 | 1 |  |
| NGC Cell H | TAI-8 | (Note 4) |  | 02 | 2 | TAI-8 | 237 | 1 | 1 |  |
| NGC Cell I | TAI-9 | 002 |  | 101 | 1 | TAI-9 | 244 | 1 | 1 |  |
| NGC Cell J | TAI-10 | 003 |  | 101 | 1 | TAI-10 | 236 | 1 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure for initial access or mobility (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column 5G-GUTI indicates GUTI included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.8.3). Note 4: Set to the same Mobile Country Code stored in EFIMSI on the test USIM card (subclause 4.8.3). |  |  |  |  |  |  |  |  |  |  |

Table 6.3.2.2-2: Default radio parameters for simulated NAS cells when cells are in same PLMN and access stratum is NR

| NAS cell ID | Frequency | NR Cell ID (Note 1) |
| --- | --- | --- |
| NGC Cell A | NRf1 | NR Cell 1 |
| NGC Cell B | NRf1 | NR Cell 2 |
| NGC Cell C | NRf1 | NR Cell 4 |
| NGC Cell D | NRf1 | NR Cell 11 |
| NGC Cell E | NA | NA |
| NGC Cell F | NRf2 | NR Cell 3 |
| NGC Cell G | NA | NA |
| NGC Cell H | NA | NA |
| NGC Cell I | NA | NA |
| NGC Cell J | NRf2 | NR Cell 12 |
| Note 1: Default NR parameters for simulated NR cells are as specified in Table 4.4.2-2. Note 2: For signalling tests, simultaneous co-existence of NGC Cells B and D is not allowed (in line with Table 4.4.2-1) |  |  |

Table 6.3.2.2-3: Default PLMN and radio parameters for simulated NAS cells when cells are in different PLMNs and access stratum is NR

| NAS cell ID | PLMN | Frequency | NR Cell ID (Note 1) |
| --- | --- | --- | --- |
| NGC Cell A | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 1 |
| NGC Cell B | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 2 |
| NGC Cell C | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 4 |
| NGC Cell D | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 11 |
| NGC Cell E | MCC=002 MNC=101 | NRf2 | NR Cell 3 |
| NGC Cell F | MCC=003 MNC=101 | NRf4 | NR Cell 14 |
| NGC Cell G | MCC = MCC in USIM MNC=02 | NRf2 | NR Cell 12 |
| NGC Cell H | MCC = MCC in USIM MNC=02 | NRf2 | NR Cell 23 |
| NGC Cell I | MCC=002 MNC=101 | NRf3 | NR Cell 6 |
| NGC Cell J | MCC=002 MNC=101 | NRf3 | NR Cell 13 |
| Note 1: Default NR parameters for simulated NR cells are as specified in Table 4.4.2-2 Note 2: for signalling tests, simultaneous co-existence of NGC Cells B and D is not allowed (In line with Table 4.4.2-1) |  |  |  |

### 6.3.3	Cell configuration types

#### 6.3.3.1	Introduction

For the purpose of test equipment resource management, different types of cell configurations are defined with different capabilities.

For E-UTRA cells, please refer to TS 36.508 [2] clauses 6.3.3 and 6.3.4.

The default NR cell configuration type is Full Cell: this NR cell configuration has, in minimum, all DL and UL physical channels and physical signals configured, as defined in subclause 4.2.2.

When not mentioned explicitly in a test case prose, an NR cell is of type Full Cell. The following subclauses define different NR cell types with lower capabilities than the Full Cell.

#### 6.3.3.2	SCell types

When testing NR CA, the following NR SCell types may be specifically mentioned in the test case prose:

-	Active SCell: An NR cell that may become an SCell at any point of time during the test case and which, while being an SCell, may be activated.

-	Inactive SCell: An NR cell that may become an SCell at any point of time during the test case but is never activated while being an SCell.

SCell activation is defined as SS sending an SCell Activation/Deactivation MAC CE to the UE to activate the SCell.

Note that an Active SCell will not become an Inactive SCell if the SCell is deactivated via an SCell Activation/Deactivation MAC CE or after the sCellDeactivationTimer timer expires.