3GPP TSG-RAN WG5 Meeting #92-e	R5-215452r1

# Electronic meeting, 16th - 27th Aug, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1362 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 6.3.3.6 for SRS time mask test for BW 70MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test requirement for SRS time mask is incomplete. Furthermore the test for SRS time mask of channel bandwidth 70MHz is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the test requirement for SRS time mask. - Add SRS time mask test for channel bandwidth 70MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test of SRS time mask will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.6.3, 6.3.3.6.4.1, 6.3.3.6.4.3, 6.3.3.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-215452r1: Undo the changes in Table 6.3.3.6.5-1 and merge to R5-214325r1. |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

6.3.3.6	SRS time mask

6.3.3.6.1	Test purpose

To verify that the SRS time mask meets the requirements given in 6.3.3.6.5.

The time mask for SRS time mask defines the transient period(s) allowed between transmit OFF power and transmit ON power when transmitting the SRS.

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3.3.6.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.3.3.6.3	Minimum conformance requirements

For SRS transmission mapped to one OFDM symbol, the ON power is defined as the mean power over the symbol duration excluding any transient period; See Figure 6.3.3.6.3-1

Figure 6.3.3.6.3-1: Single SRS time mask for NR UL transmission

For SRS transmission mapped to two or more OFDM symbols the ON power is defined as the mean power for each symbol duration excluding any transient period. For consecutive SRS transmissions without power change, Figure 6.3.3.6.3-2 applies.

Figure 6.3.3.6.3-2: Consecutive SRS time mask for the case when no power change is required with SRS usage other than antenna switching

When power change between consecutive SRS transmissions is required, then Figure 6.3.3.6.3-3 and Figure 6.3.3.6.3-4 apply.

Figure 6.3.3.6.3-3: Consecutive SRS time mask for the case when power change is required and when 15kHz and 30kHz SCS is used in FR1 with SRS usage other than antenna switching

Figure 6.3.3.6.3-4: Consecutive SRS time mask for the case when power change is required and when 60kHz SCS is used in FR1

Figure 6.3.3.6-5: FR1 Time mask for 15 kHz and 30 kHz SCS for the case when consecutive SRS switching usage is between antenna switching & other sets

where “other sets” belongs to a “usage set” other than the set for antenna switching. The usage sets for SRS switching are defined in section 6.2.1 of TS 38.214 [12].

NOTE:	Guard period of one symbol is defined between two SRS resources of an SRS resource set for antenna switching for 15kHz, 30kHz and 60kHz SCS in Table 6.2.1.2-1 of TS 38.214 [10].

The above transient period applies to all the transmit CCs in CA with the CC sounding SRS. UE RF requirements do not apply during this transient period.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.3.6.

6.3.3.6.4	Test description

6.3.3.6.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in table 6.3.3.6.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.3.6.4.1-1: Test Configuration Table

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 | Mid range (NOTE 1) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 | Lowest, Highest |  |
| SRS configuration | Paired Spectrum | Unpaired Spectrum |
| c-SRS (SRS bandwidth configuration) | 7 (24 RB for BW 5 MHz) 14 (52 RB for BW 10 MHz) 20 (76 RB for BW 15 MHz) 25 (104 RB for BW 20 MHz) 33 (132 RB for BW 25 MHz) 42 (160 RB for BW 30 MHz) 51 (216 RB for BW 40MHz) 60 (264 RB for BW 50MHz) for SCS 15 KHz | 7 (for BW 5 MHz) 13 (for BW 10 MHz) 19 (for BW 15 MHz) 25 (for BW 20 MHz) 30 (for BW 25 MHz) 40 (for BW 30 MHz) 51 (for BW 40MHz) 60 (for BW 50MHz) for SCS 15 KHz |
|  | 1 (8 RB for BW 5 MHz) 7 (24 RB for BW 10 MHz) 10 (36 RB for BW 15 MHz) 13 (48 RB for BW 20 MHz) 17 (64 RB for BW 25 MHz) 20 (76 RB for BW 30 MHz) 25 (104 RB for BW 40MHz) 33 (132 RB for BW 50MHz) 42 (160 RB for BW 60MHz) 45 (184 RB for BW 70MHz) 51 (216 RB for BW 80MHz) 56 (240 RB for BW 90MHz) 63 (272 RB for BW 100MHz) for SCS 30 KHz | 1 (for BW 5 MHz) 6 (for BW 10 MHz) 10 (for BW 15 MHz) 12 (for BW 20 MHz) 17 (for BW 25 MHz) 20 (for BW 30 MHz) 25 (for BW 40MHz) 30 (for BW 50MHz) 40 (for BW 60MHz) 45 (for BW 70MHz) 51 (for BW 80MHz) 53 (for BW 90MHz) 61 (for BW 100MHz) for SCS 30 KHz |
|  | 1 (8 RB for BW 10 MHz) 4 (16 RB for BW 15 MHz) 7 (24 RB for BW 20 MHz) 8 (28 RB for BW 25 MHz) 10 (36 RBfor BW 30 MHz) 13 (48 RB for BW 40MHz) 17 (64 RB for BW 50MHz) 20 (76 RB for BW 60MHz) 22 (88 RB for BW 70MHz) 25 (104 RB for BW 80MHz) 29 (120 RB for BW 90MHz) 33 (132 RB for BW 100MHz) for SCS 60 KHz | 1 (for BW 10 MHz) 4 (for BW 15 MHz) 7 (for BW 20, 25 MHz) 9 (for BW 30 MHz) 13 (for BW 40MHz) 17 (for BW 50MHz) 19 (for BW 60MHz) 22 (for BW 70MHz) 25 (for BW 80MHz) 27 (for BW 90MHz) 30 (for BW 100MHz) for SCS 60 KHz |
| b-SRS | 0 | 0 |
| b-hop | 3 | 0 |
| freqDomainPosition | 0 | 0 |
| SRS-PeriodicityAndOffset | sl5 for SCS 15 KHz | sl5 for SCS 15 KHz |
|  | sl10 for SCS 30 KHz | sl10 for SCS 30 KHz |
|  | sl20 for SCS 60 KHz | sl20 for SCS 60 KHz |
| transmissionComb | n2 | n2 |
| CombOffset | 0 | 0 |
| cyclicShift | 0 | 0 |
| startPosition | 0 for SCS 15 KHz | 0 for SCS 15 KHz |
|  | 1 for SCS 30 KHz | 1 for SCS 30 KHz |
|  | 3 for SCS 60 KHz | 3 for SCS 60 KHz |
| nrofSymbols | n1 for SCS 15 KHz | n1 for SCS 15 KHz |
|  | n2 for SCS 30 KHz | n2 for SCS 30 KHz |
|  | n4 for SCS 60 KHz | n4 for SCS 60 KHz |
| NOTE 1: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3.3.6.4.3.

6.3.3.6.4.2	Test procedure

1.	The SS measure the UE transmission OFF power during the 13 OFDM symbols for 15kHz SCS, 12 OFDM symbols for 30kHz SCS and 10 OFDM symbols for 60kHz SCS, preceding the SRS symbol excluding a transient period of 10 μs.

2.	Measure the output power of the transmitted SRS transmission during 1 OFDM symbols for 15kHz SCS, 2 OFDM symbols for 30kHz SCS and 4 OFDM symbols for 60kHz SCS.

3.	Measure the UE transmission OFF power during the slot following the SRS under test, excluding a transient period of 10 µs

6.3.3.6.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.3 with the following exceptions:

Table 6.3.3.6.4.3-0: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1[5], Table 4.6.3-7C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| srs-Config | SRS-Config in Table 6.3.3.6.4.3-1 |  |  |
| } |  |  |  |

Table 6.3.3.6.4.3-1: SRS-Config: SRS time mask measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SEQUENCE { | 1 entry |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| alpha | alpha08 |  |  |
| p0 | -100 |  |  |
| } |  |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SEQUENCE { | 1 entry |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  | SCS 15k |
|  | 1 |  | SCS 30k |
|  | 3 |  | SCS 60k |
| nrofSymbols | n1 |  | SCS 15k |
|  | n2 |  | SCS 30k |
|  | n4 |  | SCS 60k |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS |  |  |  |
|  | 7 (for BW 5 MHz) 14 (for BW 10 MHz) 20 (for BW 15 MHz) 25 (for BW 20 MHz) 33 (for BW 25 MHz) 42 (for BW 30 MHz) 51 (for BW 40MHz) 60 (for BW 50MHz) |  | Paired Spectrum for SCS 15 KHz |
|  | 1 (for BW 5 MHz) 7 (for BW 10 MHz) 10 (for BW 15 MHz) 13 (for BW 20 MHz) 17 (for BW 25 MHz) 20 (for BW 30 MHz) 25 (for BW 40MHz) 33 (for BW 50MHz) 42 (for BW 60MHz) 45 (for BW 70MHz) 51 (for BW 80MHz) 56 (for BW 90MHz) |  | Paired Spectrum for SCS 30 KHz |
|  | 1 (for BW 10 MHz) 4 (for BW 15 MHz) 7 (for BW 20 MHz) 8 (for BW 25 MHz) 10 (for BW 30 MHz) 13 (for BW 40MHz) 17 (for BW 50MHz) 20 (for BW 60MHz) 22 (for BW 70MHz) 25 (for BW 80MHz) 29 (for BW 90MHz) 33 (for BW 100MHz) |  | Paired Spectrum for SCS 60 KHz |
|  | 7 (for BW 5 MHz) 13 (for BW 10 MHz) 19 (for BW 15 MHz) 25 (for BW 20 MHz) 30 (for BW 25 MHz) 40 (for BW 30 MHz) 51 (for BW 40MHz) 60 (for BW 50MHz) |  | Unpaired Spectrum for SCS 15 KHz |
|  | 1 (for BW 5 MHz) 6 (for BW 10 MHz) 10 (for BW 15 MHz) 12 (for BW 20 MHz) 17 (for BW 25 MHz) 20 (for BW 30 MHz) 25 (for BW 40MHz) 30 (for BW 50MHz) 40 (for BW 60MHz) 45 (for BW 70MHz) 51 (for BW 80MHz) 53 (for BW 90MHz) 61 (for BW 100MHz) |  | Unpaired Spectrum for SCS 30 KHz |
|  | 1 (for BW 10 MHz) 4 (for BW 15 MHz) 7 (for BW 20, 25 MHz) 9 (for BW 30 MHz) 13 (for BW 40MHz) 17 (for BW 50MHz) 19 (for BW 60MHz) 22 (for BW 70MHz) 25 (for BW 80MHz) 27 (for BW 90MHz) 30 (for BW 100MHz) |  | Unpaired Spectrum for SCS 60 KHz |
| b-SRS | 0 |  |  |
| b-hop | 3 |  | Paired Spectrum |
|  | 0 |  | Unpaired Spectrum |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| periodicityAndOffset-p CHOICE{ |  |  |  |
| sl5 | 4 |  | SCS 15 KHz |
| sl10 | 8 |  | SCS 30KHz |
| sl20 | 16 |  | SCS 60 KHz |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.3.6.4.3-2: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

6.3.3.6.5	Test requirement

The requirement for the power measured in steps (1), (2) and (3) of the test procedure shall not exceed the values specified in Table 6.3.3.6.5-1.

Table 6.3.3.6.5-1: SRS time mask

|  | SCS [kHz] | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |
| Expected SRS Transmission ON Measured power | 15 | -3.8 | -0.4 | 1.2 | 2.6 | 3.6 | 4.4 | 5.7 | 6.6 | N/A | N/A | N/A | N/A |
|  | 30 | -5.6 | -0.8 | 1.0 | 2.2 | 3.5 | 4.2 | 5.6 | 6.6 | 7.5 | 8.8 | 9.2 | 9.8 |
|  | 60 | N/A | -2.5 | 0.5 | 2.2 | 2.9 | 4.0 | 5.2 | 6.5 | 7.2 | 8.6 | 9.2 | 9.6 |
| ON Power Tolerance |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3.3.6.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3.3.6.5-2: Test Tolerance (Transmit OFF power and SRS time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

==============================================================

<< End of changes >>