3GPP TSG-RAN5 Meeting # 92-e	R5-215539r1

# Electronic Meeting,  16th - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1168 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Message contents addition for intra-band non contiguous EN-DC SEM, A-SEM and ACLR test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no message exception definition for test cases 6.5B.2.2.1, 6.5B.2.2.2, 6.5B.2.2.3.despite they are LTE non-anchor agnostic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new message contents in clause 6.5B.2.2.1, 6.5B.2.2.2, 6.5B.2.2.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE power configuration will be incorrect while running these test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: - Changes in 6.5B.2.2.1 and 6.5B.2.2.3 have been removed to solve the overlapping with R5-215211 and R5-215212 respectively. - Message contents for A-SEM test have been modified to limit the configuration to overlapping transmission. - Limit messages contents exceptions to Wgap < NR ΔfOOB + E-UTRA ΔfOOB. |  |  |  |  |  |  |  |  |

<< Start of change 1>>

##### 6.5B.2.2.2	Additional Spectrum emissions mask for intra-band non-contiguous EN-DC

Editor's note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	For Wgap < NR ΔfOOB + E-UTRA ΔfOOB, test description and test requirements are FFS.

6.5B.2.2.2.1	Test purpose

Same test purpose as in clause 6.5.2.3 in TS 38.521-1 [8] for the NR carrier.

6.5B.2.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.2.2.2.3	Minimum conformance requirements

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. If for some frequency an individual CC spectrum emission mask overlaps with the bandwidth of another CC then the emission mask does not apply for that frequency.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.2.2.

No exception requirements applicable to NR or LTE when Wgap > NR ΔfOOB + E-UTRA ΔfOOB. LTE anchor agnostic approach is applied when Wgap > NR ΔfOOB + E-UTRA ΔfOOB.

Exception requirements for both NR and E-UTRA are defined for this test when Wgap < NR ΔfOOB + E-UTRA ΔfOOB and therefore LTE anchor agnostic approach is not applied when Wgap < NR ΔfOOB + E-UTRA ΔfOOB.

6.5B.2.2.2.4	Test description

For Wgap > NR ΔfOOB + E-UTRA ΔfOOB:

Same test description as in clause 6.5.2.3.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.5.2.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test procedure as in clause 6.5.2.3.4.2 in TS 38.521-1 [8] with the following steps exception:

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

For Wgap < NR ΔfOOB + E-UTRA ΔfOOB:

FFS.

6.5B.2.2.2.4.3	Message Content

Message contents are according to TS 36.508-1 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.2.2.2.4.3-1: RRCConnectionReconfiguration: nr-Config-r15 when Wgap < NR ΔfOOB + E-UTRA ΔfOOB.

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.2.2.2.4.3-2: PhysicalCellGroupConfig when Wgap < NR ΔfOOB + E-UTRA ΔfOOB

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.2.2.2.4.3-3: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band when Wgap < NR ΔfOOB + E-UTRA ΔfOOB

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

<< End of change >>