3GPP TSG-RAN5 Meeting #92-e	R5-215592r2

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0023 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MCVideo Test Case 6.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Revised test case 6.2.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Edited Test Purposes 1, 3, 5, 6. - In Conformance Requirements section, edited 1st paragraph; from 24.281,added 10.2.2.2.1, additional text in 10.2.2.2.2, and 10.2.2.3.1.3; deleted 10.2.2.2.4, 10, 10.2.2.2.5,. From TS 24.581,.added 6.3.5.4.9. - Edited Main Behavior Table, including notes format and exceptions. - In Specific Contents Section, edited Table 6.2.2.3.3-2, 6.2.2.3.3-3, 6.2.2.3.3-5, 6.2.2.3.3 6, 6.2.2.3.3-8; voided Tables 6.2.2.3.3-9, -11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test specification would have incorrect formats and information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: corrected heading styles, deleted unnecessary ros in the Main Behaviour Table, corrected paths in the Specific Message Contents tables R2: corrected language in EXCEPTION step after step 14; properly made step 21 VOID |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 6.2.2	On-network / Private Call / On-demand / Automatic Commencement Mode / With Transmission Control / Upgrade to Emergency Call / Cancellation of Emergency on User request / Client Terminated (CT)

#### 6.2.2.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorised for MCVideo Service, including authorised to receive private and private emergency calls with Automatic Commencement }

ensure that {

when { the McVideo User requests to establish a MCVideo private call, On-demand Automatic Commencement Mode, no force of Automatic Commencement, applying End-to-end communication security with Transmission Control }

then { UE (MCVideo Client) sends a SIP INVITE message to establish the Private Call and receives a SIP 200 (OK) from the SS (MCVideo Server that accepts the establishment of an MCVideo private call, On-demand Automatic Commencement Mode applying End-to-end communication security with Transmission Control, and notifies the user of the call establishment }

}

(2)

with { UE (MCVideo Client) having an ongoing On-demand Automatic Commencement Mode private call with Transmission Control }

ensure that {

when { the MCVideo User engages in communication with the inviting MCVideo User }

then { UE (MCVideo Client) respects the Transmission Control imposed by the MCVideo Server (Transmission granting/release/reject/revoke) applying Transmission Control confidentiality and integrity protection }

}

(3)

with { UE (MCVideo Client) having an ongoing On-demand Automatic Commencement Mode private call with Transmission Control }

ensure that {

when { the MCVideo User receives a request for upgrade of the ongoing MCVideo private call to a MCVideo emergency private call with Transmission Control }

then { UE (MCVideo Client) accepts the request and, upon sending SIP 200 (OK) message, considers the call as being upgraded to emergency private call (emergency private call state = "MEPC 3: emergency-pc-granted") and notifies the MCVideo User of the upgraded call if pc_DisplayIfoEmergencyCall }

}

(4)

with { UE (MCVideo Client) having an On-demand Automatic Commencement Mode private call with Transmission Control upgraded to an emergency private call }

ensure that {

when { the MCVideo User continues communication with the invited MCVideo User }

then { UE (MCVideo Client) respects the transmission control imposed by the MCVideo Server including being able to handle override requested by the inviting MCVideo user and applying Transmission Control confidentiality and integrity protection }

}

(5)

with { UE (MCVideo Client) having an On-demand Automatic Commencement Mode private call with Transmission Control that was upgraded to an emergency private call }

ensure that {

when { the MCVideo User receives a request to cancel the ongoing MCVideo emergency condition on a private call }

then { UE (MCVideo Client) accepts the request and after sending a SIP 200 (OK) response considers the emergency condition cancelled and the call being reverted back to MCVideo private call, notifies the MCVideo User of the downgraded call, and respects Reception Control (Transmission }

}

(6)

with { UE (MCVideo Client) having an ongoing On-demand Pre-arranged Group Call with Manual  Commencement Mode }

ensure that {

when { the MCVideo User requests to end the Private Call, the UE (MCVideo Client) sends a Transmission End Request message and, upon receiving a Transmission End Response from the SS (MCVideo Server) }

then { the UE (MCVideo Client) sends a SIP BYE request and the SS (MCVideo Server) responds with a SIP 200 (OK) and leaves the MCVideo  session }

}

#### 6.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.281 clauses 10.2.2.2.1, 10.2.2.2.2, 10.2.2.2.3, 10.2.2.3.1.3, 6.2.3.1.1; in TS 24.581, clause 6.3.5.4.9. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated these are Rel-14 requirements.

[TS 24.281, clause 10.2.2.2.1]

Upon receiving a request from an MCVideo user to establish an MCVideo private call the MCVideo client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [11], with the clarifications given below.

The MCVideo client:

1)	shall set the Request-URI of the SIP INVITE request to a public service identity of the participating MCVideo function serving the MCVideo user;

2)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [11];

3)	shall include the g.3gpp.mcvideo media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcvideo" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [22];

4)	shall include an Accept-Contact header field containing the g.3gpp.mcvideo media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [20];

5)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in 3GPP TS 24.229 [11]), in a P-Preferred-Service header field according to IETF RFC 6050 [14] in the SIP INVITE request;

6)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref contain with the value of "urn:urn-7:3gpp-service.ims.icsi.mcvideo" along with parameters "require" and "explicit" according to IETF RFC 3841 [20];

7)	for the establishment of a private call shall insert in the SIP INVITE request a MIME resource-lists body with the MCVideo ID of the invited MCVideo user, according to rules and procedures of IETF RFC 5366 [37];

8)	if an end-to-end security context needs to be established and if the MCVideo user is initiating a private call then:

a)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [8];

b)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [8];

c)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect private call communications and with the remaining twenty-eight bits being randomly generated as described in 3GPP TS 33.180 [8];

d)	shall encrypt the PCK to a UID associated to the MCVideo client using the MCVideo ID and KMS URI of the invited user as determined by the procedures of subclause 6.2.8.3.9 and a time related parameter as described in 3GPP TS 33.180 [8];

e)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [8]; and

g)	shall add the MCVideo ID of the originating MCVideo to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [8]; and

f)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCVideo user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [8].

9)	shall include an SDP offer according to 3GPP TS 24.229 [11] with the clarification given in subclause 6.2.1 and with a media stream of the offered media-transmission control entity;

10)	if implicit transmission control is required, shall comply with the conditions specified in subclause 6.4;

11)	if the MCVideo user is initiating a private call then:

a)	if force of automatic commencement mode at the invited MCVideo client is requested by the MCVideo user, shall include in the SIP INVITE request a Priv-Answer-Mode header field with the value "Auto" according to the rules and procedures of IETF RFC 5373 [27];

b)	if force of automatic commencement mode at the invited MCVideo client is not requested by the MCVideo user and:

i)	if automatic commencement mode at the invited MCVideo client is requested by the MCVideo user, shall include in the SIP INVITE request an Answer-Mode header field with the value "Auto" according to the rules and procedures of IETF RFC 5373 [27]; and

ii)	if manual commencement mode at the invited MCVideo client is requested by the MCVideo user, shall include in the SIP INVITE request an Answer-Mode header field with the value "Manual" according to the rules and procedures of IETF RFC 5373 [27]; and

c)	shall contain an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <session-type> element set to a value of "private";

12)	if the MCVideo emergency private call state is set to either "MVEPC 2: emergency-pc-requested" or "MVEPC 3: emergency-pc-granted" or the MCVideo emergency private priority state for this private call is set to "MVEPP 2: in-progress", the MCVideo client shall comply with the procedures in subclause 6.2.8.3.3; and

13)	shall send SIP INVITE request towards the MCVideo server according to 3GPP TS 24.229 [11].

Upon receiving a SIP 183(Session Progress) response to the SIP INVITE request the MCVideo client:

1)	may indicate the progress of the session establishment to the inviting MCVideo user.

Upon receiving a SIP 200 (OK) response to the SIP INVITE request the MCVideo client:

1)	shall interact with the media plane as specified in 3GPP TS 24.581 [5];

2)	if the MCVideo emergency private call state is set to "MVEPC 2: emergency-pc-requested" or "MVEPC 3: emergency-pc-granted", shall perform the actions specified in subclause 6.2.8.3.4; and

3)	shall notify the user that the call has been successfully established.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	if the MCVideo emergency private call state is set to "MVEPC 2: emergency-pc-requested"; or

2)	if the MCVideo emergency private call state is set to "MVEPC 3: emergency-pc-granted";

the MCVideo client shall perform the actions specified in subclause 6.2.8.3.5.

On receiving a SIP INFO request where the Request-URI contains an MCVideo session ID identifying an ongoing session, the MCVideo client shall follow the actions specified in subclause 6.2.8.3.7.

[TS 24.281, clause 10.2.2.2.2]

Upon receipt of an initial SIP INVITE request, the MCVideo client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [11] with the clarifications below.

The MCVideo client:

1)	may reject the SIP INVITE request if any of the following conditions are met:

a)	MCVideo client is already occupied in another session and the number of simultaneous sessions exceeds <MaxCall>, the maximum simultaneous MCVideo session for private call, as specified in TS 24.484 [25];

b)	MCVideo client does not have enough resources to handle the call; or

c)	any other reason outside the scope of this specification;

otherwise, continue with the rest of the steps.

NOTE 1:	If the SIP INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <emergency-ind> element set to a value of "true", the participating MCVideo function can choose to accept the request.

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCVideo function either with appropriate reject code as specified in 3GPP TS 24.229 [11] and warning texts as specified in subclause 4.4.2 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorized to restrict the reason for failure according to <allow-failure-restriction> as specified in 3GPP TS 24.484 [25] and skip the rest of the steps of this subclause;

3)	if the SIP INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication that this is a SIP INVITE request for an MCVideo emergency private call and:

i)	should display the MCVideo ID of the originator of the MCVideo emergency private call contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	if the <alert-ind> element is set to "true", should display to the MCVideo user an indication of the MCVideo emergency alert and associated information; and

b)	shall set the MCVideo emergency private priority state to "MVEPP 2: in-progress" for this private call;

4)	if the SDP offer of the SIP INVITE request contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCVideo ID of the originating MCVideo client from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [8];

b)	shall convert the MCVideo ID to a UID as described in 3GPP TS 33.180 [8];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [8];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP INVITE request with a SIP 488 (Not Acceptable Here) response as specified in IETF RFC 4567 [34], and include warning text set to "136 authentication of the MIKEY-SAKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.4; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [8]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [8];

NOTE 2:	With the PCK successfully shared between the originating MCVideo client and the terminating MCVideo client, both clients are able to use SRTP/SRTCP to create an end-to-end secure session.

5)	may check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [11];

6)	may display to the MCVideo user the MCVideo ID of the inviting MCVideo user;

7)	shall perform the automatic commencement procedures specified in subclause 6.2.3.1.1 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCVideo service setting at the invited MCVideo client for answering the call is set to automatic commencement mode;

b)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCVideo service setting at the invited MCVideo client for answering the call is set to manual commencement mode, yet the invited MCVideo client is willing to answer the call with automatic commencement mode; or

c)	SIP INVITE request contains a Priv-Answer-Mode header field with the value of "Auto"; and

8)	shall perform the manual commencement procedures specified in subclause 6.2.3.2.1 if either of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCVideo service setting at the invited MCVideo client for answering the call is set to manual commencement mode;

b)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCVideo service setting at the invited MCVideo client for answering the call is set to automatic commencement mode, yet the invited MCVideo client allows the call to be answered with manual commencement mode; or

c)	SIP INVITE request contains a Priv-Answer-Mode header field with the value of "Manual".

Upon receiving the SIP CANCEL request cancelling a SIP INVITE request for which a dialog exists at the MCVideo client and a SIP 200 (OK) response has not yet been sent to the SIP INVITE request then the MCVideo client:

1)	shall send a SIP 200 (OK) response to the SIP CANCEL request according to 3GPP TS 24.229 [11]; and

2)	shall send a SIP 487 (Request Terminated) response to the SIP INVITE request according to 3GPP TS 24.229 [11].

Upon receiving a SIP BYE request for an established dialog, the MCVideo client:

1)	shall follow the procedures in clause 10.2.5.2.

[TS 24.281, clause 10.2.2.2.3]

This subclause covers on-demand session.

Upon receipt of a SIP re-INVITE request for an existing private call session, the MCVideo client shall:

1)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication that this is a SIP re-INVITE request to upgrade this call to an MCVideo emergency private call and:

i)	should display the MCVideo ID of the originator of the MCVideo emergency private call contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	if the <alert-ind> element is set to "true", should display to the MCVideo user an indication of the MCVideo emergency alert and associated information; and

b)	shall set the MCVideo emergency private priority state to "MVEPP 2: in-progress" for this private call;

2)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <emergency-ind> element set to a value of "false":

a)	should display to the MCVideo user an indication that this is a SIP re-INVITE request to downgrade this emergency private call to a normal priority private call and:

i)	should display the MCVideo ID of the sender of the SIP re-INVITE request contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	if the <alert-ind> element is set to "false" should display to the MCVideo user an indication that the MCVideo emergency alert is cancelled;

iii)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body including an <originated-by> element:

A)	should display to the MCVideo user the MCVideo ID contained in the <originated-by> element of the MCVideo user that originated the MCVideo emergency alert; and

B)	if the MCVideo ID contained in the <originated-by> element is the MCVideo ID of the receiving MCVideo user, shall set the MCVideo emergency alert state to "MVPEA 1: no-alert";

b)	shall set the MCVideo emergency private priority state to "MVEPP 1: no-emergency" for this private call; and

c)	if the MCVideo emergency private call state of the call is set to "MVEPC 3: emergency-call-granted", shall set the MCVideo emergency private call state of the call to "MVEPC 1: emergency-pc-capable";

3)	may check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [11];

4)	may display to the MCVideo user the MCVideo ID of the inviting MCVideo user;

NOTE 1:	As this is a re-INVITE for an existing MCVideo private call session, there is no attempt made to change the answer-mode from its current state.

5)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [11];

6)	if the SIP re-INVITE request was received within an on-demand session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [11] with the clarifications given in subclause 6.2.2;

7)	shall send the SIP 200 (OK) response towards the MCVideo server according to rules and procedures of 3GPP TS 24.229 [11]; and

8)	shall interact with the media plane as specified in 3GPP TS 24.581 [5].

[TS 24.281, clause

###### 10.2.2.3.1.3]

This clause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request for an existing MCVideo private call session the participating MCVideo function:

1)	may reject the SIP re-INVITE request depending on the value of the Resource-Priority header field if the Resource-Priority header field is included in the received SIP re-INVITE request according to rules and procedures specified in IETF RFC 4412 [33] and skip the rest of the steps;

2)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the "SIP INVITE request for originating participating MCVideo function" with a SIP 500 (Server Internal Error) response. The participating MCVideo function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [15];

NOTE 1:	If the SIP re-INVITE request contains an emergency indication, the participating MCVideo function can choose to accept the request.

3)	shall determine the MCVideo ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP INVITE request and shall authorise the user;

NOTE 2:	The MCVideo ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in clause 7.3.

4)	shall validate the media parameters and if the MCVideo video media codec is not offered in the SIP re-INVITE request shall reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

NOTE 3:	If the received SIP re-INVITE request is received within a pre-established session, associated with an MCVideo private call, the media-level section for the offered MCVideo video media stream and the media-level section of the offered media-transmission control entity are expected to be the same as was negotiated in the existing pre-established session.

5)	shall generate a SIP re-INVITE request as specified in clause 6.3.2.1.9;

6)	shall set the <mcvideo-calling-user-id> element in an application/vnd.3gpp.mcvideo-info+xml MIME body of the SIP re-INVITE request to the MCVideo ID of the calling user;

7)	shall, if the SIP re-INVITE request was received within an on-demand session include in the SIP re-INVITE request an SDP containing the current media parameters used by the existing session;

8)	shall, if the SIP re-INVITE request was received within a pre-established session, include in the SIP re-INVITE request an SDP offer based upon the previously negotiated SDP for the pre-established session as specified in clause 6.3.2.1.1.2;

9)	shall include a Resource-Priority header field according to rules and procedures of 3GPP TS 24.229 [11] set to the value indicated in the Resource-Priority header field if included in the SIP re-INVITE request from the MCVideo client; and

10)	shall forward the SIP re-INVITE request, according to 3GPP TS 24.229 [11].

Upon receiving a SIP 200 (OK) response, the participating MCVideo function:

1)	shall generate a SIP 200 (OK) response as specified in the clause 6.3.2.1.5.2;

2)	if the SIP 200 (OK) response is to be sent within an on-demand session shall include in the SIP 200 (OK) response an SDP answer as specified in the clause 6.3.2.1.2.1;

3)	if the SIP 200 (OK) response is to be sent within a pre-established session shall include in the SIP 200 (OK) response an SDP answer based upon the previously negotiated SDP for the pre-established session;

4)	shall include Warning header field(s) received in the incoming SIP 200 (OK) response;

5)	shall include the P-Asserted-Identity header field received in the incoming SIP 200 (OK) response into the outgoing SIP 200 (OK) response;

6)	shall send the SIP 200 (OK) response to the MCVideo client according to 3GPP TS 24.229 [11]; and

7)	shall interact with the media plane as specified in 3GPP TS 24.581 [5].

The participating MCVideo function shall forward any other SIP response that does not contain SDP, including any MIME bodies contained therein, along the signalling path to the originating network according to 3GPP TS 24.229 [11].

[TS 24.281, clause 6.2.3.1.1]

When performing the automatic commencement mode procedures, the MCVideo client:

1)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [11];

2)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

3)	shall include the g.3gpp.mcvideo media feature tag in the Contact header field of the SIP 200 (OK) response;

4)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcvideo" in the Contact header field of the SIP 200 (OK) response;

5)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [23]. The "refresher" parameter in the Session-Expires header field shall be set to "uas";

6)	shall, if the incoming SIP INVITE request contains a Replaces header field, include in the SDP answer in the SIP 200 (OK) response to the SDP offer the parameters used for the pre-established session identified by the contents of the Replaces header field;

7)	shall, if the incoming SIP INVITE request does not contain a Replaces header field, include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [11] with the clarifications given in subclause 6.2.2;

NOTE:	In the case of a new emergency call where the terminating client is using a pre-established session, the SIP INVITE request containing a Replaces header is used to replace the pre-established session.

8)	shall send the SIP 200 (OK) response towards the MCVideo server according to rules and procedures of 3GPP TS 24.229 [11];

9)	shall, if the incoming SIP INVITE request contains a Replaces header field, release the pre-established session identified by the contents of the Replaces header field; and

10)	shall interact with the media plane as specified in 3GPP TS 24.581 [5].

When NAT traversal is supported by the MCVideo client and when the MCVideo client is behind a NAT, generation of SIP responses is done as specified in this subclause and as specified in IETF RFC 5626 [35].

[TS 24.581, clause 6.3.5.4.9]

When a Transmission Grant message is received from the transmission control arbitration logic in the MCVideo server, the transmission control interface towards the MCVideo client in the transmission control server:

1.	shall forward the Transmission Grant messages to the associated transmission participant;

2.	may set the first bit in the subtype of the Transmission Grant message to '1' (Acknowledgment is required) as described in subclause 9.2.2.1;

NOTE:	It is an implementation option to handle the receipt of the Transmission control Ack message and what action to take if the Transmission control Ack message is not received.

3.	shall enter the state 'U: permitted' as specified in subclause 6.3.5.5.2.

#### 6.2.2.3	Test description

##### 6.2.2.3.1	Pre-test conditions

System Simulator:

-	SS (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCPTT configuration document).

IUT:

-	UE (MCVideo client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2A.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCVideo Client Application has been activated and User has registered-in as the MCVideo User with the Server as active user at the Client.

##### 6.2.2.3.2	Test procedure sequence

Table 6.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCVIDEO call establishment are described in TS 36.579-1 [2], subclause 5.4.4A Generic Test Procedure for MCVideo CT communication in E-UTRA'. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 1 | The SS (MCVideo server) sends a SIP INVITE to request establishment of an MCVideo private call, On-demand Automatic Commencement Mode, no force of Automatic Commencement, applying End-to-end communication security with Transmission Control including a "text/plain" MIME body. | <-- | SIP INVITE | - | - |
|  | EXCEPTION: Step 2a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE message with a SIP 100 (Trying) message. |  | - |  |  |
| 2a1 | The UE (MCVideo Client) sends a SIP 100 (Trying) message. | --> | SIP 100 (Trying) | - | - |
| 3 | Check: Does the UE (MCVideo client) send a SIP 200 (OK) accepting the establishment of an MCVideo private call, On-demand Automatic Commencement Mode applying End-to-end communication security with Transmission Control? | --> | SIP 200 (OK) | 1 | P |
| 4 | The SS responds with a SIP ACK to acknowledge the SIP 200 (OK) message? | <-- | SIP ACK | - | - |
| 5 | The SS sends a Transmission Granted message. | <-- | Transmission Granted | - | - |
| 6 | Check: Does the UE send a Transmission Control ACK message? | --> | Transmission Control ACK | 1 | P |
| 7 | The SS (MCVideo server) sends SIP re-INVITE requesting the establishment (upgrade) of an MCVideo private emergency call on-demand Automatic Commencement Mode offering a media-level section for a media-transmission control entity. | <-- | SIP re-INVITE | - | - |
| - | EXCEPTION: Step 8a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE message with a SIP 100 (Trying) message. | - | - | - | - |
| 8a1 | The UE (MCVideo Client) sends a SIP 100 (Trying) message. | --> | SIP 100 (Trying) | - | - |
| 9 | Check: Does the UE MCVideo client) sends a SIP 200 (OK)? | --> | SIP 200 (OK) | 3 | P |
| 10 | The SS sends a SIP ACK in acknowledgement of the SIP 200 (OK) from the UE. | <-- | SIP ACK | - | - |
| 11 | The SS sends a Transmission Granted message | <-- | Transmission Granted | - | - |
| 12 | Check: Does the UE send a Transmission Control ACK message? | --> | Transmission Control ACK | 4 | P |
|  |  |  | - |  |  |
| 13 | Check: Does the UE (McVideo client) notify the user about the upgrade of the private call to an emergency private call? (NOTE 1). (NOTE 2) | - | - | 3 | P |
| 14 | The SS (MCVideo server) sends SIP re-INVITE to downgrade the emergency. | <-- | SIP re-INVITE | - | - |
| - | EXCEPTION: Step 15a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE message with a SIP 100 (Trying) message. |  |  |  |  |
| 15a1 | The UE (MCVideo Client) send a SIP 100 (Trying) message. | --> | SIP 100 (Trying) | 5 | P |
| 16 | Check: Does the UE (MCVideo client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | 5 | P |
| 17 | The SS responds with a SIP ACK to the UE (MCVideo Client). | <-- | SIP ACK | - | - |
| 18 | The SS sends a Transmission Granted message | <-- | Transmission Granted | - | - |
| 19 | Check: Does the UE send a Transmission Control ACK message? | --> | Transmission Control ACK | 4 | P |
|  |  |  | - |  |  |
| 20a1 | Check: Does the UE (MCVideo client) notify the user about the downgrade of the emergency private call to a normal priority private call? (NOTE 1) (NOTE 2) . | - | - | 5 | P |
| 21 | VOID |  |  |  |  |
| 22 | Make the UE (MCVideo Client) end the Group Call. (NOTE 1). | - | - | - | - |
| 22A | Does the UE (MCVideo Client) send the Transmission End Request message? | --> | Transmission End Request | 6 | P |
| 22B | The SS (MCVideo Server) sends a Transmission End Response, | <-- | Transmission End Response | - | - |
| 23 | Check: Does the UE (MCVideo Client) send a SIP BYE request? | --> | SIP BYE | 6 | P |
| 24 | The SS (MCVideo Server) sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| - | EXCEPTION: The SS (MCVideo Server) waits 2 seconds before the SS (MCVideo Server) deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 3) | - | - | - | - |
| NOTE 1: This action is expected to be done via a suitable implementation-dependent MMI. NOTE 2: The display information may include - indication for upgrade of the private call to an emergency private call - the MCVideo ID of the sender of the SIP re-INVITE request. NOTE 3: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. |  |  |  |  |  |

##### 6.2.2.3.3	Specific message contents

Table 6.2.2.3.3-1: SIP INVITE (Step 1, Table 6.2.2.3.2-1)

| Derivation Path: TS 36.579-1, Table 5.5.2.5.2-1, conditions MCVIDEO, PRIVATE-CALL, AUTO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.2.2.3.3-2 |  |  |  |

Table 6.2.2.3.3-2: MCVideo-Info in SIP INVITE (Table 6.2.2.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2.-2, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Message-body |  |  |  |  |  |
| MIME body part |  | MCVIDEO-Info |  |  |  |
| session-type | "private" |  |  |  |  |

Table 6.2.2.3.3-3: SIP 200 (OK) (Step 3, 16, Table 6.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition MCVIDEO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVIDEO-Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.2.2.3.3-2 |  |  |  |

Table 6.2.2.3.3-4: SIP re-INVITE (Step 7, Table 6.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, conditions MCVIDEO, EMERGENCY-CALL, AUTO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  | MCVideo-Info |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcvideo-info+xml" |  |  |  |
| MCVideo-Info | As described in Table 6.2.2.3.3-5 |  |  |  |

Table 6.2.2.3.3-5: MCVideo-INFO in SIP re-INVITE (Table 6.2.2.3.3-5)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-2, conditions EMERGENCY-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Message-body |  |  |  |  |  |
| MIME body part |  | MCVIDEO-Info |  |  |  |
| emergency-ind | "true" |  |  |  |  |

Table 6.2.2.3.3-6: SIP 200 (OK) (Step 9, Table 6.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.11-1, condition MCVIDEO, EMERGENCY-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVIDEO-Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.2.2.3.3-5 |  |  |  |

Table 6.2.2.3.3-7: SIP re-INVITE (Step 14, Table 6.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition MCVIDEO, AUTO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  | MCVideo-Info |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcvideo-info+xml" |  |  |  |
| MCVideo-Info | As described in Table 6.2.2.3.3-8 |  |  |  |

Table 6.2.2.3.3-8: MCVideo-Info in SIP INVITE (Table 6.2.2.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-2, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Message-body |  |  |  |  |  |
| MIME body part |  | MCVIDEO-Info |  |  |  |
| emergency- | not present or if present then="false" |  |  |  |  |

Table 6.2.2.3.3-9: VOID

Table 6.2.2.3.3-10: SIP BYE (Step 23, Table 6.2.2.3.2-1)

Derivation Path: TS 36.579-1, Table 5.5.2.2.1-1, condition MO_CALL

Table 6.2.2.3.3-11:  VOID

<END OF CHANGES>