Source:	CAICT, Nokia, Nokia Shanghai Bell

Title:	Discussion on test point selection for Maximum input level in FR2

# Introduction

The purpose of this paper is to provide an analysis for the parameters in test configuration table and a selection of test points for test environment, test frequencies, channel bandwidths, SCS, RB allocation and modulation for NR Maximum input level test case in FR2.

# Discussion

Maximum input level for FR2 will be defined in [1] where it is explained the scope of the Maximum input level as follows:

Maximum input level tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to a g-NodeB.

Based on the Minimum conformance requirements for Maximum input level for FR2in [2]:

7.4	Maximum input level

The maximum input level is defined as the maximum mean power, for which the throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The maximum input level is defined as a directional requirement. The requirement is verified in beam locked mode in the direction where peak gain is achieved.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal) with parameters specified in Table 7.4-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in transmission bandwidth configuration | dBm | -25 (NOTE 2) -27 (NOTE 3) |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. NOTE 2: Reference measurement channel is specified in Annex A.3.3.2: QPSK, R=1/3 variant with one sided dynamic OCNG Pattern as described in Annex A. NOTE 3: Reference measurement channel is specified in Annex A.3.3.5: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern as described in Annex A. |  |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink and downlink modulations and RB configurations.

## Test Environment

In LTE, environment condition for Maximum input level test case is specified for Normal Condition (NC). Due to NR Maximum input level test for FR1 has been proposed to be defined in same way as LTE, it is reasonable for FR2 to use the same criteria as LTE and FR1.

PROPOSAL 1: Select Test Environment as Normal Condition for Maximum input level in FR2.

## Test Frequencies

In LTE, Maximum input level is required to be performed at Mid range frequency for all bands. It is reasonable to keep the same criteria for NR Maximum input level for FR2, furthermore this same assumption has been adopted for FR1.

PROPOSAL 2: Select Mid range test frequency for Maximum input level in FR2.

## Test Channel Bandwidth

In LTE, Maximum input level is tested for Lowest, 5 MHz, and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE receiver related to the different channel bandwidths.

Follow similar strategy, NR Maximum input level for FR2 shall be tested for Lowest, Mid and Highest channel bandwidth across all NR FR2 band.

PROPOSAL 3: Select Lowest, Mid and Highest test channel bandwidth for Maximum input level in FR2.

## Test Subcarrier Spacing

In LTE and NR FR1，uplink configurations of Maximum input level test are according to the uplink configurations for reference sensitivity. It is reasonable to keep the same criteria for NR Maximum input level for FR2. Uplink configurations for reference sensitivity in [2] are extracted as bellow:

Table 7.3.2.1-2: Uplink configuration for reference sensitivity

| Operating band | NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | SCS | Duplex Mode |
| n257 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | 120 kHz | TDD |

PROPOSAL 4: Select 120 kHz subcarrier spacing for Maximum input level in FR2.

## Test Modulation and RB allocation

The selection of test points for modulation and RB allocation shall be based on the Minimum conformance requirements for Maximum input level in [2], which are extracted as bellow:

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in transmission bandwidth configuration | dBm | -25 (NOTE 2) -27 (NOTE 3) |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. NOTE 2: Reference measurement channel is specified in Annex A.3.3.2: QPSK, R=1/3 variant with one sided dynamic OCNG Pattern as described in Annex A. NOTE 3: Reference measurement channel is specified in Annex A.3.3.5: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern as described in Annex A. |  |  |  |  |  |

According to the above Minimum conformance requirements for Maximum input level, when QPSK is used for downlink for FR2 Maximum input level, downlink RB allocation shall be set according to Annex A.3.3.2 in [2]. When 256QAM is used for downlink for FR2 Maximum input level, downlink RB allocation shall be set according to Annex A.3.3.5 in [2].

In LTE, QPSK is used for uplink for all bands and all channel bandwidths testing for Maximum input level due to its robustness. It is reasonable to follow LTE philosophy for NR Maximum input level for FR2.

As discussed in subclause 2.4, uplink RB allocation shall be set according to Table 7.3.2.1-2 in [2],

In reference [3], the spectrum utilization for different modulations, subcarrier spacing and channel bandwidths are discussed in detail. The conclusion is that for a specific SCS and ChBw the number of resource blocks for DFT-s modulations is equal or lower than the used by CP modulations. According to Annex A.3.3 in [2], full RB allocation shall be used for downlink configuration for Maximum input level in order to achieve maximum spectrum utilization. So it is reasonable to use CP-OFDM waveform for downlink configuration.

Considering NR Maximum input level evaluates the receptor characteristics, the selection of the uplink modulations does not seem so critical. However, this receiver characteristic is measured by measuring downlink Throughput due to the HARQ feedback sent in PUSCH, so it is important to select the right uplink modulation to avoid any issue with uplink. So it is reasonable to use DFT-s-OFDM waveform which presents lower PAPR compared to CP-OFDM to avoid any uplink issues. Besides this kind of waveform is recommended in case of high power level due to it is more power efficient.

PROPOSAL 5: Select CP-OFDM QPSK and 256QAM for downlink configurations for Maximum input level in FR2.

PROPOSAL 6: Select DFT-s-OFDM QPSK for uplink configurations for Maximum input level in FR2.

PROPOSAL 7: Uplink RB allocation shall be set according to Table 7.3.2.1-2 in [2] for Maximum input level in FR2.

PROPOSAL 8: Downlink RB allocation shall be set according to Annex A.3.3.2 for QPSK and Annex A.3.3.5 for 256QAM in [2] for Maximum input level in FR2.

## Testability

The requirement in this test case is not testable due to maximum input level achievable in OTA test setup. Thus the test case will not be tested.

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR2 Maximum input level test case definition:

PROPOSAL 1: Select Test Environment as Normal Condition for Maximum input level in FR2.

PROPOSAL 2: Select Mid range test frequency for Maximum input level in FR2.

PROPOSAL 3: Select Lowest, Mid and Highest test channel bandwidth for Maximum input level in FR2.

PROPOSAL 4: Select 120 kHz subcarrier spacing for Maximum input level in FR2.

PROPOSAL 5: Select CP-OFDM QPSK and 256QAM for downlink configurations for Maximum input level in FR2.

PROPOSAL 6: Select DFT-s-OFDM QPSK for uplink configurations for Maximum input level in FR2.

PROPOSAL 7: Uplink RB allocation shall be set according to Table 7.3.2.1-2 in [2] for Maximum input level in FR2.

PROPOSAL 8: Downlink RB allocation shall be set according to Annex A.3.3.2 for QPSK and Annex A.3.3.5 for 256QAM  in [2] for Maximum input level in FR2.

# Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR2.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) 1 | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 1 | 2 | 6 |

# References

- 3GPP TS 38.521-2 v15.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 2 Standalone (Release 15).

- 3GPP TS 38.101-2 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 2 Standalone (Release 15).

- R5-182044, "Discussion on test point selection for NR ACS in FR2" (Keysight Technologies).

- R5-185805, “Testability issue of maximum input level and ACS (case 2) in FR2 (2)” Anritsu