3GPP TSG-RAN5 Meeting #93-e	R5-217041

# Electronic Meeting, 08th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2118 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to DIRECT LINK IDENTIFIER UPDATE REQUEST msg |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, The content of DIRECT LINK IDENTIFIER UPDATE REQUEST msg is still FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Default content of DIRECT LINK IDENTIFIER UPDATE REQUEST msg is updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No Default DIRECT LINK IDENTIFIER UPDATE REQUEST msg can be used for NR V2X TCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.7.4	Contents of V2X messages

…

–	DIRECT LINK IDENTIFIER UPDATE REQUEST

Table 4.7.4-23: DIRECT LINK IDENTIFIER UPDATE REQUEST

| Derivation Path: TS 24.587 Table 7.3.18.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK IDENTIFIER UPDATE REQUEST message identity |  | '0001 0011'B |  |  |
| Sequence number |  | Set in the test case, increase by 1 when a PC5 signalling message being sent or received | 0~255, uniquely identify a PC5 signalling message being sent or received |  |
| MSB of KNRP-sess ID |  | Not checked |  |  |
| Source layer-2 ID |  | '00 00 10'H |  |  |
| Source user info |  |  |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 01 00'H | Application Layer ID in initiating UE side |  |
| Source link local IPv6 address |  | Not present |  |  |

<End of modified section 1>