3GPP TSG-RAN5 Meeting #94-e                                                    R5-221275

Electronic Meeting, 8th – 19th November 2021

Title: 	Beam correspondence Measurement Uncertainties and test tolerances derivation

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.1.17

Document for:	Discussion and Endorsement

# 1.	Introduction

Measurement uncertainties for FR2 beam correspondence tolerance were discussed in RAN5#93 ([1], [2], [3]) although they were not finalized in test specifications ([4], [5]).

This document makes a proposal to finalize the measurement uncertainties for such test.

# 2.	Measurement uncertainties contributions

After discussions in RAN5#93e, the remaining work in terms of measurement uncertainty is to:

- Finalize the value of measurement uncertainty factor for RF power measurement equipment (agreed as [+/- 0.4dB] expanded uncertainty in RAN5#93e).

- Finalize the influence of noise

2.1 RF power measurement equipment

Some concerns were raised in the previous meeting regarding the value proposed for RF power measurement equipment due to the large dynamic range.

It should be noticed that the dynamic range is not that large as:

- On one hand the measurement grid points are only the ones that pass spherical coverage test as highlighted below:

- On the other hand, the requirement for beam correspondence tolerance indicates that for 85% of the measurement grid points the difference between EIRP1 and EIRP2 should not be higher than 3.2 dB.

Hence, the proposal will be to remove square brackets for the value of RF Power measurement equipment.

Proposal 1: Adopt +/- 0.4dB (as expanded uncertainty) for Uncertainty of the RF Power measurement equipment as proposed in [6].

2.2 Influence of noise

Regarding influence of noise, in [1], 4 options were discussed as shown below:

Proposal 3: In the absence of a CDF reference curve and to enable progress on this priority 1 test case, select one of the 4 options below: - Option 3.1: Adopt 1.0dB (systematic error) as influence of noise(SNR) for FR2 Beam Correspondence tolerance measurement MU as proposed in [3] for FR2 relative power measurement MU. - Option 3.2: Open an action point for UE vendors to provide typical ∆EIRPBC CDFs knowing the complete ERP1 vs link angle and EIRP2 vs link angle data sets so further analysis can be carried out. - Option 3.3: ΔSNRBC=10*log101+1010*log1010∆SNRSC10-1+Max∆EIRPBC10 , where MaxEIRPBC is the 85%-tile requirement value for the beam correspondence tolerance requirement and ΔSNRSC is the influence of noise of EIRP spherical coverage test for a given frequency range. - Option 3.4 ΔSNRBC=∆SNRSC-10*log101+1010*log1010∆SNRSC10-1+Max∆EIRPBC10 , where MaxEIRPBC is the 85%-tile requirement value for the beam correspondence tolerance requirement and ΔSNRSC is the influence of noise of EIRP spherical coverage test for a given frequency range. However, as SNRBC would be negative, the worst case for the MU will be that influence of noise is fully cancelled out, i.e. SNRBC=0.

After some additional discussions after RAN5#93ee with some companies, Keysight would like to propose to go for option 3.3 described above.

Proposal 2: In the absence of a CDF reference curves and to enable progress on this priority 1 test case, select option 3.3 for the influence of noise:

- Option 3.3:

ΔSNRBC=10*log101+1010*log1010∆SNRSC10-1+Max∆EIRPBC10

, where MaxEIRPBC is the 85%-tile requirement value for the beam correspondence tolerance requirement and ΔSNRSC is the influence of noise of EIRP spherical coverage test for a given frequency range.

# 3.	Test Tolerances

As for maximum output power or spherical coverage, it is proposed to derive test tolerances as TT = 0.60 x (MTSUIFF - ΔSNRmr),  being ΔSNRmr the systematic offset due to noise when measuring at minimum requirement level (ΔSNRmr= ΔSNRBC).

Proposal 3: Derive Test tolerances for beam correspondence tolerance test as TT = 0.60 x (MTSUIFF - ΔSNRmr),  being ΔSNRmr the systematic offset due to noise when measuring at minimum requirement level (ΔSNRmr= ΔSNRBC).

# 4. 	Conclusion

This document describes how to finalize FR2 beam correspondence tolerance measurement MU and makes the following proposals:

Proposal 1: Adopt +/- 0.4dB (as expanded uncertainty) for Uncertainty of the RF Power measurement equipment as proposed in [6].

Proposal 2: In the absence of a CDF reference curves and to enable progress on this priority 1 test case, select option 3.3 for the influence of noise:

- Option 3.3:

ΔSNRBC=10*log101+1010*log1010∆SNRSC10-1+Max∆EIRPBC10

, where MaxEIRPBC is the 85%-tile requirement value for the beam correspondence tolerance requirement and ΔSNRSC is the influence of noise of EIRP spherical coverage test for a given frequency range.

Proposal 3: Derive Test tolerances for beam correspondence tolerance test as TT = 0.60 x (MTSUIFF - ΔSNRmr),  being ΔSNRmr the systematic offset due to noise when measuring at minimum requirement level (ΔSNRmr= ΔSNRBC).

# 5.	References

[1] R5-218410, “Beam correspondence Measurement Uncertainties derivation”, Keysight Technologies, RAN5#93e

[2] R5-218407, “38.521-2 Beam correspondence Measurement Uncertainties”, Keysight Technologies, RAN5#93e

[3] R5-218408, “38.903 Beam correspondence Measurement Uncertainties”, Keysight Technologies, RAN5#93e

[4] 38.521-2 v16.9.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”

[5] 38.903 v16.9.0 “Derivation of test tolerances and measurement uncertainty for User equipment (UE) conformance test cases”

[6] R5-210845, “On FR2 relative power measurement uncertainty”, Anritsu, RAN5#90-e