3GPP TSG-RAN5 Meeting #94-e	R5-221499

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2207 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to generic test procedures for NR SL MIMO tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To update generic test procedures for NR SL MIMO tests - Change all the wording “simulated NR SL UE” to "NR-SS-UE". - Step of UTC time reset should be removed and added back before the steps of unicast link establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Generic test procedures are updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.1, 4.5.2, 4.5.4, 4.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-220568r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.5.1	General

The generic procedures are used by test cases to get UE under test into SWITCHED_OFF, RRC_IDLE/Ipsec SA not established, RRC_INACTIVE, RRC_CONNECTED/Ipsec SA or Out of Coverage established state.

A test case controls the SS by specifying the required RRC state and a set of generic procedure parameters applicable for the intended testing.

The connectivity EN-DC is MR-DC via E-UTRA-NR Dual Connectivity. This is a UE connected to the EPC. The connectivity E-UTRA/5GC, NR, NGEN-DC, NE-DC and NR-DC are all a UE connected to the 5GC.

The connectivity E-UTRA/EPC is E-UTRA connected to the EPC as specified in the present document.

Table 4.5.1-1: Generic procedure parameters

| Parameter | Values | Description | Parameter condition |
| --- | --- | --- | --- |
| Connectivity | E-UTRA/5GC | E-UTRA connected to 5GC | Mandatory |
|  | NR | NR connected to 5GC |  |
|  | EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |  |
|  | NGEN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to 5GC |  |
|  | NE-DC | NR E-UTRA Dual Connectivity |  |
|  | NR-DC | NR-NR Dual Connectivity |  |
|  | WLAN | Un trusted non 3GPP access over WLAN |  |
|  | E-UTRA/EPC | E-UTRA connected to EPC |  |
| Bearers | MCG(s) and SCG | MCG and SCG | Mandatory when Connectivity is set to EN-DC, NGEN-DC, NE-DC and NR-DC and when the generic procedures are used by test cases to get UE under test into RRC_CONNECTED state. s=1 if MULTI_PDN= FALSE and s=2 if MULTI_PDN=TRUE. N/A otherwise. |
|  | MCG(s) and split | MCG and split |  |
| Test Mode | On | UE test mode active as specified in TS 38.509 [11], clause 5.2.2. | Optional |
| Test Loop Function | On | UE test mode active with one of the UE test loop modes activated as specified in TS 38.509 [11], clauses 5.2.2 and 5.3.2. | Optional |
| Connected without release | On | Enter RRC_Connected with Ipsec_SA_Established and without any release. | Optional N/A for NR-DC. |
| Interworking without N26 interface supported | On | The NWK sets the REGISTRATION ACCEPT message, IE 5GS network feature support, IWK N26 (octet 3, bit 7) = 1 | Optional, Depends on test case scenario. Default message content for REGISTRATION ACCEPT is set to Interworking without N26 interface NOT supported |
| Unrestricted nr PDN | On | Allow unrestricted numbers of PDNs. | Optional for Connectivity E-UTRA/EPC. N/A otherwise. |
| Sidelink | On | NR sidelink | Optional |
| Cast type | Unicast | To establish unicast sidelink and PC5-RRC connection. | Optional |
| GNSS Sync | On | To use GNSS as the synchronization reference source. | Optional |
| SL MIMO | On | To transmit PSSCH with 2 spatial layers, i.e. SL MIMO | Optional |

<End of modified section 1>

<Start of modified section 2>

4.5.2	RRC_IDLE

4.5.2.1	Initiation

The SS shall:

1>	if connectivity is EN-DC:

2>	use 1 E-UTRA cell and 1 NR cell, default parameters;

2>	if connected without release is not present:

3>	perform according to the table 4.5.2.2-1: E-UTRA RRC_IDLE;

1>	if connectivity is E-UTRA/EPC:

2>	use 1 E-UTRA cell, default parameters;

2>	if unrestricted nr PDN is not present:

3>	perform according to the table 4.5.2.2-1: E-UTRA RRC_IDLE;

2>	else:

3>	perform according to the table 4.5.2.2-5: E-UTRA RRC_IDLE Unrestricted nr PDN:

3>	if pc_noOf_PDNsNewConnection > 0:

4>	perform according to the table 4.5.2.2-6: E-UTRA RRC_IDLE Unrestricted nr PDN Extension;

1>	if connectivity is NR:

2>	use 1 NR cell, default parameters;

2> if sidelink is On

3>	use 1 NR-SS-UE;

3> if GNSS Sync is On

4> use 1 GNSS simulator.

2>	perform according to the table 4.5.2.2-2: NR RRC_IDLE;

2> if pc_noOf_PDUsNewConnection > 0

3> perform according to the table 4.5.2.2-4: NR RRC_IDLE Extension;

1>	if connectivity is WLAN:

2>	use 1 WLAN cell, default parameters;

2>	if connected without release is not present:

3>	perform according to the table 4.5.2.2-3: WLAN Ipsec_SA_Released;

2>	else:

3>	Not defined:

1>	if connectivity is NR-DC:

2>	use 2 NR cells, default parameters;

2>	perform according to the table 4.5.2.2-2: NR RRC_IDLE.

1>	if connectivity is NE-DC:

2>	use 1 E-UTRA cell and 1 NR cell, default parameters;

2>	perform according to the table 4.5.2.2-2: NR RRC_IDLE.

<End of modified section 2>

<Start of modified section 3>

Table 4.5.2.2-2: NR RRC_IDLE

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | - | <-- | NR RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5 | The SS transmits a DLInformationTransfer message and an AUTHENTICATION REQUEST message. | <-- | NR RRC: DLInformationTransfer 5GMM: AUTHENTICATION REQUEST | - | - |
| 6 | The UE transmits an ULInformationTransfer message and an AUTHENTICATION RESPONSE message. | --> | NR RRC: ULInformationTransfer 5GMM: AUTHENTICATION RESPONSE | - | - |
| 7 | Void | - | - | - | - |
| 8 | The SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 9 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Step 9Aa1 to 9Aa2 describes behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported and the procedure parameter Interworking without N26 interface supported is not present. | - | - | - | - |
| 9Aa1 | IF UE_S1_SUPPORTED the SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 9Aa2 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: Steps 9a1 to 9a2 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 9a1 | IF Test Mode = On OR Test Loop Function = On, the SS transmits an ACTIVATE TEST MODE message to activate UE radio bearer test mode procedure. The ACTIVATE TEST MODE message is using the associated condition for the test loop. | <-- | RRC: DLInformationTransfer TC: ACTIVATE TEST MODE | - | - |
| 9a2 | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer TC: ACTIVATE TEST MODE COMPLETE | - | - |
| 10 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 11 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 12 | The SS transmits a UECapabilityEnquiry message. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 13 | The UE transmits a UECapabilityInformation message. | --> | NR RRC: UECapabilityInformation | - | - |
| 14 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 15 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| 16 | Void | - | - | - | - |
| 17 | Void | - | - | - | - |
| 18 | Void | - | - | - | - |
| - | EXCEPTION: Step 19a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 19a1 | IF pc_noOf_PDUsSameConnection > 0 THEN the generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. | - | - | - | - |
| - | EXCEPTION: Steps 19Aa1 to 19Aa2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 19Aa1 | IF connected without release is On AND Test Loop Function=On AND pc_noOf_PDUsNewConnection=0 THEN the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 19Aa2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| - | EXCEPTION: Step 20a1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 20a1 | IF connected without release is not present THEN, the SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| - | EXCEPTION: Step 20Aa1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 20Aa1 | IF GNSS Sync = On THEN trigger UE to reset or clear the current UTC time that has been calculated from GNSS. NOTE: The UTC time can be reset or clear on the UE using AT command (+CUTCR). | - | - | - | - |
| - | EXCEPTION: Step 21a1 and 21b1 depend on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 21a1 | IF Sidelink = On AND Cast type = Unicast AND UE initiated unicast mode NR sidelink comunication THEN generic procedure specified in subclause 4.9.22 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. |  |  |  |  |
| 21b1 | ELSE IF Sidelink = On AND Cast type = Unicast AND NR-SS-UE initiated unicast mode NR sidelink comunication THEN generic procedure specified in subclause 4.9.23 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. |  |  |  |  |
| 22a1 | Void | - | - |  |  |

<End of modified section 3>

<Start of modified section 4>

4.5.4	RRC_CONNECTED

4.5.4.1	Initiation

The SS shall:

1>	perform according to clause 4.5.2 RRC_IDLE;

1>	if connectivity is EN-DC:

2>	use 1 E-UTRA cell and 1 NR cell, default parameters;

2>	if connected without release is On:

3>	perform according to the table 4.5.4.2-2: RF E-UTRA RRC_CONNECTED;

2>	else:

3>	perform according to the table 4.5.4.2-1: E-UTRA RRC_CONNECTED;

1>	if connectivity is E-UTRA/EPC:

2>	use 1 E-UTRA cell, default parameters;

2>	perform according to the table 4.5.4.2-1: E-UTRA RRC_CONNECTED;

1>	if connectivity is NR:

2>	use 1 NR cell, default parameters;

2> if sidelink is On

3>	use 1 NR-SS-UE;

3> if GNSS Sync is On

4> use 1 GNSS simulator.

2>	if connected without release is not present:

3>	perform according to the table 4.5.4.2-3: NR RRC_CONNECTED;

1>	if connectivity is WLAN:

2>	use 1 WLAN cell, default parameters;

2>	if connected without release is not present:

3>	perform according to the table 4.5.4.2-4: WLAN IPsec_SA_Established;

2>	else:

3>	Not defined;

1>	if connectivity is NR-DC:

2>	use 2 NR cells, default parameters;

2>	perform according to the table 4.5.4.2-5: NR-DC RRC_CONNECTED;

1>	if connectivity is NE-DC:

2>	use 1 E-UTRA cell and 1 NR cell, default parameters;

2>	perform according to the table 4.5.4.2-6: NE-DC RRC_CONNECTED;

<End of modified section 4>

<Start of modified section 5>

4.5.7	Out of Coverage

4.5.7.1	Initiation

The SS shall:

1>	If sidelink is On

2>	use neither NR cell nor E-UTRA cell;

2>	use 1 GNSS simulator;

2>	use 1 NR-SS-UE;

2>	perform the procedure according to the table 4.5.7.2-1: Out of Coverage (NR sidelink).

4.5.7.2	Procedures

Table 4.5.7.2-1: Out of Coverage (NR sidelink)

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 1a1 | IF Test Mode = On OR Test Loop Function = On THEN the SS triggers UE to activate UE test mode E. NOTE: The activation of UE test mode may be performed by MMI or AT command (+CATM). | - | - | - | - |
| 1A | Trigger UE to reset or clear the current UTC time that has been calculated from GNSS. NOTE: The UTC time can be reset or clear on the UE using AT command (+CUTCR). | - | - | - | - |
| - | EXCEPTION: Steps 2a1 and 2b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 2a1 | IF Cast type = Unicast AND UE initiated unicast mode NR sidelink comunication THEN generic procedure specified in subclause 4.9.22 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. |  |  |  |  |
| 2b1 | ELSE IF Cast type = Unicast AND NR-SS-UE initiated unicast mode NR sidelink comunication THEN generic procedure specified in subclause 4.9.23 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. |  |  |  |  |
| - | EXCEPTION: Steps 3a1 and 3b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 3a1 | IF Test Loop Function = On AND the NR-SS-UE is configured as transmitting UE THEN the SS triggers UE to close UE test loop mode E (Receive Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). |  |  |  |  |
| 3b1 | IF Test Loop Function = On AND NR-SS-UE is configured as receiving UE AND SL MIMO ≠ On THEN the SS triggers UE to close UE test loop mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). |  |  |  |  |
| 3c1 | IF Test Loop Function = On AND NR-SS-UE is configured as receiving UE AND SL MIMO = On THEN the SS triggers UE to close UE test loop mode E (Transmit Mode with SL-MIMO). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). |  |  |  |  |
| 4 | Void | - | - |  |  |

4.5.7.3	Specific message contents

All specific message contents shall be according subclause 4.7B with the exceptions below.

Table 4.5.7.3-1: +CATM (Table 4.5.7.2-1, Step 1a1)

Derivation Path: +CATM specified in subclause 4.7B with condition Activation

Table 4.5.7.3-2: +CCUTLE (Table 4.5.7.2-1, Step 3a1)

Derivation Path: Table 4.7B-1 with condition Close and Receive

Table 4.5.7.3-3: +CCUTLE (Table 4.5.7.2-1, Step 3b1)

Derivation Path: Table 4.7B-1 with condition Close and Transmit

Table 4.5.7.3-4: +CCUTLE (Table 4.5.7.2-1, Step 3c1)

Derivation Path: Table 4.7B-1 with condition Close, Transmit and SL_MIMO

<End of modified section 5>