3GPP TSG-RAN WG5 Meeting # 95-e	R5-222742

# Electronic Meeting, 9th May – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1699 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update AMPR for NS_04 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Israel Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A-MPRE values for Low channel with Outer_full RB test configurations for NS_04 are misaligned with TS 38.101-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | New A-MPR values and Lower TX powerlimits are corrected in Table 6.2.3.5-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_07, NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

<< Unchanged sections skipped>>

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

Table 6.2.3.4.1-1: Test Configuration table for NS_06 and NS_35

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_35 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 16 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 22 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 25 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void NOTE 4: This configuration is only applicable for NS_06 power class 1 UE. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2: Test Configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 3 (Note 3) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 4 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 7 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 8 (Note 4, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 9 (Note 4) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 10 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 11 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 12 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 14 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 16 | Low |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 17 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 18 | High |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 23 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 25 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 26 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 27 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 28 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 29 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 30 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 31 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 32 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 33 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 34 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 35 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 36 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 37 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 38 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 39 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 40 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 41 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 42 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 43 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 44 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 45 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 46 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 47 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 48 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 49 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |

| A-MPR test parameters for NS_04 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 50 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 51 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 52 | Low |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 53 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 54 | High |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 55 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 56 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 57) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 58 | Low |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 59 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 60 | High |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth |  |  |  |  |  |  |

<< Unchanged sections skipped>>

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2.3.5-1. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.3.5-1: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-2: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 26 | 3,5 | 5.5 | 1.5 | 19 | 3.5 | 2 | 28+TT | 15.5-TT |
| 9 | 26 | 3,5 | 3.5 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 10 | 26 | 3,5 | 3.5 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 19.5-TT |
| 11 | 26 | 0.5 | 0 | 0 | 25.5 | 2.0 | 2 | 28+TT | 23.5-TT |
| 12 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 13 | 26 | 0.5 | 0 | 0 | 25.5 | 2.0 | 2 | 28+TT | 23.5-TT |
| 14 | 26 | 3,5 | 6 | 1.5 | 18.5 | 4.0 | 2 | 28+TT | 14.5-TT |
| 15 | 26 | 3,5 | 4.5 | 1.5 | 20 | 2.5 | 2 | 28+TT | 17.5-TT |
| 16 | 26 | 3,5 | 4.5 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 18.5-TT |
| 17 | 26 | 1 | 0 | 0 | 25 | 2.0 | 2 | 28+TT | 23-TT |
| 18 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 19 | 26 | 1 | 0 | 0 | 25 | 2.0 | 2 | 28+TT | 23-TT |
| 20 | 26 | 3,5 | 6 | 1.5 | 18.5 | 4.0 | 2 | 28+TT | 14.5-TT |
| 21 | 26 | 3,5 | 5 | 1.5 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 22 | 26 | 3,5 | 5 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 18.0-TT |
| 23 | 26 | 2 | 0 | 0 | 24 | 2.0 | 2 | 28+TT | 22-TT |
| 24 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 25 | 26 | 2 | 0 | 0 | 24 | 2.0 | 2 | 28+TT | 22-TT |
| 26 | 26 | 3,5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 27 | 26 | 3,5 | 5 | 1.5 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 28 | 26 | 3,5 | 5 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 18.0-TT |
| 29 | 26 | 2.5 | 0 | 0 | 23.5 | 2.0 | 2 | 28+TT | 21.5-TT |
| 30 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 31 | 26 | 2.5 | 0 | 0 | 23.5 | 2.0 | 2 | 28+TT | 21.5-TT |
| 32 | 26 | 4.5 | 8 | 1.5 | 16.5 | 5.0 | 2 | 28+TT | 11.5-TT |
| 33 | 26 | 4.5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 34 | 26 | 4.5 | 6.5 | 1.5 | 20 | 2.5 | 2 | 28+TT | 16.5-TT |
| 35 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 2 | 28+TT | 19.5-TT |
| 36 | 26 | 4.5 | 0 | 1.5 | 20 | 2.5 | 2 | 28+TT | 17.5-TT |
| 37 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 2 | 28+TT | 19.5-TT |
| 38 | 26 | 3,5 | 7.5 | 1.5 | 17 | 5.0 | 2 | 28+TT | 12-TT |
| 39 | 26 | 3,5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 40 | 26 | 3,5 | 6.5 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 16.5-TT |
| 41 | 26 | 3 | 0 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 42 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 43 | 26 | 3 | 0 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 44 | 26 | 3,5 | 7.5 | 1.5 | 17 | 5.0 | 2 | 28+TT | 12-TT |
| 45 | 26 | 3,5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 46 | 26 | 3,5 | 6.5 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 16.5-TT |
| 47 | 26 | 3 | 0 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 48 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 49 | 26 | 3 | 0 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 50 | 26 | 3,5 | 7.5 | 1.5 | 17 | 5.0 | 2 | 28+TT | 12-TT |
| 51 | 26 | 3,5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 52 | 26 | 3,5 | 6.5 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 16.5-TT |
| 53 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 54 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 55 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 56 | 26 | 6.5 | 10 | 1.5 | 14.5 | 5.0 | 2 | 28+TT | 9.5-TT |
| 57 | 26 | 6.5 | 7.5 | 1.5 | 17 | 5.0 | 2 | 28+TT | 12-TT |
| 58 | 26 | 6.5 | 7.5 | 1.5 | 18 | 3.5 | 2 | 28+TT | 15-TT |
| 59 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 60 | 26 | 6.5 | 0 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_04) for band n41 (Step 3) and UE Power Class 1.5 and Power Class 2 test requirements (NS_04) for band n41 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 3.5 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 3 | 23 | -3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 4 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 2.0 | 2 | 28+TT | 22.8-TT |
| 5 | 23 | -3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 2.0 | 2 | 28+TT | 22.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 0 | 1.5 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 10 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 11 | 23 | 0 | 0.5 | 0 | 1.5 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 13 | 23 | 0 | 1 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 14 | 23 | 0 | 1 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 15 | 23 | 0 | 1 | 0 | 1.5 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 16 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 17 | 23 | 0 | 1 | 0 | 1.5 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 19 | 23 | 0 | 2 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 2 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 21 | 23 | 0 | 2 | 0 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 22 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 23 | 23 | 0 | 2 | 0 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 2.5 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 27 | 23 | 0 | 2.5 | 0 | 1.5 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 28 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 29 | 23 | 0 | 2.5 | 0 | 1.5 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 31 | 23 | 0 | 4.5 | 6 | 1.5 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 33 | 23 | 0 | 4.5 | 0 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 38 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 40 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 41 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 43 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 45 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 46 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 47 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 3.5 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3.5 | 0 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 52 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 53 | 23 | 0 | 3.5 | 0 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 55 | 23 | 0 | 6.5 | 8 | 1.5 | 13.5 | 5.0 | 2 | 25+TT | 8.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 1.5 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 57 | 23 | 0 | 6.5 | 0 | 1.5 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 58 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 59 | 23 | 0 | 6.5 | 0 | 1.5 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>