3GPP TSG-RAN5 Meeting #95-e	R5-223272

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0753 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change FR2 SEM verification test metric |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Optimization of FR2 SEM test with objective to save test time by modification of the test metric to EIRP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update conformance requirements and test procedure section to indicate EIRP-based test measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEM test will remain unoptimized with existing TRP based test metric |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of Changes>

#### 6.5.2.1	Spectrum Emission Mask

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

6.5.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

6.5.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.2.1.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than FOOB as specified in Table 6.5.2.1.3-1 the spurious requirements in clause 6.5.3 are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.1.3-1 for the specified channel bandwidth. The requirement is verified in beam locked mode with the test metric of  of EIRP at the beam peak direction modified by the power difference between peak EIRP and TRP.

Table 6.5.2.1.3-1: General NR spectrum emission mask for Range 2.

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | 1 MHz |
| 40-100 | -13 | -13 | -13 | -13 | 1 MHz |
| 100-200 |  | -13 | -13 | -13 | 1 MHz |
| 200-400 |  |  | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | 1 MHz |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.5.2.1.

<Unchanged sections skipped>

6.5.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.2.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms for the UE to reach maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure the  EIRPof the transmitted signal at the Tx Beam Peak direction with a measurement filter of bandwidths according to Table 6.5.2.1.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table.  EIRPshall be recorded for each step as EIRPstep. The measurement period shall capture the active time slots.   EIRPis calculated considering both polarizations, theta and phi.

6.	Total Radidated Power at each step (TRPstep) will be calculated as EIRPstep- ΔP.

where ΔP = Peak EIRP(dBm) - TRP(dBm) ; is the antenna directivity which can be calculated from the measurements performed using any one test point defined in clause 6.2.1

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in Table 6.5.2.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE 2:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

<Unchanged sections skipped>

6.5.2.1.5	Test requirement

The  determined EIRP of any UE emission derived in step 6, shall fulfil requirements in Table.6.5.2.1.5-1.

<End of changes >