3GPP TSG RAN WG5#95-e	R5-223352

Electronic Meeting, 9 May – 20 May 2022

Title:	Proposing a new WI for inter-system mobility test cases between untrusted Non-3GPP and 3GPP system

Source:	China telecom

Agenda item:	4.1

Document for:	Endorsement

- Introduction

The purpose of this document is to discuss about adding inter-system mobility test cases between untrusted Non-3GPP and 3GPP system in 36.523 and 38.523.

- Discussion

During 5G era, higher spectrum deployment lead to high cost to achieve deep and wide-area network coverage.  Wi-Fi can be a good supplement to cellular coverage especially for indoor scenarios. Indoor voice call issues are in the top list of customer complaints.Wi-Fi Calling provides an opportunity to address the 5G indoor coverage issues in a cost-efficient and effective way. Seamlessly mobility between cellular and Wi-Fi can provide user better service experience when making call indoor.

According to GSMA statistic, there are more than sixty operators all over the world deploying Wi-Fi calling with untrusted non-3GPP access. Inter system mobility between 3GPP and non-3GPP is very critical feature for securing call continuity.

The SA2 has already finished the core spec for non-3GPP access to 5G System in Rel 15. CT1 have defined protocol requirement for access procedures to the 5G System via non-3GPP access networks. The RAN5 also have defined the UE conformance test cases for non-3gpp access through N3IWF to 5GC including mobility management part and session management part. However, the inter system mobility between 3GPP and non-3GPP is not covered in RAN5 specification. The situation is the same for non-3GPP access to EPS.

There are already some platforms supporting non-3GPP and 3GPP access to 5GC/EPS in one system. Therefore, we have the following proposals:

Proposal 1: RAN5 start a new WI to introduce inter-system mobility test cases between untrusted Non-3GPP and 3GPP system in 36.523 and 38.523. The test cases should cover the aspects as follow：

- Handover between the NR and 5GS /N3IWF

- Handover between EPS and 5GS /N3IWF

- Handover between EPC/ePDG and 5GS

- Handover between LTE and EPC/ePDG

The parent WIs are listed as below where objective 1/2/3 associated with 5GS_Ph1-CT and objective 4 associated with SAES-SA-FP_n3GPP.

| Parent Work / Study Items |  |  |  |  |
| --- | --- | --- | --- | --- |
| Release | Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| Rel 15 | 5GS_Ph1-CT | CT1 | 780012 | 5G Non-3GPP access networks |
| Rel 15 | 5GS_Ph1-CT | CT1 | 780013 | 5G Interworking with EPC |
| Rel 8 | SAES-SA-FP_n3GPP | CT1 | 380030 | Stage 3 for Inter-system mobility between E-UTRAN and non 3GPP accesses |

We estimate the total number of test cases will be 8 if we only consider the normal case.  Error case such PDN connection rejected by non-3GPP access may also be considered if necessary. One example of test case is provided in the annex for better understanding.

The suggested time line is as below, we estimate the test cases to be finished in RAN#99 meeting:

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.508-1 | Introduction of generic procedures, test environment, default message contents | TSG RAN#99 (Mar-23) |  |
| 38.508-2 | Introduction of specific ICS | TSG RAN#99 (Mar-23) |  |
| 38.523-1 | Introduction of new protocol test cases for handover between Untrusted Non-3GPP Access and 3GPP Access | TSG RAN#99 (Mar-23) |  |
| 38.523-2 | Applicability statements of the new protocol test cases | TSG RAN#99 (Mar-23) |  |
| 38.523-3 | Introduction of test model for new protocol test cases. | TSG RAN#99 (Mar-23) | Progress of TTCN development of the new protocol test cases is tracked in MCC TF160 reports to RAN5/RAN. |
| 36.508 | Introduction of generic procedures, test environment, default message contents | TSG RAN#99 (Mar-23) |  |
| 36.523-1 | Introduction of new protocol test cases for handover between LTE and EPC/ePDG | TSG RAN#99 (Mar-23) |  |
| 36.523-2 | Applicability statements of the new protocol test cases. | TSG RAN#99 (Mar-23) |  |
| 36.523-3 | Introduction of test model for new protocol test cases. | TSG RAN#99 (Mar-23) | Progress of TTCN development of the new protocol test cases is tracked in MCC TF160 reports to RAN5/RAN. |

Proposal 2: The test cases to be finished in RAN#99 meeting.

- Proposal

Proposal 1: RAN5 start a new WI to introduce inter-system mobility test cases between untrusted Non-3GPP and 3GPP system in 36.523 and 38.523. The test cases should cover the aspects as follow：

- Handover between the NR and 5GS /N3IWF

- Handover between EPS and 5GS /N3IWF

- Handover between EPC/ePDG and 5GS

- Handover between LTE and EPC/ePDG

Proposal 2: The test cases to be finished in RAN#99(Mar-23) meeting.

- Annex

Example Test case for Handover from 5GS to EPC/ePDG

Test Purpose (TP)

with { UE in state 5GMM-REGISTERED and 5GMM-CONNECTED on a 5GC NR cell, UE has established at least one PDU session with SMF/UPF, NWK support interworking between 5GS and ePDG/EPS, the UE supports N1 mode and IP address preservation }

ensure that {

when { When UE performs a handover of existing PDU session to ePDG/EPC }

then { UE shall indicate IP address allocated by 5GC and Handover Attach }

}

Conformance requirements

24.302 7.2.2

Once the ePDG has been selected, the UE shall initiate the IPsec tunnel establishment procedure using the IKEv2 protocol as defined in IETF RFC 7296 [28] and 3GPP TS 33.402 [15].

The UE shall send an IKE_SA_INIT request message to the selected ePDG in order to setup an IKEv2 security association. Upon receipt of an IKE_SA_INIT response, the UE shall send an IKE_AUTH request message to the ePDG, including:

-	The type of IP address (IPv4 address or IPv6 prefix or both) that needs to be configured in an IKEv2 CFG_REQUEST Configuration Payload. If the UE requests for both IPv4 address and IPv6 prefix, the UE shall send two configuration attributes in the CFG_REQUEST Configuration Payload: one for the IPv4 address and the other for the IPv6 prefix;

-	The "IDr" payload, containing the APN in the Identification Data, for non-emergency session establishment. For emergency session establishment, the UE shall format the "IDr" payload according to clause 7.2.5. The UE shall set the ID Type field of the "IDr" payload to ID_FQDN as defined in IETF RFC 7296 [28]. The UE indicates a request for the default APN by omitting the "IDr" payload, which is in accordance with IKEv2 protocol as defined in IETF RFC 7296 [28]; and

-	The "IDi" payload containing the NAI.

If the UE supports N1 mode, the UE shall indicate the PDU session ID in the IKE_AUTH request message. If N1 mode capability is disabled, the UE may indicate the PDU session ID in the IKE_AUTH request message.

In order to indicate the PDU session ID in the IKE_AUTH request message, the UE shall include the N1_MODE_CAPABILITY Notify payload as defined in clause 8.2.9.15 in the IKE_AUTH request message and shall:

-	if the UE is establishing a PDN connection not related to any existing PDU session or any existing PDN connection, allocate a PDU session ID which is not currently being used by another PDU session over either 3GPP access or non-3GPP access, set the PDU Session ID field of the N1_MODE_CAPABILITY Notify payload to the allocated PDU session ID, and associate the allocated PDU session ID with the PDN connection that is being established;

if the UE is transferring an existing PDU session from 5GS, set the PDU Session ID field of the N1_MODE_CAPABILITY Notify payload to the PDU session ID of the existing PDU session that is being transferred, and associate the PDU session ID with the PDN connection that is being established. If the existing PDU session is a non-emergency PDU session, the UE shall in addition associate the S-NSSAI of the existing PDU session that is being transferred and the related PLMN ID with the PDN connection that is being established; or

-	if the UE is transferring an existing PDN connection from EPS and a PDU session ID is associated with the PDN connection that is being transferred, set the PDU Session ID field of the N1_MODE_CAPABILITY Notify payload to the PDU session ID associated with the existing PDN connection. If the existing PDN connection is a non-emergency PDN connection and an S-NSSAI and a related PLMN ID are associated with the existing PDN connection, the UE shall in addition associate the S-NSSAI and the related PLMN ID with the PDN connection that is being established.

During the IKEv2 authentication and security association establishment for handover, the UE supporting IP address preservation for NBM, shall provide an indication about Attach Type, which indicates Handover Attach. During the IKEv2 authentication and security association establishment for transfer of an existing PDU session from 5GS, the UE shall provide an indication about Attach Type, which indicates Handover Attach. To indicate attach due to handover, the UE shall include the previously allocated home address information during the IPSec tunnel establishment. Depending on the IP version, the UE shall include either the INTERNAL_IP4_ADDRESS or the INTERNAL_IP6_ADDRESS attribute or both in the CFG_REQUEST Configuration Payload within the IKE_AUTH request message to indicate the home address information which is in accordance with IKEv2 protocol as defined in IETF RFC 7296 [28]. If the previously allocated home address information consists of both an IPv4 address and an IPv6 prefix, then the UE shall include the INTERNAL_IP4_ADDRESS attribute and the INTERNAL_IP6_ADDRESS attribute in the CFG_REQUEST configuration payload within the IKE_AUTH request message. If the previously allocated home address information consists of an IPv4 address only, then the UE shall include the INTERNAL_IP4_ADDRESS attribute and shall not include the INTERNAL_IP6_ADDRESS attribute in the CFG_REQUEST configuration payload within the IKE_AUTH request message. If the previously allocated home address information consists of an IPv6 prefix only, then the UE shall include the INTERNAL_IP6_ADDRESS attribute and shall not include the INTERNAL_IP4_ADDRESS attribute in the CFG_REQUEST configuration payload within the IKE_AUTH request message. The UE shall support IPSec ESP (see IETF RFC 4303 [32]) in order to provide secure tunnels between the UE and the ePDG as specified in 3GPP TS 33.402 [15].

Test description

        - Pre test conditions

- System Simulator:

-	One WLAN Cell.

-  One NR Cell

- UE:

-	None

- Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Void | - | - | - | - |
| 1 | The UE is initially attached to the 3GPP Access network and establish at least an IMS PDU session with 5GC network. | - | - | - | - |
| 2 | The UE performs the IMS registration procedure according TS 34.229-1 [43] subclause C.2c (steps 2-9). |  |  |  |  |
| 3 | Make the UE attempt an MTSI MO Speech Call. | - |  |  |  |
| 4 | Set NR cell to unavailable WiFi available | - |  | - | - |
| 4 | UE associates with the WLAN AP and obtains the local IP address. This step may happen in parallel with step 1. | - | - | - | - |
| 5 | Check: Does the UE transmit a DNS Query message with QNAME set to FQDN of the ePDG? | --> | DNS Query | 1 | P |
| 6 | The SS transmits a DNS Response message with the IP address of the ePDG. | <-- | DNS Response | - | - |
| 7 | Check: Does the UE transmit an IKE_SA_INIT message to the ePDG? | --> | IKE_SA_INIT | 2 | P |
| 8 | The SS transmits an IKE_SA_INIT message | <-- | IKE_SA_INIT Response |  |  |
| 9 | Check: Does UE transmit an IKE_AUTH request ? The UE shall include its address (IPv4 address or IPv6 prefix /address or both) allocated when it's attached to 3GPP Access into the CFG_Request, the "IDr" payload, containing the DNN of the PDU session and the "IDi" payload containing the NAI UE shall set the PDU Session ID field of the N1_MODE_CAPABILITY Notify payload to the PDU session ID of the existing PDU session that is being transferred. UE shall provide an indication about Attach Type, which indicates Handover Attach. | --> | IKE_AUTH Request | 3 | P |
| 10 | The SS transmits an IKE_AUTH Response message including an EAP-Request/AKA-Challenge. | <-- | IKE_AUTH Response | - | - |
| 11 | Check: Does the UE transmit an IKE_AUTH Request message including the EAP-Response/AKA-Challenge? | --> | IKE_AUTH Request | 4 | P |
| 12 | The SS transmits an IKE_AUTH Response message including EAP-Success. | <-- | IKE_AUTH Response | - | - |
| 13 | Check: Does the UE transmit an IKE_AUTH Request message with Authentication payload? | --> | IKE_AUTH Request | 5 | P |
| 14 | The SS transmits an IKE_AUTH Response message with Authentication and Configuration payloads. In the IKEv2 Authentication Response message, the ePDG sends S-NSSAI and the PLMN ID that the S-NSSAI relates to, to the UE. | <-- | IKE_AUTH Response | - | - |
| 15 | If there are multiple PDU sessions, the UE establishes connectivity for each PDU session that need transferred to non-3GPP access. |  |  | 6 | P |
| 16 | UE is triggered by MMI to release the call. | - | - | - | - |