3GPP TSG-RAN5 Meeting #95-e	R5-223353

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2998 | rev | 1 | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC CAG testcase 6.5.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose Step 24 does not add any value or helps to achieve TP2. According to TP2, the UE should automatically camp on the cell whose CAG ID is present in the Allowed CAG list. NR Cell 4 CAG ID is not present in Allowed CAG list, so this check does not make sense. It is proposed to void Step 24. It is also proposed that power level T1 be changed and NR Cell 2 be set at -88dB in Table 6.5.2.2.3.2-1 which will facilitate Steps 26-44a1 Since NRCell 4 is used only in Step 24, which is proposed to be voided, there is no point in creating NR Cell 4 for this testcase. Hence it is proposed to remove NR Cell 4 from the testcase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Voided Steps 24 and 25. Voided T2 setting in Tables 6.5.2.2.3.2-1 and 6.5.2.2.3.2-2. Updated the Prose to remove all instances of NR Cell 4. R5-223279r1 : Added MediaTek as co-source. Removed T2 from cl 6.5.2.2.3.2. Also voided T2 and NR Cell 4 from Tables 6.5.2.2.3.2-1/2. Restored Step numbers 24 and 25 in column st of Table 6.5.2.2.3.2-3 Also Updated the Length of CAG information list contents in Table 6.5.2.2.3.3-1 and 6.5.2.2.3.3-2 to 2 bytes. R5-223279r2 : In order to ensure that the testcase is run by a UE that supports CAG, introduced special message content for REGISTRATION REQ in preamble (Table 6.5.2.2.3.3-3)which checks for support of CAG. R5-223353 : Final Tdoc number for the withdrawn R5-223279r2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose is not optimized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.5.2.2	CAG Selection in Automatic Mode

6.5.2.2.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and provisioned with an empty "CAG information list” }

ensure that {

when { UE is switched on or returned to coverage, and a cell which is not a CAG cell becomes available }

then { UE selects the cell }

}

(2)

with { UE in Automatic network selection mode provisioned with a "CAG information list” }

ensure that {

when { UE is switched on or returned to coverage, and a CAG cell with a CAG-ID present in the "Allowed CAG list" becomes available }

then { UE selects the CAG cell }

}

(3)

with { UE in Automatic network selection mode provisioned with a "CAG information list” }

ensure that {

when { UE is switched on and a cell which is not a CAG cell becomes available, and there is an entry with the PLMN ID of the PLMN in the "CAG information list" but the "indication that the MS is only allowed to access 5GS via CAG cells" is not included in the entry }

then { UE selects the cell }

}

6.5.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 clauses 3.8, and 4.4.3.1.1. Unless otherwise stated these are Rel-16 requirements.

[TS 23.122, clause 3.8]

The MS may support CAG.

If the MS supports CAG, the MS can be provisioned with a "CAG information list", consisting of zero or more entries, each containing:

a)	a PLMN ID;

b)	an "Allowed CAG list". The "Allowed CAG list" contains zero or more CAG-IDs; and

c)	an optional "indication that the MS is only allowed to access 5GS via CAG cells".

The "CAG information list" is stored in the ME.

NOTE: 	When the UE is registering or registered to a PLMN other than the HPLMN or EHPLMN, then the HPLMN will send a "CAG information list" consisting of CAG subscription information related to the serving PLMN only. When the UE is registering or registered to the HPLMN or EHPLMN then the HPLMN or EHPLMN can send CAG subscription information related to any PLMN in the "CAG information list".

If the MS supports CAG and a PLMN is selected as described in subclause 4.4.3.1.1, the automatic CAG selection is performed as part of subclause 4.4.3.1.1.

If the MS supports CAG and a PLMN is selected as described in subclause 4.4.3.1.2, the manual CAG selection is performed as part of subclause 4.4.3.1.2.

The NAS shall provide the AS with a "CAG information list", if available. If the contents of the "CAG information list" have changed, the NAS shall provide an updated "CAG information list" to the AS.

[TS 23.122, clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order) or stored in the ME (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

When following the above procedure the following requirements apply:

…

m)	In i to v, if the MS supports CAG and:

1)	is provisioned with a non-empty "CAG information list", the MS shall consider a PLMN indicated by an NG-RAN cell only if:

A)	the cell is a CAG cell and broadcasts a CAG-ID for the PLMN such that there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is included in the "Allowed CAG list" of the entry; or

B)	the cell is not a CAG cell and:

-	there is no entry with the PLMN ID of the PLMN in the "CAG information list"; or

-	there exists an entry with the PLMN ID of the PLMN in the "CAG information list" but the "indication that the MS is only allowed to access 5GS via CAG cells" is not included in the entry; or

2)	is provisioned with an empty "CAG information list" or is not provisioned with a "CAG information list", the MS shall consider a PLMN indicated by an NG-RAN cell only if the cell is not a CAG cell.

n)	In i to v, if the MS only supports control plane CIoT 5GS optimization (see 3GPP TS 23.501 [62]) and the MS camps on an E-UTRA cell connected to 5GCN, which is not NB-IoT cell (see 3GPP TS 36.304 [43], 3GPP TS 36.331 [22]), the MS shall not consider PLMNs which do not advertise support of 5GS services with control plane CIoT 5GS optimization.

o)	In i to v, if the MS supports CIoT 5GS optimizations, the MS shall not consider the PLMN/access technology combinations for which the MS preferred CIoT network behaviour is not advertised as supported by the PLMN/access technology combination (see 3GPP TS 24.501 [64]).

NOTE 7:	As an implementation option, the MS supporting CIoT 5GS optimizations that was not able to select any PLMN according to o) can perform a second iteration of i to v with no restriction.

p)	In iii, the MS shall use the PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" stored in the ME, if the last received steering of roaming information contains the "list of preferred PLMN/access technology combinations"(see annex C) and is stored in the ME. Otherwise, the MS shall use the "Operator Controlled PLMN Selector with Access Technology" list retrieved from the SIM.

If successful registration is achieved, the MS indicates the selected PLMN.

If registration cannot be achieved because no PLMNs are available and allowable, and the MS does not support access to RLOS, the MS indicates "no service" to the user, waits until a new PLMN is available and allowable and then repeats the procedure.

If there were one or more PLMNs which were available and allowable, but an LR failure made registration on those PLMNs unsuccessful or an entry in any of the lists "forbidden location areas for roaming", "forbidden tracking areas for roaming", "5GS forbidden tracking areas for roaming", "forbidden location areas for regional provision of service", "forbidden tracking areas for regional provision of service", "5GS forbidden tracking areas for regional provision of service", or "CAG information list" prevented a registration attempt, the MS selects the first such PLMN again and enters a limited service state.

…

6.5.2.2.3	Test description

6.5.2.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 11.

-	NR Cell 2 is CAG cells.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used for NR Cell 1, NR Cell 2 and NR Cell 11.

-	The PLMNs are identified in the test by the identifiers in Table 6.5.2.2.3.1-1 and the PLMN settings are defined in TS 36.523-1 [13] table 6.0.1-1.

Table 6.5.2.2.3.1-1: PLMN identifiers

| NR Cell | PLMN name | CAG IDs |
| --- | --- | --- |
| 2 | PLMN2 | 1 |
| 11 | PLMN2 | - |

UE:

-	The UE is in Automatic PLMN selection mode.

-	There is an empty “CAG information list” provisioned in the UE.

Preamble:

-	The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes an empty CAG information list before it is in state Switched OFF (State 0N-B).

6.5.2.2.3.2	Test procedure sequence

Table 6.5.2.2.3.2-1 for FR1 and 6.5.2.2.3.2-2 for FR2 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", and "T3" are to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.2.2.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | void | NR Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" |  | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 |  | "Off" |  |
| void |  |  |  |  |  |  |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" |  | -88 |  |

Table 6.5.2.2.3.2-2: Cell configuration changes over time for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | void | NR Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | "Off" |  | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" |  | "Off" |  |
| void |  |  |  |  |  |  |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" |  | FFS |  |

Table 6.5.2.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | Check: Does the UE send a RRCSetupRequest on NR Cell 1? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 3-13 | Steps 3 to 13 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. NOTE: The UE starts registration. | - | - | - | - |
| 14 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing the CAG information list with the CAG ID of NR Cell 2. | <-- | NR RRC: DLInformationTransfer 5G MM: REGISTRATION ACCEPT | - | - |
| 15-20a1 | Steps 15 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. NOTE: The UE completes registration and the RRC connection is released. | - | - | - | - |
| 21 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4] | - | - | - | - |
| 22 | SS adjusts cell levels according to row T1 of table 6.5.2.2.3.2-1 for FR1 and table 6.5.2.2.3.2-2 for FR2. | - | - | - | - |
| 23 | Power on the UE. | - | - | - | - |
| 24 | Void | - | - | - | - |
| 25 | Void | - | - | - | - |
| 26 | Check: Does the UE send a RRCSetupRequest on NR Cell 2? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 27-44a1 | Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 2. NOTE: The UE completes registration and the RRC connection is released. | - | - | - | - |
| 45 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4] | - | - | - | - |
| 46 | SS adjusts cell levels according to row T3 of table 6.5.2.2.3.2-1 for FR1 and table 6.5.2.2.3.2-2 for FR2. | - | - | - | - |
| 47 | Power on the UE. | - | - | - | - |
| 48 | Check: Does the UE send a RRCSetupRequest on NR Cell 11? | --> | NR RRC: RRCSetupRequest | 3 | P |
| 49-66a1 | Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 11. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

6.5.2.2.3.3	Specific message contents

Table 6.5.2.2.3.3-1: REGISTRATION ACCEPT (step 14, Table 6.5.2.2.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  | MCC : 002; MNC : 11; CAG only : 0; CAG-ID 1 : 1 |  |
| Length of CAG information list contents | ‘0000 0000 0000 1001’B |  |  |
| Entry 1 |  |  |  |
| Length of entry contents | ‘0000 1000’B |  |  |
| MCC | MCC of NR Cell 2 | See Table 6.5.2.2.3.1-1 |  |
| MNC | MNC of NR Cell 2 | See Table 6.5.2.2.3.1-1 |  |
| CAG only | ‘0’B |  |  |
| CAG_ID 1 | 1 | CAG ID is coded as a 32 bit BITSTRING |  |

Table 6.5.2.2.3.3-2: REGISTRATION ACCEPT (Preamble)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  |  |  |
| Length of CAG information list contents | ‘0000 0000 0000 0000’B |  |  |
| Entry 1 | Not present |  |  |

Table 6.5.2.2.3.3-3: REGISTRATION REQUEST (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '0000 0001'B | Initial registration |  |
| 5GMM capability |  |  |  |
| CAG | ‘1’B | CAG supported |  |