3GPP TSG-RAN5 Meeting #95-e	R5-223360

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2333 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default configuration of SCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To clarify the configuration value of fields in 1st stage SCI: - DMRS pattern - PSFCH overhead indication - To clarify the configuration value of fields in 2nd stage SCI: - HARQ feedback enabled/disabled indicator |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Configuration value of DMRS pattern and PSFCH overhead indication in 1st stage SCI are clarified. - Configuration value of HARQ feedback enabled/disabled indicator in 2nd stage SCI are clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion is caused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-222642r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.6.2	Sidelink physical layer parameters

4.3.6.2.1	Physical layer parameters for scheduling of PSSCH on PSCCH

4.3.6.2.1.1	Physical layer parameters for SCI format 1-A

SCI format 1-A is used to schedule PSSCH transmission.

Default physical layer parameters for SCI format 1-A are specified in Table 4.3.6.2.1.1-1.

Table 4.3.6.2.1.1-1: Physical layer parameters for SCI format 1-A

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Priority | Dependent on test parameters | - |  |
| Frequency resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | only one PSCCH/PSSCH occasion is scheduled per grant | "00000"B |  |
| Resource reservation period | Not present | - |  |
| DMRS pattern | Indicating the first entry of sl-PSSCH-DMRS-TimePatternList-r16 to be used | "0"B |  |
| 2nd-stage SCI format | SCI format 2-A | “00”B |  |
|  | SCI format 2-B | “01”B | DISTANCE-BASED HARQ |
| Beta_offset indicator | Indicating the first entry of sl-BetaOffsets2ndSCI to be used | "00"B |  |
| Number of DMRS port | Single port p = 1000 is used | "0"B |  |
|  | Two ports p = 1000 and 1001 are used | "1"B | SL MIMO |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| Additional MCS table indicator |  | Not present |  | - |  |  |  |
| PSFCH overhead indication |  | "1"B if mod (i, nPSFCH) = 0 "0"B if mod (i, nPSFCH ) ≠ 0 where i is the logical slot index belong to resource pool per 1024 frame as specified in 38.214 [21] clause 8. |  |  |  | nPSFCH = 2 OR nPSFCH = 4 (Note 1) |  |
|  |  | Not present |  |  |  |  |  |
| Reserved |  | Number of reserved bits is given by sl-NumReservedBits-r16 |  | "00" |  |  |  |
| Note 1: nPSFCH is the PSFCH period (in slots) provided by sl-PSFCH-Period-r16. nPSFCH = 0 if sl-PSFCH-Period-r16 is not configured. |  |  |  |

| Condition | Explanation |
| --- | --- |
| DISTANCE-BASED HARQ | For distance-based HARQ test |
| SL MIMO | For SL MIMO test |

4.3.6.2.2	Physical layer parameters for scheduling on PSSCH

4.3.6.2.2.1	Physical layer parameters for SCI format 2-A

SCI format 2-A is used to provide HARQ information, to provide Souce/Destination L1 ID or to trigger CSI reporting.

Default physical layer parameters for SCI format 2-A are specified in Table 4.3.6.2.2.1-1.

Table 4.3.6.2.2.1-1: Physical layer parameters for SCI format 2-A

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| HARQ process number | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| Source ID | Dependent on test parameters | - |  |
| Destination ID | Dependent on test parameters | - |  |
| HARQ feedback enabled/disabled indicator | HARQ feedback is enabled | “1”B |  |
| Cast type indicator | broadcast | "00"B | BROADCAST |
|  | groupcast with ACK/NACK HARQ | "01"B | GROUPCAST_ACK_NACK |
|  | unicast | "10"B | UNICAST |
|  | groupcast with NACK-only HARQ | "11"B | GROUPCAST_NACK_ONLY |
| CSI request | Not ask for SL CSI report | "0"B |  |
|  | Ask for SL CSI report | "1"B | SL_CSI |

| Condition | Explanation |
| --- | --- |
| BROADCAST | For broadcast SL communication test |
| GROUPCAST_ACK_NACK | For groupcast SL communication with ACK/NACK HARQ feedback test |
| GROUPCAST_NACK_ONLY | For groupcast SL communication with NACK-only HARQ feedback test |
| UNICAST | For unicast SL communication test |
| SL_CSI | For SL CSI reporting test |

4.3.6.2.2.2	Physical layer parameters for SCI format 2-B

SCI format 2-B is used to provide HARQ information, Souce/Destination L1 ID or distance-based HARQ related parameters.

Default physical layer parameters for SCI format 2-B are specified in Table 4.3.6.2.2.2-1.

Table 4.3.6.2.2.2-1: Physical layer parameters for SCI format 2-B

| Parameter | Value | Value in binary |
| --- | --- | --- |
| HARQ process number | Dependent on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |
| Redundancy version | Dependent on test parameters | - |
| Source ID | Dependent on test parameters | - |
| Destination ID | Dependent on test parameters | - |
| HARQ feedback enabled/disabled indicator | HARQ feedback enabled | “1”B |
| Zone ID | Dependent on test parameters | - |
| Communication range requirement | Indicating the first entry of sl-ZoneConfigMCR-List to be used | "0000"B |

<End of modified section 1>