3GPP TSG-RAN5 Meeting #95-e	R5-223843

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0167 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test bands selection criteria for UL MIMO capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current D001 and D005 is reflect to general frequency bands used to all type of test cases, however, D001 and D005 are defined for UL MIMO test cases and shall reflect to UL MIMO capabilties in TS38.508-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new Dxx1 to adapt new Table A.4.3.9-12 for NR FR1 UL MIMO; - Added new Dxx2 to adapt new Table A.4.3.9-13 for NR FR2 UL MIMO; - Updated selection criteria to UL MIMO test cases; - Corrected D012 with new Table A.4.3.9-12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL MIMO test cases will stay incorrect to the PICS table. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Corresponding TS38.508-2 CR#0326 (R5-222695) with new Table A.4.3.9-12 and Table A.4.3.9-13 added. New Dxx1/Dxx2 in table 4.0-2 need defined # when implement in the final spec. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-222696 with following changes: - corrected other comments in CR coverpage; - add new Dxx1/Dxx2 instead changing D001/D005; - updated corresponding UL MIMO TC with new Dxx1/Dxx2; - corrected D012 with new table. |  |  |  |  |  |  |  |  |

## {Start of changes}

## 4.0	Test case conditions and selection criteria

For the purposes of the present document, the applicability of conformance test cases conditions given in Table 4.0-1 apply. The tested bands selection criteria given in Table 4.0-2 apply. The tested CA/DC configuration selection criteria given in Table 4.0-3 apply. The ICS proformas used in Table 4.0-1, Table 4.0-2 and Table 4.0-3 are defined in TS 38.508-2 [8] unless otherwise stated.

<Unchanged Sections Skipped>

Table 4.0-2: Tested Bands Selection Criteria

| Code | Tested Bands Selection Criteria |  |  | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| D001 | (A.4.3.1-1 OR A.4.3.1-2) AND NOT (A.4.3.1-5 OR A.4.3.1-6) |  |  | All supported FR1 Bands without SUL/SDL bands |  |  |
| D002 | A.4.3.1-4 |  |  | All supported FR1 PC2 Bands |  |  |
| D003 | A.4.3.1-5 |  |  | All supported FR1 SUL Bands |  |  |
| D004 | {1,2,3,5,7,8,12,14,20,25,28,30,34,38,39,40,41,50,51,65,66,70,71,74,75,76} |  |  | UE supported bands among n1,n2,n3,n5,n7,n8,n12,n14,n20,n25,n28,n30,n34,n38,n39,n40,n41,n50,n51,n65,n66,n70,n71,n74,n75,n76 |  |  |
| D005 | A.4.3.1-3 |  |  | All supported FR2 Bands |  |  |
| D006 | A.4.3.1-1 OR A.4.3.1-2 |  |  | All supported FR1 Bands |  |  |
| D007 | A.4.3.1-1 OR A.4.3.1-2 OR A.4.3.1-3 |  |  | All supported NR Bands |  |  |
| D008 | ANY((A.4.3.1-1) AND 10MHz) |  |  | Any FDD FR1 band within the set supporting 10 MHz UE Channel BW |  |  |
| D009 | ANY((A.4.3.1-2) AND 20MHz) |  |  | Any TDD FR1 band within the set supporting 20 MHz UE Channel BW |  |  |
| D010 | ANY((A.4.3.1-2) AND 40MHz) |  |  | Any TDD FR1 band within the set supporting 40 MHz UE Channel BW |  |  |
| D011 | A.4.3.9-4a OR A.4.3.9-4b |  |  | All supported 4 Rx antenna ports Bands |  |  |
| D012 | ((A.4.3.9-12) AND FDD) AND NOT (A.4.3.1-5 OR A.4.3.1-6) |  |  | All supported FDD FR1 bands with UL MIMO capabilities and not SUL/SDL bands |  |  |
| D013 | ANY((A.4.3.1-3) AND 50MHz) |  |  | Any TDD FR2 band within the set supporting 50 MHz UE Channel BW |  |  |
| D014 | ANY((A.4.3.1-3) AND 100MHz) |  |  | Any TDD FR2 band within the set supporting 100 MHz UE Channel BW |  |  |
| D015 | ANY((A.4.3.1-3) AND 200MHz) |  |  | Any TDD FR2 band within the set supporting 200 MHz UE Channel BW |  |  |
| D016 | A.4.3.1-9 |  |  | All supported FR1 sidelink bands |  |  |
| D017 | {40, 41, 77, 78, 79} |  |  | UE supported TDD bands among n40, n41, n77, n78, n79 |  |  |
| D018 | A.4.3.1-2/2e OR A.4.3.1-2/12 |  |  | All supported FR1 Bands for operation with shared spectrum channel access |  |  |
| D019 |  | {34, 38, 39, 48, 90} AND 10MHz |  |  | TDD FR1 bands among n34, n38, n39, n48, n90 supporting 10MHz UE Channel BW |  |
| Dxx1 |  | A.4.3.9-12 AND NOT (A.4.3.1-5 OR A.4.3.1-6) | All supported FR1 Bands with UL MIMO capabilities and not SUL/SDL bands |
| Dxx2 |  | A.4.3.9-13 | All supported FR2 Bands with UL MIMO capabilities |
| NOTE 1: Band Selection is based on set theory. For each feature, item number shall correspond to the Band number. The result is the set of bands for which the test shall be conducted. The following operators are used: AND: Set intersection ( ). {1,2} AND {2,3} = {2} OR: Set union ( ). {1,2} OR {2,3} = {1,2,3} NOT: Set complement (\), full set being all bands. NOT{1} = {2 ...256} Also note that this is set without repetitions so {1} AND {1} = {1} The following basic sets are used: {1,2}: Explicitly given band set 10MHz: All bands supporting 10 MHz FDD: All bands in FDD mode |  |  |  |  |  |  |

<Unchanged Sections Skipped>

## 4.1	RF conformance test cases

NOTE:	To determine applicability of a test case, supported CBW and SCS in the RF-Parameters IE (see TS 38.331 [11]) which conveys RF related capabilities for NR operation is taken into account.

### 4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC1.5 PC2 PC3 |  |
|  | <Unchanged Sections Skipped> |  |  |  |  |  |  |
| 6.2A.2.1 | UE maximum output power reduction for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  | Test execution is not necessary if TS 38.521-1 TC 6.5A.2.4.1.1 is executed. |
| 6.2D.1.1 | UE maximum output power - EIRP and TRP for UL MIMO | Rel-15 | C151 | UEs supporting 5GS FR2 and UL-MIMO | Dxx1 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2D.1.2 | UE maximum output power - Spherical coverage for UL MIMO | Rel-15 | C151 | UEs supporting 5GS FR2 and UL-MIMO | Dxx1 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2A.3.1 | UE additional maximum output power reduction for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  | Test execution is not necessary if TS 38.521-1 TC 6.5A.2.3 and 6.5A.3.3 are executed. |
| 6.2A.4.1 | Configured transmitted power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.2C.1 | Configured transmitted power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2C.3 | UE maximum output power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2C.4 | UE maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | Test execution is not necessary if TS 38.521-1 TC 6.5C.2.4.1 is executed. |
| 6.2C.5 | UE additional maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2D.1 | UE maximum output power for UL MIMO | Rel-15 | N/A | Not recommended due to no test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4. |  |  | Maximum Output Power for UL MIMO is tested as part of the MPR test case with using MPR=1.5dB suggested by RAN4. |
| 6.2D.2 | UE maximum output power reduction for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.4.1 is executed. |
| 6.2D.3 | UE additional maximum output power reduction for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.3 and 6.5D.3.3 are executed. |
| 6.2D.4 | Configured transmitted power for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.2E.2.1 | UE maximum output power reduction for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | Test execution is not necessary if TS 38.521-1 TC 6.5E.2.4.1 is executed. |
|  | <Unchanged Sections Skipped> |  |  |  |  |  |  |
| 6.3C.4.3 | Aggregate power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3D.1 | Minimum output power for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.3D.3 | Transmit ON/OFF time mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.3D.4.1 | Absolute power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.3D.4.2 | Relative power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.3D.4.3 | Aggregate power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.3E.1.1 | Minimum output power for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |
|  | <Unchanged Sections Skipped> |  |  |  |  |  |  |
| 6.4C.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK for SUL | Rel-16 | C112 | UEs supporting 5GS FR1 and SUL and pi/2-BPSK modulation scheme and low PAPR DMRS | D003 |  |  |
| 6.4D.1 | Frequency error for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.4D.2.1 | Error vector magnitude for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.4D.2.2 | Carrier leakage for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.4D.2.3 | In-band emissions for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.4D.2.4 | EVM equalizer spectrum flatness for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.4D.3 | Time alignment error for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.4D.4 | Requirements for coherent UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
|  | <Unchanged Sections Skipped> |  |  |  |  |  |  |
| 6.5C.4 | Transmit intermodulation for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5D.1 | Occupied bandwidth for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.1_1 | Occupied bandwidth for UL MIMO (Rel-16 onward) | Rel-16 | C003b | UEs supporting 5GS FR1 and UL MIMO and ULFPTx | Dxx1 |  |  |
| 6.5D.2.2 | Spectrum Emission Mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.2.3 | Additional spectrum emission mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.2.4.1 | NR ACLR for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.2.4.2 | UTRA ACLR for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.2_1.4.2 | UTRA ACLR for UL MIMO (Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.3.1 | General spurious emissions for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.3.2 | Spurious emissions for UE co-existence for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.3.3 | Additional spurious emissions for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.3_1.1 | General spurious emissions for UL MIMO (Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.3_1.2 | Spurious emission for UE co-existence for UL MIMO (Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.3_1.3 | Additional spurious emissions for UL MIMO(Rel-16 onward) | Rel-16 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5D.4 | Transmit intermodulation for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | Dxx1 |  |  |
| 6.5E.2.2.1 | Spectrum emission mask for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |
|  | <Unchanged Sections Skipped> |  |  |  |  |  |  |
| 6.5E.2.3.1 | Additional Spectrum emission mask for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |
| 6.5E.2.4.1 | Adjacent channel leakage ratio for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |  |
| 6.5E.3.2.1 | Spurious emissions for UE co-existence for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |
| 6.5E.3.3.1 | Additional spurious emissions requirements for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |
| 6.5F.2.2 | Spectrum emission mask for operation with shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |
| 6.5F.2.4 | Adjacent channel leakage ratio for operation with shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |
| 6.5F.3.1 | General spurious emissions | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |
| 7 | Receiver Characteristics |  |  |  |  |  |  |
| 7.3.2 | Reference sensitivity power level | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | 2Rx 4Rx |  |
| 7.3A.1 | Reference sensitivity power level for 2DL CA without exception | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.3A.1_1 | Reference sensitivity power level for 2DL CA exceptions | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.3A.2 | Reference sensitivity level for CA (3DL CA) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.3A.3 | Reference sensitivity level for CA (4DL CA) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.3C.2 | Reference sensitivity power level for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 7.3D.2 | Reference sensitivity power level for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.3E.2 | Reference sensitivity for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |
| 7.3F.2 | Reference sensitivity for shared spectrum channel access | Rel-16 | C001c | UEs supporting 5GS FR1 and operation with shared spectrum channel access | D018 |  |  |
| 7.4 | Maximum input level | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.4 if UE supports NSA and TS 38.521-3 TC 7.4B.3 or 7.4B.4 has been executed. |
| 7.4A.1 | Maximum input level for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.4A.2 | Maximum input level for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.4A.3 | Maximum input level for CA (4DL CA) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4D | Maximum input level for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.5 | Adjacent channel selectivity | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | NOTE 1 Skip TC 7.5 if UE supports NSA and TS 38.521-3 TC 7.5B.2 or 7.5B.3 has been executed. |
| 7.5A.1 | Adjacent channel selectivity for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.5A.2 | Adjacent channel selectivity for 3DL CA | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.5A.3 | Adjacent channel selectivity for 4DL CA | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.5D | Adjacent channel selectivity for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.6.2 | In-band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.6.2 if UE supports NSA and TS 38.521-3 TC 7.6B.2.2 or 7.6B.2.3 has been executed. |
| 7.6.3 | Out-of-band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 7.6.4 | Narrow band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D004 |  | Skip TC 7.6.4 if UE supports NSA and TS 38.521-3 TC 7.6B.4.2 or 7.6B.4.3 has been executed. |
| 7.6A.2.1 | In-band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.6A.2.2 | In-band blocking for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.6A.2.3 | In-band blocking for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | NOTE 1 Skip TC 7.6A.2.3 if UE supports NSA and TS 38.521-3 TC 7.6B.2.3_1.3 has been executed. |
| 7.6A.3.1 | Out-of-band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.6A.3.2 | Out-of-band blocking for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.6A.3.3 | Out-of-band blocking for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |
| 7.6A.4.1 | Narrow band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.6A.4.2 | Narrow band blocking for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.6A.4.3 | Narrow band blocking for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | Skip TC 7.6A.4.3 if UE supports NSA and TS 38.521-3 TC 7.6B.4.3_1.3 has been executed. |
| 7.6C.2 | In-band blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 7.6C.2_1 | Inband Blocking for SUL with DL CA | Rel-17 | C078 | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E031 |  |  |
| 7.6C.3 | Out-of-band blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 7.6C.3_1 | Out-of-band blocking for SUL with DL CA | Rel-17 | C078 | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E031 |  |  |
| 7.6D.2 | In-band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.6D.3 | Out-of-band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.6D.4 | Narrow band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.7 | Spurious response | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 7.7A.1 | Spurious response for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.7A.2 | Spurious response for CA (3DL CA) | Rel-16 | FFS | UEs supporting 5GS FR1 and CA (3DL CA) | FFS |  | NOTE 1 |
| 7.7A.3 | Spurious response for CA (4DL CA) | Rel-16 | FFS | UEs supporting 5GS FR1 and CA (4DL CA) | FFS |  | NOTE 1 |
| 7.7D | Spurious response for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.8.2 | Wide band Intermodulation | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.8.2 if UE supports NSA and TS 38.521-3 TC 7.8B.2.2 or 7.8B.2.3 has been executed. |
| 7.8A.2.1 | Wide band Intermodulation for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.8A.2.2 | Wide band Intermodulation for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.8A.2.3 | Wide band Intermodulation for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |
| 7.8D.2 | Wide band Intermodulation for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.9 | Spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.9 if UE supports NSA and TS 38.521-3 TC 7.9B.1 or 7.9B.2 or 7.9B.3 has been executed. |
| 7.9A.1 | Spurious emissions for CA (2DL CA) | Rel-15 | C005 | UEs supporting 5GS FR1 and inter-band 2DL CA with a DL-only band | E002 |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-1. |  |  |  |  |  |  |  |

Table 4.1.1-1a: Void

Table 4.1.1-1b: Void

Table 4.1.1-1c: Void

### 4.1.2	FR2 standalone conformance test cases

Table 4.1.2-1: Applicability of RF SA FR2 conformance test cases, ref. TS 38.521-2 [2]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1.1 | UE maximum output power - EIRP and TRP | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | Skip TC 6.2.1.1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4.1 has been executed. |
|  | <Unchanged Sections Skipped> |  |  |  |  |  |  |
| 6.3A.4.4.7 | Aggregate power tolerance for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.3D.1 | Minimum output power for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | Dxx2 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3D.2 | Transmit OFF power for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | Dxx2 |  | NOTE 1 |
| 6.3D.3.1 | General ON/OFF time mask for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | Dxx2 |  | NOTE 1 |
| 6.3D.3.4 | SRS time mask for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | Dxx2 |  | NOTE 1 |
| 6.4.1 | Frequency error | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.4.1 if UE supports NSA and TS 38.521-3 TC 6.4B.1.4 has been executed. |
|  | <Unchanged Sections Skipped> |  |  |  |  |  |  |
| 6.4A.2.3.7 | In-band emissions for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4D.1 | Frequency error for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | Dxx2 |  | NOTE 1 |
| 6.4D.3 | Time alignment error for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | Dxx2 |  | NOTE 1 |
| 6.5.1 | Occupied bandwidth | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.5.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4 has been executed. |

## {End of changes}