3GPP TSG-RAN5 Meeting #96-e	R5-224046
Electronic Meeting, 15th August – 26th August 2022

Title: 	Discussion on Rel-16 Power Boost test definition

Source:	        Keysight Technologies UK Ltd, Apple Portugal

Agenda Item:	5.3.15.8

Document for:	Discussion and Endorsement

# 1.	Introduction

As part of the Rel-16 FR2 RF enhancements, RAN4 has defined a mechanism to enable UL power boosting in exchange for suspended In-Band Emissions requirements ([1], [2], [3], [4]).

This document discusses how to introduce such core requirements into test specifications ([5]).

# 2.	Discussion

# 2.1 Core requirements analysis

The changes in core requirements related to Power Boost are the ones highlighted in yellow in tables below:

### 6.2.4 Configured transmitted power The UE can configure its maximum output power. The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds PPowerclass + DPIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax with PPowerclass the UE power class as specified in sub-clause 6.2.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1, MPRf,c as specified in sub-clause 6.2.2 , A-MPRf,c as specified in sub-clause 6.2.3, ΔMBP,n the peak EIRP relaxation as specified in clause 6.2.1 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1. DPIBE is 1.0 dB if UE declares support for mpr-PowerBoost-FR2-r16, UL transmission is QPSK, MPRf,c = 0 and when NS_200 applies and the network configures the UE to operate with mpr-PowerBoost-FR2-r16. Otherwise DPIBE is 0.0 dB. The requirement is verified in beam peak direction. maxUplinkDutyCycle-FR2, as defined in TS 38.306 [14], is a UE capability to facilitate electromagnetic power density exposure requirements. This UE capability is applicable to all FR2 power classes. If the field of UE capability maxUplinkDutyCycle-FR2 is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE follows the uplink scheduling and can apply P-MPRf,c. If the field of UE capability maxUplinkDutyCycle-FR2 is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means. P-MPRf,c is the power management maximum output power reduction. The UE shall apply P-MPRf,c for carrier f of serving cell c only for the cases described below. For UE conformance testing P-MPRf,c shall be 0 dB. a) ensuring compliance with applicable electromagnetic power density exposure requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications; b) ensuring compliance with applicable electromagnetic power density exposure requirements in case of proximity detection is used to address such requirements that require a lower maximum output power. NOTE 1: P-MPRf,c was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions. NOTE 2: P-MPRf,c and maxUplinkDutyCycle-FR2 may impact the maximum uplink performance for the selected UL transmission path. NOTE 3: MPE P-MPR Reporting, as defined in TS 38.306 [14], is an optional UE capability to report P-MPRf,c when the reporting conditions configured by gNB are met. This UE capability is applicable to all FR2 power classes. The tolerance T(∆P) for applicable values of ∆P (values in dB) is specified in Table 6.2.4-1. Table 6.2.4-1: PUMAX,f,c tolerance Operating Band; ∆P (dB); Tolerance T(∆P) (dB). n257, n258, n259, n260, n261, n262; P = 0; 0. 0 < P ≤ 2; 1.5. 2 < P ≤ 3; 2.0. 3 < P ≤ 4; 3.0. 4 < P ≤ 5; 4.0. 5 < P ≤ 10; 5.0. 10 < P ≤ 15; 7.0. 15 < P ≤ X; 8.0. NOTE: X is the value such that Pumax,f,c lower bound, PPowerclass - P – T(P) = minimum output power specified in clause 6.3.1.

#### 6.4.2.3 In-band emissions ##### 6.4.2.3.1 General The in-band emission is defined as the average across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The IBE requirement does not apply if UE declares support for mpr-PowerBoost-FR2-r16, UL transmission is QPSK,MPRf,c = 0 and when NS_200 applies, and the network configures the UE to operate with mpr-PowerBoost-FR2-r16. The basic in-band emissions measurement interval is identical to that of the EVM test. The requirement is verified with the test metric of In-band emission (Link=TX beam peak direction, Meas=Link angle).

Observation 1: Core requirements modification for Power Boost affect Configured output power and in-band emissions.

Observation 2: In order to exercise Power Boost requirements, the UE shall support mpr-PowerBoost-FR2-r16 UE capability ([6]):

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| mpr-PowerBoost-FR2-r16 Indicates whether UE supports uplink transmission power boost by suspension of in-band emission (IBE) requirements as specified in TS 38.101-2 [3]. | Band | No | TDD only | FR2 only |

Observation 3: In order to exercise Power Boost requirements, the network needs to configure the UE to operate with mpr-PowerBoost-FR2-r16 ([7]):

ServingCellConfig information element

UplinkConfig ::=                    SEQUENCE {

initialUplinkBWP                    BWP-UplinkDedicated                                                     OPTIONAL,   -- Need M

uplinkBWP-ToReleaseList             SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                              OPTIONAL,   -- Need N

uplinkBWP-ToAddModList              SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink                          OPTIONAL,   -- Need N

firstActiveUplinkBWP-Id             BWP-Id                                                                  OPTIONAL,   -- Cond SyncAndCellAdd

pusch-ServingCellConfig             SetupRelease { PUSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

carrierSwitching                    SetupRelease { SRS-CarrierSwitching }                                   OPTIONAL,   -- Need M

...,

[[

powerBoostPi2BPSK                   BOOLEAN                                                                 OPTIONAL,   -- Need M

uplinkChannelBW-PerSCS-List         SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

enablePL-RS-UpdateForPUSCH-SRS-r16  ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUSCH0-0-r16 ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUCCH-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForSRS-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

uplinkTxSwitching-r16               SetupRelease { UplinkTxSwitching-r16 }                                  OPTIONAL,   -- Need M

mpr-PowerBoost-FR2-r16              ENUMERATED {true}                                                       OPTIONAL    -- Need R

]]

Observation 4: PIBE applies to EIRP metric only.

Finally, PIBE is applicable to all power classes and channel bandwidths as:

- No restriction in the definition

- All power classes and channel bandwidths allow MPRf,c = 0 under certain conditions

Observation 5: PIBE is applicable to all power classes and channel bandwidths as:

- No restriction in the definition

- All power classes and channel bandwidths allow MPRf,c = 0 under certain conditions

# 2.2 Test case applicability

As new Rel-16 signalling and new Rel-16 UE capabilities are required to exercise Power Boost requirements, associated test case should be applicable from Rel-16 onwards.

Proposal 1: As new Rel-16 signalling and new Rel-16 UE capabilities are required to exercise Power Boost requirements, Power Boost associated testing should be applicable from Rel-16 and onwards for devices supporting mpr-PowerBoost-FR2-r16 UE capability.

# 2.3 Candidate test cases

Observing the applicability of the configured output power test case in [5], it can be seen that configured output power core requirements are tested indirectly through 6.2.1 Maximum output power, 6.2.2 Maximum output power reduction and 6.2.3 UE maximum output power with additional requirements:

6.2.4.2 Test applicability The requirements of this test are covered in test cases 6.2.1 Maximum output power, 6.2.2 Maximum output power reduction and 6.2.3 UE maximum output power with additional requirements to all types of NR UE release 15 and forward.

Hence one of these 3 test cases should be used as a baseline to define new test case.

Considering that PIBE is only applicable if NS_200 is used, 6.2.3 UE maximum output power with additional requirements test shall be discarded as an option:

On the other hand, considering that PIBE≠0 is only applicable for QPSK modulation when MPRf,c=0, 6.2.1 Maximum output power test shall be used as a baseline to add Power Boost requirements.

According to MPR requirements in section 6.2.2.3 ([5]), MPRf,c=0 implies:

- Inner region 1

- DFTs-OFDM waveform

- QPSK or Pi/2 BPSK

Hence, current test point definition for 6.2.1 Maximum output power could also be leveraged:

Proposal 2: Use 6.2.1.1 Maximum output power test as a baseline to define test for Rel-16 FR2 RF enhancement related to Power Boost.

When evaluating whether changes should be done over existing test case vs a new test case, considering applicability analysis in section 2.2, it seems the better approach is to define a new test case. To be taken into account that PIBE applies only to EIRP metric, even when using 6.2.1.1 Maximum output power test as a baseline, there is no need to run TRP measurement.

Proposal 3: Define a new test case to test Power Boost requirements: 6.2.4_1 Configured transmitted power with Power Boost.

Taking into account that existing 6.2.1.1 UE maximum output power - EIRP and TRP test:

- Includes TRP measurement

- Tests the EIRP power measurements when the network is not configuring mpr-PowerBoost-FR2-r16

applicability of 6.2.1.1 UE maximum output power - EIRP and TRP test (already defined as applicable to all types of NR UE release 15 and forward) does not require any change.

Observation 6: Applicability of 6.2.1.1 UE maximum output power - EIRP and TRP test defined as applicable to all types of NR UE release 15 and forward does not require any update.

Regarding In band Emissions core requirements, the change indicates to exempt In-band emissions requirements if device supports mpr-PowerBoost-FR2-r16 and network configures mpr-PowerBoost-FR2-r16. This implies that no additional testing is needed nor modifications on existing test case as current test definition does not include any configuration of mpr-PowerBoost-FR2-r16 from the network and hence already defined requirements apply. However, minimum conformance requirements need some update on the highlighted text in yellow below to be aligned with core requirements.

6.4.2.3.3	Minimum conformance requirements

The in-band emission is defined as the average across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The IBE requirement does not apply if UE declares support for mpr-PowerBoost-FR2-r16, UL transmission excluding Pi/2 BPSK is such that MPRf,c = 0 and when NS_200 applies, and the network configures the UE to operate with mpr-PowerBoost-FR2-r16.

The basic in-band emissions measurement interval is identical to that of the EVM test.

The requirement is verified with the test metric of In-band emission (Link=TX beam peak direction, Meas=Link angle).

The relative in-band emission shall not exceed the values specified in Table 6.4.2.3.3-1 for power class 1 UEs.

The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4.2.3.3-1 for power class 1, Table 6.4.2.3.3-2 for power class 2, Table 6.4.2.3.3-3 for power class 3 and Table 6.4.2.3.3-4 for power class 4 UEs.

Table 6.4.2.3.3-1: Requirements for in-band emissions for power class 1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 27 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 27 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 17 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | 4 dBm ≤ Output power ≤ 17 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Section 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB= 1 or RB= -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

The relative in-band emission shall not exceed the values specified in Table 6.4.2.3.3-2 for power class 2.

Table 6.4.2.3.3-2: Requirements for in-band emissions for power class 2

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 16 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 16 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 6 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 6 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Section 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB= 1 or RB= -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

The relative in-band emission shall not exceed the values specified in Table 6.4.2.3.3-3 for power class 3 UEs.

Table 6.4.2.3.3-3: Requirements for in-band emissions for power class 3

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Section 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB= 1 or RB= -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

The relative in-band emission shall not exceed the values specified in Table 6.4.2.3.3-4 for power class 4 UEs.

Table 6.4.2.3.3-4: Requirements for in-band emissions for power class 4

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 21 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 21 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 11 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤11 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed in the RBs containing the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Section 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB= 1 or RB= -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.4.2.3.

Observation 7: No change is required to test updates related to Rel-16 FR2 RF enhancements Power Boost for In-band emissions core requirements.

Proposal 4: Update In-band emissions minimum conformance requirements to align with core requirements.

# 2.4 Test point analysis

As described in section 2.3, test point analysis applicability for test 6.2.1.1 could be used for new test 6.2.4_1 Configured transmitted power with Power Boost.

Proposal 5: Extend 38.905 ([8]) test point analysis applicability for test 6.2.1.1 to new test 6.2.4_1 Configured transmitted power with Power Boost.

# 2.5 Message contents

By default, NS-200 is applied according to 38.508-1 ([9]) and 38.521-2 ([5]):

Hence, there is no need for specific configuration.

On the other side, by default mpr-PowerBoost-FR2-r16 is set to “Not present” in 38.508-1. Hence, specific message contents would be required to activate mpr-PowerBoost-FR2-r16:

Proposal 6: Define specific message contents to activate mpr-PowerBoost-FR2-r16 in ServingCellConfig information element in new test case 6.2.4_1 Configured transmitted power with Power Boost.

# 3. 	Conclusion

This document discusses how to introduce Rel-16 FR2 RF enhancements Power Boost requirements into test specifications ([5]) and makes the following observation and proposals:

Observation 1: Core requirements modification for Power Boost affect Configured output power and in-band emissions.

Observation 2: In order to exercise Power Boost requirements, the UE shall support mpr-PowerBoost-FR2-r16 UE capability ([6]):

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| mpr-PowerBoost-FR2-r16 Indicates whether UE supports uplink transmission power boost by suspension of in-band emission (IBE) requirements as specified in TS 38.101-2 [3]. | Band | No | TDD only | FR2 only |

Observation 3: In order to exercise Power Boost requirements, the network needs to configure the UE to operate with mpr-PowerBoost-FR2-r16 ([7]):

ServingCellConfig information element

UplinkConfig ::=                    SEQUENCE {

initialUplinkBWP                    BWP-UplinkDedicated                                                     OPTIONAL,   -- Need M

uplinkBWP-ToReleaseList             SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                              OPTIONAL,   -- Need N

uplinkBWP-ToAddModList              SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink                          OPTIONAL,   -- Need N

firstActiveUplinkBWP-Id             BWP-Id                                                                  OPTIONAL,   -- Cond SyncAndCellAdd

pusch-ServingCellConfig             SetupRelease { PUSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

carrierSwitching                    SetupRelease { SRS-CarrierSwitching }                                   OPTIONAL,   -- Need M

...,

[[

powerBoostPi2BPSK                   BOOLEAN                                                                 OPTIONAL,   -- Need M

uplinkChannelBW-PerSCS-List         SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

enablePL-RS-UpdateForPUSCH-SRS-r16  ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUSCH0-0-r16 ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUCCH-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForSRS-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

uplinkTxSwitching-r16               SetupRelease { UplinkTxSwitching-r16 }                                  OPTIONAL,   -- Need M

mpr-PowerBoost-FR2-r16              ENUMERATED {true}                                                       OPTIONAL    -- Need R

]]

Observation 4: PIBE applies to EIRP metric only.

Observation 5: PIBE is applicable to all power classes and channel bandwidths as:

- No restriction in the definition

- All power classes and channel bandwidths allow MPRf,c = 0 under certain conditions

Proposal 1: As new Rel-16 signalling and new Rel-16 UE capabilities are required to exercise Power Boost requirements, Power Boost associated testing should be applicable from Rel-16 and onwards for devices supporting mpr-PowerBoost-FR2-r16 UE capability.

Proposal 2: Use 6.2.1.1 Maximum output power test as a baseline to define test for Rel-16 FR2 RF enhancement related to Power Boost.

Proposal 3: Define a new test case to test Power Boost requirements: 6.2.4_1 Configured transmitted power with Power Boost.

Observation 6: Applicability of 6.2.1.1 UE maximum output power - EIRP and TRP test defined as applicable to all types of NR UE release 15 and forward does not require any update.

Observation 7: No change is required to test updates related to Rel-16 FR2 RF enhancements Power Boost for In-band emissions core requirements.

Proposal 4: Update In-band emissions minimum conformance requirements to align with core requirements.

Proposal 5: Extend 38.905 ([8]) test point analysis applicability for test 6.2.1.1 to new test 6.2.4_1 Configured transmitted power with Power Boost.

Proposal 6: Define specific message contents to activate mpr-PowerBoost-FR2-r16 in ServingCellConfig information element in new test case 6.2.4_1 Configured transmitted power with Power Boost.

All above proposals are implemented in [10], [11] and [12].

Equivalent EN-DC test case is proposed in [13].

# 4.	References

[1] 3GPP TS 38.101-2 v16.12.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[2] R4-2011905, “CR to 38.101-2: FR2 UE EIRP increase with IBE relaxation”, Qualcomm, Nokia, AT&T, T-Mobile, Verizon, RAN4#96-e

[3] R4-1912975, “WF on UL Power Boost and IBE Relaxation”, Qualcomm, Skyworks, RAN4#92-bis, Chongqing, China

[4] R4-2006782, “FR2 UE EIRP increase with IBE relaxation”, Qualcomm, Nokia, AT&T, T-Mobile, Verizon, RAN4#95-e.

[5] 3GPP TS 38.521-2 v16.12.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

[6] 3GPP TS 38.306 v17.1.0 User Equipment (UE) radio access capabilities

[7] 3GPP TS 38.331 v17.1.0 Radio Resource Control (RRC) protocol specification

[8] 3GPP TS 38.905 v17.5.0 Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases

[9] 3GPP TS 38.508-1 v17.5.0 User Equipment (UE) conformance specification; Part 1: Common test environment.

[10] R5-224071, “CR to 38.521-2 on New test case addition: 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[11] R5-224072, “CR to 38.521-2: In-band emissions minimum conformance requirements update”, Keysight Technologies UK, RAN5#96-e.

[12] R5-224073, “CR to 38.905: Test point analysis for new test 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[13] R5-224074, “CR to 38.521-3 on New test case addition: 6.2B.4.1.4_1	Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC).”, Keysight Technologies UK, RAN5#96-e.