3GPP TSG-RAN5 Meeting #96-e	R5-224159

# Electronic meeting, 15 – 26 Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1403 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of DC_3A-7A-20A_n8A reference sensitivity 2-band fallback test requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introducing reference sensitivity test requirements for DC_3A-7A-20A_n8A based on test point analysis in the associated CR to TR 38.905 as indicated below. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding following changes: - MSD test points for DC_3A_n8A intermodulation - Test configuration for DC_3A_n8A - Add also DC_3A-7A, which is not implemented - Test requirements for DC_3A_n8A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No complete test coverage for DC_3A-7A-20A_n8A reference sensitivity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.0.3.5.1, 7.3B.2.3.4.2.1, 7.3B.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0631 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

7.3B.2.0.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.0.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration |  | EUTRA or NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) |  | UL LCRB |  | DL Fc (MHz) |  | MSD (dB) |  | IMD order |
| DC_1_n3 |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 23 |  | IMD3 |
|  |  | n3 |  | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |
| DC_1A_n8A |  | 1 |  | 1965 |  | 5 |  | 25 |  | 2155 |  | 6.0 |  | IMD4 |
|  |  | n8 |  | 887.5 |  | 5 |  | 25 |  | 932.5 |  | N/A |  | N/A |
| DC_1A_n77A |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 29.8 |  | IMD23 |
| DC_1A_SUL_n77A- |  |  |  |  |  |  |  |  |  |  |  | 32.54 |  |  |
| n84A |  | n77, n78 |  | 4090 |  | 10 |  | 50 |  | 4090 |  | N/A |  | N/A |
| DC_1A_n77A |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 8.0 |  | IMD43 |
| DC_1A-SUL_n77A-n84A |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A |  | n77, n78 |  | 3710 |  | 10 |  | 50 |  | 3710 |  | N/A |  | N/A |
| DC_1A_n78A, |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 8.0 |  | IMD43 |
| DC_1A_SUL_n78A- |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
| n84A |  | n78 |  | 3710 |  | 10 |  | 25 |  | 3710 |  | N/A |  |  |
| DC_2A_n66A |  | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |
|  |  | n66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |
| DC_2A_n66A |  | 2 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |
|  |  | n66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |
| DC_2A_n77A DC_2A-2A_n77A |  | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD2 |
|  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |
|  |  | n77 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |
|  |  | 2 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 8.0 |  | IMD4 |
|  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
|  |  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |
|  |  | 2 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 5 |  | IMD5 |
|  |  |  |  |  |  |  |  |  |  |  |  | 7.74 |  |  |
|  |  | n77 |  | 3810 |  | 10 |  | 50 |  | 3810 |  | N/A |  | N/A |
| DC_2A_n78A |  | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD23 |
|  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |
|  |  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |
|  |  | 2 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 8.0 |  | IMD43 |
|  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
|  |  | n78 |  | 3690 |  | 10 |  | 50 |  | 3690 |  | N/A |  | N/A |
| DC_3_n1 |  | 3 |  | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |
|  |  | n1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 23 |  | IMD3 |
|  |  | 3 |  | 1771 |  | 10 |  | 50 |  | 1866 |  | 4 |  | IMD4 |
| DC_3_n5 |  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | N/A |  | N/A |
|  |  | 3 |  | 1721 |  | 10 |  | 50 |  | 1816 |  | N/A |  | N/A |
|  |  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 24 |  | IMD23 |
| DC_3A_n7A |  | 3 |  | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |
| DC_3C_n7A |  | n7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 10.25 |  | IMD4 |
| DC_3A_n8A |  | n8 |  | 900 |  | 5 |  | 25 |  | 945 |  | 8 |  | IMD43 |
|  |  | 3 |  | 1755 |  | 10 |  | 50 |  | 1850 |  | N/A |  | N/A |
|  |  | n8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |
|  |  | 3 |  | 1747.5 |  | 10 |  | 50 |  | 1842.5 |  | 6.4 |  | IMD5 |
|  |  | 3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | IMD4 |
| DC_3A_n20A |  | n20 |  | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |
|  |  | 3 |  | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |
|  |  | n20 |  | 847 |  | 5 |  | 25 |  | 806 |  | 9 |  | IMD4 |
| DC_3A_n41A |  | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 8.2 |  | IMD4 |
| DC_3C_n41A |  | n41 |  | 2657.5 |  | 10 |  | 50 |  | 2657.5 |  | N/A |  | IMD4 |
| DC_3A_SUL_n41A-n80A, |  | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 8.2 |  | IMD4 |
| DC_3C_SUL_n41A-n80A |  | n41 |  | 2657.5 |  | 10 |  | 52 |  | 2657.5 |  | N/A |  | N/A |
| DC_3A_n77A |  | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 26 |  | IMD23 |
| DC_3A_SUL_n77A-n80A DC_3A_n78A |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  | n77, n78 |  | 3575 |  | 10 |  | 50 |  | 3575 |  | N/A |  | N/A |
| DC_3A_n77A |  | 3 |  | 1765 |  | 5 |  | 25 |  | 1860 |  | 8.0 |  | IMD43 |
| DC_3A_SUL_n77A-n80A, DC_3A_n78A |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  | n77, n78 |  | 3435 |  | 10 |  | 50 |  | 3435 |  | N/A |  | N/A |
| DC_5A_n66A |  | 5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD23 |
|  |  | n66 |  | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |
| DC_5A_n78A |  | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 8.3 |  | IMD4 |
|  |  | n78 |  | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A |  | N/A |
| DC_5A_n77A8 |  | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 8.3 |  | IMD4 |
|  |  | n77 |  | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A |  | N/A |
|  |  | 5 |  | 826.5 |  | 5 |  | 25 |  | 871.5 |  | 5.5 |  | IMD5 |
|  |  | n77 |  | 4177.5 |  | 10 |  | 50 |  | 4177.5 |  | N/A |  | N/A |
| DC_7_n3 |  | 7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 13 |  | IMD4 |
|  |  | n3 |  | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |
| DC_7_n5 |  | 7 |  | 2547 |  | 10 |  | 50 |  | 2667 |  | N/A |  | N/A |
|  |  | n5 |  | 834 |  | 5 |  | 25 |  | 879 |  | 12 |  | IMD33 |
| DC_7A_n77A |  | 7 |  | 2540 |  | 5 |  | 25 |  | 2660 |  | 7.1 |  | IMD4 |
|  |  | n77 |  | 3870 |  | 10 |  | 50 |  | 3870 |  | N/A |  | N/A |
| DC_7A_n66A |  | 7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 15 |  | 4th IMD |
| DC_7A-7A_n66A DC_7C_n66A |  | n66 |  | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |
| DC_8A_n1A |  | 8 |  | 887.5 |  | 5 |  | 25 |  | 932.5 |  | N/A |  | N/A |
|  |  | n1 |  | 1965 |  | 5 |  | 25 |  | 2155 |  | 6 |  | IMD4 |
|  |  | 8 |  | 900 |  | 5 |  | 25 |  | 945 |  | 8 |  | IMD43 |
| DC_8A_n3A |  | n3 |  | 1755 |  | 10 |  | 50 |  | 1850 |  | N/A |  | N/A |
|  |  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |
|  |  | n3 |  | 1747.5 |  | 10 |  | 50 |  | 1842.5 |  | 6.4 |  | IMD5 |
| DC_8A_n20A | n20 |  | 849.5 |  | 5 |  | 25 |  | 808.5 |  | 25 |  | IMD33 |  |
|  | 8 |  | 890.5 |  | 5 |  | 25 |  | 935.5 |  | N/A |  | N/A |  |
|  | n20 |  | 847.5 |  | 5 |  | 25 |  | 806.5 |  | N/A |  | N/A |  |
|  | 8 |  | 892.5 |  | 5 |  | 25 |  | 937.5 |  | 25 |  | IMD33 |  |
| DC_8A_n41A |  | 8 |  | 882.5 |  | 5 |  | 25 |  | 927.5 |  | 12.1 |  | IMD33 |
|  |  | n41 |  | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |
| DC_8A_SUL_n41A- |  | 8 |  | 882.5 |  | 5 |  | 25 |  | 927.5 |  | 12.1 |  | IMD33 |
| n81A |  | n41 |  | 2685 |  | 10 |  | 52 |  | 2685 |  | N/A |  | N/A |
| DC_8A_n77A DC_8A_n78A |  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 8.3 |  | IMD4 |
| DC_8A-n79C, DC_8A-SUL_n78A-n81A |  | n77, n78 |  | 3635 |  | 10 |  | 50 |  | 3635 |  | N/A |  | H4 |
| DC_12_n78 |  | 12 |  | 710 |  | 5 |  | 25 |  | 740 |  | 5.5 |  | IMD5 |
|  |  | n78 |  | 3580 |  | 10 |  | 50 |  | 3580 |  | N/A |  | N/A |
| DC_13A_n77A |  | 13 |  | 784.5 |  | 5 |  | 20 |  | 753.5 |  | 5.5 |  | IMD5 |
|  |  | n77 |  | 3891.5 |  | 10 |  | 50 |  | 3891.5 |  | N/A |  | N/A |
|  |  | 20 |  | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |
| DC_20A_n3A |  | n3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | IMD4 |
|  |  | 20 |  | 847 |  | 5 |  | 25 |  | 806 |  | 9 |  | IMD4 |
|  |  | n3 |  | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |
| DC_8A_n79A |  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 4.8 |  | IMD5 |
| DC_8A-SUL_n79A-n81A |  | n79 |  | 4532.5 |  | 40 |  | 216 |  | 4532.5 |  | N/A |  | N/A |
| DC_18A_n77A DC_18A_n78A |  | 18 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |
|  |  | n77, n78 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |
| DC_12A_n77A |  | 12 |  | 702 |  | 5 |  | 20 |  | 732 |  | 5.5 |  | IMD5 |
|  |  | n77 |  | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |
| DC_19A_n78A |  | 19 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |
|  |  | n78 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |
| DC_20_n7 | 20 |  | 851 |  | 5 |  | 25 |  | 810 |  | 12 |  | IMD33 |  |
|  | n7 |  | 2512 |  | 10 |  | 50 |  | 2632 |  | N/A |  | N/A |  |
| DC_20A_n8A |  | 20 |  | 849.5 |  | 5 |  | 25 |  | 808.5 |  | 21 |  | IMD3 |
|  |  | n8 |  | 892.5 |  | 5 |  | 25 |  | 937.5 |  | 21 |  | IMD3 |
| DC_20A_n77A DC_20A_n78A, |  | 20 |  | 850 |  | 5 |  | 25 |  | 809 |  | 11 |  | IMD4 |
| DC_20A-SUL_n78A-n82A |  | n77, n78 |  | 3359 |  | 10 |  | 50 |  | 3359 |  | N/A |  | N/A |
| DC_20A_n77A |  | 20 |  | 840 |  | 5 |  | 25 |  | 799 |  | 6.5 |  | IMD54 |
|  |  | n77 |  | 4159 |  | 10 |  | 50 |  | 4159 |  | N/A |  | N/A |
| DC_21A_n79A |  | 21 |  | 1457.5 |  | 5 |  | 25 |  | 1505.5 |  | 18.4 |  | IMD3 |
|  |  | n79 |  | 4420.5 |  | 40 |  | 216 |  | 4420.5 |  | N/A |  | N/A |
| DC_26A_n41A |  | 26 |  | 839 |  | 5 |  | 25 |  | 884 |  | 15.6 |  | IMD3 |
|  |  | n41 |  | 2562 |  | 10 |  | 50 |  | 2562 |  | N/A |  | N/A |
|  |  | 28 |  | 730 |  | 10 |  | 50 |  | 775 |  | 15.3 |  | IMD 2 |
|  |  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |
| DC_28_n50 |  | 28 |  | 740 |  | 10 |  | 50 |  | 785 |  | 6 |  | IMD 4 |
|  |  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |
|  |  | 28 |  | 740 |  | 10 |  | 50 |  | 785 |  | 0.5 |  | IMD 5 |
|  |  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |
| DC_28A_n51A |  | 28 |  | 742.3 |  | 5 |  | 25 |  | 797.3 |  | 5 |  | IMD 4 |
|  |  | n51 |  | 1429.5 |  | 5 |  | 25 |  | 1429.5 |  | N/A |  | N/A |
| DC_25A_n77A DC_25A-25A_n77A |  | 25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD2 |
|  |  | n77 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |
|  |  | 25 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 8 |  | IMD4 |
|  |  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |
|  |  | 25 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 5 |  | IMD5 |
|  |  | n77 |  | 3810 |  | 10 |  | 50 |  | 3810 |  | N/A |  | N/A |
| DC_26A_n77A, |  | 26 |  | 836.5 |  | 5 |  | 25 |  | 881.5 |  | 11.1 |  | IMD4 |
| DC_26A_n78A |  | n77, n78 |  | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |
| DC_28A_n77A, |  | 28 |  | 705.5 |  | 5 |  | 25 |  | 760.5 |  | 5.5 |  | IMD5 |
| DC_28A_n78A, DC_28A-SUL_n78A-n83A |  | n77, n78 |  | 3582.5 |  | 10 |  | 50 |  | 3582.5 |  | N/A |  | N/A |
| DC_48A_n66A |  | 48 |  | 3630 |  | 20 |  | 100 |  | 3630 |  | N/A |  | N/A |
|  |  | n66 |  | 1715 |  | 5 |  | 25 |  | 2115 |  | 4 |  | IMD5 |
|  |  | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |
| DC_66A_n2A |  | n2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |
|  |  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |
|  |  | n2 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |
| DC_66A_n5A |  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD23 |
|  |  | 66 |  | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |
|  |  | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |
|  |  | n25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |
| DC_66A_n25A |  | 66 |  | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | 23 |  | IMD3 |
|  |  | n25 |  | 1912.5 |  | 5 |  | 25 |  | 1992.5 |  | N/A |  | N/A |
|  |  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |
|  |  | n25 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |
| DC_66A_n71A |  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 5 |  | IMD4 |
|  |  | n71 |  | 675 |  | 5 |  | 25 |  | 629 |  | N/A |  | N/A |
| DC_66A_n77A DC_66-66_n77A DC_66-66-66_n77A |  | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | 31.0 |  | IMD2 |
|  |  | n77 |  | 3950 |  | 10 |  | 50 |  | 3950 |  | N/A |  | N/A |
|  |  | 66 |  | 1760 |  | 5 |  | 25 |  | 2160 |  | 5.0 |  | IMD5 |
|  |  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |
| DC_66A_n78A |  | 66 |  | 1730 |  | 5 |  | 25 |  | 2130 |  | 5.0 |  | IMD5 |
|  |  | n78 |  | 3660 |  | 5 |  | 50 |  | 3660 |  | N/A |  | N/A |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

Table 7.3B.2.3.4.2.1-5: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to "Range" columns. |  |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | DC Configuration / NRB_agg |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |
|  | DC Configuration |  |  |  |  |  | E-UTRA Ch BW |  | NR Ch BW |  | CC MOD E-UTRA/NR | E-UTRA & NR RB allocation |  | CC MOD E-UTRA/NR |  | E-UTRA & NR allocations (LCRB @ RBstart) |  |  |  |
|  | E-UTRA |  |  |  | NR |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |
|  | Band |  | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  | Mid |  | Y | Mid | 5 |  | Mid |  | QPSK /CP-OFDM QPSK | All RBs |  | QPSK/ DFT-s-OFDM QPSK |  | REFSENS |  |  | REFSENS |
| Test Settings for DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 8 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 7 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 8 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_3A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 20 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_3A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 77/ 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_7A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Note 5 |  | 66 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Low |  | 77 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A-n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 1 | High | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 41 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A_n77A/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| 2 | 8 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_8A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Note 5 |  | 79 | Note 5 | 5 |  | 40 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 216@0 |
| Test Settings for DC_12A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | All RBs |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 8 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_20A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 2 | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Note 5 |  | 79 |  | Note 5 | 5 | 40 |  | Note 7 |  | All RBs |  |  | Note 7 |  | 25@0 | 216@0 |  |
| Test Settings for DC_28A_n50A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 50 | Note 5 | 10 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 50@0 |
| Test Settings for DC_28A_n51A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 51 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A_n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for CA_7A_20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Note 5 |  | 20 | Note 5 | 10 |  | 5 |  | QPSK | All RBs |  | QPSK |  | 50@0 |  |  | 25@0 |
| Note 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. Note 2: Use DC Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the DC Configuration. E.g. for DC_1A-n3A, X=1, Y=3. Note 4: REFSENS refers to the E_UTRA bands and NR band NRB 's single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2 of TS 36.521-1 [10] and Table 7.3.2.4.1-3 of TS 38.521-1 [8], respectively Note 5: Test frequency for each DC configuration shall follow Table 7.3B.2.0.3.5.1-1. If test configurations of each ID in a DC configuration are same, test frequency shall follow the order of Table 7.3B.2.0.3.5.1-1. Note 6: Not applicable if the UE only supports Bandwidth Combination Set 1. Note 7: Same as default. Note 8: RBSTART = 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  | SCG -NR |  |  |  |  |
|  | Band |  | Range | NRB |  |  |  | Band |  | Range | NRB |  |
|  | UL MOD |  | DL MOD | CH BW |  |  | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 |  | Mid |  |  |  |  | n3 |  | Mid |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 40 MHz | All RBs / 0 |
| 24,10 | 1 |  | UL 1950 / DL 2140 |  |  |  |  | n3 |  | UL 1760 / DL 1855 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 36 | 1 |  | Low |  |  |  |  | n3 |  | High |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 40 MHz | All RBs / 0 |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  | UL 1965 / DL 2155 |  |  |  |  | n8 |  | UL 887.5 / DL 932.5 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  | UL 1950 / DL 2140 |  |  |  |  | n78 |  | 3710 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 2 | UL 1855 / DL 1935 |  |  |  |  | n66 |  | 2175 |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 24 |  | 2 | UL 1883.3 /DL1963.3 |  |  |  |  | n66 |  | 2150 |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  | DL 1980 MHZ |  |  |  |  | n71 |  | UL 665.5 MHz |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 |  |
| 23 | 2 |  | DL1980 MHZ |  |  |  |  | n71 |  | UL 673 MHz |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |  |
| 35 | 2 |  | UL 1881 MHZ |  |  |  |  | n71 |  | Low |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  | 1860 |  |  |  |  | n77 |  | 3720 |  |  |  |
|  | QPSK |  | QPSK | 20 |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 2 |  | 1860 |  |  |  |  | n77 |  | 3700 |  |  |  |
|  | QPSK |  | QPSK | 20 |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 3 8 | 2 |  | 1860 |  |  |  |  | n77 |  | 3400 |  |  |  |
|  | QPSK |  | QPSK | 20 |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 45 | 2 |  | DL Mid |  |  |  |  | n77 |  | 3920 |  |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| 4a 9 | 2 |  | DL 1950 |  |  |  |  | n77 |  | 4000 |  |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| 54 | 2 |  | UL 1855/DL 1935 |  |  |  |  | n77 |  | 3790 |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 64 | 2 |  | UL 1900/DL 1980 |  |  |  |  | n77 |  | 3720 |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 74 | 2 |  | UL 1885/DL 1965 |  |  |  |  | n77 |  | 3810 |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  | UL 1870 MHZ |  |  |  |  | n78 |  | 3740 MHZ |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 2 |  | UL 1885 MHz |  |  |  |  | n78 |  | 3740 MHz |  |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 34 | 2 |  | UL 1855 MHz/ DL 1935 MHz |  |  |  |  | n78 |  | UL/DL 3790 MHz |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 44 | 2 |  | UL 1885 MHz/ DL 1965 MHz |  |  |  |  | n78 |  | UL/DL 3690 MHz |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 |  | Mid |  |  |  |  | n1 |  | Mid |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| 26 | 3 |  | High |  |  |  |  | n1 |  | Low |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| 34 | 3 |  | UL 1760 / DL 1855 |  |  |  |  | n1 |  | UL 1950 / DL 2140 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  | UL 1771 / DL 1866 |  |  |  |  | n5 |  | UL 838 / DL 883 |  |  |
|  | QPSK |  | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 24 | 3 |  | UL 1721 / DL 1816 |  |  |  |  | n5 |  | UL 838 / DL 883 |  |  |
|  | QPSK |  | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  | UL 1730 / DL 1825 |  |  |  |  | n7 |  | UL 2535 / DL 2655 |  |  |
|  | QPSK |  | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  | UL 1755 / DL 1850 |  |  |  |  | n8 |  | UL 900 / DL 945 |  |  |
|  | QPSK |  | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 24 | 3 |  | UL 1747.5 / DL 1842.5 |  |  |  |  | n8 |  | UL 897.5 / DL 942.5 |  |  |
|  | QPSK |  | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 |  | High |  |  |  |  | n41 |  | Low |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 26 | 3 |  | High |  |  |  |  | n41 |  | Low |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| 34, 10 | 3 |  | UL 1740 / DL 1835 |  |  |  |  | n41 |  | 2657.5 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 |  | Mid |  |  |  |  | n78 |  | 3495 |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 23 | 3 |  | Mid |  |  |  |  | n78 |  | 3525 |  |  |
|  | QPSK |  | QPSK | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |
| 35 | 3 |  | Mid |  |  |  |  | n78 |  | 3685 |  |  |
|  | N/A |  | QPSK | 20 MHz |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 20 MHz | All RBs / REFSENS_ENDC_2 |
| 3a9 | 3 |  | Low |  |  |  |  | n78 |  | High |  |  |
|  | QPSK |  | QPSK | 20 |  |  | All RBs/0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 20 | All RBs / REFSENS_ENDC_2 |
| 44 | 3 |  | UL 1740 / DL 1835 |  |  |  |  | n78 |  | 3575 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 54 | 3 |  | UL 1765 / DL 1860 |  |  |  |  | n78 |  | 3435 |  |  |
|  | QPSK |  | QPSK | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |

| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 5 |  |  | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 |  |  | 829 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3316 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 23 | 5 |  |  | 829 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 4145 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 38 | 5 |  |  | 840 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3600 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 44, 13 | 5 |  |  | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3421 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 54, 13 | 5 |  |  | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 4177.5 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  | DL 2655 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | DL 1825 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 10 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  | DL 2667 |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  | DL 879 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 10 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 7 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  | Low |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  | DL 932.5 |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  | DL 2155 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 5 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  | DL 945 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | DL 1850 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 5 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 |  |  | DL 942.5 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | DL 1842.5 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  |  | 5 MHz |  |  |  | N/A |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  | UL 890.5 / DL 935.5 |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  | UL 847.5 / DL 806.5 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3590 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 2,8 | 8 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3520 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 8 |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3635 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  | 4457.7 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 11 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  | 4512.7 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 13 |  |  | 782 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3910 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 1a8 | 13 |  |  | 782 |  |  |  |  | 10 |  |  |  |  |  |  | n77 |  |  |  | 3600 |  |  | 100 NHz |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 10 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 25 | 13 |  |  | DL Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3755 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 2a9 | 13 |  |  | DL 760 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3950 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 34 | 13 |  |  | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3891.5 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  | Low |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 19 |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  | 4445.02 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | UL 1775 / DL 1870 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 20 |  |  | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  | UL 1735 / DL 1830 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  | UL 851 / DL 810 |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  | UL 2512 / DL 2632 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 20 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  | 3388 MHz |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 20 |  |  | UL 850 / DL 809 MHz |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  | 3359 MHz |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  | 2524.5 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  | QPSK |  |  | 50 MHz |  |  |  | All RBs / 0 |
| 2,8 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  | 2572 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  | QPSK |  |  | 50 MHz |  |  |  | All RBs / 0 |
| 34 | 26 |  |  | UL 839 |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  | 2562 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | 3366 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  | QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |
| 2,8 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | 3446 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  | QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |
| 34 | 26 |  |  | UL 836.5 |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | 3391 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  | 4432.5 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 60 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  | 4470.5 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 15 MHz |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 60 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  | Low |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  | CP-OFDM QPSK |  |  | 20 MHz |  |  |  | All RBs / 0 |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 28 |  | Low |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3540 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |  |  |
| 28 |  | 28 |  | Low |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |  |  |
| 34 |  | 28 |  | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3582.5 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| 45 |  | 28 |  | Low |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3815 |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  | QPSK |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  | 3525 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| 2,9 | 40 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  | Mid |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | 3750 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| 26 | 41 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | Low |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 41 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  | Low |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 48 |  |  | 3557.6MHZ |  |  |  |  |  |  |  | n66 |  |  |  |  | 1778.8 MHZ |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |
| 23 |  |  | 48 |  |  | 3581.0 MHz |  |  |  |  |  |  |  | n66 |  |  |  |  | 1777.5 MHz |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |
| 34 |  |  | 48 |  |  | 3630 MHz |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1715 / DL 2115 MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 |  |  | 66 |  |  | UL1775 MHz/ DL2175 MHz |  |  |  |  |  |  |  | n2 |  |  |  |  | UL 1855 / DL 1935MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 24, 10 |  |  | 66 |  |  | UL 1750 MHz/ DL 2150 MHz |  |  |  |  |  |  |  | n2 |  |  |  |  | UL 1883.3 / DL 1963.3MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 66 |  | UL 1750 MHz/ DL 2150 MHz |  |  |  |  |  |  |  |  | n71 |  |  |  |  | UL 675 MHz/ DL 629 MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |

| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 66 | 1720 |  |  | n77 | 3440 |  |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 66 | 1720 |  |  | n77 | 3420 |  |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 38 | 66 | 1720 |  |  | n77 | 3600 |  |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 44 | 66 | UL 1775/ DL 2175 |  |  | n77 | 3950 |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 54 | 66 | UL 1760 / DL 2160 |  |  | n77 | 3720 |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 66 | UL 1755 MHZ |  |  | n78 | 3510 MHZ |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 66 | UL 1755 MHz |  |  | n78 | 3480 MHz |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 34 | 66 | UL 1730 MHz/ DL2130 MHz |  |  | n78 | UL/DL 3660 MHz |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n8A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n77A | 2 | N/A | -71.3 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -89.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n78A | 2 | N/A | -71.37 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.37 | - | - | - | - | N/A | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| DC_3A_n8A | 3 | N/A | - | REFSENS | - | - | - | N/A | FDD |
|  | n8 | 15 | -88.3 | - | - | - | - | IMD43 | FDD |
|  | 3 | N/A | - | -86.9 | - | - | - | IMD5 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| CA_3A-n20A | 3 | N/A | -92.3 | - | - | - | - | IMD4 | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | -88.0+TT | - | - | - | - | IMD4 | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  |  |  | -90.87 | - | - | - | - |  |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -70.3 | - | - | - | - | IMD23 | FDD |
|  |  |  | -70.67 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  |  |  | -88.37 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n66A | 5 | N/A | -67.3 | - | - | - | - | IMD23 | FDD |
|  | n66 |  | REFSENS | - | - | - | - | N/A | FDD |
| DC_5A_n77A8 | 5 | N/A | -89 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 5 | N/A | -91.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n78A | 5 | N/A | -89.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n3A | 7 | N/A | - | -81.3 | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n20A | n20 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 8 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | H4 | TDD |
| DC_8A_n79A DC_8A-SUL_n79A-n81A | 8 | N/A | -91.5 | - | - | - | - | IMD5 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20_n7 | 20 | N/A | -84.3 | - | - | - | - | IMD3 | FDD |
|  | n7 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20_n8 | 20 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
|  | n8 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_20A_n77A | 20 | N/A | -85.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 20 | N/A | -89.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_28_n50 | 28 | N/A | - | -79.5 | - | - | - | IMD 2 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -88.8 | - | - | - | IMD 4 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -94.3 | - | - | - | IMD 5 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n77A | 66 | N/A | -67.8 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: Void NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

-------- End of changes -------