3GPP TSG-RAN WG5 Meeting #96-e                                                       R5-224323

# Electronic Meeting, 15 – 26 August 2022

Source:	Qualcomm Incorporated, HiSilicon, China Unicom

Title:	Discussion paper for handling legacy test case for Redcap Only UE

# Document for:	Agreement

Agenda Item:	6.3.26.6

## 1.	Introduction

This discussion is to further progress handling of legacy test case for RedCap only UE. During RAN5#95, 2 feasible proposals were discussed as way forward to handling SNPN Only UE [11].

Based on earlier discussion a modified criteria was developed to scope our RedCap test cases, which is now captured in ““RedCap Applicable Legacy TC_v2.0_16.11.1”  analysis of this data shall help to conclude on way forward for RedCap tests

## 2.	Discussion

Attachment “RedCap Applicable Legacy TC_v2.0_16.11.1” contains list of all test cases as per TS38.523-2 (v16.11.1) and provides response to several key parameters. A set of common test environment has been identified, along with its impact, as below –

|  | Parameter | Details | Impact |
| --- | --- | --- | --- |
| 1 | SIB1 | SIB1 Change to allow for RedCap Operation - RedCap-ConfigCommonSIB-r17 IE; intraFreqReselectionRedCap - TS 38.331 | TS 38.508-1 |
| 2 | CBW | Set Max BW to 20Mhz for all bands under test- Modify Cl6 in TS38.508-1 | TS 38.508-1 |
| 3 | Early Identification UE | Two new LCIDs for CCCH/1 are introduced for RedCap UEs, for early identification via Msg3 or MsgA PUSCH, Whenever UE sends CCCH/1 data in Msg3 or MsgA PUSCH, it uses the RedCap-specific LCID, regardless of whether Msg1/A early identification is used or not. LCID=36 or LCID=35 for UL according to TS 38.321 Table 6.2.1-2 depending on whether it is a CCCH1 or a CCCH message for a RedCap UE | TS 38.523-3 (?) |
| 4 | Initial BWP | RedCap UEs shall use only the RedCap-specific initial UL BWP, if configured by network, to perform RACH, includes CD-SSB PDCCH-ConfigCommon of the RedCap-specific initial DL BWP includes common search space configuration for RAR | TS 38.523-3 (?) |
| 5 | RACH Config | Seperate RACH configuration: if seperate RACH configuration for REDCAP UE. eg different max number of preamble transmission , different power ramping , different back of timer after an attempt for REDcap UE |  |
| 6 | SN Field in PDCP and RLC Header | Mandatory support for 12-bit SN field in RLC & PDCP header. Optional support for 18-bit SN | TS 38.508-1, TS38.508-2 |
| 7 | DRB | Mandatory support for 8 DRBs. Optional support for 16 DRBs | No Impact |

There are prose level changes, majority in Layer 2 test cases, which are also identified

Also identified are the features and test cases excluded from the scoping

| Capabilities NOT supported Confirmed |  |  |  |
| --- | --- | --- | --- |
|  | Capability | Details | Impact |
| 1 | EN-DC | TS38.300 |  |
| 2 | NE-DC | TS38.300 |  |
| 3 | NR-DC | TS38.300 |  |
| 4 | CA | TS38.300 |  |
| 4 | R-16 DAPS | TS38.300 |  |
| 5 | CPAC | TS38.300 |  |
| 6 | IAB | TS38.300 |  |
| Capabilities TBD – Based on market needs |  |  |  |
|  | Capability | Details | Impact |
| 1 | Rel-16 Features | All Rel-16 are TBD |  |
| 2 | TS34.229-5 | VoNR and IMS related |  |

## 3.	Proposal/Way Forward

- Discussion if either proposal in [11] can be applied based on data collected and analyzed

## 4.	References

[1]	TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	 TS 36.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; EUTRA; User Equipment (UE) conformance specification; Part 1: Protocol

[3]	TS 38.508-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception User Equipment (UE) conformance specification;

[4]	TS 38.300: NR; NR and NG-RAN Overall description; Stage-2; Rel-16 (cl 16.6.1)

[5]	R5-213408: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#91e

[6]	R5-214736: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#92e

[7]	R5-220837: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#94e

[8] 	R5-222032: LS to GCF, PTCRB - Conformance Test Case Development for 3GPP Rel-16 Standalone Non-Public Network (SNPN) and other Verticals UE, RAN5#94e

[9] 	R5-222258	Addition of SNPN only applicability	Qualcomm CDMA Technologies

[10] 	R5-222259	Addition of applicable R15 tests for SNPN-only UE in a new clause	Qualcomm CDMA Technologies

[11] 	R5-223351	Addition of applicable R15 tests for SNPN-only UE in a new clause	Qualcomm CDMA Technologies